# April 11, 2002

# Tom Micek

Subj: STIL multi DUT flow considerations and suggestions.

Massive parallel testing in a multi-flow (ie: speed sort) should be considered as part of the STIL flow standard.  Single DUT testing

in a speed sort environment is straight forward but the complexity of

how STIL should handle multi site is a challenge.  As an example,

if we have the following:   example testing X4

Start  __T1__ T2__T3  _________T4  __ T5____Pass

|        |        |                        |          |     |

# F        |        |                       F        F     |

# |____|___ T6 ______________|

|                               |

|_ T7____________|

|

F

Where:  F = Fail path

If we are testing X4 (for ease of description)

if DUT1 & DUT4 devices passes T1& T2

DUT2 & DUT3 pass T1 & fails T2

The question is the following – How will the Test Engineer determine the path/paths as different devices take different directions within a flow? Do we continue with the T3 path devices then T6 path devices or perhaps T6 then back to the T3 device path?

### Suggestions

## The flow control can get complicated when multi DUT’s go different

direction paths depending on individual DUT pass/fails.  The Test Engineer should have the ability to establish the algorithm for handling multi site DUT paths.  Perhaps there should be a default path

flow with optional branch path control.  On branch paths perhaps a run

time stack can be  implemented with the PUSH/POP of the flow

control stack determined by the Test Engineer.  The preceeding test

will control which DUT’s are active or inactive.  This is still being

discussed on our end and open for suggestions when time allows within

the STIL.4 discussions.

- Each test to have a DUT pass/fail array whose size is determined by the multi site DUT count.  Each element of the Test DUT pass/fail array will

contain the pass/fail of the individual DUT’s during the flow.  This

would be very helpful for post analysis.  T1 pass/fail array would be

PFFP from the example above.

- In order for real time control of which DUT’s are active or

inactive (depending on pass or fail) there should be consideration

for a global DUT Active/Inactive array and also a per test active/inactive

array.  The global active/inactive DUT array could be the real time

control of which devices to continue testing with the local test active/

inactive array having the ability to turn off tests during the flow by the

user.

Tom Micek

-- 
Tom Micek     Dept: RU621       Phone: (512) 895-7478 
Motorola        Mail Stop: OE21  Fax:     (512) 895-3701 
6501 William Cannon Drive West  email:  tom.micek@motorola.com 
Austin, Tx.  78735-8598