Title:			Liaison Statement to IEEE P802.1

Date:			October 11, 2004

Meeting Location:	Barcelona, Spain

- Contacts:		Nan Chen, President MEF (nan_chen@atrica.com)

- Paul Bottorff, VP, TC Co-Chair (pbottorf@nortelnetworks.com)

- Bob Klessig VP, TC Co-Chair (bklessig@cisco.com)

- To:			Mick Seaman, Chair P802.1 Interworking Task Group
			(mailto:mick_seaman@ieee.org)

- Tony Jeffree, Chair P802.1 Working Group
			(mailto:tony@jeffree.co.uk)

From: 			Metro Ethernet Forum

The Metro Ethernet Forum is defining a new protocol called the Ethernet Local Management Interface (E-LMI).[1] The purpose of the protocol is to allow the Metro Ethernet Network to convey information about services available at a UNI to the Customer Edge attached to the UNI. In MEF terminology, this means that the MEN and the CE run E-LMI and the MEN provides information about the Ethernet Virtual Connections (EVCs). The information carried by the E-LMI includes:

- EVC status, e.g., new, active, inactive,

- EVC parameters, e.g., VLAN ID value that identifies the EVC, Bandwidth Profiles (policing), and

- Provisioning information, e.g., EVC identifier.

The E-LMI messages will be carried in Ethernet frames and the MEF would like to identify the E-LMI protocol via a well known destination MAC address and Ethertype.

Ultimately, we would like IEEE 802.1 to reserve a well known MAC address for E-LMI. For now we want to inform you about our E-LMI work and solicit your advice about the selection of a well known destination MAC address.

Our reading of IEEE P802.1ad/D2.4 leads us to believe that the destination must be recognized by a C-VLAN aware device such as the CE since a standard Ethernet device will run E-LMI. It must also be recognized by the C-VLAN aware component in a Provider Bridge in the Customer-tagged interface model (Figure 15-4 in D2.4) since this is where the information about the EVCs will be known in this case. It also must be recognized by the S-VLAN aware component in a Provider Bridge in the Port-based interface model (Figure 15-2 in D2.4) since this is where the information about the EVC will be known in this case.

Table 8-2, page 44 of IEEE P802.1ad/D2.4 shows the following addresses meet the above requirements:

- 01-80-C2-00-00-06,

- 01-80-C2-00-00-07,

- 01-80-C2-00-00-09, and

- 01-80-C2-00-00-0A.

Is our analysis valid? If yes, is it possible for IEEE 802.1 to reserve one of the above well known MAC addresses so that it can be used, not exclusively, for E-LMI?

References

[1]  MEF D00037_002, MEF UNI Type 2, Approved Draft 2

This document is attached, and also may be found on the MEF website.

MEF Approved Drafts, ITU-T access:

http://www.metroethernetforum.org/_root/members/drafts.htm

Username:  mef

Password: 3030