| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 660 Rev.1 (PLEN/15) |
|  |  | English only Original: English |
| Question(s): | 9/15 | 10-21 September 2012 |
| TD |  |  |
| Source: | Editors draft G.8021.1/Y.1341.1 |  |
| Title: | Draft Recommendation ITU-T G.8021.1/Y.1341.1 version 0.6 (for Consent, September 2012) |  |

This document contains the proposed text for Draft New Recommendation G.8021.1 : 
TYPES AND CHARACTERISTICS OF ETHERNET (ETH) EQUIPMENT.

Version History

| 0.0 | Q9/15, June 2011 | Initial text |
| --- | --- | --- |
| 0.1 | November, 2011 | Editors version updated following meeting discussions |
| 0.2 | SG15, Dec 2011 | Pasted Annex A figs from C.1582 Pasted Annex B from C.1590 Added Table 8.1 from C.1585 Replaced Sect 8.1 and 8.2 from C.1588. Figure will be harmonised from C.1779. Change bars relative to initial TD615. |
| 0.3 | Q9/15, May 2012 | Replaced sect 6 with text from WD22. Added figures in section 7 based on WD69 |
| 0.4 | June 2012 | Cleanup following May interim Completion of figures for clause 7 Section 7 reordered to match table 8-1 and Appendix I order. Acronyms, References and Summary added. |
| 0.5 | July 2012 | Cleanup based on correspondence Base text for consent |
| 0.6 | Sept 2012, SG15 meeting | Cleanup following discussion in question including: - deletion of Appendix C and “OAM modified” figures in clause 7 - swapping clauses 7 & 8 - fixed technical errors in B-BEB (removed ISID FD) & I-BEB (replaces ETH_SVI with ETH_SV) figures - aligned RCSI figures in Appendix A with 802.1Qbc |

G.8021.1 : TYPES AND CHARACTERISTICS OF ETHERNET (ETH) EQUIPMENT

Summary

Recommendation G.8021.1 describes examples of Ethernet equipment types derived from IEEE 802.1Q bridge types.

Keywords

Ethernet equipment, Bridge

# Scope

This recommendation describes an overview of the functions of Ethernet equipment and provides  examples of various Ethernet equipment types. The recommendation is based on the Ethernet transport network architecture specified in Recommendation G.8010, the Bridge Types specified in IEEE 802.1Q and equipment functional blocks specified in Recommendation G.8021. It is by no means restricting on the way equipment may be built.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

| [IEEE 802.1aq] | IEEE Std. 802.1aq-2012, Shortest Path Bridging |
| --- | --- |
| [IEEE 802.1Q] | IEEE Std. 802.1Q-2011, Local and Metropolitan Area Networks – Virtual Bridged Local Area Networks. |
| [IEEE 802.1Qbc] | IEEE Std. 802.1Qbc-2011, Provider Bridging - Remote Customer Service Interface |
| [IEEE 802.3] | IEEE Std. 802.3-2008, Part 3: Carrier sense multiple access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications |
| [ITU-T G.800] | ITU-T Recommendation G.800 (02/12), Unified functional architecture of transport networks |
| [ITU-T G.8010] | ITU-T Recommendation G.8010 (02/04), Architecture of Ethernet layer networks |
| [ITU-T G.8011] | ITU-T Recommendation G.8011 (01/09), Ethernet service characteristics |
| [ITU-T G.8012] | ITU-T Recommendation G.8012 (08/04), Ethernet UNI and Ethernet NNI |
| [ITU-T G.8021] | ITU-T Recommendation G.8021 (05/12), Characteristics of Ethernet transport network equipment functional blocks |
| [ITU-T G.8031] | ITU-T Recommendation G.8031 (06/11), Ethernet linear protection switching |
| [ITU-T G.8032] | ITU-T Recommendation G.8032 (02/12), Ethernet ring protection switching |
| [ITU-T G.supp] | ITU-T Recommendation G.supp (09/12), Ethernet ring protection switching supplement |
| [ITU-T Y.1731] | ITU-T Recommendation Y.1731 (07/11), OAM functions and mechanisms for Ethernet based networks |
| [MEF 6.1] | Metro Ethernet Forum 6.1 Ethernet Services Definitions - Phase 2 (April 2008) |
| [MEF 26.1] | Metro Ethernet Forum 26.1, External Network Network Interface (ENNI)–Phase 2 (January 2012) |
| [MEF 30] | Metro Ethernet Forum 30, Service OAM Fault Management Implementation Agreement (January 2011) |
| [MEF 35] | Metro Ethernet Forum 35, Service OAM Performance Monitoring Implementation Agreement (April 2012) |
| [IETF RFC 6060] | IETF RFC 6060 (2011), Generalized Multiprotocol Label Switching (GMPLS) Control of Ethernet Provider Backbone Traffic Engineering (PBB-TE) |

# Definitions

This Recommendation does not define any terms.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B-BEB | B-type Backbone Edge Bridge |
| --- | --- |
| BCB | Backbone Core Bridge |
| B-MAC | Backbone MAC address |
| CBP | Customer Backbone Port |
| CE | Customer Equipment [IEEE 802.1], Customer Edge [MEF] |
| CEP | Customer Edge Port |
| CEP IB-BEB | IB-BEB with Customer Edge Port |
| CEP I-BEB | I-BEB with Customer Edge Port |
| CNP | Customer Network Port |
| DSL | Digital Subscriber Line |
| EC | Ethernet Connection |
| E-NNI | External Network to Network Interface |
| ESP | Ethernet Switched Path |
| ETH | Ethernet MAC layer network |
| EVC | Ethernet Virtual Connection |
| GMPLS | GeneralizedMultiprotocol Label Switching |
| IB-BEB | IB-type Backbone Edge Bridge |
| I-BEB | I-type Backbone Edge Bridge |
| I-SID | Backbone Service Identifier field in an I-TAG |
| MEF | Metro Ethernet Forum |
| MEG | Maintenance Entity Group |
| MPLS | Multiprotocol Label Switching |
| MPLS-TP | MPLS Transport Profile |
| NNI | Network to Network Interface |
| OAM | Operations and Maintenance |
| OTN | Optical Transport Network |
| OVC | Operator Virtual Connection |
| PAP | Provider Access Port |
| PB | Provider Bridge |
| PBB | Provider Backbone Bridging |
| PBB-TE | Provider Backbone Bridging with Traffic Engineering |
| PDH | Plesiochronous Digital Hierarchy |
| PEB | Provider Edge Bridge |
| PHY | Physical layer |
| PIP | Provider Instance Port |
| PNP | Provider Network Port |
| PON | Passive Optical Network |
| RCAP | Remote Customer Access Port |
| RCSI | Remote Customer Service Interface |
| RCSI IB-BEB | IB-type Backbone Edge Bridge with Remote Customer Service Interface |
| RCSI I-BEB | I-type Backbone Edge Bridge with Remote Customer Service Interface |
| RCSI PEB | Provider Edge Bridge with Remote Customer Service Interface |
| SDH | Synchronous Digital Hierarchy |
| SPB | Shortest Path Bridging [IEEE 802.1aq] |
| TAG | The attributes of priority information, and optionally, VLAN identification information, associated with an Ethernet frame |
| TB-BEB | TB-type Backbone Edge Bridge |
| T-BEB | T-type Backbone Edge Bridge |
| UNI | User to Network Interface |
| VLAN | Virtual Local Area Network |
| VID | VLAN Identifier |

# Conventions

There are no special conventions in this Recommendation.

# ETH LAYER HIERARCHY, DOMAINS, TOPOLOGY AND CONNECTIONS

Figure 6-1 shows a much simplified view of an Ethernet Network consisting of two domains (depicted by the cloud edges) and showing generic Ethernet bridge equipment within each domain.

Figure 6-1: Ethernet Generic Equipment

Ethernet is an extremely flexible technology and it is virtually impossible to diagram many possible deployments. In the figure the network is constructed from Ethernet Provider Bridges (PB) interconnected by Ethernet Links.  See Appendix II for additional [IEEE 802.1Q] Provider Network considerations. A Provider Edge Bridge (PEB) provides for connection to Ethernet Customer Equipment (“CE”) via an MEF UNI interface.

Figure 6-2: Ethernet Over SDH Example

Those links between bridges are not necessarily optical fibers, and may be provided by Ethernet over X technology networks.  For example, a link could be provided by Ethernet over SDH, or by Ethernet over an OTN network. The generic equipment would then also contain some SDH or OTN components, ranging from a simple termination to a complete cross connect function.

## 6.1 MEF Global Interconnect

The Metro Ethernet Forum [MEF 6.1] defines three basic Ethernet service types – E-Line, E-Tree, E-LAN (Figure 6-3) – which can be offered by a single service provider in a single metro network or between multiple metro networks, or by multiple service providers. In the latter case the UNI ports may be distributed globally.

Figure 6-3 –MEF’s E-Line, E-LAN and E-Tree service examples

The Ethernet service connectivity between UNIs is referred to as an Ethernet Virtual Connection (EVC). There are point-to-point EVCs, rooted multipoint EVCs and multipoint-to-multipoint EVCs.

Figure 6-4: Connections across multiple operators

An EVC starts and ends at midpoints of UNI links and spans one or more service provider/carrier networks. For the case that an EVC spans multiple service provider/carrier networks, these carrier networks are interconnected via [MEF26.1] based Ethernet E-NNI links. Interconnection may be direct or via a Carrier Ethernet Exchange. The segment of an EVC within one service provider/carrier network is referred to as an Operator Virtual Connection (OVC). An OVC starts and ends at the midpoints of UNI and E-NNI links.

## 6.2 Ethernet Maintenance Entities

Ethernet Virtual Connections are supported by Ethernet Connections (EC) within an Ethernet Services Layer, which is providing Ethernet bridge functionality. This Ethernet Services Layer is supported by a Transport Services Layer. The connections in this transport services layer carry one or more ECs. The ECs pass the E-NNIs, while the transport services layer connections terminate before those E-NNIs.

The ECs are monitored by means of [ITU-T Y.1731] or CFM [IEEE 802.1Q] Ethernet OAM, allowing customers, service providers and network operators to monitor the EC segment within their domain via a Maintenance Entity Group/Association (MEG/MA) level (Figure 6.5).

The transport services layer may consist of Ethernet, MPLS, MPLS-TP, PDH, SDH, OTN, DSL and PON technologies. The UNI and E-NNI links are supported by [IEEE 802.3] technologies. The E-NNI links carry S-Tagged EC signals.

Figure 6.5 – Ethernet Maintenance entities

7	INTERFACE PORTS ON ETHERNET TRANSPORT NETWORK EQUIPMENT

The figures in this section illustrate examples of different types of ports that could be used in ETH equipment. The term ‘port’ is used here in a logical context and does not imply any partitioning of functions to hardware modules. A hardware module may implement one port, many ports, or a portion of the functions for one or more ports.

These types of interfaces depend on the type of ETH Equipment as shown in the following table:

Table 7-1 – Bridge Types and Interfaces

| ETH Equipment / 802.1Q Bridge Type | UNI Service Interface | NNI Service Interface |
| --- | --- | --- |
| PEB | C-tagged (Individual, Bundled) | S-tagged |
| RCSI PEB | C-tagged (Individual, Bundled) | S-tagged |
|  | Port-based (Untagged, Priority C-/S-tagged) |  |
| PB | Port-based (Untagged, Priority C-/S-tagged) | S-tagged |
|  | S-tagged (Peering) |  |
| I-BEB | Port-based (Untagged, Priority C-/S-tagged) | I-tagged |
|  | S-tagged (Individual, Bundled) |  |
| B-BEB | I-tagged | B-tagged |
| IB-BEB | Port-based (Untagged, Priority C-/S-tagged) | B-tagged |
|  | S-tagged (Individual, Bundled) |  |
| IB-BEB with CEP | C-tagged (Individual, Bundled) | B-tagged |
| I-BEB with CEP | C-tagged (Individual, Bundled) | I-tagged |
| RCSI IB-BEB | C-tagged (Individual, Bundled) | B-tagged |
|  | Port-based (Untagged, Priority C-/S-tagged) |  |
| RCSI I-BEB | C-tagged (Individual, Bundled) | I-tagged |
|  | Port-based (Untagged, Priority C-/S-tagged) |  |
| T-BEB | Transparent (Individual) | I-tagged |
| TB-BEB | Transparent (Individual) | B-tagged |
| BCB | N/A | B-tagged |

Notes:

- “Peering” here means that any OAM flows associated with the client signal share the same Maintenance Domain Level space as used by the operator’s OAM flows because no tag is added by the UNI mapping.

An S-TAG is identical to a B-TAG in all respects.

7.1   UNI

Table 7-1 above provides a summary of the twelve [IEEE 802.1Q] edge bridge types and their supported UNI interfaces. Those UNI interfaces include the following five types: C-Tagged, S-Tagged, I-Tagged, Port Based, Transparent.

In general, when an interface is referred to as “X-Tagged”, it means that the interface is configured to accept a client signal which carries a tag type of “X” following the destination and source address fields (DA/SA). Note that other addresses and tags may follow that outer X tag.

A Port based interface can be thought of as the opposite of X-Tagged, it is configured to accept client signals which are untagged (i.e., no tag follows the destination and source address fields) or priority tagged (i.e., only a priority value is carried in the tag, the VLAN identifier is zero).

A Transparent interface is configured to accept any client with properly formed destination and source address fields and frame format, regardless of the tag type.

Figure 7-1 below illustrates how an Ethernet Virtual Connection can be mapped by the five UNI interface types. The numbers beside each interface refer to the clauses in [IEEE 802.1Q] and [IEEE 802.1Qbc] where they are defined. For each interface, a pictorial representation shows the sequence of tag processing (e.g., S-TAG) and Backbone MAC addresses processing (B-MACs). The ovals in between the processing elements provide labels typically used to refer to the signals formed at those points (e.g., a Backbone Service Instance). Note the figure is not intended to depict [ITU-T G.800] or [ITU-T G.8010] (sub) layers.

The row of blocks below the various processing elements shows the [IEEE 802.1Q] edge bridge types (e.g. a PEB) supporting the connected UNI interfaces.

This representation provides a simple means to identify which type of [IEEE 802.1Q] edge bridge supports a certain UNI type.

For illustration purposes, an [IEEE 802.3] Ethernet MAC is shown mapped into either an [IEEE 802.3] PHY or an OTN PHY. The full set of Ethernet mappings is described in [ITU-T G.8012].

Figure 7-1: EVC over PB, PBB, PBB-TE (grouped by UNI interface)

7.2   NNI

Figure 7-2 below illustrates the same mappings of an EVC into the five UNI interface types, except shown from an NNI perspective, grouped into the three NNI interface types: S-Tagged, B-Tagged, I-Tagged. Note that since a B-TAG is identical to an S-TAG in all respects, there are actually only two NNI types.

This representation provides a simple means to identify which type of [IEEE 802.1Q] edge bridge supports a certain NNI type.

Again, for illustration purposes, an [IEEE 802.3] Ethernet MAC is shown mapped into either an [IEEE 802.3] PHY or an OTN PHY.

The only other type of [IEEE 802.1Q] provider bridge is the Backbone Core Bridge, which supports a S-/B-Tagged NNI service interface (see Appendix I). Since a BCB does not support a UNI service interface it isn’t included in Figures 7-1, 7-2. It can be viewed as a bridge fabric (i.e., an ETH FD) connected to peering S-Tagged NNI service interfaces.

Figure 8-2: EVC over PB, PBB, PBB-TE (grouped by NNI interface)

8	BASIC ETH EQUIPMENT TYPES

Several examples of ETH equipment are described in this section, illustrating different ways to combine the atomic functions defined in [G.8021].   These particular combinations are aligned with the set of IEEE 802.1Q Bridge types.  This set is defined in [IEEE 802.1Q] and is summarized in Appendix I.

The full set of basic ETH equipment types is presented together in Figure 8 using layer processor and edge processor functions. Each equipment type is then shown individually in the remainder of this section, first comprised of layer processor and edge processor functions, followed by an expansion into G.8021 atomic functions as outlined in Appendix III.

Figure 8 – Layer processor and edge processor based model for basic ETH equipment types

8.1	Provider Edge Bridge (PEB)

Figure 8-1a – PEB –layer processor and edge processor functions

Figure 8-1b – PEB – G.8021 atomic functions

8.2	RCSI Provider Edge Bridge (PEB)

Figure 8-2a – RCSI PEB –layer processor and edge processor functions

Figure 8-2b – RCSI PEB – G.8021 atomic functions

8.3	Provider Bridge (PB)

Figure 8-3a – PB –layer processor and edge processor functions

Figure 8-3b – PB – G.8021 atomic functions

8.4	I-type Backbone Edge Bridge (I-BEB)

Figure 8-4a – I-BEB –layer processor and edge processor functions

Figure 8-4b – I-BEB – G.8021 atomic functions

8.5	B-type Backbone Edge Bridge (B-BEB)

Figure 8-5a – B-BEB –layer processor and edge processor functions

Figure 8-5b – B-BEB – G.8021 atomic functions

8.6	IB-type Backbone Edge Bridge (IB-BEB)

Figure 8-6a – IB-BEB –layer processor and edge processor functions

Figure 8-6b – IB-BEB – G.8021 atomic functions

8.7	IB-BEB with CEP

Figure 8-7a – IB-BEB with CEP –layer processor and edge processor functions

Figure 8-7b – IB-BEB with CEP – G.8021 atomic functions

8.8	I-BEB with CEP

\

Figure 8-8a – I-BEB with CEP –layer processor and edge processor functions

Figure 8-8b – I-BEB with CEP – G.8021 atomic functions

8.9	RCSI IB-type Backbone Edge Bridge (RCSI IB-BEB)

Figure 8-9a – RCSI IB-BEB–layer processor and edge processor functions

Figure 8-9b – RCSI IB-BEB– G.8021 atomic functions

8.10	RCSI I-type Backbone Edge Bridge (RCSI I-BEB)

Figure 8-10a – RCSI I-BEB  –layer processor and edge processor functions

Figure 8-10b – RCSI I-BEB – G.8021 atomic functions

8.11	T-type Backbone Edge Bridge (T BEB)

Figure 8-11a – T-BEB –layer processor and edge processor functions

Figure 8-11b – T-BEB – G.8021 atomic functions

8.12	TB-type Backbone Edge Bridge (TB BEB)

Figure 8-12a – TB-BEB  –layer processor and edge processor functions

Figure 8-12b – TB-BEB – G.8021 atomic functions

8.13	Backbone Core Bridge (BCB)

Figure 8-13a – BCB–layer processor and edge processor functions

Figure 8-13b – BCB – G.8021 atomic functions

9	OPERATIONS, ADMINISTRATION, MAINTENANCE AND PROVISIONING

For detailed information on OAM processes for Ethernet Layers, refer to [Y.1731], CFM [IEEE 802.1Q], [MEF 30] and [MEF 35].

10	PROTECTION SWITCHING

For detailed information on Ethernet  Protection Switching, refer to [ITU-T G.8031] (Linear Protection), [ITU-T G.8032] (Ring Protection), and [ITU-T G.supp] (Ring Protection Supplement).

11	CONTROL PLANE

Typically transport Ethernet Equipment does not implement xSTP from [IEEE 802.1Q].  Instead each node is directly provisioned using the management plane.

However, it may be beneficial to implement the more advanced SPB control plane defined in [IEEE 802.1aq], or use GMPLS specifically for PBB-TE [IEEE 802.1Q] defined in [IETF RFC 6060].

Appendix I:  [IEEE 802.1Q] Equipment Examples

(This appendix does not form an integral part of this Recommendation)

These figures provide diagrams showing the IEEE 802.1Q Bridge types.  These diagrams correspond to the set of figures in Section 8.

Figure I.1 – Provider Edge Bridge (PEB)

Figure I.2 – RCSI Provider Edge Bridge (PEB)

Figure I.3 – Provider Bridge (PB)

Figure I.4 – I-type Backbone Edge Bridge (I-BEB)

Figure I.5 – B-type Backbone Edge Bridge (B-BEB)

Figure I.6 – IB-type Backbone Edge Bridge (IB-BEB)

Figure I.7 – IB-BEB with CEP

Figure I.8 – I-BEB with CEP

Figure I.9 – RCSI IB-type Backbone Edge Bridge (RCSI IB-BEB)

Figure I.10 – RCSI I-type Backbone Edge Bridge (RCSI I-BEB)

Figure I.11 – T-type Backbone Edge Bridge (T BEB)

Figure I.12 – TB-type Backbone Edge Bridge (TB BEB)

Figure I.13 – Backbone Core Bridge (BCB)

Appendix II:  [IEEE 802.1Q] Provider Network Considerations

(This appendix does not form an integral part of this Recommendation)

II.1    OAM

In a PBB or PBB-TE network, interconnected via physical links (e.g., [IEEE 802.3], OTN), OAM may be instrumented at several layers, such as:

- S-VLAN, per S-VID

- Backbone Service Instance (BSI), per I-SID

- B-VLAN, per B-VID

- Ethernet Switched Path (ESP), per <ESP-DA, ESP-SA, ESP-VID>  tuple

- Link maintenance domain levels

To support exhaustive fault isolation capabilities within PBB or PBB-TE networks, OAM at both the BSI and B-VLAN/ESP layers is necessary. Refer to [IEEE 802.1Q] clauses 26.8 and 26.9 for more details.

PBB networks can support a peering NNI handoff, interconnected via I-tagged interfaces. Refer to [IEEE 802.1Q] clause 26.6.2 for more details. In order to provide for fault isolation between peered PBB networks, it is important that BSI I-SID layer OAM is instrumented.

II.2   Network Hierarchy and Scalability

To interconnect customer networks in a scalable manner, PBB / PBB-TE was defined to address the 4094 services scaling limitation in metro PB networks. This was achieved by supporting the multiplexing of PB S-VLANs into PBB BSIs, followed by the multiplexing of those BSIs into PBB B-VLANs or PBB-TE ESPs. Further, a hierarchal structure for PBB / PBB-TE was defined in order to allow multiple levels of PBB / PBB-TE networks. Refer to [IEEE 802.1Q] clause 26.6.1 for more details. Specifically, Figure 26-1 from [IEEE 802.1Q] illustrates this network hierarchy.

Note that both PBB B-VLANs and PBB-TE ESPs can be deployed in the same network by partitioning the B-VID space accordingly.

Obviously if BSIs are not multiplexed into PBB B-VLANs or PBB-TE ESPs, and are instead mapped 1-to-1 (I-SID↔B-VID/ESP), then the network scalability is significantly reduced since only a combined maximum of 4094 BSIs (i.e., the B-VID space) can be supported.

Further, if the PBB or PBB-TE network is overlaid on an existing PB network, rather than deployed between subtending PB networks, then the network scalability is reduced yet again. This is because the combination of BSIs and PB services is limited to 4094 (i.e., the B-VID space and S-VID space are one in the same because the tags are identical).

Appendix III:  Layer processor and edge processor to G.8021 atomic functions expansion

(This appendix does not form an integral part of this Recommendation)

This recommendation uses several large functions to simplify the diagrams. These functions are not atomic, and each contained function is available for use individually should that be required.

Contained functions which are light grey are optional and need not appear in all implementations. This allows for MIPs and MEPs to be instantiated as required while not requiring a separate drawing for each and every possible OAM configuration.

These symbols contain transport functions providing for full layer encapsulation.

These symbols contain functions that only provide tag stacking and do not create a new layer boundary.

_______________________