IEEE P802.11
Wireless LANs

Performance of the Proposed 2.4-GHz PHY

Date:	May 1, 1998

Author:	John H. Cafarella
	MICRILOR, Inc.
	17 Lakeside Office Park, Wakefield, MA 01880
	Phone: 781-246-0103
	Fax: 781-246-0157
	e-Mail: JohnCafarella@worldnet.att.net

Abstract

This is a revised draft documenting the performance of the MICRILOR 2.4-GHz proposal.

# 1	Performance Summary

## 1.1	Implementation

# RF/IF PRISM® chipset with PA replaced by lower power unit

Baseband chip  25K gates modem function + 10K gates control

+ Interoperability  8K gates

Diversity requires alternate antenna and incurs switch loss.

## 1.2	Immunity to Multipath and Noise

### 1.1.1	Without Diversity

Multipath Only

Using the 802.11-mandated Diffuse Rayleigh Multipath model, incorporation of a Channel Matched Filter (CMF), as dictated by maximum-likelihood processing applied to the diffuse multipath channel, extends the delay-spread tolerance to 275 ns for 10% PER (insensitive to packet length).  The proposal is based upon inclusion of the CMF.

Noise Only

The Gaussian Channel performance: 10% PER for 1000-byte packet at an input SNR of 1.7 dB.

Noise and Multipath

At 275 ns delay spread, a 1000-byte packet experiences 20% PER at input SNR of 21.4 dB.  At 150 ns a 1000-byte packet experiences 10% PER at an input SNR of  20.6 dB.

### 1.1.2	With Diversity

Gains approximately 1 to 2 dB reduction in required SNR.

## 1.2	Overhead Related Parameters

PLCP Preamble & Header:  < 24 s           (with antenna diversity)

Slot size: 10 s                                             (with antenna diversity)

SIFS could be short, depending upon processor.  Drop-down FEC mode incurs 7.5-s latency, but end of reception can be timed from that point.

## 1.3	Spectral Efficiency and Cell Density Related Parameters

Channelization:

Frequency					Code

Two 10-Mbps frequency channels			Eight search-code channels

One 10-Mbps + one 5-Mbps channel

One 10-Mbls + one 2-Mbps channel		48 cyclic data code channels

Three channels, any mix of 2 & 5 Mbps		64K pseudorandom data code channels

Three 1-/2-Mbps + 1 10-Mbps

Cell Planning: BSAs operated with substantial spatial overlap of user areas if on different frequency channels; BSAs on same frequency channel is reasonable separation; always separate code channels as a matter of practice.  Mix of 2-Mbps with 10-/5-Mbps channels.   It is possible to operate three legacy 1-/2-Mbps channels independently, and also operate a 10-Mbps high-rate channel on top of, and interoperating with, the middle legacy channel.

Adjacent channel interference:  On same frequency channel (CCI) , desired signal must exceed interfering signal by  2 dB.  For different frequency channels (ACI) there is extra 35 dB isolation ( as in low-rate standard).

Interference Immunity

>14-dB processing gain against CW

12-dB processing gain against >25% BW Gaussian noise

## 1.4	Critical Points

- Phase noise not an issue (non-coherent receiver)

- Power consumption very attractive (PA efficienty; EB/N0)

- Complexity not an issue (35K to 43K gates)

- RF PA backoff not required (MSK spreading modulation)

- Antenna diversity included

## 1.5	Intellectual Property

Letter to IEEE.  Patent number not yet available.  Will comply with IEEE guidelines.  Call Dr. Stanley Reible.

## 1.6	Interoperability/Coexistence

Enhanced CCA possible for non-cooperative coexistence techniques described in D_97/128.  Recommended coexistence based upon high-rate deferral when required.

Interoperability with 1-/2-Mbps DSSS via CCA matched filter.  Dual-preamble not required; chip can process both; coordination via RTS/CTS exchange.

# 2	Multipath Models

## 2.1	Diffuse Rayleigh Channel

The Diffuse Rayleigh path model assumes that the channel impulse response comprises Rayleigh-distributed paths at discrete, uniformly spaced delays, filled in delay

hc(t)=k=0¥akd(t−kts)

where ts is the delay spacing, and ak is a complex amplitude whose real and imaginary components are Gaussian-distributed, with the following equivalent distributions:

p(Reak)=12πσαke−Reak22σαk2p(Imak)=12πσαke−Imak22σαk2	real/imaginary components

- - - - -

p(ak)=akσαk2e−ak22σαk2p(Argak)=12π		magnitude & phase

The path strengths are exponentially distributed in amplitude vs. delay

σαk2=σ02e−kτsTRMS with σ02=1−e−τsTRMS2

where TRMS is the multipath RMS delay spread.  The normalization of the path strengths holds the total received signal power constant as other parameters are varied.  The multipath-delay spacing is the shorter of the spread-spectrum chip time Tc or half the delay spread TRMS.

## 2.2	Other Channel Models

The Diffuse Rayleigh channel represents an extreme of channel behavior which stresses receiver design.  For example, when the multipath is of more specular nature, then excellent performance can be achieved using a relatively simple receiver structure.  Other channels have been proposed in the literature, but the 802.11 committee has not been able to agree on alternates.  As a result, data for only the Diffuse Rayleigh channel is presented herein.

# 3	Signal Model

The transmit waveform has the baseband representation

s(t)=mn=0M−1jnCmnpT(t−(n+mM)Tc)

where

Tc is the inverse of the chip frequency,

n is the chip index within a symbol,

M is the number of chips per symbol,

m is the symbol index,

Cmn is the chip code, and

pT(t) is the single-chip waveform:

During the acquisition preamble the chip value is

Cmn=CSn

where

CSn is the search PN code, 0£n<M-1.

In the data portion of a frame the chip value is

Cmn=dmPmnWKmn

where

dm is the polarity specified by the DBPSK component of the 16-ary DBOK signaling,

Pmn is the PN code during the mth symbol,

WKmn is the Walsh function during the mth symbol, and

Km is specified by the 16-ary OK signaling component.

When transmitted, s(t) is convolved with the channel impulse response to yield the received waveform

r(t)=k=0∞akmn=0M−1jnCmnpT(t−kτs−(n+mM)Tc)

At the receiver complex Gaussian noise z(t) is added, which obeys

z(t)=0z(t)z(t')=0z(t)z(t')=2N0δ(t−t')

where N0 is the one-sided noise spectral density.  The waveform is then convolved with the aggregate receive filter to form x(t)=w(t)+q(t).  The deterministic part w(t) (i.e., r(t) filtered) is

r(t)=k=0∞akmn=0M−1jnCmnpR(t−kτs−(n+mM)Tc)

where pR(t) is the chip-pulse waveform after receive filtering.

The noise component q(t) (i.e., z(t) filtered) has variance

q(t)2=σN2=2N0BN

where BN is the noise bandwidth of the receiver.

The filtered receive waveform is sampled at the chip rate to produce the sequence xn=wn+qn.

wn=r(t0+nTc)=k=0∞akmn'=0M−1jn'Cmn'pR(t0+nTc−kτs−(n'+mM)Tc)

The statistics of samples of the noise qn will be the same as q(t).  During acquisition the strongest path (at some delay k0s) is selected for demodulation.  We explicitly define the sampling time reference to correspond to the correct sampling time for the first chip of symbol m0 on this strongest signal component; that is, we select t0=k0s+m0MTc.  Thus, the signal sequence is

wn=k=0∞akmn'=0M−1jn'Cmn'pR((k0−k)τs+(n−n'+(m0−m)M)Tc)

The correlator reference sample sequence is j-nBm0n; the correlator output is

YBm0=n=0M−1j−lBm0nwn=k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'pR((k0−k)τs+(n−n'+(m0−m)M)Tc)

A number of chip values can contribute to the nth sample because of the width of pR(t) and also because of the multipath delay spread.  It is most natural to measure the delay spread in multiples, or sub-multiples, of the chip time because it simplifies analysis and simulations.  For 0 delay spread we have a Gaussian channel; when the delay spread is a single chip time, then it is necessary to half-chip sampling of the multipath profile; for integer multiples greater than 1 the multipath profile is sampled at the chip time.  The multipath will thus be sampled at Tc or ½Tc; the possible sampled values of pR(t) are

pR0=pR(0)=1pR12=pR(T2)=pR(−T2)pR1=pR(T)=pR(−T)

Samples at 1.5Tc or greater from the time of the peak are approximately zero.  We can make the replacement

pR((k0−k)τs+(n−n'+(m0−m)M)Tc)=δn−n'−(m0−m)M+(k0−k)τsTc+pR12δn−n'−(m0−m)M+(k0−k)τsTc+12+δn−n'−(m0−m)M+(k0−k)τsTc−12+pR1δn−n'−(m0−m)M+(k0−k)τsTc+1+δn−n'−(m0−m)M+(k0−k)τsTc−1

where the function x=1 if the index equals 0, and x=0 otherwise).

YBm0=k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+pR12k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+12+pR12k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc−12+pR1k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc+1+pR1k=0∞akmn'=0M−1n=0M−1jn'−nBm0nCmn'δn−n'+(m0−m)M+(k0−k)τsTc−1

Observations:

-  The first term is the principal contribution for each path, i.e., samples at the peak of the chip waveform.

-  The second two terms (having coefficients pR½) are introduced when the multipath is sampled at half the chip time.

-  The last two terms (coefficients pR1) reflect the inter-chip effects of using, MSK or filtered PSK.

This may be further developed by specializing to multipath sampling at the chip time or half that value.  In the following two equations, the expression in braces {} is a required constraint on remaining variables to be consistent with the original limits on the summation over n’.  Note that in all cases this constraint has the form {0 £ n+£ M-1}.

For s=Tc

YBm0=k=0∞akmn=0M−1j−(m0−m)M−(k0−k)Bm0nCm,n+(m0−m)M+(k0−k) 0≤n+(m0−m)M+(k0−k)≤M−1+pR1k=0∞akmn=0M−1j−(m0−m)M−(k0−k)−1Bm0nCm,n+(m0−m)M+(k0−k)+1 0≤n+(m0−m)M+(k0−k)+1≤M−1+pR1k=0∞akmn=0M−1j−(m0−m)M−(k0−k)+1Bm0nCm,n+(m0−m)M+(k0−k)−1 0≤n+(m0−m)M+(k0−k)−1≤M−1

For s=½Tc

k−k0even∞n=0M−1j−(m0−m)M−(k0−k)2+1Bm0nCmn+(m0−m)M+(k0−k)2−1 0≤n+(m0−m)M+(k0−k)2−1≤M−1

We introduce the acyclic correlation function RBC as follows:

RBmCm'Δ=0 Δ≤−M=j−Δn=−ΔM−1BmnCm',n+Δ −M<Δ<0j−Δn=0M−1−ΔBmnCm',n+Δ 0≤Δ<M0 Δ≥M

This is a standard definition of the crosscorrelation of code sequences, except that the j- factor introduced by MSK signaling has also been absorbed into the definition.  With this, the above equations become

These equations represent the outputs Y of correlators matched to reference codes B, due to input codes C at various multipath delays.  It is necessary to further specialize these to consider signal demodulation and detection performance; this will be done in the next two sections.

The receiver filtering will be intentionally broadened relative to ideal matched filtering to avoid excessive inter-chip ISI (keep pR1 small); thus, the noise will be uncorrelated from sample to sample.  The noise variances of correlator outputs is

σY2=MσN2=2MN0BN

For s=Tc

YBm0=k=0∞akmRBm0Cm,(m0−m)M+(k0−k)+pR1k=0∞akmRBm0Cm,(m0−m)M+(k0−k)+1+pR1k=0∞akmRBm0Cm,(m0−m)M+(k0−k)−1

When multiple correlator outputs are considered, the correlator noise is uncorrelated between correlators because the reference functions are orthogonal.

For s=½Tc

k−k0even∞

In the following sections, we shall focus on delay spreads greater than or equal to 2Tc.  This avoids considerable analytical complexity, since sampling at s=Tc avoids handling correlations induced by sampling the multipath at half the chip rate.  This restricts our attention to delay spreads ³62.5 ns for the 32-Mchip/s modulation.

# 4	Demodulation Performance

## 4.1	Demodulation Mode Formulation

We begin with the expression for the correlator outputs specialized to the case of M-ary signaling, where the transmitted waveforms are Cm=dmPmWKm and the correlators are matched to Bm=PmWK.  The correlator outputs for symbol m0 are

YPm0WK=k=0∞akmdmRPm0WK,PmWKm,(m0−m)M+(k0−k)+pR1k=0∞akmdmRPm0WK,PmWKm,(m0−m)M+(k0−k)+1+pR1k=0∞akmdmRPm0WK,PmWKm,(m0−m)M+(k0−k)−1

From here on we shall neglect the terms in pR1.  These reflect inter-chip effects due to bandwidth reduction, for example as  encountered when PSK signals are filtered, and in any case for MSK-like signals.  The value of pR1 is approximately .2; by contrast we must consider correlation side lobes of order .5 or higher relative to the autocorrelation peak, and multipath amplitudes which can be equal to the path being demodulated.  Thus, the effects reflected by carrying the pR1 terms are of second-order in significance, and carrying them would complicate the analysis without benefit.  The correlator outputs are

YPm0WK=k=0∞akmdmRPm0WK,PmWKm,(m0−m)M+(k0−k)

We expect that symbol waveform m0 can be significantly effected by the two immediately preceding symbols and the following symbol, for any multipath model.  For 2 Msymbol/s signaling this is a span of 2 s.  Over this range of symbols we expand the sum

YPm0WK=k=0∞ak[dm0+1RPm0WK,Pm0+1WKm0+1,−M+(k0−k)+dm0RPm0WK,PmWKm,(k0−k)+dm0−1RPm0WK,Pm0−1WKm0−1,M+(k0−k)+dm0+2RPm0WK,Pm0−2WKm0−2,2M+(k0−k)]

In addition to the above signal components there is thermal noise.  The correlator outputs are Rician-distributed with SNR

γK=YPm0WK2σY2=YPm0WK22MN0BN

The probability of symbol error, conditioned on the specific waveforms s=Pm0+1WKm0+1,Pm0WKm0,Pm0−1WKm0−1,Pm0−2WKm0−2 and the multipath profile a=α0,α0,α0 . . ., is

Pe|s,a=1−0∞drKm0 rKm0σ2 e−rKm022σ2−γKm0 I0(rKm0σ22γKm0)K≠Km00rKm0drK rKσ2 e−rK22σ2−γK I0(rKσ22γK)

In general it is necessary to average this probability of error over relevant distributions for s and a.

## 1.2	Gaussian channel

For the Gaussian channel we have |0|2=1 and (hence k0=0) and k=0 (k>0), so the signal component of the detection correlator outputs are

YPm0WJ=a0RPm0WJ,Pm0WKm0,0=Mα0δJKm0

where nm is the conventional Kronecker delta.  These are the outputs of M correlators matched to the waveform set generated by combining the PN code used for the mth symbol Pm0 with each of the M Walsh functions WJ.  The output SNR is

γ=YPm0WKm02σY2=M2α022MN0BN=M SNRIN

Most communications texts evaluate the error probability for M-ary OK using the union bound

Union

Bound

Numerical

Figure 1 – Pe approximations

Pe(γ)≈M−12 e−γ2

An alternate estimate is obtained using a Padé-like approximation which preserves the correct behavior as ®0.

Pe(γ)≈M−1M+2(eγ2−1)

The exact probability of error is given by

Pe=1−0∞dr1 r1σ2 e−r122σ2−γ I0(r1σ22γ)0r1dr2 r2σ2 e−r222σ2M−1

where r1 is the Rician variate of the correct correlator output and r2 represents the Rayleigh variates of the M-1 incorrect correlator outputs.  This has no closed-form solution, which is, of course, why the approximations are needed.  Figure 1 compares numerical evaluation of the integrals to the approximations for M=16.

## 1.3	Diffuse Rayleigh Channel

Unlike the cases of Gaussian-channel demodulation and detection in general, it is not possible to procede on a purely analytical basis for demodulation of signals in the Rayleigh-channel case.  This is due to the nonlinear interplay between multipath profiles and Walsh/PN coset cross-correlation side lobes in the symbol-decision process.  We take the approach here of using simulation to construct the sufficient statistic derrived in , then reverting to analytical techniques for subsequent evaluation.  The key feature of this approach is minimization of the dimensionality to be handled via simulation.  In particular, with noise as the only random element, simulation requires relatively long run times to produce meaningful results for low probabilities of error.  In the present case, the dimensionality of the random multipath is much larger than that of the thermal noise, and the signal waveforms themselves possess substantial dimensionality over which we must average.  This makes simulation difficult.  The approach used herein employs simulation to handle the averaging over many random selections of multipath profile and signal waveform, but uses standard analytical results via the union bound to handle thermal noise aspects without relying on simulation.  This enables generation of a curve of symbol- or packet-error probability vs. SNR for the effort of averaging over signal and multipath parameters.

Rayleigh-distributed generation of random multpath envelopes was implemented by generating a discrete amplitude distribution having equal probabilities per amplitude bin, and whose amplitudes were derrived by averaging the amplitude over the amplitude bins according to the Rayleigh density.  The results presented here employed 100 bins; re-running several of the cases using 1000 bins showned no difference.  For each multipath component the amplitude was determined by selecting the amplitude bin using a uniform-distribution random number generator.  The phase of each component was also generated as uniformly pseudorandom.  These amplitude and phase components were used to obtain the real and imaginary components of the multipath samples.

Constaints imposed on the simulation by agreement among the 802.11 members were that the multipath samples must extend to 10 TRMS and that multipath must be sampled at the finer of TRMS/2 or Tc.  When the multipath was sampled at sub-chip intervals, it was convolved with the receiver pulse (single-chip waveform) response at the same resolution, then subsequently decimated to the single-sample-per-chip sample rate used in receiver processing.  This approach properly handles the continuous analog effects.

For each randomization there were generated a random complex multipath profile and a multi-symbol waveform with random data, and a single-symbol statistic was used to compute the probability of symbol error PSE vs. SNR for a range of values as described in .  The resulting curve must be considered a conditional PSE vs. SNR, conditioned upon the specific multipath profile and signal waveform employed.  The randomization was iterated for 4, 8 or 16 thousand cycles, and the curves vs. SNR averaged to arrive at the final curve of PSE vs. SNR.  The PER performance was calculated from these averaged probability of symbol error curves as also described in .

The following parameters were employed in simulations:

|  | Without Channel Matched Filter | With Channel Matched Filter |
| --- | --- | --- |
| Multipath Extent | 64 samples max. | 128 samples max. |
| Signal Extent | 64 chips = 4 symbols | 128 chips = 8 symbols |
| Channel Matched Filter Span | N/A | 8 chips = ½ symbol |
| Symbol Demodulated | 3rd | 6th |

Longer spans for signal and multipath were used with the Channel Matched Filter because it tolerated larger TRMS.

1.4	Performance

Figure 2 - PER vs. delay spread for noise-less case

Figure 2 shows the probability of packet error vs. delay spread for the noise-less case.  The triangles indicate performance without a channel matched filter, while the rectangles indicate that with a channel matched filter.  A 10% PER occurs at 275 ns delay spread using the channel matched filter.  The increase in delay-spread tolerance from about 80 ns to 275 ns is dramatic enough to warrant including the CMF in the baseline proposal.

As an independent check on the computation approach outlined above, a complete simulation was carried out, for thenoise-free case, in which 64-byte packets were fully demodulated for each random multipath profile.  The results, shown in Figure 3, confirm the 10% PER at 275 ns.  In addition, the full-frame simulation also enabled extraction of the conditional probability PCD described in  .  The value estimated for the symbol-error-based computation was 1/32=.03125.  The value of PCD extracted from the full-frame demodulation was .0283 at 275 ns and .0323 at 300 ns.  Thus, the full-frame simulation and symbol-error-based computation are consistent.

Figure 3 - Demodulation of 64-byte packet for noise-less case

Figures 4 and 5 present the probability of packet error vs. input SNR for a Rayleigh channel with 275 ns delay spread, and for a Gaussian channel.

Figure 4

Figure 5

# 2	Detection Performance

## 2.1	Acquisition Mode Formulation

The signal model was formulated to support analysis of 16-ary Noncoherent BiOrthogonal Keying (16-ary DBOK) with PN codes changing from symbol to symbol.  In this section we specialize to the acquisition signal; i.e., a 16-chip code CSn repeated from symbol to symbol.  The codes and their cyclic correlations are tabulated in section . The cyclic correlation of two codes is

RBmCm'Δ=j−Δn=0M−1BmnCm', n+Δ(modM)

Consider the primary contribution (peak of chip waveform) to a correlator output YB for symbol m0 due to a unit-amplitude signal component at delay ks

YB=mRBmCm',(m0−m)M+(k0−k)=RBmCm',(k0−k)+RBmCm',(M+k0−k) k>k0=RBmCm',(k0−k)+RBmCm',(−M+k0−k) k<k0RBmCm',(k0−k)

where we have recognized that the two acyclic correlations in each case actually make up the full cyclic correlation (using jM=1).  The correlator output, with the reference matched to and aligned with the signal at lag k0c, is

YB=k=0∞akRSS,(k0−k)+pR1k=0∞akRSS,(k0−k)+1+pR1k=0∞akRSS,(k0−k)−1

During the search phase, the signal is repeated continually; the cyclic correlation function, in effect, aliases multipath components delayed by one or multiple symbols, whereas during demodulation these become intersymbol interference.  We now explore this fold-over effect.

There are only MTc possible sample times within the periodic symbol timing (assuming delay spread ³2Tc).  Multipath at lag (mM+n)Tc appears summed with multipath at lag nTc, for all m.  Because of this, we may re-cast the multipath description by summing multipath components separated by multiples of MTc.  For path models having strengths which are complex Gaussian, such a summation will also be complex Gaussian (Rayleigh amplitude).  For example, the re-cast (cyclic) IDR channel impulse response is

Hc(t)=k=0M−1βkδ(t−kτs) τs=Tc

where

σβk2=σ02e−kTcTRMS1−e−MTcTRMS

## 2.2	Gaussian channel

For the Gaussian channel we have |0|2=1 and (hence k0=0) and k=0 (k>0), so the signal component of the detection correlator output is

YB=RSS0=Mα0

and the output SNR is

γ=YB2σY2=M2α022MN0BN=M SNRIN

The probability of detection vs.  for a given probability of false alarm may be found as the non-fluctuating-target case in books on radar, e.g., DiFranco and Rubin, although the reader is cautioned that radar texts generally use R=2 as the signal to noise, whereas communication texts use .

## 2.3	Diffuse Rayleigh Channel, TRMS³2Tc

Although the search correlator employs the same single-sample-per-chip computation as does the demodulation correlator, this is actually stepped along at half-chip intervals to limit the “straddling loss” to a fraction of a dB.  During demodulation k0 corresponds to the strongest multipath; during acquisition k0 can be considered a hypothesis to be tested, i.e., whether the signal at delay k0s exceeds the threshold.  Thus, we may compute the probability of detection as one minus the probability that correlator outputs for k0=0 to M-1 all fall below the threshold.  The correlator outputs for the k0 are

YBk0=k=0M−1βkRSS,(k0−k)+pR1k=0M−1βkRSS,(k0−k)+1+pR1k=0M−1βkRSS,(k0−k)−1

We ignore autocorrelation side lobes, assuming

RSSΔ=Mδ0Δ

which results in

YBk0=βk0+pR1βk0+1+pR1βk0−1βk0

To the same level of approximation, we can set pR1=0 for detection since it is lower than expected autocorrelation side lobes.  For a single-component Rayleigh-fluctuating signal the probability of detection vs. mean signal-to-noise ratio and probability of false alarm is given by

Pd=Pfa1(1+γ)

Where  is the mean SNR.  For detection we test correlator amplitudes for M times of arrival, and the signal is missed only if detection fails on all M.  For signal timing corresponding to k0, the mean path SNR for the IDR channel is

γk0=YBk02σY2=M2βk022MN0BN=M2σβk02 SNRTOT

The definition of SNRTOT comes from

k0=0M−1YBk02=k0=0M−12σβk02=2σ021−e−MTcTRMSk0=0M−1e−kTcTRMS=2σ021−e−TcTRMS=1

independent of TRMS for the IDR channel.  Thus, the probability of detection in multipath is

Pd=1−k0=0M−11−Pfa1(1+M2σβk02 SNR)

## 2.4	Performance

Figure 6 shows the probability of missed detection vs. mean input SNR for the IDR channel, and vs. SNR for the Gaussian channel, for a detection criterion requiring threshold crossings on three successive symbols, at overall probability of false alarm of 10-6. Requiring multiple threshold crossings enables improvement of detection performance, relative to single-symbol detection, without incurring the complexity of video combining before testing against a threshold.  If we budget 1% of frame loss to missed detection, then it is clear from the figure that IDR multipath spreads from about 60 to 250 ns will incur 4- to 5-dB of fading loss, but that detectability improves monotonically with SNR.

Figure 6 - Detection Performance

# 3	Interference Rejection

Previous sections assessed link performance in noise and multipath.  This section considers interference rejection in the absence of multipath propagation effects.

## 3.1	Interference Rejection Formulation

From  the signal response for the mth  symbol is

YPmWJ=a0RPWJ,PWKm,0=Mα0δJKm

where Pm is the PN code, J represents the Walsh-function correlator outputs and Km the signal variant transmitted, respectively, 0 is the signal strength and nm is the Kronecker delta.

The baseband representation of the interfering signal is x(t)ej2πυt, where x(t) is the envelope and  is the offset frequency.  It is assumed that the bandwidth of x(t) is small enough, and that  is restricted to be sufficiently close to the signal carrier frequency, that no substantial roll-off due to receive filtering can be included in the processing gain calculation.  The interference is sampled and applied to the correlator bank to produce

YPmWJ=RPWJ,X=n=0M−1PmnWJnxnej2πnυTc

We consider next the cases of narrowband CW interference and of band-limited Gaussian interference.

## 3.2	Narrowband CW Interference

We assume the interference to be I ej2πυt, where I is the complex amplitude of the interference.  The correlator outputs are

YPmWJ=RPWJ,X=I n=0M−1PmnWJnej2πnυTc

As shown in document IEEE P802.11-97/116, for CW interference there is an abrupt drop to zero probability of error for some CW interference level when the largest possible interference output can no longer influence the data decision.  We may determine this threshold by finding

Max over m,J n=0M−1PmnWJnej2πnυTc

Figure 7 - Correlator output responses.

for the (M=16 bit) PN codes to be used.  The 8 PN codes (coset leaders) have identical statistics in this regard.  We show here the case of Pmn=0158H.  Figure 7 shows YPmWJ2 (measured in dB relative to coherent response) vs. Tc for all 16 correlator outputs.  The worst-case situation occurs for  within a symbol bandwidth of center frequency, where 2 correlator outputs produce outputs only 6-dB down from the coherent signal. As the CW interference level approaches the zero-error threshold, it is only the two channels having –6-dB responses for the interference which are of interest.  When the signal would emerge from one of these two channel, which happens with probability 2/16, then it is possible for the CW interference to produce an amplitude out of an incorrect channel while simultaneously lowering the amplitude of the correct channel.  This will occur when

I2+α0≤I2

Taking the arbitrary phase angle between the interference and signal to be , an error occurs for

I2α0+1≤I2α0 or 1+14Iα02+Iα0cos(Θ)≤14Iα02

If a solution to this inequality exists, we may identify a boundary angle B between angles corresponding to making an error and angles for which no error occurs.  The solution for this angle is

cos(ΘB)=−Iα0−1=−γ where γ≡α02I2

is the signal-to-interference ratio.  There clearly can be no error for >1; for <1 the solution for B is

ΘB=±cos−1(−γ)

The random phase  is distributed over 2, so the probability of error is simply the fraction of 2 for which an error will occur times the 1/8 probability that the correct channel is one of the two channels under consideration, or

Pe(γ)=18πcos−1(γ)

It must be remembered that this is the asymptotic behavior near =1; for low signal-to-interference ratio the probability of error is larger than indicated by the above expression.

## 3.3	Narrowband Gaussian Interference

In this case we assume that x(t) is a Gaussian random variable.  The correlator outputs due to noise are

YPmWJ=n=0M−1PmnWJnxnej2πnυTc

It is assumed that the noise bandwidth is small enough, and that the frequency offset is confined, such that no substantial roll-off due to receive filtering is included as processing gain.

The processed interference remains Gaussian, since the correlation process is linear, so it remains only to find the mean-square correlator outputs.

YPmWJ2=n=015PmnWJnznn'=015Pmn'WJn'zn'=n=015n'=015PmnWJnPmn'WJn'znzn'=2σ2 n=015n'=015PmnWJnPmn'WJn' e−an−n'Tc

In the well-known case of white Gaussian noise, the correlator outputs are equal-variance, uncorrelated Gaussian variables.  The above equation can be used to explore the effect of the bandwidth of the Gaussian interference.  Pm=0158H was used, combined with the 16 Walsh functions, to calculate the correlator outputs shown in Figure 8.

Figure 8 - Correlator output varainces, normalized to 22, in dB relative to coherent response.

For very narrowband Gaussian interference (a®0) anomalous results are obtained.  Two of the correlator outputs produce interference outputs suppressed only 6 dB relative to the coherent signal; this is due to lack of one-zero  balance in the code for those correlator channels.  On the other hand, six of the correlator outputs provide essentially infinite rejection of narrowband interference.  The remaining eight outputs yield the same 12-dB suppression of narrowband interference as is the case for wideband interference.

Asymptotically (a®) all 16 correlator outputs produce the 12-dB suppression of the Gaussian interference expected from the 16 chips per symbol.  It is clear that for noise bandwidths greater than approximately 25% of the chip rate (or 4 times the symbol rate) the processing gain is essentially the nominal 12 dB.

## 3.4	Performance

The S/I performance is summarized in Figure 9.  CW interference produces a large error probability until the S/I approaches 0 dB, at which point the probability of error drops abruptly to zero.  For Gaussian interference the probability of error was estimated using the union bound, with the signal emerging from the correlator channel having the smallest variance; that is, summing the binary-orthogonal error probabilities for S/I set by the (largest) individual channel responses to the interference.  This was done for 5%, 10% and 20% noise bandwidth relative to the chip rate (fnTc).  Also shown is the asymptotic behavior if the noise were decorrelated from chip to chip.  At 20% noise bandwidth the performance is within a fraction of a dB of the asymptotic behavior for large noise bandwidth.

GAUSSIAN

FnTc=5%

CW

=10%

=20%

Figure 9 – Symbol error probability for CW and Gaussian interference

Interpreting this curve in terms of the “processing gain” test, the dashed curve (decorrelated-noise limit) represents the 12-dB which is ten times log10 of the number of chips per symbol; CW interference would appear to have PG of 14 dB, while Gaussian noise at 10% and 5% would yield 10.9-dB and 9.7-dB, respectively.

# 4	Appendix: Approximate MSK

The approximation to MSK to be implemented departs from an ideal, matched MSK system in the following:

- the generation process combines a staircase approximation to a cosine pulse followed by a filter to offer an inexpensive implementation;

- the transmit spectrum has lower side bands than ideal MSK;

- the receive processor does not exactly match the chip waveform, preferring to keep the bandwidth somewhat higher for better multipath resolution.

The ideal MSK waveform has the baseband representation

s(t)=njncnpMSK(t−nTc)

where

Tc is the inverse of the chip frequency,

n is the chip index,

cn is the chip value, and

pMSK(t) is the single-chip MSK waveform:

pMSK(t)=cos(pt2Tc)t<Tc0t<Tc

Ideal processing of the MSK waveform at the receiver begins with the (analog) chip matched filter (CMF) for the MSK pulse.

pCMF(t)=1Tccos(pt2Tc)t<Tc0t<Tc

which results in the baseband waveform

w(t)=k=0¥aknjncnRMSK(t−kts−nTc)

where the chip autocorrelation function is

RMSK(t)=(1−t2Tc)cos(pt2Tc)+1psin(pt2Tc)t<2Tc0t>Tc

By contrast, the proposed implementation begins with a generator for the chip waveform which produces

pG(t)=1 t<.5Tc.37 .5Tc<t<Tc0 t>Tc

The transmit filtering is designed to minimize the energy taken out of the main spectral lobe while suppressing the side lobes.  This may be accomplished using, e.g., 5th–order baseband filters plus some IF bandpass filter.  As a result of this filtering, the generated pulse shape is replaced by the transmitter (equivalent-baseband) pulse shape pT(t), where

pT(t)=pG(t)∗hT(t)

so that the baseband representation of the transmitter waveform is

s(t)=njncnpT(t−nTc)

On receive, the waveform is passed through equivalent IF and baseband filtering.  The resulting baseband complex signal is

s(t)=njncnpR(t−nTc)

where pR(t) is

pR(t)=pG(t)∗hT(t)∗hR(t)

Of significance is that the pulse shape of PR(t) is of considerably less time extent that would be the case for a true MSK pulse passed through an exact chip matched filter.  Chip-to-chip amplitude overlap with this approach is typically less than .2, compared to .5 for true MSK.  This is important for good multipath performance.

# 5	Appendix: Search Codes

The 16-bit codes selected for use during search and CSn are given in the following table, along with their cyclic autocorrelation functions, symmetric about the main lobe.

| CSn (hex) | Rss0 | Rss1 | Rss2 | Rss3 | Rss4 | Rss5 | Rss6 | Rss7 | Rss8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44BC | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| A0DC | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| D223 | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| 0A76 | 16 | 0 | 0 | 0 | 0 | 0 | -4 | 0 | -4 |
| 425C | 16 | 0 | 0 | 0 | 0 | 4 | 0 | -4 | 0 |
| 23A4 | 16 | 0 | 0 | 0 | 0 | 4 | 0 | -4 | 0 |
| 245C | 16 | 0 | 0 | 0 | 0 | -4 | 0 | 4 | 0 |
| A243 | 16 | 0 | 0 | 0 | 0 | -4 | 0 | 4 | 0 |

These codes have excellent correlation properties for signal detection and selection of the strongest multipath component, having 4 or 5 zero values for autocorrelation side lobes nearest to the main lobe.  When it is required to select codes for independent operation of BSAs, it is important to consider the peak and average cross-correlation values between the different codes (dB relative to main lobe) as shown below.

|  | 44BC |  | A0DC |  | D223 |  | 0A76 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak | rms | peak | Rms | Peak | rms | Peak | rms |
| 44BC | 0 | -19.3 | -2.5 | -11.3 | -6.0 | -11.3 | -6.0 | -11.3 |
| A0DC | -2.5 | -11.3 | 0 | -19.3 | -6.0 | -11.3 | -6.0 | -11.3 |
| D223 | -6.0 | -11.3 | -6.0 | -11.3 | 0 | -19.3 | -2.5 | -11.3 |
| 0A76 | -6.0 | -11.3 | -6.0 | -11.3 | -2.5 | -11.3 | 0 | -19.3 |
| 425C | -4.1 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| 23A4 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| 245C | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| A243 | -4.1 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |

|  | 425C |  | 23A4 |  | 245C |  | A243 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Peak | rms | peak | Rms | Peak | rms | Peak | rms |
| 44BC | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| A0DC | -8.5 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -8.5 | -12.0 |
| D223 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 | -4.1 | -12.0 |
| 0A76 | -4.1 | -12.0 | -8.5 | -12.0 | -8.5 | -12.0 | -4.1 | -12.0 |
| 425C | 0 | -18.1 | -6.0 | -11.1 | -6.0 | -13.3 | -6.0 | -13.3 |
| 23A4 | -6.0 | -11.1 | 0 | -18.1 | -6.0 | -13.3 | -6.0 | -13.3 |
| 245C | -6.0 | -13.3 | -6.0 | -13.3 | 0 | -18.1 | -6.0 | -11.1 |
| A243 | -6.0 | -13.3 | -6.0 | -13.3 | -6.0 | -11.1 | 0 | -18.1 |

# 6	Appendix: Error Probability Evaluation

## 6.1	Probability of Error Computation

The presence of multipath generally causes all correlator outputs to produce some signal component; Thus, the distribution of all outputs is Rician.  The probability of error is

Pe=1−0∞drKm rKmσ2 e−rKm22σ2−γKm I0(rKm0σ22γKm)K≠Km0rKm0drK rKσ2 e−rK22σ2−γK I0(rKσ22γK)

where Km is the correct correlator channel.  To evaluate this we make substitutions of the form x=r2σ2.

Pe=1−0∞du e−u−γKm I0(4uγKm)K≠Km0udvK e−vK−γK I0(4vKγK)

The union bound can be used for all situations in which the probability of error would be acceptable.

Pe=K≠Km0∞du e−u−γKm I0(4uγKm)u∞dvK e−vK−γK I0(4vKγK)

## 6.2	Probability One Rician Variate Exceeds Another

We use the Marcum Q function to compute the binary probabilities PK, i.e., the probability that vK exceeds u.  If we define a=γK and b=γKm then we have

PK=121−Qb,a+Qa,b

For a=b this gives PK=½.  We assume from here on that b>a.

PK=12b−ab+ab+a2∞e−uI02ubab+adu b>a

If a=0 we have the familiiar result that PK=12e− b2.  More generally, we substitute the series n=0∞1n!2x22n for I0(x)

PK=12b−ab+an=0∞bab+a2nn!2b+a2∞u2ne−udu  and carry out the integral

PK=12b−ab+ae−b+a2n=0∞2n!n!2bab+a2nm=02n1m!b+a2m

This form can be recursively computed, as can be seen by defining K=ba/(b+a) and C=(b+a)/2.

### 6.2.1	For use when b+a is small

Define: C=(b+a)/2

K=b*a/(b+a)^2

F=.5*(b-a)/(b+a)

Init: S=1,G=1,H=1,TNF=1,C2N=1

Recursion in N:

TNF=TNF*N*(4*N-2)

G=G*K(4*N-2)/N

H=H+C2N*C*(2*N+C)/TNF

C2N=C2N*C^2

D=G*H

S=S+D

Terminate recursion: D/S<

Result: P=EXP(-C)*F*S

PK=b−a2(b+a)e−b+a2n=0∞2n!n!2Knm=02n1m!Cm

If we let SN be the sum over n carried to N terms, and HN be the sum over m carried to 2N terms

SN=n=0∞2n!n!2Knm=02n1m!Cm=SN−1+2(2N−1)NKKN−1HN−1+(C2+2NC)C2(N−1)(2N)!

For each n the series over m is carried to completion, so it is not a source of error.  Since the series in n has the variable ba/(b+a)2 raised to the nth power, and since this variable is never larger than .25, we expect that the series in n can be readily truncated without large error.  Unfortunately, the sum over m is approaching exp[(b+a)/2] for large n, and this results in overflow problems for the inner summation as well as underflow in the exponential.  The recursion for b+a small uses the equations directly, while that for b+a large associates the exponential with the sum over m and employs logarithms.

### 6.2.2	For use when b+a is large

Define: C=(b+a)/2

K=b*a/(b+a)^2

F=.5*(b-a)/(b+a)

LC=Ln(C)

LK=Ln(K)

Init: LS=0,LG=0,LH=0,LTNF=0,LC2N=0

Recursion in N:

LTNF=LTNF +Ln(2*N*(2*N-1))

LG=LG+LK+Ln((4*N-2)/N)

LQ=LC2N+LC+Ln(2*N+C)-LTNF

H=H+Ln(1+Exp(LQ))

LC2N=LC2N+2*LC

LD=LG+LH

LS=LS+Ln(1+Exp(LD-LS)

Terminate recursion: LD-LS<Ln()

Result: P=F*Exp(S-C)

We now have a robust algorithm for computing the probability of error when both variates are Rician for all values of a and b.

## 6.3	Probability One Gaussian Variate Exceeds Another in Magnitude

In the case of coherent reception, a similar formulation may be used and this section applies to the computation of symbol-error probability.  Assume x=2σ2γx>y=2σ2γy>0; if the the means are reversed, then interchange the roles of x and y, and complement the probability.  In order to compute the probability correctly we must sum probabilities of mutually exclusive events.

|  | P(x>0)=1−12 erfcγx | P(x<0)=12 erfcγx |  |
| --- | --- | --- | --- |
| P(y>0)=1−12 erfcγy | P(y>x)=12 erfcγx2−γx2 |  | P(y>−x)=1−12 erfcγx2+γx2 |
| P(y<0)=12 erfcγy | P(y<−x)=12 erfcγx2+γx2 |  | P(y<x)=1−12 erfcγx2−γx2 |

Here we have used directly the well-know results for making correct sign decisions.  The relative-magnitude probabilities recognize the sum/difference of the means as well as the corresponding doubling of the noise variance.  The probability is

Py>x=1−12 erfcγx1−12 erfcγy12 erfcγx2−γy2+12 erfcγx1−12 erfcγy1−12 erfcγx2+γy2+1−12 erfcγx12 erfcγy12 erfcγx2+γy2+12 erfcγx12 erfcγy1−12 erfcγx2−γy2

This can be evaluated for the Gaussian-channel limit

Py>x=12erfcγx21−12 erfcγx+erfcγx1−12 erfcγx2

## 6.4	Probability of Packet Failure

The probability of symbol error results from the statistical average over the distributions for the thermal noise, for the data and spreading codes, and for the multipath.  There are three situations to consider:

Noise Only:  In this case the random distribution is that of the thermal noise; for a given SNR the symbol errors are independent from symbol to symbol, with probability PeN.  The probability that packet having Ns symbols will fail is

PF=1−(1−PeN)Ns

Multipath Only:  In this case the random distributions are those of the multipath and code/data combination; the SNR is infinite, and the residual probability of symbol error Pe is the product of the probability PCB that the multipath profile is one which causes errors in the absence of noise and the probability PCD that the code and data combination are sensitive to this bad channel condition.  However, when this happens, the symbol errors are highly correlated throughout the packet because the multipath is static over a packet.  For 16-bit codes we assume that the conditional probability of error, given a bad channel condition, is PCD=1/8, for some coset.  However, since there are four cosets used to form any code channel, and since it is likely that only one of  the cosets is disturbed by a particular “bad” multipath profile, the correct value for calculations is PCD=1/32.  The probability that packet having Ns symbols will fail is

PF=(1−(1−PCD)Ns)PCB

Multipath and Noise: In this case the random distributions of both the noise and multipath must be accommodated.  When the SNR is very high the multipath-only behavior must be exhibited, while the noise-only behavior should dominate when Pe>>Pe.  We seek a procedure for calculating PF consistent with the above, and which may be applied to computations without requiring a decision as to the dominant behavior (i.e., multipath or noise). We may separate the probability of symbol error into mutually exclusive good- and bad-channel cases, then employ the appropriate probability of packet failure.  The packet-failure probability is then

PF=1−(1−Pen)Ns1−PCB +(1−(1−PCD)Ns(1−Pen)Ns)PCB

The non-thermal error probability must be subtracted from Pe, and that PCD=1/8PeI can be substituted, to yield

PF=1−(1−Pen+PeI)Ns1−32PeI +(1−(3132)Ns(1−Pen+PeI)Ns)32PeI

## 6.5	Noise Re-Normalization for Channel Matched Filter

The performance computation used is a hybrid between simulation and analytical techniques.  The channel impulse response, data and spreading codes are repeatedly selected simulation-style from a large space in order to generate possible received waveforms.  However, because simulation is particularly inefficient in computing low probabilities of symbol error, this is done using the signal amplitudes and analytical expressions for the probability of symbol error vs. SNR.  The insertion of a channel matched filter (CMF) causes some difficulty for the hybrid calculation, as opposed to simulation, because the noise is re-normalized in the process.

### 6.5.1	Standard Receiver

A “largest-of” receiver correlates against the possible received waveforms, then selects as the correct hypothesis that which produces the largest magnitude for the correlation.  In a white-noise environment the signal components are

Sm=An=0Nc−1CmnCm'n

where A is the unknown complex amplitude, Nc is the number of chips, Cmn is the code pattern and m’ corresponds to the data (i.e., the actual waveform transmitted).  The noise variance is

Nm2=n=0Nc−1znCmn2=n'Nc−1n=0Nc−1zn'znCmn'Cmn=2σ2nNc−1Cmn2

which is the same for all channels.  The SNR is found by using the magnitude squared of the output for the correct channel

Sm'2Nm'2=An=0Nc−1CmnCm'n22σ2nNc−1Cmn2=A22σ2 Nc2Nc=NcA22σ2

Since the noise is added at the receiver, while the signal is convolved with the channel impulse response before entering the receiver, the probability of symbol error can be computed using the standard expressions by simply computing the magnitudes of the correlator outputs.  The noise is unaffected in this process.  Of course, the channel distortions generally cause the SNR in the incorrect channels to be non-zero, but this is handled using the union bound with pair-wise error probabilities for two Rician variates.

### 6.5.2	Receiver with CMF

The form of the above SNR makes clear the difficulty incurred when a CMF is inserted.  Unlike the previous case, which left the noise unaltered, the presence of a CMF inside the receiver means that the noise is affected.  Thus, the denominator of the SNR expression must be changed.  The cascade of the CMF and the correlators must be considered as the overall linear processing reference function.  Thus, we define

gmn=Cmn∗dn=n'=0,n−Ntminn,Nc−1Cmn'dn−n'

where dn is the impulse response of the CMF (assumed not to perfectly match) and Nt is the number of filter taps.  The received waveform is rmn=Cm'n∗cn where cn is the true channel impulse response.  With this definition we may write the SNR

Sm'2Nm'2=A22σ2 n=0Nc+Nt−1rmngm'n2nNc+Nt−1gm'n2

We use the previous formulation (i.e., union bound with pair-wise error probabilities) with the SNR per correlator output replaced by the above ratio of summations.  However, note that the noise component now varies for each channel; this is inconsistent with the dual-Rician error formulation used, which assumes equal variances. A reasonable approximation can be used to avoid a considerably more complex formulation: we shall compute the noise variance summation as statistically averaged over random codes, then use this single variance for all channels.  This will take the noise re-normalization into account in a reasonable manner.  We write the noise summation

ngm'n2=nn'n} }  cSup {}  {} C rSub { size 8{ ital mn'} } C rSub { size 8{ ital mn
              dn−n'dn−n} }  rSup { size 8{*} } } {

We use the average over all codes by asserting that Cmn'Cmn} } }} =δ rSub { size 8{n'n which leads to

ngm'n2=nn'n} }  cSup {}  {} δ rSub { size 8{n'ndn−n'dn−n} }  rSup { size 8{*} } = Sum cSub { size 8{n} }  cSup {}  {}  Sum cSub { size 8{n'} }  cSup {}  {}  lline d rSub { size 8{n - n'} }  rline  rSup { size 8{2} } =N rSub { size 8{c} }  Sum cSub { size 8{n'} }  cSup {}  {}  lline d rSub { size 8{n} }  rline  rSup { size 8{2} } } {

This result requires careful consideration of the limits of the summations.  A simple approach to handling the noise re-normalization is to require n'dn2=1 which avoids any required change in original computation procedure.

See Appendix on MSK Approximation.

Although the correlator reference will be properly aligned to the signal, it is necessary to provide for mismatch of the reference and signal to later support 16-ary Orthogonal demodulation.

Because of the linearity of filtering, the chip pulse is approximately symmetric.

This is the Kronecker delta x0 with half-integer values of x allowed.

This is convenient analytically, and it also means that the results can be used directly for PSK waveforms as well as for MSK.

0 still carries an unknown propagation phase.

If a true matched filter were used, then this would be the familiar E/N0.

We shall evaluate the probability of symbol error based only upon the orthogonal-signaling component; in the absence of such errors, the error probability for the DBPSK component is completely negligible.

We use ~ (tilde) to distinguish cyclic correlations from acyclic.

Note that the normalization is preserved, i.e., summing k over the corresponding range of k.

0 still carries an unknown propagation phase.

If a true matched filter were used, then this would be the familiar E/N0.

DiFranco and Rubin, Radar Detection.

Op Cit, DiFranco and Rubin.

We have ignored here the pR1 terms as second-order.

This ignores the receive filtering reduction on the noise, but this limit is simply for comparison.

The implementation loss is assumed the same for all cases.

The filter’s lack of causality is of no concern for present purposes.

M. Schwartz, W. R. Bennett and S. Stein, Communications Systems and Techniques.  New York: McGraw-Hill, 1966, pp. 585-587.

If b<a, then we interchange the roles of a and b and complement the resulting computed probability to find PK.

There is no loss in generality in assuming that the means are positive.

This was shown in document 97-120 for a specific code being used as an example; since the codes are statistically similar, it is reasonable to use this value for our calculations.