IEEE P802.11
Wireless LANs

Performance Comparison of Joint BreezeCom+NEC Proposal vs BreezeCom Proposal

Date:	March 10, 1998 Update of March 18, 1998

Author:	Tal Kaitz                                         Naftali Chayat

BreezeCom

Atidim Technology Park, Bldg 1

Tel Aviv 61131, ISRAEL

email: {talk,naftalic}@breezecom.co.il

Introduction

Shortly before the March 98 meeting BreezeCom and NEC decided to present a joint proposal based on Offset Quadrature Modulation. Since the merger and until the presentation at 802.11 we did not prepare a separate evaluation of the joint proposal, hinging on the similarity of the OQM modulation in the joint vs. previous BreezeCom proposal (as presented in 98/76 and 97/76r1). The assumption was that only ACI data will change.

This document is an update on performance evaluation of the joint proposal. Contrary to our previous assumptions, there is slight degradation in multipath performance when SRRC pulse is used, versus the situation when GMSK-like pulse is used. The change is slight, yet there is a need to understand its source; this will be performed in the near future.

In the performance update equalizer with 16 feedforward taps and 23 feedback taps was compared. For initial comparison 64 byte packets were addressed.

General Description

| Parameter | Value(s) |
| --- | --- |
| Data Rates Supported | 20.9677 Mbit/s (mandatory), 25.0000 Mbit/s (mandatory), 41.9355 Mbit/s (optional), 50.0000 Mbit/s (optional), 62.9032 Mbit/s (optional/impractical), 75.0000 Mbit/s (optional/impractical), 83.8710 Mbit/s (optional/impractical), 100.0000 Mbit/s (optional/impractical) |
| Channel Spacing (Breeze proposal) | 25 MHz |
| Channel Spacing (joint proposal) | 20 MHz |
| Center Frequencies (Breeze proposal) | lower: 5.175, 5.200, 5.225, 5.250 GHz middle 5.275, 5.300, 5.325 GHz upper: 5.750, 5.775, 5.800 GHz |
| Center Frequencies (joint proposal) | lower: 5.170, 5.190, 5.210, 5.230, 5.250 GHz middle 5.270, 5.290, 5.310, 5.330 GHz upper: 5.745, 5.765, 5.795, 5.815 GHz |
| Power Levels | Resulting from 12.5 MHz bandwidth, Lower band: 30 mW Middle band: 150 mW Upper band: 600 mW |
| Sensitivities | 20.9677 Mbit/s: -77 dBm 25.0000 Mbit/s: -75 dBm 41.9355 Mbit/s: -67 dBm 50.0000 Mbit/s: -65 dBm |
| CCA threshold |  |
| Clock Rate accuracy | 10 ppm |
| Carrier Frequency accuracy | 10 ppm (60 kHz) |
| Waveform implementation accuracy specification method | RMS residual ISI when optimizing with respect to slack parameters – frequency, phase and timing offset, and a short equalizer (joint Breeze+NEC proposal) |
| Power Backoff in RF PA | Saturated in Breeze proposal 1 dB to 6 dB depending on modulation and U-NII subband (Breeze+NEC) |
| Implementation Complexity | 100-200 Kgates, depending on equalizer length. The power consumption for the implementation with long equalizer is expected to be 350 mW with equalizer adaptation and 250 mW without adaptation along the packet. (based on 0.25 micron process) |

Per-Rate Feature Summary

| Parameter | 21 Mbit/s | 25 Mbit/s | 42 Mbit/s | 50 Mbit/s |
| --- | --- | --- | --- | --- |
| Data rate | 20.9677 Mbit/s | 25.0 Mbit/s | 41.9355 Mbit/s | 50.0 Mbit/s |
| ECC method | Hamming | none | Hamming | none |
| Interleaving method | write rows, encode columns, read rows depth 8 | None | write rows, encode columns, read rows depth 16 | none |
| Suggested minimal sensitivity | -77 dBm | -75 dBm | -67 dBm | -65 dBm |
| Suggested Co-Channel rejection |  |  |  |  |
| Suggested Adjacent Channel rejection |  |  |  |  |
| Suggested Alternate Channel rejection |  |  |  |  |
| Implementation Accuracy (joint propsal) | 23 dB residual ISI | 23 dB residual ISI | 30 dB residual ISI | 30 dB residual ISI |

Per-Rate Performance Summary

The data relates to DFE receiver with 16 taps in feed-forward filter and 15 decision feedback taps. Data for shorter equalizers will be provided soon.

| Parameter | 21 Mbit/s | 25 Mbit/s | 42 Mbit/s | 50 Mbit/s |
| --- | --- | --- | --- | --- |
| Eb/No at PER=10%, AWGN, 64b | 7.2 dB | 9.5 dB | 10 dB | 12.5 dB |
| Trms at PER=10%, noise free, 64b | 240 nsec | 230 nsec | 120 nsec | 100 nsec |
| Eb/No @ 20%, with Trms @ 10%, 64b |  |  |  | 17 dB |
| Eb/No at PER=10%, AWGN, 1000b | 8.5 dB | 11.2 dB | 12.5 dB | 14.2 dB |
| Trms at PER=10%, noise free, 1000b | 185 nsec | 170 nsec | 95 nsec | 70 nsec |
| Eb/No @ 20%, with Trms @ 10%, 1000b | 15 dB? | 17 dB? | 19 dB | 22 dB? |
| CCI immunity [dB] | -13 dB | -15 dB | -17 dB | -20 dB |
| ACI immunity [dB] for BreezeCom proposal | 40 dB 28 dB (sat adj) | 42 dB 30 dB (sat adj) | 34 dB | 36 dB |
| ACI immunity [dB] for joint Breeze+NEC proposal (P=1 PA model) | 25-26 dB at 2 dB backoff | 23-24 dB at 2 dB backoff | 17 dB at 6 dB backoff | 15 dB at 6 dB backoff |
| CW jammer immunity [dB] | -10 dB | -11 dB | -18.5 dB | -21 dB |
| Narrowband Gaussian noise immunity [dB] |  |  |  |  |
| Phase noise tolerance, (BW=50 kHz), rad2 [dBc] at which PER becomes 10% | -10 dB | -12 dB | -16 dB | -20 dB |

Timing and Overhead related parameters

Attach verbal explanation of the assumptions taken for each parameter

| Attribute | Suggested Value |
| --- | --- |
| aSlotTime | 6.0 ms in Breezecom 7.4 ms in joint proposal |
| aCCATime | 3.0 ms |
| aRxTxTurnaroundTime | 1.4 ms |
| aTxPLCPDelay | 0.4 ms |
| aRxTxSwitchTime | 0.4 ms. |
| aTxRampOnTime | 0.4 ms. |
| aTxRFDelay | 0.4 ms. |
| aSIFSTime | 12.0 ms. in Breezecom 13.4 ms in joint proposal |
| aRxRFDelay | 1.0 ms. |
| aRxPLCPDelay | 7.0 ms. |
| aMACProcessingDelay | 0.6 ms. in Breezecom 2.0 ms in joint proposal |
| aTxRampOffTime | 0.4 ms. |
| aPreambleLength | 10.24 ms in Breezecom 12.8 ms in joint proposal |
| aPLCPHdrLength | 3.2 ms |
| aMPDUDurationFactor | 1.1923 (if ECC used) |
| aAirPropagationTime | 0.8 ms |
| aCWmin | 15 |
| aCWmax | 1023 |

Fig.1: Packet Error Rate vs. multipath and Eb/No, coded binary 21 Mbit/s, BreezeCom.

Fig.2: Packet Error Rate vs. multipath and Eb/No, coded binary 21 Mbit/s, Breeze+NEC.

Fig.3: Packet Error Rate vs. multipath and Eb/No, uncoded binary 25 Mbit/s, BreezeCom.

Fig.4: Packet Error Rate vs. multipath and Eb/No, uncoded binary 25 Mbit/s, Breeze+NEC.

Fig.5: Packet Error Rate vs. multipath and Eb/No, coded binary 42 Mbit/s, BreezeCom.

Fig.6: Packet Error Rate vs. multipath and Eb/No, coded binary 42 Mbit/s, Breeze+NEC.

Fig.7: Packet Error Rate vs. multipath and Eb/No, uncoded binary 50 Mbit/s, BreezeCom.

Fig.8: Packet Error Rate vs. multipath and Eb/No, uncoded binary 50 Mbit/s, Breeze+NEC.

Fig.9: Packet Error Rate vs. phase noise, BreezeCom.

Fig.10: Packet Error Rate vs. phase noise, Breeze+NEC.