IEEE P802.11
Wireless LANs

Annex E proposed text

Date:	April 25, 1999

Author:	Tal Kaitz and Naftali Chayat
	BreezeCom
	Atitim Technology Park, Bldg. 1, Tel-Aviv 61131, Israel
	Phone: +972-3-645-6262
	Fax:     +972-3-645-6290
	e-mail : [talk naftalic]@breezecom.co.il

Abstract

This document serves as a proposed text to be inserted as Annex E in the 802.11a standard. The purpose of this annex is to bring an example of encoding a frame for the OFDM PHY. The example goes through the all the encoding details defined by the standard.

Annex E

The purpose of this annex is to bring an example of encoding a frame for the OFDM PHY described in Clause 17 of the 802.11 standard. The encoding illustration goes through the following stages:

- Generating the short training sequences section of the preamble.

- Generating the long preamble sequence section of the preamble.

- Generating the SIGNAL field bits.

- Coding and interleaving the SIGNAL field bits.

- Mapping the SIGNAL field into frequency domain.

- Pilot insertion.

- Transforming into time domain.

- Delineation of the data octet stream into a bit stream.

- Prepending the SERVICE field and adding the pad bits, forming thus the DATA.

- Scrambling and zeroing the tail bits.

- Encoding the DATA with a convolutional encoder and puncturing.

- Mapping into complex QAM16 symbols.

- Pilots Insertion.

- Transforming from frequency to time and adding a circular prefix.

- Concatenating the OFDM symbols into a single time-domain signal.

In the description of time domain waveforms a complex baseband signal at 20 Msamples per second shall be used.

The example uses the 36 Mbit/s data rate and a message of 100 octets. These parameters are chosen in order to illustrate as many nontrivial aspects of the processing:

- Use of several bits per symbol (4 in our case)

- Puncturing of the convolutional code

- Interleaving which uses non-trivially the LSB-MSB swapping stage

- Scrambling of the pilot subcarriers

# The message

The message being encoded is the first 72 characters of the well-known “Ode to Joy” by F. Schiller:

Joy, bright spark of divinity,

Daughter of Elysium,

Fire-insired we tread

Thy sanctuary.

Thy magic power re-unites

All that custom has divided,

All men become brothers

Under the sway of thy gentle wings…

The message is converted to ASCII, prepended with an appropriate MAC header and a CRC32 is added. The resulting 100 octets PSDU is shown in table 1.

| # | Val | Val | Val | Val | Val |
| --- | --- | --- | --- | --- | --- |
| 1… 5 | 04 | 02 | 00 | 2e | 00 |
| 6… 10 | 60 | 08 | cd | 37 | a6 |
| 11…15 | 00 | 20 | d6 | 01 | 3c |
| 16…20 | f1 | 00 | 60 | 08 | ad |
| 21…25 | 3b | af | 00 | 00 | 4a |
| 26…30 | 6f | 79 | 2c | 20 | 62 |
| 31…35 | 72 | 69 | 67 | 68 | 74 |
| 36…45 | 20 | 73 | 70 | 61 | 72 |
| 41…45 | 6b | 20 | 6f | 66 | 20 |
| 46…50 | 64 | 69 | 76 | 69 | 6e |
| 51…55 | 69 | 74 | 79 | 2c | 0a |
| 56…60 | 44 | 61 | 75 | 67 | 68 |
| 61…55 | 74 | 65 | 72 | 20 | 6f |
| 66…60 | 66 | 20 | 45 | 6c | 79 |
| 71…55 | 73 | 69 | 75 | 6d | 2c |
| 76…60 | 0a | 46 | 69 | 72 | 65 |
| 81…55 | 2d | 69 | 6e | 73 | 69 |
| 86…60 | 72 | 65 | 64 | 20 | 77 |
| 91…55 | 65 | 20 | 74 | 72 | 65 |
| 96…100 | 61 | da | 57 | 99 | ed |

Table 1: The message.

# Generation of the preamble

## Generation of the short sequences

The short sequences section of the preamble is described by its frequency domain representation, given in table 2.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0 | 0 |  | -16 | -1.414 | -1.414 |  | 0 | 0 | 0 |  | 16 | 1.414 | 1.414 |
| -31 | 0 | 0 |  | -15 | 0 | 0 |  | 1 | 0 | 0 |  | 17 | 0 | 0 |
| -30 | 0 | 0 |  | -14 | 0 | 0 |  | 2 | 0 | 0 |  | 18 | 0 | 0 |
| -29 | 0 | 0 |  | -13 | 0 | 0 |  | 3 | 0 | 0 |  | 19 | 0 | 0 |
| -28 | 0 | 0 |  | -12 | -1.414 | -1.414 |  | 4 | 1.414 | 1.414 |  | 20 | -1.414 | 1.414 |
| -27 | 0 | 0 |  | -11 | 0 | 0 |  | 5 | 0 | 0 |  | 21 | 0 | 0 |
| -26 | 0 | 0 |  | -10 | 0 | 0 |  | 6 | 0 | 0 |  | 22 | 0 | 0 |
| -25 | 0 | 0 |  | -9 | 0 | 0 |  | 7 | 0 | 0 |  | 23 | 0 | 0 |
| -24 | 1.414 | 1.414 |  | -8 | 1.414 | -1.414 |  | 8 | 1.414 | 1.414 |  | 24 | 1.414 | 1.414 |
| -23 | 0 | 0 |  | -7 | 0 | 0 |  | 9 | 0 | 0 |  | 25 | 0 | 0 |
| -22 | 0 | 0 |  | -6 | 0 | 0 |  | 10 | 0 | 0 |  | 26 | 0 | 0 |
| -21 | 0 | 0 |  | -5 | 0 | 0 |  | 11 | 0 | 0 |  | 27 | 0 | 0 |
| -20 | 1.414 | 1.414 |  | -4 | -1.414 | -1.414 |  | 12 | -1.414 | -1.414 |  | 28 | 0 | 0 |
| -19 | 0 | 0 |  | -3 | 0 | 0 |  | 13 | 0 | 0 |  | 29 | 0 | 0 |
| -18 | 0 | 0 |  | -2 | 0 | 0 |  | 14 | 0 | 0 |  | 20 | 0 | 0 |
| -17 | 0 | 0 |  | -1 | 0 | 0 |  | 15 | 0 | 0 |  | 31 | 0 | 0 |

Table 2: Frequency domain representation of the short sequences

One period of the IFFT on the contents of Table 2 is given in Table 3.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.044 | 0.044 |  | 16 | 0.044 | 0.044 |  | 32 | 0.044 | 0.044 |  | 48 | 0.044 | 0.044 |
| 1 | -0.109 | -0.045 |  | 17 | -0.109 | -0.045 |  | 33 | -0.109 | -0.045 |  | 49 | -0.109 | -0.045 |
| 2 | -0.044 | 0.062 |  | 18 | -0.044 | 0.062 |  | 34 | -0.044 | 0.062 |  | 50 | -0.044 | 0.062 |
| 3 | 0.013 | 0.126 |  | 19 | 0.013 | 0.126 |  | 35 | 0.013 | 0.126 |  | 51 | 0.013 | 0.126 |
| 4 | -0.133 | -0.044 |  | 20 | -0.133 | -0.044 |  | 36 | -0.133 | -0.044 |  | 52 | -0.133 | -0.044 |
| 5 | -0.095 | 0.052 |  | 21 | -0.095 | 0.052 |  | 37 | -0.095 | 0.052 |  | 53 | -0.095 | 0.052 |
| 6 | -0.018 | 0.062 |  | 22 | -0.018 | 0.062 |  | 38 | -0.018 | 0.062 |  | 54 | -0.018 | 0.062 |
| 7 | 0.075 | -0.066 |  | 23 | 0.075 | -0.066 |  | 39 | 0.075 | -0.066 |  | 55 | 0.075 | -0.066 |
| 8 | 0.133 | 0.044 |  | 24 | 0.133 | 0.044 |  | 40 | 0.133 | 0.044 |  | 56 | 0.133 | 0.044 |
| 9 | -0.042 | 0.071 |  | 25 | -0.042 | 0.071 |  | 41 | -0.042 | 0.071 |  | 57 | -0.042 | 0.071 |
| 10 | -0.044 | -0.062 |  | 26 | -0.044 | -0.062 |  | 42 | -0.044 | -0.062 |  | 58 | -0.044 | -0.062 |
| 11 | 0.013 | -0.152 |  | 27 | 0.013 | -0.152 |  | 43 | 0.013 | -0.152 |  | 59 | 0.013 | -0.152 |
| 12 | -0.044 | -0.044 |  | 28 | -0.044 | -0.044 |  | 44 | -0.044 | -0.044 |  | 60 | -0.044 | -0.044 |
| 13 | 0.069 | 0.099 |  | 29 | 0.069 | 0.099 |  | 45 | 0.069 | 0.099 |  | 61 | 0.069 | 0.099 |
| 14 | 0.107 | -0.062 |  | 30 | 0.107 | -0.062 |  | 46 | 0.107 | -0.062 |  | 62 | 0.107 | -0.062 |
| 15 | 0.075 | -0.085 |  | 31 | 0.075 | -0.085 |  | 47 | 0.075 | -0.085 |  | 63 | 0.075 | -0.085 |

Table 3: One period of IFFT of the short sequences

The single period of the short training sequence is extended periodically for 161 samples ( about 8 microseconds), and then multiplied by the window function

W(k)=0.5k=011≤k<1600.5k=160

The last element serves as an overlap with the following sections. The 161 samples vector is shown in Table 4.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.022 | 0.022 |  | 40 | 0.133 | 0.044 |  | 80 | 0.044 | 0.044 |  | 120 | 0.133 | 0.044 |
| 1 | -0.109 | -0.045 |  | 41 | -0.042 | 0.071 |  | 81 | -0.109 | -0.045 |  | 121 | -0.042 | 0.071 |
| 2 | -0.044 | 0.062 |  | 42 | -0.044 | -0.062 |  | 82 | -0.044 | 0.062 |  | 122 | -0.044 | -0.062 |
| 3 | 0.013 | 0.126 |  | 43 | 0.013 | -0.152 |  | 83 | 0.013 | 0.126 |  | 123 | 0.013 | -0.152 |
| 4 | -0.133 | -0.044 |  | 44 | -0.044 | -0.044 |  | 84 | -0.133 | -0.044 |  | 124 | -0.044 | -0.044 |
| 5 | -0.095 | 0.052 |  | 45 | 0.069 | 0.099 |  | 85 | -0.095 | . |  | 125 | 0.069 | 0.099 |
| 6 | -0.018 | 0.062 |  | 46 | 0.107 | -0.062 |  | 86 | -0.018 | 0.062 |  | 126 | 0.107 | -0.062 |
| 7 | 0.075 | -0.066 |  | 47 | 0.075 | -0.085 |  | 87 | 0.075 | -0.066 |  | 127 | 0.075 | -0.085 |
| 8 | 0.133 | 0.044 |  | 48 | 0.044 | 0.044 |  | 88 | 0.133 | 0.044 |  | 128 | 0.044 | 0.044 |
| 9 | -0.042 | 0.071 |  | 49 | -0.109 | -0.045 |  | 89 | -0.042 | 0.071 |  | 129 | -0.109 | -0.045 |
| 10 | -0.044 | -0.062 |  | 50 | -0.044 | 0.062 |  | 90 | -0.044 | -0.062 |  | 130 | -0.044 | 0.062 |
| 11 | 0.013 | -0.152 |  | 51 | 0.013 | 0.126 |  | 91 | 0.013 | -0.152 |  | 131 | 0.013 | 0.126 |
| 12 | -0.044 | -0.044 |  | 52 | -0.133 | -0.044 |  | 92 | -0.044 | -0.044 |  | 132 | -0.133 | -0.044 |
| 13 | 0.069 | 0.099 |  | 53 | -0.095 | 0.052 |  | 93 | 0.069 | 0.099 |  | 133 | -0.095 | 0.052 |
| 14 | 0.107 | -0.062 |  | 54 | -0.018 | 0.062 |  | 94 | 0.107 | -0.062 |  | 134 | -0.018 | 0.062 |
| 15 | 0.075 | -0.085 |  | 55 | 0.075 | -0.066 |  | 95 | 0.075 | -0.085 |  | 135 | 0.075 | -0.066 |
| 16 | 0.044 | 0.044 |  | 56 | 0.133 | 0.044 |  | 96 | 0.044 | 0.044 |  | 136 | 0.133 | 0.044 |
| 17 | -0.109 | -0.045 |  | 57 | -0.042 | 0.071 |  | 97 | -0.109 | -0.045 |  | 137 | -0.042 | 0.071 |
| 18 | -0.044 | 0.062 |  | 58 | -0.044 | -0.062 |  | 98 | -0.044 | 0.062 |  | 138 | -0.044 | -0.062 |
| 19 | 0.013 | 0.126 |  | 59 | 0.013 | -0.152 |  | 99 | 0.013 | 0.126 |  | 139 | 0.013 | -0.152 |
| 20 | -0.133 | -0.044 |  | 60 | -0.044 | -0.044 |  | 100 | -0.133 | -0.044 |  | 140 | -0.044 | -0.044 |
| 21 | -0.095 | 0.052 |  | 61 | 0.069 | 0.099 |  | 101 | -0.095 | 0.052 |  | 141 | 0.069 | 0.099 |
| 22 | -0.018 | 0.062 |  | 62 | 0.107 | -0.062 |  | 102 | -0.018 | 0.062 |  | 142 | 0.107 | -0.062 |
| 23 | 0.075 | -0.066 |  | 63 | 0.075 | -0.085 |  | 103 | 0.075 | -0.066 |  | 143 | 0.075 | -0.085 |
| 24 | 0.133 | 0.044 |  | 64 | 0.044 | 0.044 |  | 104 | 0.133 | 0.044 |  | 144 | 0.044 | 0.044 |
| 25 | -0.042 | 0.071 |  | 65 | -0.109 | -0.045 |  | 105 | -0.042 | 0.071 |  | 145 | -0.109 | -0.045 |
| 26 | -0.044 | -0.062 |  | 66 | -0.044 | 0.062 |  | 106 | -0.044 | -0.062 |  | 146 | -0.044 | 0.062 |
| 27 | 0.013 | -0.152 |  | 67 | 0.013 | 0.126 |  | 107 | 0.013 | -0.152 |  | 147 | 0.013 | 0.126 |
| 28 | -0.044 | -0.044 |  | 68 | -0.133 | -0.044 |  | 108 | -0.044 | -0.044 |  | 148 | -0.133 | -0.044 |
| 29 | 0.069 | 0.099 |  | 69 | -0.095 | 0.052 |  | 109 | 0.069 | 0.099 |  | 149 | -0.095 | 0.052 |
| 30 | 0.107 | -0.062 |  | 70 | -0.018 | 0.062 |  | 110 | 0.107 | -0.062 |  | 150 | -0.018 | 0.062 |
| 31 | 0.075 | -0.085 |  | 71 | 0.075 | -0.066 |  | 111 | 0.075 | -0.085 |  | 151 | 0.075 | -0.066 |
| 32 | 0.044 | 0.044 |  | 72 | 0.133 | 0.044 |  | 112 | 0.044 | 0.044 |  | 152 | 0.133 | 0.044 |
| 33 | -0.109 | -0.045 |  | 73 | -0.042 | 0.071 |  | 113 | -0.109 | -0.045 |  | 153 | -0.042 | 0.071 |
| 34 | -0.044 | 0.062 |  | 74 | -0.044 | -0.062 |  | 114 | -0.044 | 0.062 |  | 154 | -0.044 | -0.062 |
| 35 | 0.013 | 0.126 |  | 75 | 0.013 | -0.152 |  | 115 | 0.013 | 0.126 |  | 155 | 0.013 | -0.152 |
| 36 | -0.133 | -0.044 |  | 76 | -0.044 | -0.044 |  | 116 | -0.133 | -0.044 |  | 156 | -0.044 | -0.044 |
| 37 | -0.095 | 0.052 |  | 77 | 0.069 | 0.099 |  | 117 | -0.095 | 0.052 |  | 157 | 0.069 | 0.099 |
| 38 | -0.018 | 0.062 |  | 78 | 0.107 | -0.062 |  | 118 | -0.018 | 0.062 |  | 158 | 0.107 | -0.062 |
| 39 | 0.075 | -0.066 |  | 79 | 0.075 | -0.085 |  | 119 | 0.075 | -0.066 |  | 159 | 0.075 | -0.085 |
|  |  |  |  |  |  |  |  |  |  |  |  | 160 | 0.0221 | 0.0221 |

Table 4: Time domain representation of short sequences

## Generation of the long sequences

The frequency domain representation of the long sequences section is preamble is given in Table 5.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0.000 | 0.000 |  | -16 | 1.000 | 0.000 |  | 0 | 0.000 | 0.000 |  | 16 | 1.000 | 0.000 |
| -31 | 0.000 | 0.000 |  | -15 | 1.000 | 0.000 |  | 1 | 1.000 | 0.000 |  | 17 | -1.000 | 0.000 |
| -30 | 0.000 | 0.000 |  | -14 | 1.000 | 0.000 |  | 2 | -1.000 | 0.000 |  | 18 | -1.000 | 0.000 |
| -29 | 0.000 | 0.000 |  | -13 | 1.000 | 0.000 |  | 3 | -1.000 | 0.000 |  | 19 | 1.000 | 0.000 |
| -28 | 0.000 | 0.000 |  | -12 | 1.000 | 0.000 |  | 4 | 1.000 | 0.000 |  | 20 | -1.000 | 0.000 |
| -27 | 0.000 | 0.000 |  | -11 | -1.000 | 0.000 |  | 5 | 1.000 | 0.000 |  | 21 | 1.000 | 0.000 |
| -26 | 1.000 | 0.000 |  | -10 | -1.000 | 0.000 |  | 6 | -1.000 | 0.000 |  | 22 | -1.000 | 0.000 |
| -25 | 1.000 | 0.000 |  | -9 | 1.000 | 0.000 |  | 7 | 1.000 | 0.000 |  | 23 | 1.000 | 0.000 |
| -24 | -1.000 | 0.000 |  | -8 | 1.000 | 0.000 |  | 8 | -1.000 | 0.000 |  | 24 | 1.000 | 0.000 |
| -23 | -1.000 | 0.000 |  | -7 | -1.000 | 0.000 |  | 9 | 1.000 | 0.000 |  | 25 | 1.000 | 0.000 |
| -22 | 1.000 | 0.000 |  | -6 | 1.000 | 0.000 |  | 10 | -1.000 | 0.000 |  | 26 | 1.000 | 0.000 |
| -21 | 1.000 | 0.000 |  | -5 | -1.000 | 0.000 |  | 11 | -1.000 | 0.000 |  | 27 | 0.000 | 0.000 |
| -20 | -1.000 | 0.000 |  | -4 | 1.000 | 0.000 |  | 12 | -1.000 | 0.000 |  | 28 | 0.000 | 0.000 |
| -19 | 1.000 | 0.000 |  | -3 | 1.000 | 0.000 |  | 13 | -1.000 | 0.000 |  | 29 | 0.000 | 0.000 |
| -18 | -1.000 | 0.000 |  | -2 | 1.000 | 0.000 |  | 14 | -1.000 | 0.000 |  | 30 | 0.000 | 0.000 |
| -17 | 1.000 | 0.000 |  | -1 | 1.000 | 0.000 |  | 15 | 1.000 | 0.000 |  | 31 | 0.000 | 0.000 |

Table 5: Frequency domain representation of the long sequences

The time domain representation is derived by performing IFFT on the contents of Table 5, cyclically extending the result to get the cyclic prefix and the multiplying with the window function of section 2.1 . The resulting 161 points vector is shown in Table 6. The  samples are appended to the short sequence section by overlapping and adding element 160 of Table 4 to element 0 of Table 6.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -0.078 | 0.000 |  | 40 | 0.098 | -0.026 |  | 80 | 0.062 | 0.062 |  | 120 | -0.035 | -0.151 |
| 1 | 0.012 | -0.098 |  | 41 | 0.053 | 0.004 |  | 81 | 0.119 | 0.004 |  | 121 | -0.122 | -0.017 |
| 2 | 0.092 | -0.106 |  | 42 | 0.001 | -0.115 |  | 82 | -0.022 | -0.161 |  | 122 | -0.127 | -0.021 |
| 3 | -0.092 | -0.115 |  | 43 | -0.137 | -0.047 |  | 83 | 0.059 | 0.015 |  | 123 | 0.075 | -0.074 |
| 4 | -0.003 | -0.054 |  | 44 | 0.024 | -0.059 |  | 84 | 0.024 | 0.059 |  | 124 | -0.003 | 0.054 |
| 5 | 0.075 | 0.074 |  | 45 | 0.059 | -0.015 |  | 85 | -0.137 | 0.047 |  | 125 | -0.092 | 0.115 |
| 6 | -0.127 | 0.021 |  | 46 | -0.022 | 0.161 |  | 86 | 0.001 | 0.115 |  | 126 | 0.092 | 0.106 |
| 7 | -0.122 | 0.017 |  | 47 | 0.119 | -0.004 |  | 87 | 0.053 | -0.004 |  | 127 | 0.012 | 0.098 |
| 8 | -0.035 | 0.151 |  | 48 | 0.062 | -0.062 |  | 88 | 0.098 | 0.026 |  | 128 | -0.156 | 0.000 |
| 9 | -0.056 | 0.022 |  | 49 | 0.037 | 0.098 |  | 89 | -0.038 | 0.106 |  | 129 | 0.012 | -0.098 |
| 10 | -0.060 | -0.081 |  | 50 | -0.057 | 0.039 |  | 90 | -0.115 | 0.055 |  | 130 | 0.092 | -0.106 |
| 11 | 0.070 | -0.014 |  | 51 | -0.131 | 0.065 |  | 91 | 0.060 | 0.088 |  | 131 | -0.092 | -0.115 |
| 12 | 0.082 | -0.092 |  | 52 | 0.082 | 0.092 |  | 92 | 0.021 | -0.028 |  | 132 | -0.003 | -0.054 |
| 13 | -0.131 | -0.065 |  | 53 | 0.070 | 0.014 |  | 93 | 0.097 | -0.083 |  | 133 | 0.075 | 0.074 |
| 14 | -0.057 | -0.039 |  | 54 | -0.060 | 0.081 |  | 94 | 0.040 | 0.111 |  | 134 | -0.127 | 0.021 |
| 15 | 0.037 | -0.098 |  | 55 | -0.056 | -0.022 |  | 95 | -0.005 | 0.120 |  | 135 | -0.122 | 0.017 |
| 16 | 0.062 | 0.062 |  | 56 | -0.035 | -0.151 |  | 96 | 0.156 | 0.000 |  | 136 | -0.035 | 0.151 |
| 17 | 0.119 | 0.004 |  | 57 | -0.122 | -0.017 |  | 97 | -0.005 | -0.120 |  | 137 | -0.056 | 0.022 |
| 18 | -0.022 | -0.161 |  | 58 | -0.127 | -0.021 |  | 98 | 0.040 | -0.111 |  | 138 | -0.060 | -0.081 |
| 19 | 0.059 | 0.015 |  | 59 | 0.075 | -0.074 |  | 99 | 0.097 | 0.083 |  | 139 | 0.070 | -0.014 |
| 20 | 0.024 | 0.059 |  | 60 | -0.003 | 0.054 |  | 100 | 0.021 | 0.028 |  | 140 | 0.082 | -0.092 |
| 21 | -0.137 | 0.047 |  | 61 | -0.092 | 0.115 |  | 101 | 0.060 | -0.088 |  | 141 | -0.131 | -0.065 |
| 22 | 0.001 | 0.115 |  | 62 | 0.092 | 0.106 |  | 102 | -0.115 | -0.055 |  | 142 | -0.057 | -0.039 |
| 23 | 0.053 | -0.004 |  | 63 | 0.012 | 0.098 |  | 103 | -0.038 | -0.106 |  | 143 | 0.037 | -0.098 |
| 24 | 0.098 | 0.026 |  | 64 | -0.156 | 0.000 |  | 104 | 0.098 | -0.026 |  | 144 | 0.062 | 0.062 |
| 25 | -0.038 | 0.106 |  | 65 | 0.012 | -0.098 |  | 105 | 0.053 | 0.004 |  | 145 | 0.119 | 0.004 |
| 26 | -0.115 | 0.055 |  | 66 | 0.092 | -0.106 |  | 106 | 0.001 | -0.115 |  | 146 | -0.022 | -0.161 |
| 27 | 0.060 | 0.088 |  | 67 | -0.092 | -0.115 |  | 107 | -0.137 | -0.047 |  | 147 | 0.059 | 0.015 |
| 28 | 0.021 | -0.028 |  | 68 | -0.003 | -0.054 |  | 108 | 0.024 | -0.059 |  | 148 | 0.024 | 0.059 |
| 29 | 0.097 | -0.083 |  | 69 | 0.075 | 0.074 |  | 109 | 0.059 | -0.015 |  | 149 | -0.137 | 0.047 |
| 30 | 0.040 | 0.111 |  | 70 | -0.127 | 0.021 |  | 110 | -0.022 | 0.161 |  | 150 | 0.001 | 0.115 |
| 31 | -0.005 | 0.120 |  | 71 | -0.122 | 0.017 |  | 111 | 0.119 | -0.004 |  | 151 | 0.053 | -0.004 |
| 32 | 0.156 | 0.000 |  | 72 | -0.035 | 0.151 |  | 112 | 0.062 | -0.062 |  | 152 | 0.098 | 0.026 |
| 33 | -0.005 | -0.120 |  | 73 | -0.056 | 0.022 |  | 113 | 0.037 | 0.098 |  | 153 | -0.038 | 0.106 |
| 34 | 0.040 | -0.111 |  | 74 | -0.060 | -0.081 |  | 114 | -0.057 | 0.039 |  | 154 | -0.115 | 0.055 |
| 35 | 0.097 | 0.083 |  | 75 | 0.070 | -0.014 |  | 115 | -0.131 | 0.065 |  | 155 | 0.060 | 0.088 |
| 36 | 0.021 | 0.028 |  | 76 | 0.082 | -0.092 |  | 116 | 0.082 | 0.092 |  | 156 | 0.021 | -0.028 |
| 37 | 0.060 | -0.088 |  | 77 | -0.131 | -0.065 |  | 117 | 0.070 | 0.014 |  | 157 | 0.097 | -0.083 |
| 38 | -0.115 | -0.055 |  | 78 | -0.057 | -0.039 |  | 118 | -0.060 | 0.081 |  | 158 | 0.040 | 0.111 |
| 39 | -0.038 | -0.106 |  | 79 | 0.037 | -0.098 |  | 119 | -0.056 | -0.022 |  | 159 | -0.005 | 0.120 |
|  |  |  |  |  |  |  |  |  |  |  |  | 160 | 0.078 | 0 |

Table 6: Time domain representation of the long sequences

# Generation of the SIGNAL field

## SIGNAL field bit assignment

The assignment follows Clause 17.3.4 and table 111. The transmitted bits are shown in Table 7 where bit 0 is transmitted first

| ### | Bit | meaning |  | ### | Bit | meaning |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | LENGTH (LSB) |  | 12 | 1 | RATE: R1 |
| 1 | 0 |  |  | 13 | 0 | RATE: R2 |
| 2 | 1 |  |  | 14 | 1 | RATE: R3 |
| 3 | 0 |  |  | 15 | 1 | RATE: R4 |
| 4 | 0 |  |  | 16 | 0 | Reserved |
| 5 | 1 |  |  | 17 | 0 | Parity |
| 6 | 1 |  |  | 18 | 0 | SIGNAL TAIL |
| 7 | 0 |  |  | 19 | 0 | SIGNAL TAIL |
| 8 | 0 |  |  | 20 | 0 | SIGNAL TAIL |
| 9 | 0 |  |  | 21 | 0 | SIGNAL TAIL |
| 10 | 0 |  |  | 22 | 0 | SIGNAL TAIL |
| 11 | 0 | LENGTH (MSB) |  | 23 | 0 | SIGNAL TAIL |

Table 7: Bit assignment for SIGNAL field

## Coding the SIGNAL field bits.

The bits are encoded by the rate ½ convolutional encoder to yield the 48 bits given in Table 8.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 8 | 1 |  | 16 | 1 |  | 24 | 0 |  | 32 | 1 |  | 40 | 0 |
| 1 | 0 |  | 9 | 1 |  | 17 | 1 |  | 25 | 0 |  | 33 | 0 |  | 41 | 1 |
| 2 | 0 |  | 10 | 0 |  | 18 | 1 |  | 26 | 0 |  | 34 | 1 |  | 42 | 1 |
| 3 | 0 |  | 11 | 0 |  | 19 | 1 |  | 27 | 1 |  | 35 | 0 |  | 43 | 1 |
| 4 | 1 |  | 12 | 1 |  | 20 | 1 |  | 28 | 0 |  | 36 | 0 |  | 44 | 0 |
| 5 | 1 |  | 13 | 0 |  | 21 | 0 |  | 29 | 0 |  | 37 | 0 |  | 45 | 0 |
| 6 | 0 |  | 14 | 0 |  | 22 | 0 |  | 30 | 0 |  | 38 | 1 |  | 46 | 0 |
| 7 | 1 |  | 15 | 0 |  | 23 | 1 |  | 31 | 1 |  | 39 | 0 |  | 47 | 0 |

Table 8: SIGNAL field bits after encoding

## Interleaving the SIGNAL field bits.

The encoded bits are interleaved according to the interleaver of section 17.3.5.6. A detailed breakdown of the interleaving operation is deferred to section 6. The interleaved SIGNAL field bits are shown in Table 9.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 8 | 1 |  | 16 | 0 |  | 24 | 1 |  | 32 | 1 |  | 40 | 0 |
| 1 | 1 |  | 9 | 0 |  | 17 | 0 |  | 25 | 0 |  | 33 | 0 |  | 41 | 0 |
| 2 | 1 |  | 10 | 1 |  | 18 | 0 |  | 26 | 0 |  | 34 | 1 |  | 42 | 0 |
| 3 | 0 |  | 11 | 0 |  | 19 | 0 |  | 27 | 1 |  | 35 | 1 |  | 43 | 0 |
| 4 | 1 |  | 12 | 1 |  | 20 | 1 |  | 28 | 0 |  | 36 | 1 |  | 44 | 0 |
| 5 | 0 |  | 13 | 1 |  | 21 | 1 |  | 29 | 1 |  | 37 | 0 |  | 45 | 0 |
| 6 | 0 |  | 14 | 0 |  | 22 | 1 |  | 30 | 0 |  | 38 | 0 |  | 46 | 1 |
| 7 | 1 |  | 15 | 1 |  | 23 | 0 |  | 31 | 0 |  | 39 | 0 |  | 47 | 0 |

Table 9: SIGNAL field bits after interleaving

## SIGNAL field frequency domain

The encoded and interleaved bits are BPSK modulated to yield the frequency domain representation given in Table 10

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0.000 | 0.000 |  | -16 | -1.000 | 0.000 |  | 0 | 0.000 | 0.000 |  | 16 | -1.000 | 0.000 |
| -31 | 0.000 | 0.000 |  | -15 | 1.000 | 0.000 |  | 1 | 1.000 | 0.000 |  | 17 | -1.000 | 0.000 |
| -30 | 0.000 | 0.000 |  | -14 | -1.000 | 0.000 |  | 2 | -1.000 | 0.000 |  | 18 | -1.000 | 0.000 |
| -29 | 0.000 | 0.000 |  | -13 | 1.000 | 0.000 |  | 3 | -1.000 | 0.000 |  | 19 | -1.000 | 0.000 |
| -28 | 0.000 | 0.000 |  | -12 | 1.000 | 0.000 |  | 4 | 1.000 | 0.000 |  | 20 | -1.000 | 0.000 |
| -27 | 0.000 | 0.000 |  | -11 | -1.000 | 0.000 |  | 5 | -1.000 | 0.000 |  | 21 | 0.000 | 0.000 |
| -26 | -1.000 | 0.000 |  | -10 | 1.000 | 0.000 |  | 6 | 1.000 | 0.000 |  | 22 | -1.000 | 0.000 |
| -25 | 1.000 | 0.000 |  | -9 | -1.000 | 0.000 |  | 7 | 0.000 | 0.000 |  | 23 | -1.000 | 0.000 |
| -24 | 1.000 | 0.000 |  | -8 | -1.000 | 0.000 |  | 8 | -1.000 | 0.000 |  | 24 | -1.000 | 0.000 |
| -23 | -1.000 | 0.000 |  | -7 | 0.000 | 0.000 |  | 9 | -1.000 | 0.000 |  | 25 | 1.000 | 0.000 |
| -22 | 1.000 | 0.000 |  | -6 | -1.000 | 0.000 |  | 10 | 1.000 | 0.000 |  | 26 | -1.000 | 0.000 |
| -21 | 0.000 | 0.000 |  | -5 | -1.000 | 0.000 |  | 11 | -1.000 | 0.000 |  | 27 | 0.000 | 0.000 |
| -20 | -1.000 | 0.000 |  | -4 | 1.000 | 0.000 |  | 12 | 1.000 | 0.000 |  | 28 | 0.000 | 0.000 |
| -19 | -1.000 | 0.000 |  | -3 | 1.000 | 0.000 |  | 13 | 1.000 | 0.000 |  | 29 | 0.000 | 0.000 |
| -18 | 1.000 | 0.000 |  | -2 | 1.000 | 0.000 |  | 14 | 1.000 | 0.000 |  | 30 | 0.000 | 0.000 |
| -17 | 1.000 | 0.000 |  | -1 | -1.000 | 0.000 |  | 15 | -1.000 | 0.000 |  | 31 | 0.000 | 0.000 |

Table 10: Frequency domain representation of SIGNAL field

Four pilots subcarriers are added by taking the values {1.0,1.0,1.0,-1.0}, multiplying them by the first element of sequence p0…126, given in equation (25), and inserting into location {-21,-7,7,21} respectively. The  resulting frequency domain values are given in Table 11.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0.000 | 0.000 |  | -16 | -1.000 | 0.000 |  | 0 | 0.000 | 0.000 |  | 16 | -1.000 | 0.000 |
| -31 | 0.000 | 0.000 |  | -15 | 1.000 | 0.000 |  | 1 | 1.000 | 0.000 |  | 17 | -1.000 | 0.000 |
| -30 | 0.000 | 0.000 |  | -14 | -1.000 | 0.000 |  | 2 | -1.000 | 0.000 |  | 18 | -1.000 | 0.000 |
| -29 | 0.000 | 0.000 |  | -13 | 1.000 | 0.000 |  | 3 | -1.000 | 0.000 |  | 19 | -1.000 | 0.000 |
| -28 | 0.000 | 0.000 |  | -12 | 1.000 | 0.000 |  | 4 | 1.000 | 0.000 |  | 20 | -1.000 | 0.000 |
| -27 | 0.000 | 0.000 |  | -11 | -1.000 | 0.000 |  | 5 | -1.000 | 0.000 |  | 21 | -1.000 | 0.000 |
| -26 | -1.000 | 0.000 |  | -10 | 1.000 | 0.000 |  | 6 | 1.000 | 0.000 |  | 22 | -1.000 | 0.000 |
| -25 | 1.000 | 0.000 |  | -9 | -1.000 | 0.000 |  | 7 | 1.000 | 0.000 |  | 23 | -1.000 | 0.000 |
| -24 | 1.000 | 0.000 |  | -8 | -1.000 | 0.000 |  | 8 | -1.000 | 0.000 |  | 24 | -1.000 | 0.000 |
| -23 | -1.000 | 0.000 |  | -7 | 1.000 | 0.000 |  | 9 | -1.000 | 0.000 |  | 25 | 1.000 | 0.000 |
| -22 | 1.000 | 0.000 |  | -6 | -1.000 | 0.000 |  | 10 | 1.000 | 0.000 |  | 26 | -1.000 | 0.000 |
| -21 | 1.000 | 0.000 |  | -5 | -1.000 | 0.000 |  | 11 | -1.000 | 0.000 |  | 27 | 0.000 | 0.000 |
| -20 | -1.000 | 0.000 |  | -4 | 1.000 | 0.000 |  | 12 | 1.000 | 0.000 |  | 28 | 0.000 | 0.000 |
| -19 | -1.000 | 0.000 |  | -3 | 1.000 | 0.000 |  | 13 | 1.000 | 0.000 |  | 29 | 0.000 | 0.000 |
| -18 | 1.000 | 0.000 |  | -2 | 1.000 | 0.000 |  | 14 | 1.000 | 0.000 |  | 30 | 0.000 | 0.000 |
| -17 | 1.000 | 0.000 |  | -1 | -1.000 | 0.000 |  | 15 | -1.000 | 0.000 |  | 31 | 0.000 | 0.000 |

Table 11: Frequency domain representation of SIGNAL field with pilots inserted

## SIGNAL field time domain

The time domain representation is derived by cyclic extension and multiplying by the window function:

W(k)=0.5k=011≤k<800.5k=80

The resulting 81 samples vector is shown in Table 12.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -0.016 | -0.016 |  | 20 | 0.024 | -0.112 |  | 40 | -0.138 | 0.116 |  | 60 | 0.058 | -0.119 |
| 1 | 0.060 | -0.089 |  | 21 | 0.110 | -0.014 |  | 41 | -0.032 | 0.063 |  | 61 | 0.087 | 0.029 |
| 2 | -0.045 | -0.112 |  | 22 | -0.003 | -0.033 |  | 42 | 0.136 | 0.019 |  | 62 | 0.200 | -0.036 |
| 3 | -0.061 | -0.030 |  | 23 | -0.101 | -0.041 |  | 43 | -0.023 | -0.080 |  | 63 | -0.085 | -0.074 |
| 4 | -0.058 | 0.014 |  | 24 | -0.050 | -0.072 |  | 44 | -0.024 | 0.021 |  | 64 | -0.031 | -0.031 |
| 5 | 0.055 | -0.069 |  | 25 | -0.015 | -0.117 |  | 45 | -0.040 | 0.063 |  | 65 | 0.060 | -0.089 |
| 6 | 0.055 | -0.025 |  | 26 | 0.055 | 0.025 |  | 46 | 0.005 | -0.086 |  | 66 | -0.045 | -0.112 |
| 7 | -0.015 | 0.117 |  | 27 | 0.055 | 0.069 |  | 47 | 0.098 | -0.059 |  | 67 | -0.061 | -0.030 |
| 8 | -0.050 | 0.072 |  | 28 | -0.058 | -0.014 |  | 48 | 0.031 | 0.000 |  | 68 | -0.058 | 0.014 |
| 9 | -0.101 | 0.041 |  | 29 | -0.061 | 0.030 |  | 49 | 0.098 | 0.059 |  | 69 | 0.055 | -0.069 |
| 10 | -0.003 | 0.033 |  | 30 | -0.045 | 0.112 |  | 50 | 0.005 | 0.086 |  | 70 | 0.055 | -0.025 |
| 11 | 0.110 | 0.014 |  | 31 | 0.060 | 0.089 |  | 51 | -0.040 | -0.063 |  | 71 | -0.015 | 0.117 |
| 12 | 0.024 | 0.112 |  | 32 | -0.031 | 0.031 |  | 52 | -0.024 | -0.021 |  | 72 | -0.050 | 0.072 |
| 13 | -0.081 | -0.033 |  | 33 | -0.085 | 0.074 |  | 53 | -0.023 | 0.080 |  | 73 | -0.101 | 0.041 |
| 14 | 0.054 | -0.116 |  | 34 | 0.200 | 0.036 |  | 54 | 0.136 | -0.019 |  | 74 | -0.003 | 0.033 |
| 15 | 0.050 | 0.133 |  | 35 | 0.087 | -0.029 |  | 55 | -0.032 | -0.063 |  | 75 | 0.110 | 0.014 |
| 16 | -0.094 | 0.000 |  | 36 | 0.058 | 0.119 |  | 56 | -0.138 | -0.116 |  | 76 | 0.024 | 0.112 |
| 17 | 0.050 | -0.133 |  | 37 | 0.130 | 0.091 |  | 57 | -0.152 | -0.075 |  | 77 | -0.081 | -0.033 |
| 18 | 0.054 | 0.116 |  | 38 | -0.153 | -0.047 |  | 58 | -0.153 | 0.047 |  | 78 | 0.054 | -0.116 |
| 19 | -0.081 | 0.033 |  | 39 | -0.152 | 0.075 |  | 59 | 0.130 | -0.091 |  | 79 | 0.050 | 0.133 |
|  |  |  |  |  |  |  |  |  |  |  |  | 80 | -0.047 | 0.000 |

Table 12: Time domain representation of the SIGNAL field

The SIGNAL field samples are appended with one sample overlap to the preamble, given in Table 6.

# Generating  the DATA bits

## Delineating, SERVICE field prepending, and zero padding

The transmitted message, shown in figure 1, contains 100 octets, or equivalently 800 bits. The bits are prepended by the 16 SERVICE field bits and are appended, by 6 tail bits. The resulting 822 bits are appended by zero bits to yield an integer number of OFDM symbols. For 36Mb/s mode, there are 144 data bits per OFDM symbol, the overall number of bits is ceil(822/6)*144 = 864. Hence 864-822=42 zero bits are appended.

The data bits are shown in Table 13 and Table 14. For clarity only the first and last 144 bits are shown.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 24 | 0 |  | 48 | 0 |  | 72 | 1 |  | 96 | 0 |  | 120 | 0 |
| 1 | 0 |  | 25 | 0 |  | 49 | 0 |  | 73 | 1 |  | 97 | 0 |  | 121 | 0 |
| 2 | 0 |  | 26 | 0 |  | 50 | 0 |  | 74 | 0 |  | 98 | 0 |  | 122 | 0 |
| 3 | 0 |  | 27 | 0 |  | 51 | 0 |  | 75 | 0 |  | 99 | 0 |  | 123 | 0 |
| 4 | 0 |  | 28 | 0 |  | 52 | 0 |  | 76 | 1 |  | 100 | 0 |  | 124 | 0 |
| 5 | 0 |  | 29 | 0 |  | 53 | 0 |  | 77 | 1 |  | 101 | 0 |  | 125 | 0 |
| 6 | 0 |  | 30 | 1 |  | 54 | 0 |  | 78 | 0 |  | 102 | 0 |  | 126 | 0 |
| 7 | 0 |  | 31 | 0 |  | 55 | 0 |  | 79 | 1 |  | 103 | 0 |  | 127 | 1 |
| 8 | 0 |  | 32 | 0 |  | 56 | 0 |  | 80 | 0 |  | 104 | 0 |  | 128 | 0 |
| 9 | 0 |  | 33 | 0 |  | 57 | 1 |  | 81 | 0 |  | 105 | 0 |  | 129 | 0 |
| 10 | 0 |  | 34 | 0 |  | 58 | 1 |  | 82 | 1 |  | 106 | 1 |  | 130 | 1 |
| 11 | 0 |  | 35 | 0 |  | 59 | 0 |  | 83 | 1 |  | 107 | 0 |  | 131 | 1 |
| 12 | 0 |  | 36 | 0 |  | 60 | 0 |  | 84 | 0 |  | 108 | 0 |  | 132 | 1 |
| 13 | 0 |  | 37 | 0 |  | 61 | 0 |  | 85 | 1 |  | 109 | 0 |  | 133 | 1 |
| 14 | 0 |  | 38 | 0 |  | 62 | 0 |  | 86 | 1 |  | 110 | 0 |  | 134 | 0 |
| 15 | 0 |  | 39 | 0 |  | 63 | 0 |  | 87 | 1 |  | 111 | 0 |  | 135 | 0 |
| 16 | 0 |  | 40 | 0 |  | 64 | 0 |  | 88 | 1 |  | 112 | 1 |  | 136 | 1 |
| 17 | 0 |  | 41 | 0 |  | 65 | 0 |  | 89 | 0 |  | 113 | 1 |  | 137 | 1 |
| 18 | 0 |  | 42 | 1 |  | 66 | 0 |  | 90 | 1 |  | 114 | 0 |  | 138 | 1 |
| 19 | 0 |  | 43 | 0 |  | 67 | 0 |  | 91 | 0 |  | 115 | 1 |  | 139 | 1 |
| 20 | 0 |  | 44 | 1 |  | 68 | 1 |  | 92 | 0 |  | 116 | 0 |  | 140 | 0 |
| 21 | 1 |  | 45 | 1 |  | 69 | 0 |  | 93 | 1 |  | 117 | 1 |  | 141 | 0 |
| 22 | 0 |  | 46 | 1 |  | 70 | 0 |  | 94 | 1 |  | 118 | 1 |  | 142 | 0 |
| 23 | 0 |  | 47 | 0 |  | 71 | 0 |  | 95 | 0 |  | 119 | 0 |  | 143 | 1 |

Table 13: First 144 DATA bits

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 720 | 0 |  | 744 | 0 |  | 768 | 0 |  | 792 | 0 |  | 816 | 0 |  | 840 | 0 |
| 721 | 0 |  | 745 | 0 |  | 769 | 1 |  | 793 | 1 |  | 817 | 0 |  | 841 | 0 |
| 722 | 1 |  | 746 | 1 |  | 770 | 1 |  | 794 | 0 |  | 818 | 0 |  | 842 | 0 |
| 723 | 0 |  | 747 | 0 |  | 771 | 0 |  | 795 | 1 |  | 819 | 0 |  | 843 | 0 |
| 724 | 0 |  | 748 | 0 |  | 772 | 0 |  | 796 | 0 |  | 820 | 0 |  | 844 | 0 |
| 725 | 0 |  | 749 | 0 |  | 773 | 1 |  | 797 | 1 |  | 821 | 0 |  | 845 | 0 |
| 726 | 0 |  | 750 | 0 |  | 774 | 0 |  | 798 | 1 |  | 822 | 0 |  | 846 | 0 |
| 727 | 0 |  | 751 | 0 |  | 775 | 1 |  | 799 | 1 |  | 823 | 0 |  | 847 | 0 |
| 728 | 0 |  | 752 | 0 |  | 776 | 0 |  | 800 | 1 |  | 824 | 0 |  | 848 | 0 |
| 729 | 1 |  | 753 | 1 |  | 777 | 1 |  | 801 | 0 |  | 825 | 0 |  | 849 | 0 |
| 730 | 1 |  | 754 | 1 |  | 778 | 1 |  | 802 | 0 |  | 826 | 0 |  | 850 | 0 |
| 731 | 1 |  | 755 | 1 |  | 779 | 0 |  | 803 | 1 |  | 827 | 0 |  | 851 | 0 |
| 732 | 0 |  | 756 | 0 |  | 780 | 0 |  | 804 | 1 |  | 828 | 0 |  | 852 | 0 |
| 733 | 1 |  | 757 | 1 |  | 781 | 0 |  | 805 | 0 |  | 829 | 0 |  | 853 | 0 |
| 734 | 1 |  | 758 | 0 |  | 782 | 0 |  | 806 | 0 |  | 830 | 0 |  | 854 | 0 |
| 735 | 1 |  | 759 | 0 |  | 783 | 1 |  | 807 | 1 |  | 831 | 0 |  | 855 | 0 |
| 736 | 0 |  | 760 | 0 |  | 784 | 1 |  | 808 | 1 |  | 832 | 0 |  | 856 | 0 |
| 737 | 1 |  | 761 | 1 |  | 785 | 1 |  | 809 | 1 |  | 833 | 0 |  | 857 | 0 |
| 738 | 1 |  | 762 | 1 |  | 786 | 0 |  | 810 | 1 |  | 834 | 0 |  | 858 | 0 |
| 739 | 0 |  | 763 | 1 |  | 787 | 1 |  | 811 | 0 |  | 835 | 0 |  | 859 | 0 |
| 740 | 0 |  | 764 | 0 |  | 788 | 1 |  | 812 | 1 |  | 836 | 0 |  | 860 | 0 |
| 741 | 1 |  | 765 | 0 |  | 789 | 0 |  | 813 | 1 |  | 837 | 0 |  | 861 | 0 |
| 742 | 0 |  | 766 | 1 |  | 790 | 1 |  | 814 | 0 |  | 838 | 0 |  | 862 | 0 |
| 743 | 1 |  | 767 | 0 |  | 791 | 0 |  | 815 | 1 |  | 839 | 0 |  | 863 | 0 |

Table 14: Last 144 DATA bits

## Scrambling

The 864 bits are scrambled by the scrambler of figure 113. The initial state of the scrambler is the state 1011101. The generated scrambling sequence is given in Table 15.

| 0 | 0 |  | 16 | 1 |  | 32 | 0 |  | 48 | 1 |  | 64 | 0 |  | 80 | 0 |  | 96 | 0 |  | 112 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  | 17 | 0 |  | 33 | 1 |  | 49 | 1 |  | 65 | 1 |  | 81 | 0 |  | 97 | 0 |  | 113 | 0 |
| 2 | 1 |  | 18 | 1 |  | 34 | 1 |  | 50 | 1 |  | 66 | 1 |  | 82 | 1 |  | 98 | 1 |  | 114 | 0 |
| 3 | 0 |  | 19 | 0 |  | 35 | 0 |  | 51 | 1 |  | 67 | 1 |  | 83 | 1 |  | 99 | 0 |  | 115 | 1 |
| 4 | 1 |  | 20 | 1 |  | 36 | 1 |  | 52 | 0 |  | 68 | 0 |  | 84 | 1 |  | 100 | 0 |  | 116 | 1 |
| 5 | 1 |  | 21 | 0 |  | 37 | 0 |  | 53 | 1 |  | 69 | 0 |  | 85 | 0 |  | 101 | 1 |  | 117 | 0 |
| 6 | 0 |  | 22 | 0 |  | 38 | 0 |  | 54 | 0 |  | 70 | 0 |  | 86 | 1 |  | 102 | 0 |  | 118 | 0 |
| 7 | 0 |  | 23 | 1 |  | 39 | 0 |  | 55 | 0 |  | 71 | 1 |  | 87 | 1 |  | 103 | 0 |  | 119 | 0 |
| 8 | 0 |  | 24 | 1 |  | 40 | 0 |  | 56 | 1 |  | 72 | 1 |  | 88 | 1 |  | 104 | 0 |  | 120 | 1 |
| 9 | 0 |  | 25 | 1 |  | 41 | 1 |  | 57 | 0 |  | 73 | 1 |  | 89 | 1 |  | 105 | 0 |  | 121 | 0 |
| 10 | 0 |  | 26 | 0 |  | 42 | 0 |  | 58 | 1 |  | 74 | 1 |  | 90 | 0 |  | 106 | 0 |  | 122 | 1 |
| 11 | 1 |  | 27 | 0 |  | 43 | 1 |  | 59 | 0 |  | 75 | 1 |  | 91 | 0 |  | 107 | 0 |  | 123 | 1 |
| 12 | 1 |  | 28 | 1 |  | 44 | 0 |  | 60 | 0 |  | 76 | 1 |  | 92 | 1 |  | 108 | 1 |  | 124 | 1 |
| 13 | 0 |  | 29 | 1 |  | 45 | 1 |  | 61 | 0 |  | 77 | 1 |  | 93 | 0 |  | 109 | 0 |  | 125 | 0 |
| 14 | 0 |  | 30 | 1 |  | 46 | 0 |  | 62 | 1 |  | 78 | 0 |  | 94 | 1 |  | 110 | 0 |  | 126 | 1 |
| 15 | 1 |  | 31 | 1 |  | 47 | 1 |  | 63 | 1 |  | 79 | 0 |  | 95 | 1 |  | 111 | 0 |  |  |  |

Table 15: Scrambling sequence for seed 1011101

After scrambling,  the bits in location 816 (i.e. the 817th bit ) to 822 (the 823rd bit) are zeroed. The first and last 144 scrambled bits are shown in Table 16 and Table 17 respectively.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 24 | 1 |  | 48 | 1 |  | 72 | 0 |  | 96 | 0 |  | 120 | 1 |
| 1 | 1 |  | 25 | 1 |  | 49 | 1 |  | 73 | 0 |  | 97 | 0 |  | 121 | 0 |
| 2 | 1 |  | 26 | 0 |  | 50 | 1 |  | 74 | 1 |  | 98 | 1 |  | 122 | 1 |
| 3 | 0 |  | 27 | 0 |  | 51 | 1 |  | 75 | 1 |  | 99 | 0 |  | 123 | 1 |
| 4 | 1 |  | 28 | 1 |  | 52 | 0 |  | 76 | 0 |  | 100 | 0 |  | 124 | 1 |
| 5 | 1 |  | 29 | 1 |  | 53 | 1 |  | 77 | 0 |  | 101 | 1 |  | 125 | 0 |
| 6 | 0 |  | 30 | 0 |  | 54 | 0 |  | 78 | 0 |  | 102 | 0 |  | 126 | 1 |
| 7 | 0 |  | 31 | 1 |  | 55 | 0 |  | 79 | 1 |  | 103 | 0 |  | 127 | 1 |
| 8 | 0 |  | 32 | 0 |  | 56 | 1 |  | 80 | 0 |  | 104 | 0 |  | 128 | 1 |
| 9 | 0 |  | 33 | 1 |  | 57 | 1 |  | 81 | 0 |  | 105 | 0 |  | 129 | 1 |
| 10 | 0 |  | 34 | 1 |  | 58 | 0 |  | 82 | 0 |  | 106 | 1 |  | 130 | 1 |
| 11 | 1 |  | 35 | 0 |  | 59 | 0 |  | 83 | 0 |  | 107 | 0 |  | 131 | 0 |
| 12 | 1 |  | 36 | 1 |  | 60 | 0 |  | 84 | 1 |  | 108 | 1 |  | 132 | 0 |
| 13 | 0 |  | 37 | 0 |  | 61 | 0 |  | 85 | 1 |  | 109 | 0 |  | 133 | 1 |
| 14 | 0 |  | 38 | 0 |  | 62 | 1 |  | 86 | 0 |  | 110 | 0 |  | 134 | 0 |
| 15 | 1 |  | 39 | 0 |  | 63 | 1 |  | 87 | 0 |  | 111 | 0 |  | 135 | 0 |
| 16 | 1 |  | 40 | 0 |  | 64 | 0 |  | 88 | 0 |  | 112 | 0 |  | 136 | 1 |
| 17 | 0 |  | 41 | 1 |  | 65 | 1 |  | 89 | 1 |  | 113 | 1 |  | 137 | 1 |
| 18 | 1 |  | 42 | 1 |  | 66 | 1 |  | 90 | 1 |  | 114 | 0 |  | 138 | 0 |
| 19 | 0 |  | 43 | 1 |  | 67 | 1 |  | 91 | 0 |  | 115 | 0 |  | 139 | 0 |
| 20 | 1 |  | 44 | 1 |  | 68 | 1 |  | 92 | 1 |  | 116 | 1 |  | 140 | 0 |
| 21 | 1 |  | 45 | 0 |  | 69 | 0 |  | 93 | 1 |  | 117 | 1 |  | 141 | 0 |
| 22 | 0 |  | 46 | 1 |  | 70 | 0 |  | 94 | 0 |  | 118 | 1 |  | 142 | 1 |
| 23 | 1 |  | 47 | 1 |  | 71 | 1 |  | 95 | 1 |  | 119 | 0 |  | 143 | 0 |

Table 16: First 144 bits after scrambling

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 720 | 0 |  | 744 | 0 |  | 768 | 0 |  | 792 | 1 |  | 816 | 0 |  | 840 | 0 |
| 721 | 1 |  | 745 | 0 |  | 769 | 1 |  | 793 | 0 |  | 817 | 0 |  | 841 | 0 |
| 722 | 0 |  | 746 | 1 |  | 770 | 1 |  | 794 | 0 |  | 818 | 0 |  | 842 | 0 |
| 723 | 1 |  | 747 | 1 |  | 771 | 0 |  | 795 | 0 |  | 819 | 0 |  | 843 | 0 |
| 724 | 1 |  | 748 | 0 |  | 772 | 0 |  | 796 | 1 |  | 820 | 0 |  | 844 | 1 |
| 725 | 0 |  | 749 | 0 |  | 773 | 0 |  | 797 | 1 |  | 821 | 0 |  | 845 | 1 |
| 726 | 0 |  | 750 | 1 |  | 774 | 1 |  | 798 | 0 |  | 822 | 0 |  | 846 | 1 |
| 727 | 1 |  | 751 | 1 |  | 775 | 1 |  | 799 | 1 |  | 823 | 0 |  | 847 | 0 |
| 728 | 0 |  | 752 | 0 |  | 776 | 0 |  | 800 | 1 |  | 824 | 1 |  | 848 | 1 |
| 729 | 0 |  | 753 | 1 |  | 777 | 0 |  | 801 | 0 |  | 825 | 1 |  | 849 | 1 |
| 730 | 0 |  | 754 | 1 |  | 778 | 0 |  | 802 | 0 |  | 826 | 0 |  | 850 | 1 |
| 731 | 1 |  | 755 | 0 |  | 779 | 0 |  | 803 | 0 |  | 827 | 1 |  | 851 | 1 |
| 732 | 0 |  | 756 | 0 |  | 780 | 1 |  | 804 | 1 |  | 828 | 1 |  | 852 | 0 |
| 733 | 0 |  | 757 | 0 |  | 781 | 0 |  | 805 | 1 |  | 829 | 1 |  | 853 | 0 |
| 734 | 1 |  | 758 | 1 |  | 782 | 1 |  | 806 | 0 |  | 830 | 0 |  | 854 | 1 |
| 735 | 1 |  | 759 | 1 |  | 783 | 1 |  | 807 | 0 |  | 831 | 0 |  | 855 | 0 |
| 736 | 1 |  | 760 | 0 |  | 784 | 1 |  | 808 | 1 |  | 832 | 0 |  | 856 | 1 |
| 737 | 1 |  | 761 | 0 |  | 785 | 0 |  | 809 | 0 |  | 833 | 1 |  | 857 | 1 |
| 738 | 1 |  | 762 | 1 |  | 786 | 1 |  | 810 | 0 |  | 834 | 1 |  | 858 | 0 |
| 739 | 0 |  | 763 | 0 |  | 787 | 0 |  | 811 | 1 |  | 835 | 1 |  | 859 | 0 |
| 740 | 0 |  | 764 | 1 |  | 788 | 1 |  | 812 | 0 |  | 836 | 1 |  | 860 | 1 |
| 741 | 1 |  | 765 | 0 |  | 789 | 0 |  | 813 | 0 |  | 837 | 1 |  | 861 | 0 |
| 742 | 0 |  | 766 | 0 |  | 790 | 0 |  | 814 | 0 |  | 838 | 1 |  | 862 | 0 |
| 743 | 0 |  | 767 | 1 |  | 791 | 1 |  | 815 | 0 |  | 839 | 1 |  | 863 | 1 |

Table 17: Last 144 bits after scrambling

# Generating the first DATA symbol

## Coding the DATA bits

The scrambled bits are coded with a rate ¾ convolutional code. The first 144 scrambled bits of Table 16 are mapped into the 192 bits of Table 18.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 32 | 1 |  | 64 | 0 |  | 96 | 0 |  | 128 | 0 |  | 160 | 1 |
| 1 | 0 |  | 33 | 0 |  | 65 | 0 |  | 97 | 0 |  | 129 | 1 |  | 161 | 1 |
| 2 | 1 |  | 34 | 1 |  | 66 | 1 |  | 98 | 1 |  | 130 | 0 |  | 162 | 0 |
| 3 | 0 |  | 35 | 0 |  | 67 | 1 |  | 99 | 1 |  | 131 | 1 |  | 163 | 1 |
| 4 | 1 |  | 36 | 1 |  | 68 | 0 |  | 100 | 1 |  | 132 | 1 |  | 164 | 0 |
| 5 | 0 |  | 37 | 1 |  | 69 | 0 |  | 101 | 0 |  | 133 | 0 |  | 165 | 0 |
| 6 | 1 |  | 38 | 1 |  | 70 | 0 |  | 102 | 0 |  | 134 | 0 |  | 166 | 1 |
| 7 | 1 |  | 39 | 1 |  | 71 | 0 |  | 103 | 1 |  | 135 | 1 |  | 167 | 1 |
| 8 | 0 |  | 40 | 1 |  | 72 | 1 |  | 104 | 1 |  | 136 | 0 |  | 168 | 0 |
| 9 | 0 |  | 41 | 1 |  | 73 | 1 |  | 105 | 1 |  | 137 | 1 |  | 169 | 0 |
| 10 | 0 |  | 42 | 0 |  | 74 | 1 |  | 106 | 0 |  | 138 | 0 |  | 170 | 1 |
| 11 | 0 |  | 43 | 0 |  | 75 | 1 |  | 107 | 0 |  | 139 | 0 |  | 171 | 0 |
| 12 | 1 |  | 44 | 1 |  | 76 | 0 |  | 108 | 0 |  | 140 | 0 |  | 172 | 1 |
| 13 | 0 |  | 45 | 0 |  | 77 | 1 |  | 109 | 0 |  | 141 | 0 |  | 173 | 1 |
| 14 | 0 |  | 46 | 0 |  | 78 | 0 |  | 110 | 1 |  | 142 | 0 |  | 174 | 0 |
| 15 | 0 |  | 47 | 1 |  | 79 | 1 |  | 111 | 0 |  | 143 | 0 |  | 175 | 1 |
| 16 | 1 |  | 48 | 0 |  | 80 | 1 |  | 112 | 0 |  | 144 | 0 |  | 176 | 0 |
| 17 | 0 |  | 49 | 1 |  | 81 | 1 |  | 113 | 1 |  | 145 | 0 |  | 177 | 1 |
| 18 | 1 |  | 50 | 0 |  | 82 | 1 |  | 114 | 0 |  | 146 | 1 |  | 178 | 0 |
| 19 | 0 |  | 51 | 1 |  | 83 | 0 |  | 115 | 0 |  | 147 | 1 |  | 179 | 0 |
| 20 | 0 |  | 52 | 1 |  | 84 | 0 |  | 116 | 0 |  | 148 | 0 |  | 180 | 1 |
| 21 | 0 |  | 53 | 0 |  | 85 | 1 |  | 117 | 0 |  | 149 | 1 |  | 181 | 0 |
| 22 | 0 |  | 54 | 1 |  | 86 | 1 |  | 118 | 1 |  | 150 | 1 |  | 182 | 1 |
| 23 | 1 |  | 55 | 1 |  | 87 | 1 |  | 119 | 1 |  | 151 | 1 |  | 183 | 0 |
| 24 | 0 |  | 56 | 0 |  | 88 | 0 |  | 120 | 1 |  | 152 | 1 |  | 184 | 0 |
| 25 | 0 |  | 57 | 1 |  | 89 | 1 |  | 121 | 1 |  | 153 | 0 |  | 185 | 0 |
| 26 | 1 |  | 58 | 0 |  | 90 | 1 |  | 122 | 0 |  | 154 | 1 |  | 186 | 1 |
| 27 | 0 |  | 59 | 0 |  | 91 | 1 |  | 123 | 1 |  | 155 | 0 |  | 187 | 1 |
| 28 | 0 |  | 60 | 0 |  | 92 | 1 |  | 124 | 0 |  | 156 | 1 |  | 188 | 1 |
| 29 | 0 |  | 61 | 1 |  | 93 | 0 |  | 125 | 1 |  | 157 | 0 |  | 189 | 0 |
| 30 | 0 |  | 62 | 0 |  | 94 | 1 |  | 126 | 0 |  | 158 | 1 |  | 190 | 1 |
| 31 | 1 |  | 63 | 0 |  | 95 | 1 |  | 127 | 0 |  | 159 | 0 |  | 191 | 0 |

Table 18: Coded bits of first DATA symbol

## Interleaving the DATA bits

The interleaver is defined as a two permutation process. We shall denote by k the index of the coded bit before the first permutation, i shall be the index after the first and before the second permutation and j shall be the index after the second permutation, just prior to modulation mapping. The mapping from k to i is shown in Table 19and the mapping from i, to j is shown in Table 20.

| k | i |  | k | i |  | k | i |  | k | i |  | k | i |  | k | i |  | k | i |  | k | i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 24 | 97 |  | 48 | 3 |  | 72 | 100 |  | 96 | 6 |  | 120 | 103 |  | 144 | 9 |  | 168 | 106 |
| 1 | 12 |  | 25 | 109 |  | 49 | 15 |  | 73 | 112 |  | 97 | 18 |  | 121 | 115 |  | 145 | 21 |  | 169 | 118 |
| 2 | 24 |  | 26 | 121 |  | 50 | 27 |  | 74 | 124 |  | 98 | 30 |  | 122 | 127 |  | 146 | 33 |  | 170 | 130 |
| 3 | 36 |  | 27 | 133 |  | 51 | 39 |  | 75 | 136 |  | 99 | 42 |  | 123 | 139 |  | 147 | 45 |  | 171 | 142 |
| 4 | 48 |  | 28 | 145 |  | 52 | 51 |  | 76 | 148 |  | 100 | 54 |  | 124 | 151 |  | 148 | 57 |  | 172 | 154 |
| 5 | 60 |  | 29 | 157 |  | 53 | 63 |  | 77 | 160 |  | 101 | 66 |  | 125 | 163 |  | 149 | 69 |  | 173 | 166 |
| 6 | 72 |  | 30 | 169 |  | 54 | 75 |  | 78 | 172 |  | 102 | 78 |  | 126 | 175 |  | 150 | 81 |  | 174 | 178 |
| 7 | 84 |  | 31 | 181 |  | 55 | 87 |  | 79 | 184 |  | 103 | 90 |  | 127 | 187 |  | 151 | 93 |  | 175 | 190 |
| 8 | 96 |  | 32 | 2 |  | 56 | 99 |  | 80 | 5 |  | 104 | 102 |  | 128 | 8 |  | 152 | 105 |  | 176 | 11 |
| 9 | 108 |  | 33 | 14 |  | 57 | 111 |  | 81 | 17 |  | 105 | 114 |  | 129 | 20 |  | 153 | 117 |  | 177 | 23 |
| 10 | 120 |  | 34 | 26 |  | 58 | 123 |  | 82 | 29 |  | 106 | 126 |  | 130 | 32 |  | 154 | 129 |  | 178 | 35 |
| 11 | 132 |  | 35 | 38 |  | 59 | 135 |  | 83 | 41 |  | 107 | 138 |  | 131 | 44 |  | 155 | 141 |  | 179 | 47 |
| 12 | 144 |  | 36 | 50 |  | 60 | 147 |  | 84 | 53 |  | 108 | 150 |  | 132 | 56 |  | 156 | 153 |  | 180 | 59 |
| 13 | 156 |  | 37 | 62 |  | 61 | 159 |  | 85 | 65 |  | 109 | 162 |  | 133 | 68 |  | 157 | 165 |  | 181 | 71 |
| 14 | 168 |  | 38 | 74 |  | 62 | 171 |  | 86 | 77 |  | 110 | 174 |  | 134 | 80 |  | 158 | 177 |  | 182 | 83 |
| 15 | 180 |  | 39 | 86 |  | 63 | 183 |  | 87 | 89 |  | 111 | 186 |  | 135 | 92 |  | 159 | 189 |  | 183 | 95 |
| 16 | 1 |  | 40 | 98 |  | 64 | 4 |  | 88 | 101 |  | 112 | 7 |  | 136 | 104 |  | 160 | 10 |  | 184 | 107 |
| 17 | 13 |  | 41 | 110 |  | 65 | 16 |  | 89 | 113 |  | 113 | 19 |  | 137 | 116 |  | 161 | 22 |  | 185 | 119 |
| 18 | 25 |  | 42 | 122 |  | 66 | 28 |  | 90 | 125 |  | 114 | 31 |  | 138 | 128 |  | 162 | 34 |  | 186 | 131 |
| 19 | 37 |  | 43 | 134 |  | 67 | 40 |  | 91 | 137 |  | 115 | 43 |  | 139 | 140 |  | 163 | 46 |  | 187 | 143 |
| 20 | 49 |  | 44 | 146 |  | 68 | 52 |  | 92 | 149 |  | 116 | 55 |  | 140 | 152 |  | 164 | 58 |  | 188 | 155 |
| 21 | 61 |  | 45 | 158 |  | 69 | 64 |  | 93 | 161 |  | 117 | 67 |  | 141 | 164 |  | 165 | 70 |  | 189 | 167 |
| 22 | 73 |  | 46 | 170 |  | 70 | 76 |  | 94 | 173 |  | 118 | 79 |  | 142 | 176 |  | 166 | 82 |  | 190 | 179 |
| 23 | 85 |  | 47 | 182 |  | 71 | 88 |  | 95 | 185 |  | 119 | 91 |  | 143 | 188 |  | 167 | 94 |  | 191 | 191 |

Table 19: first permutation

| i | j |  | i | j |  | i | j |  | i | j |  | i | j |  | i | j |  | i | j |  | i | j |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 24 | 24 |  | 48 | 48 |  | 72 | 72 |  | 96 | 96 |  | 120 | 120 |  | 144 | 144 |  | 168 | 168 |
| 1 | 1 |  | 25 | 25 |  | 49 | 49 |  | 73 | 73 |  | 97 | 97 |  | 121 | 121 |  | 145 | 145 |  | 169 | 169 |
| 2 | 2 |  | 26 | 26 |  | 50 | 50 |  | 74 | 74 |  | 98 | 98 |  | 122 | 122 |  | 146 | 146 |  | 170 | 170 |
| 3 | 3 |  | 27 | 27 |  | 51 | 51 |  | 75 | 75 |  | 99 | 99 |  | 123 | 123 |  | 147 | 147 |  | 171 | 171 |
| 4 | 4 |  | 28 | 28 |  | 52 | 52 |  | 76 | 76 |  | 100 | 100 |  | 124 | 124 |  | 148 | 148 |  | 172 | 172 |
| 5 | 5 |  | 29 | 29 |  | 53 | 53 |  | 77 | 77 |  | 101 | 101 |  | 125 | 125 |  | 149 | 149 |  | 173 | 173 |
| 6 | 6 |  | 30 | 30 |  | 54 | 54 |  | 78 | 78 |  | 102 | 102 |  | 126 | 126 |  | 150 | 150 |  | 174 | 174 |
| 7 | 7 |  | 31 | 31 |  | 55 | 55 |  | 79 | 79 |  | 103 | 103 |  | 127 | 127 |  | 151 | 151 |  | 175 | 175 |
| 8 | 8 |  | 32 | 32 |  | 56 | 56 |  | 80 | 80 |  | 104 | 104 |  | 128 | 128 |  | 152 | 152 |  | 176 | 176 |
| 9 | 9 |  | 33 | 33 |  | 57 | 57 |  | 81 | 81 |  | 105 | 105 |  | 129 | 129 |  | 153 | 153 |  | 177 | 177 |
| 10 | 10 |  | 34 | 34 |  | 58 | 58 |  | 82 | 82 |  | 106 | 106 |  | 130 | 130 |  | 154 | 154 |  | 178 | 178 |
| 11 | 11 |  | 35 | 35 |  | 59 | 59 |  | 83 | 83 |  | 107 | 107 |  | 131 | 131 |  | 155 | 155 |  | 179 | 179 |
| 12 | 13 |  | 36 | 37 |  | 60 | 61 |  | 84 | 85 |  | 108 | 109 |  | 132 | 133 |  | 156 | 157 |  | 180 | 181 |
| 13 | 12 |  | 37 | 36 |  | 61 | 60 |  | 85 | 84 |  | 109 | 108 |  | 133 | 132 |  | 157 | 156 |  | 181 | 180 |
| 14 | 15 |  | 38 | 39 |  | 62 | 63 |  | 86 | 87 |  | 110 | 111 |  | 134 | 135 |  | 158 | 159 |  | 182 | 183 |
| 15 | 14 |  | 39 | 38 |  | 63 | 62 |  | 87 | 86 |  | 111 | 110 |  | 135 | 134 |  | 159 | 158 |  | 183 | 182 |
| 16 | 17 |  | 40 | 41 |  | 64 | 65 |  | 88 | 89 |  | 112 | 113 |  | 136 | 137 |  | 160 | 161 |  | 184 | 185 |
| 17 | 16 |  | 41 | 40 |  | 65 | 64 |  | 89 | 88 |  | 113 | 112 |  | 137 | 136 |  | 161 | 160 |  | 185 | 184 |
| 18 | 19 |  | 42 | 43 |  | 66 | 67 |  | 90 | 91 |  | 114 | 115 |  | 138 | 139 |  | 162 | 163 |  | 186 | 187 |
| 19 | 18 |  | 43 | 42 |  | 67 | 66 |  | 91 | 90 |  | 115 | 114 |  | 139 | 138 |  | 163 | 162 |  | 187 | 186 |
| 20 | 21 |  | 44 | 45 |  | 68 | 69 |  | 92 | 93 |  | 116 | 117 |  | 140 | 141 |  | 164 | 165 |  | 188 | 189 |
| 21 | 20 |  | 45 | 44 |  | 69 | 68 |  | 93 | 92 |  | 117 | 116 |  | 141 | 140 |  | 165 | 164 |  | 189 | 188 |
| 22 | 23 |  | 46 | 47 |  | 70 | 71 |  | 94 | 95 |  | 118 | 119 |  | 142 | 143 |  | 166 | 167 |  | 190 | 191 |
| 23 | 22 |  | 47 | 46 |  | 71 | 70 |  | 95 | 94 |  | 119 | 118 |  | 143 | 142 |  | 167 | 166 |  | 191 | 190 |

Table 20: second permutation

As a specific example consider the case of  k=17 ( the 18th bit after encoding and puncturing). It is mapped by first permutation to i=13 and by the second permutation to j=12 (the 13th bit before mapping).

The interleaved bits are shown in Table 21.

| ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |  | ## | Bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  | 32 | 0 |  | 64 | 1 |  | 96 | 0 |  | 128 | 0 |  | 160 | 0 |
| 1 | 1 |  | 33 | 1 |  | 65 | 0 |  | 97 | 0 |  | 129 | 1 |  | 161 | 1 |
| 2 | 1 |  | 34 | 0 |  | 66 | 0 |  | 98 | 1 |  | 130 | 1 |  | 162 | 1 |
| 3 | 0 |  | 35 | 0 |  | 67 | 0 |  | 99 | 0 |  | 131 | 1 |  | 163 | 0 |
| 4 | 0 |  | 36 | 0 |  | 68 | 1 |  | 100 | 1 |  | 132 | 0 |  | 164 | 0 |
| 5 | 1 |  | 37 | 0 |  | 69 | 0 |  | 101 | 0 |  | 133 | 0 |  | 165 | 0 |
| 6 | 0 |  | 38 | 1 |  | 70 | 0 |  | 102 | 1 |  | 134 | 0 |  | 166 | 0 |
| 7 | 0 |  | 39 | 0 |  | 71 | 0 |  | 103 | 1 |  | 135 | 0 |  | 167 | 1 |
| 8 | 0 |  | 40 | 0 |  | 72 | 1 |  | 104 | 0 |  | 136 | 1 |  | 168 | 0 |
| 9 | 0 |  | 41 | 1 |  | 73 | 0 |  | 105 | 1 |  | 137 | 1 |  | 169 | 0 |
| 10 | 1 |  | 42 | 0 |  | 74 | 1 |  | 106 | 0 |  | 138 | 1 |  | 170 | 0 |
| 11 | 0 |  | 43 | 1 |  | 75 | 1 |  | 107 | 0 |  | 139 | 0 |  | 171 | 0 |
| 12 | 0 |  | 44 | 1 |  | 76 | 0 |  | 108 | 0 |  | 140 | 0 |  | 172 | 0 |
| 13 | 0 |  | 45 | 1 |  | 77 | 1 |  | 109 | 0 |  | 141 | 0 |  | 173 | 1 |
| 14 | 1 |  | 46 | 0 |  | 78 | 0 |  | 110 | 1 |  | 142 | 1 |  | 174 | 1 |
| 15 | 0 |  | 47 | 1 |  | 79 | 1 |  | 111 | 1 |  | 143 | 0 |  | 175 | 0 |
| 16 | 1 |  | 48 | 1 |  | 80 | 0 |  | 112 | 1 |  | 144 | 1 |  | 176 | 0 |
| 17 | 0 |  | 49 | 0 |  | 81 | 1 |  | 113 | 1 |  | 145 | 0 |  | 177 | 1 |
| 18 | 1 |  | 50 | 1 |  | 82 | 1 |  | 114 | 1 |  | 146 | 1 |  | 178 | 0 |
| 19 | 0 |  | 51 | 1 |  | 83 | 1 |  | 115 | 1 |  | 147 | 0 |  | 179 | 1 |
| 20 | 0 |  | 52 | 0 |  | 84 | 1 |  | 116 | 0 |  | 148 | 0 |  | 180 | 1 |
| 21 | 1 |  | 53 | 0 |  | 85 | 1 |  | 117 | 1 |  | 149 | 1 |  | 181 | 0 |
| 22 | 1 |  | 54 | 1 |  | 86 | 1 |  | 118 | 0 |  | 150 | 0 |  | 182 | 0 |
| 23 | 1 |  | 55 | 0 |  | 87 | 1 |  | 119 | 0 |  | 151 | 0 |  | 183 | 1 |
| 24 | 1 |  | 56 | 1 |  | 88 | 1 |  | 120 | 0 |  | 152 | 0 |  | 184 | 1 |
| 25 | 1 |  | 57 | 0 |  | 89 | 0 |  | 121 | 1 |  | 153 | 1 |  | 185 | 1 |
| 26 | 1 |  | 58 | 0 |  | 90 | 1 |  | 122 | 0 |  | 154 | 1 |  | 186 | 0 |
| 27 | 0 |  | 59 | 1 |  | 91 | 1 |  | 123 | 0 |  | 155 | 1 |  | 187 | 0 |
| 28 | 1 |  | 60 | 0 |  | 92 | 1 |  | 124 | 1 |  | 156 | 0 |  | 188 | 0 |
| 29 | 1 |  | 61 | 0 |  | 93 | 1 |  | 125 | 1 |  | 157 | 0 |  | 189 | 0 |
| 30 | 1 |  | 62 | 0 |  | 94 | 0 |  | 126 | 0 |  | 158 | 1 |  | 190 | 0 |
| 31 | 0 |  | 63 | 1 |  | 95 | 1 |  | 127 | 0 |  | 159 | 0 |  | 191 | 1 |

Table 21: Interleaved bits of first data symbol

## Mapping into symbols

The frequency domain symbols are generated by grouping 4 coded bits and mapping into complex QAM16 symbols according to Table 84. For instance, the first 4 bits (0 1 1 0 ) are mapped to the complex value -0.316+0.949j, inserted at subcarrierr –26.

Four pilots subcarriers are added by taking the values {1.0,1.0,1.0,-1.0}, multiplied them by the second element of sequence p, given in equation (25) and inserted into location {-21,-7,7,21} respectively

The frequency domain is shown in Table 22.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0.000 | 0.000 |  | -16 | -0.949 | 0.949 |  | 0 | 0.000 | 0.000 |  | 16 | -0.316 | 0.316 |
| -31 | 0.000 | 0.000 |  | -15 | -0.316 | -0.316 |  | 1 | -0.949 | 0.949 |  | 17 | -0.949 | 0.949 |
| -30 | 0.000 | 0.000 |  | -14 | 0.316 | -0.316 |  | 2 | 0.949 | 0.316 |  | 18 | -0.316 | 0.949 |
| -29 | 0.000 | 0.000 |  | -13 | 0.949 | 0.316 |  | 3 | -0.316 | -0.949 |  | 19 | -0.949 | -0.316 |
| -28 | 0.000 | 0.000 |  | -12 | -0.949 | 0.949 |  | 4 | -0.949 | 0.316 |  | 20 | -0.949 | -0.949 |
| -27 | 0.000 | 0.000 |  | -11 | 0.949 | -0.316 |  | 5 | 0.316 | 0.316 |  | 21 | -1.000 | 0.000 |
| -26 | -0.316 | 0.949 |  | -10 | -0.949 | -0.316 |  | 6 | -0.316 | -0.949 |  | 22 | -0.316 | 0.949 |
| -25 | -0.316 | -0.949 |  | -9 | 0.949 | -0.949 |  | 7 | 1.000 | 0.000 |  | 23 | -0.316 | -0.316 |
| -24 | -0.949 | 0.949 |  | -8 | 0.949 | -0.949 |  | 8 | -0.316 | -0.949 |  | 24 | 0.949 | -0.316 |
| -23 | -0.949 | 0.949 |  | -7 | 1.000 | 0.000 |  | 9 | 0.316 | -0.949 |  | 25 | 0.316 | -0.949 |
| -22 | 0.949 | 0.949 |  | -6 | 0.949 | 0.316 |  | 10 | -0.316 | 0.316 |  | 26 | -0.949 | -0.316 |
| -21 | 1.000 | 0.000 |  | -5 | -0.316 | -0.316 |  | 11 | -0.949 | -0.949 |  | 27 | 0.000 | 0.000 |
| -20 | -0.316 | 0.316 |  | -4 | -0.316 | 0.316 |  | 12 | 0.316 | 0.949 |  | 28 | 0.000 | 0.000 |
| -19 | 0.316 | 0.949 |  | -3 | 0.316 | 0.316 |  | 13 | -0.949 | 0.949 |  | 29 | 0.000 | 0.000 |
| -18 | 0.316 | 0.949 |  | -2 | 0.949 | 0.316 |  | 14 | 0.949 | 0.949 |  | 30 | 0.000 | 0.000 |
| -17 | -0.316 | -0.949 |  | -1 | 0.316 | -0.316 |  | 15 | -0.316 | -0.949 |  | 31 | 0.000 | 0.000 |

Table 22: Frequency domain of first data symbol

The time domain samples are produced by performing IFFT, cyclically extending and multiplying with the window function in the same manner as described in section 3.5. The time domain samples are appended with one sample overlap to the SIGNAL field symbol.

# Generating the additional DATA symbols

The generation of the additional 5 data symbols follows the same step as in section 5. A special attention should be paid to the scrambling of the pilot subcarriers. Table 23 lists the polarity of the pilots subcarriers and the elements of the sequence p0…126 for the DATA symbols. For completeness, the pilots in the SIGNAL are included as well.

| i | OFDM symbol | element of pi | pilot at #-21 | pilot at #-7 | pilot at #7 | pilot at #21 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | SIGNAL | 1 | 1.0 +0j | 1.0 +0j | 1.0 +0j | -1.0 +0j |
| 1 | DATA 1 | 1 | 1.0 +0j | 1.0 +0j | 1.0 +0j | -1.0 +0j |
| 2 | DATA 2 | 1 | 1.0 +0j | 1.0 +0j | 1.0 +0j | -1.0 +0j |
| 3 | DATA 3 | 1 | 1.0 +0j | 1.0 +0j | 1.0 +0j | -1.0 +0j |
| 4 | DATA 4 | -1 | -1.0 +0j | -1.0 +0j | -1.0 +0j | 1.0 +0j |
| 5 | DATA 5 | -1 | -1.0 +0j | -1.0 +0j | -1.0 +0j | 1.0 +0j |
| 6 | DATA 6 | -1 | -1.0 +0j | -1.0 +0j | -1.0 +0j | 1.0 +0j |

Table 23: Polarity of the pilot subcarriers

The symbols are appended one after the other with a one sample overlap.

# The entire packet

The packet in its entirety is shown in Table 1. The short sequences section, the long sequences section, the SIGNAL field and the DATA symbols are separated by thick lines.

| ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |  | ### | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.022 | 0.022 |  | 40 | 0.133 | 0.044 |  | 80 | 0.044 | 0.044 |  | 120 | 0.133 | 0.044 |
| 1 | -0.109 | -0.045 |  | 41 | -0.042 | 0.071 |  | 81 | -0.109 | -0.045 |  | 121 | -0.042 | 0.071 |
| 2 | -0.044 | 0.062 |  | 42 | -0.044 | -0.062 |  | 82 | -0.044 | 0.062 |  | 122 | -0.044 | -0.062 |
| 3 | 0.013 | 0.126 |  | 43 | 0.013 | -0.152 |  | 83 | 0.013 | 0.126 |  | 123 | 0.013 | -0.152 |
| 4 | -0.133 | -0.044 |  | 44 | -0.044 | -0.044 |  | 84 | -0.133 | -0.044 |  | 124 | -0.044 | -0.044 |
| 5 | -0.095 | 0.052 |  | 45 | 0.069 | 0.099 |  | 85 | -0.095 | 0.052 |  | 125 | 0.069 | 0.099 |
| 6 | -0.018 | 0.062 |  | 46 | 0.107 | -0.062 |  | 86 | -0.018 | 0.062 |  | 126 | 0.107 | -0.062 |
| 7 | 0.075 | -0.066 |  | 47 | 0.075 | -0.085 |  | 87 | 0.075 | -0.066 |  | 127 | 0.075 | -0.085 |
| 8 | 0.133 | 0.044 |  | 48 | 0.044 | 0.044 |  | 88 | 0.133 | 0.044 |  | 128 | 0.044 | 0.044 |
| 9 | -0.042 | 0.071 |  | 49 | -0.109 | -0.045 |  | 89 | -0.042 | 0.071 |  | 129 | -0.109 | -0.045 |
| 10 | -0.044 | -0.062 |  | 50 | -0.044 | 0.062 |  | 90 | -0.044 | -0.062 |  | 130 | -0.044 | 0.062 |
| 11 | 0.013 | -0.152 |  | 51 | 0.013 | 0.126 |  | 91 | 0.013 | -0.152 |  | 131 | 0.013 | 0.126 |
| 12 | -0.044 | -0.044 |  | 52 | -0.133 | -0.044 |  | 92 | -0.044 | -0.044 |  | 132 | -0.133 | -0.044 |
| 13 | 0.069 | 0.099 |  | 53 | -0.095 | 0.052 |  | 93 | 0.069 | 0.099 |  | 133 | -0.095 | 0.052 |
| 14 | 0.107 | -0.062 |  | 54 | -0.018 | 0.062 |  | 94 | 0.107 | -0.062 |  | 134 | -0.018 | 0.062 |
| 15 | 0.075 | -0.085 |  | 55 | 0.075 | -0.066 |  | 95 | 0.075 | -0.085 |  | 135 | 0.075 | -0.066 |
| 16 | 0.044 | 0.044 |  | 56 | 0.133 | 0.044 |  | 96 | 0.044 | 0.044 |  | 136 | 0.133 | 0.044 |
| 17 | -0.109 | -0.045 |  | 57 | -0.042 | 0.071 |  | 97 | -0.109 | -0.045 |  | 137 | -0.042 | 0.071 |
| 18 | -0.044 | 0.062 |  | 58 | -0.044 | -0.062 |  | 98 | -0.044 | 0.062 |  | 138 | -0.044 | -0.062 |
| 19 | 0.013 | 0.126 |  | 59 | 0.013 | -0.152 |  | 99 | 0.013 | 0.126 |  | 139 | 0.013 | -0.152 |
| 20 | -0.133 | -0.044 |  | 60 | -0.044 | -0.044 |  | 100 | -0.133 | -0.044 |  | 140 | -0.044 | -0.044 |
| 21 | -0.095 | 0.052 |  | 61 | 0.069 | 0.099 |  | 101 | -0.095 | 0.052 |  | 141 | 0.069 | 0.099 |
| 22 | -0.018 | 0.062 |  | 62 | 0.107 | -0.062 |  | 102 | -0.018 | 0.062 |  | 142 | 0.107 | -0.062 |
| 23 | 0.075 | -0.066 |  | 63 | 0.075 | -0.085 |  | 103 | 0.075 | -0.066 |  | 143 | 0.075 | -0.085 |
| 24 | 0.133 | 0.044 |  | 64 | 0.044 | 0.044 |  | 104 | 0.133 | 0.044 |  | 144 | 0.044 | 0.044 |
| 25 | -0.042 | 0.071 |  | 65 | -0.109 | -0.045 |  | 105 | -0.042 | 0.071 |  | 145 | -0.109 | -0.045 |
| 26 | -0.044 | -0.062 |  | 66 | -0.044 | 0.062 |  | 106 | -0.044 | -0.062 |  | 146 | -0.044 | 0.062 |
| 27 | 0.013 | -0.152 |  | 67 | 0.013 | 0.126 |  | 107 | 0.013 | -0.152 |  | 147 | 0.013 | 0.126 |
| 28 | -0.044 | -0.044 |  | 68 | -0.133 | -0.044 |  | 108 | -0.044 | -0.044 |  | 148 | -0.133 | -0.044 |
| 29 | 0.069 | 0.099 |  | 69 | -0.095 | 0.052 |  | 109 | 0.069 | 0.099 |  | 149 | -0.095 | 0.052 |
| 30 | 0.107 | -0.062 |  | 70 | -0.018 | 0.062 |  | 110 | 0.107 | -0.062 |  | 150 | -0.018 | 0.062 |
| 31 | 0.075 | -0.085 |  | 71 | 0.075 | -0.066 |  | 111 | 0.075 | -0.085 |  | 151 | 0.075 | -0.066 |
| 32 | 0.044 | 0.044 |  | 72 | 0.133 | 0.044 |  | 112 | 0.044 | 0.044 |  | 152 | 0.133 | 0.044 |
| 33 | -0.109 | -0.045 |  | 73 | -0.042 | 0.071 |  | 113 | -0.109 | -0.045 |  | 153 | -0.042 | 0.071 |
| 34 | -0.044 | 0.062 |  | 74 | -0.044 | -0.062 |  | 114 | -0.044 | 0.062 |  | 154 | -0.044 | -0.062 |
| 35 | 0.013 | 0.126 |  | 75 | 0.013 | -0.152 |  | 115 | 0.013 | 0.126 |  | 155 | 0.013 | -0.152 |
| 36 | -0.133 | -0.044 |  | 76 | -0.044 | -0.044 |  | 116 | -0.133 | -0.044 |  | 156 | -0.044 | -0.044 |
| 37 | -0.095 | 0.052 |  | 77 | 0.069 | 0.099 |  | 117 | -0.095 | 0.052 |  | 157 | 0.069 | 0.099 |
| 38 | -0.018 | 0.062 |  | 78 | 0.107 | -0.062 |  | 118 | -0.018 | 0.062 |  | 158 | 0.107 | -0.062 |
| 39 | 0.075 | -0.066 |  | 79 | 0.075 | -0.085 |  | 119 | 0.075 | -0.066 |  | 159 | 0.075 | -0.085 |
| 160 | -0.056 | 0.022 |  | 200 | 0.098 | -0.026 |  | 240 | 0.062 | 0.062 |  | 280 | -0.035 | -0.151 |
| 161 | 0.012 | -0.098 |  | 201 | 0.053 | 0.004 |  | 241 | 0.119 | 0.004 |  | 281 | -0.122 | -0.017 |
| 162 | 0.092 | -0.106 |  | 202 | 0.001 | -0.115 |  | 242 | -0.022 | -0.161 |  | 282 | -0.127 | -0.021 |
| 163 | -0.092 | -0.115 |  | 203 | -0.137 | -0.047 |  | 243 | 0.059 | 0.015 |  | 283 | 0.075 | -0.074 |
| 164 | -0.003 | -0.054 |  | 204 | 0.024 | -0.059 |  | 244 | 0.024 | 0.059 |  | 284 | -0.003 | 0.054 |
| 165 | 0.075 | 0.074 |  | 205 | 0.059 | -0.015 |  | 245 | -0.137 | 0.047 |  | 285 | -0.092 | 0.115 |
| 166 | -0.127 | 0.021 |  | 206 | -0.022 | 0.161 |  | 246 | 0.001 | 0.115 |  | 286 | 0.092 | 0.106 |
| 167 | -0.122 | 0.017 |  | 207 | 0.119 | -0.004 |  | 247 | 0.053 | -0.004 |  | 287 | 0.012 | 0.098 |
| 168 | -0.035 | 0.151 |  | 208 | 0.062 | -0.062 |  | 248 | 0.098 | 0.026 |  | 288 | -0.156 | 0.000 |
| 169 | -0.056 | 0.022 |  | 209 | 0.037 | 0.098 |  | 249 | -0.038 | 0.106 |  | 289 | 0.012 | -0.098 |
| 170 | -0.060 | -0.081 |  | 210 | -0.057 | 0.039 |  | 250 | -0.115 | 0.055 |  | 290 | 0.092 | -0.106 |
| 171 | 0.070 | -0.014 |  | 211 | -0.131 | 0.065 |  | 251 | 0.060 | 0.088 |  | 291 | -0.092 | -0.115 |
| 172 | 0.082 | -0.092 |  | 212 | 0.082 | 0.092 |  | 252 | 0.021 | -0.028 |  | 292 | -0.003 | -0.054 |
| 173 | -0.131 | -0.065 |  | 213 | 0.070 | 0.014 |  | 253 | 0.097 | -0.083 |  | 293 | 0.075 | 0.074 |
| 174 | -0.057 | -0.039 |  | 214 | -0.060 | 0.081 |  | 254 | 0.040 | 0.111 |  | 294 | -0.127 | 0.021 |
| 175 | 0.037 | -0.098 |  | 215 | -0.056 | -0.022 |  | 255 | -0.005 | 0.120 |  | 295 | -0.122 | 0.017 |
| 176 | 0.062 | 0.062 |  | 216 | -0.035 | -0.151 |  | 256 | 0.156 | 0.000 |  | 296 | -0.035 | 0.151 |
| 177 | 0.119 | 0.004 |  | 217 | -0.122 | -0.017 |  | 257 | -0.005 | -0.120 |  | 297 | -0.056 | 0.022 |
| 178 | -0.022 | -0.161 |  | 218 | -0.127 | -0.021 |  | 258 | 0.040 | -0.111 |  | 298 | -0.060 | -0.081 |
| 179 | 0.059 | 0.015 |  | 219 | 0.075 | -0.074 |  | 259 | 0.097 | 0.083 |  | 299 | 0.070 | -0.014 |
| 180 | 0.024 | 0.059 |  | 220 | -0.003 | 0.054 |  | 260 | 0.021 | 0.028 |  | 300 | 0.082 | -0.092 |
| 181 | -0.137 | 0.047 |  | 221 | -0.092 | 0.115 |  | 261 | 0.060 | -0.088 |  | 301 | -0.131 | -0.065 |
| 182 | 0.001 | 0.115 |  | 222 | 0.092 | 0.106 |  | 262 | -0.115 | -0.055 |  | 302 | -0.057 | -0.039 |
| 183 | 0.053 | -0.004 |  | 223 | 0.012 | 0.098 |  | 263 | -0.038 | -0.106 |  | 303 | 0.037 | -0.098 |
| 184 | 0.098 | 0.026 |  | 224 | -0.156 | 0.000 |  | 264 | 0.098 | -0.026 |  | 304 | 0.062 | 0.062 |
| 185 | -0.038 | 0.106 |  | 225 | 0.012 | -0.098 |  | 265 | 0.053 | 0.004 |  | 305 | 0.119 | 0.004 |
| 186 | -0.115 | 0.055 |  | 226 | 0.092 | -0.106 |  | 266 | 0.001 | -0.115 |  | 306 | -0.022 | -0.161 |
| 187 | 0.060 | 0.088 |  | 227 | -0.092 | -0.115 |  | 267 | -0.137 | -0.047 |  | 307 | 0.059 | 0.015 |
| 188 | 0.021 | -0.028 |  | 228 | -0.003 | -0.054 |  | 268 | 0.024 | -0.059 |  | 308 | 0.024 | 0.059 |
| 189 | 0.097 | -0.083 |  | 229 | 0.075 | 0.074 |  | 269 | 0.059 | -0.015 |  | 309 | -0.137 | 0.047 |
| 190 | 0.040 | 0.111 |  | 230 | -0.127 | 0.021 |  | 270 | -0.022 | 0.161 |  | 310 | 0.001 | 0.115 |
| 191 | -0.005 | 0.120 |  | 231 | -0.122 | 0.017 |  | 271 | 0.119 | -0.004 |  | 311 | 0.053 | -0.004 |
| 192 | 0.156 | 0.000 |  | 232 | -0.035 | 0.151 |  | 272 | 0.062 | -0.062 |  | 312 | 0.098 | 0.026 |
| 193 | -0.005 | -0.120 |  | 233 | -0.056 | 0.022 |  | 273 | 0.037 | 0.098 |  | 313 | -0.038 | 0.106 |
| 194 | 0.040 | -0.111 |  | 234 | -0.060 | -0.081 |  | 274 | -0.057 | 0.039 |  | 314 | -0.115 | 0.055 |
| 195 | 0.097 | 0.083 |  | 235 | 0.070 | -0.014 |  | 275 | -0.131 | 0.065 |  | 315 | 0.060 | 0.088 |
| 196 | 0.021 | 0.028 |  | 236 | 0.082 | -0.092 |  | 276 | 0.082 | 0.092 |  | 316 | 0.021 | -0.028 |
| 197 | 0.060 | -0.088 |  | 237 | -0.131 | -0.065 |  | 277 | 0.070 | 0.014 |  | 317 | 0.097 | -0.083 |
| 198 | -0.115 | -0.055 |  | 238 | -0.057 | -0.039 |  | 278 | -0.060 | 0.081 |  | 318 | 0.040 | 0.111 |
| 199 | -0.038 | -0.106 |  | 239 | 0.037 | -0.098 |  | 279 | -0.056 | -0.022 |  | 319 | -0.005 | 0.120 |
| 320 | 0.062 | -0.016 |  | 340 | 0.024 | -0.112 |  | 360 | -0.138 | 0.116 |  | 380 | 0.058 | -0.119 |
| 321 | 0.060 | -0.089 |  | 341 | 0.110 | -0.014 |  | 361 | -0.032 | 0.063 |  | 381 | 0.087 | 0.029 |
| 322 | -0.045 | -0.112 |  | 342 | -0.003 | -0.033 |  | 362 | 0.136 | 0.019 |  | 382 | 0.200 | -0.036 |
| 323 | -0.061 | -0.030 |  | 343 | -0.101 | -0.041 |  | 363 | -0.023 | -0.080 |  | 383 | -0.085 | -0.074 |
| 324 | -0.058 | 0.014 |  | 344 | -0.050 | -0.072 |  | 364 | -0.024 | 0.021 |  | 384 | -0.031 | -0.031 |
| 325 | 0.055 | -0.069 |  | 345 | -0.015 | -0.117 |  | 365 | -0.040 | 0.063 |  | 385 | 0.060 | -0.089 |
| 326 | 0.055 | -0.025 |  | 346 | 0.055 | 0.025 |  | 366 | 0.005 | -0.086 |  | 386 | -0.045 | -0.112 |
| 327 | -0.015 | 0.117 |  | 347 | 0.055 | 0.069 |  | 367 | 0.098 | -0.059 |  | 387 | -0.061 | -0.030 |
| 328 | -0.050 | 0.072 |  | 348 | -0.058 | -0.014 |  | 368 | 0.031 | 0.000 |  | 388 | -0.058 | 0.014 |
| 329 | -0.101 | 0.041 |  | 349 | -0.061 | 0.030 |  | 369 | 0.098 | 0.059 |  | 389 | 0.055 | -0.069 |
| 330 | -0.003 | 0.033 |  | 350 | -0.045 | 0.112 |  | 370 | 0.005 | 0.086 |  | 390 | 0.055 | -0.025 |
| 331 | 0.110 | 0.014 |  | 351 | 0.060 | 0.089 |  | 371 | -0.040 | -0.063 |  | 391 | -0.015 | 0.117 |
| 332 | 0.024 | 0.112 |  | 352 | -0.031 | 0.031 |  | 372 | -0.024 | -0.021 |  | 392 | -0.050 | 0.072 |
| 333 | -0.081 | -0.033 |  | 353 | -0.085 | 0.074 |  | 373 | -0.023 | 0.080 |  | 393 | -0.101 | 0.041 |
| 334 | 0.054 | -0.116 |  | 354 | 0.200 | 0.036 |  | 374 | 0.136 | -0.019 |  | 394 | -0.003 | 0.033 |
| 335 | 0.050 | 0.133 |  | 355 | 0.087 | -0.029 |  | 375 | -0.032 | -0.063 |  | 395 | 0.110 | 0.014 |
| 336 | -0.094 | 0.000 |  | 356 | 0.058 | 0.119 |  | 376 | -0.138 | -0.116 |  | 396 | 0.024 | 0.112 |
| 337 | 0.050 | -0.133 |  | 357 | 0.130 | 0.091 |  | 377 | -0.152 | -0.075 |  | 397 | -0.081 | -0.033 |
| 338 | 0.054 | 0.116 |  | 358 | -0.153 | -0.047 |  | 378 | -0.153 | 0.047 |  | 398 | 0.054 | -0.116 |
| 339 | -0.081 | 0.033 |  | 359 | -0.152 | 0.075 |  | 379 | 0.130 | -0.091 |  | 399 | 0.050 | 0.133 |
| 400 | -0.017 | -0.009 |  | 420 | 0.003 | 0.024 |  | 440 | 0.022 | -0.041 |  | 460 | -0.012 | 0.066 |
| 401 | 0.043 | 0.065 |  | 421 | -0.072 | 0.054 |  | 441 | 0.022 | -0.134 |  | 461 | 0.058 | 0.022 |
| 402 | -0.063 | -0.014 |  | 422 | 0.023 | 0.098 |  | 442 | 0.046 | -0.087 |  | 462 | 0.035 | 0.000 |
| 403 | -0.073 | -0.071 |  | 423 | -0.018 | -0.171 |  | 443 | 0.010 | 0.108 |  | 463 | -0.071 | -0.031 |
| 404 | -0.050 | 0.028 |  | 424 | 0.044 | -0.086 |  | 444 | 0.060 | 0.083 |  | 464 | 0.059 | -0.018 |
| 405 | -0.043 | 0.129 |  | 425 | 0.080 | 0.052 |  | 445 | 0.054 | 0.104 |  | 465 | 0.043 | 0.065 |
| 406 | 0.053 | 0.115 |  | 426 | 0.020 | -0.070 |  | 446 | 0.032 | -0.094 |  | 466 | -0.063 | -0.014 |
| 407 | 0.139 | -0.058 |  | 427 | -0.016 | 0.115 |  | 447 | -0.033 | -0.123 |  | 467 | -0.073 | -0.071 |
| 408 | 0.038 | -0.058 |  | 428 | -0.143 | -0.020 |  | 448 | -0.011 | 0.168 |  | 468 | -0.050 | 0.028 |
| 409 | -0.009 | 0.030 |  | 429 | -0.066 | -0.166 |  | 449 | 0.014 | 0.082 |  | 469 | -0.043 | 0.129 |
| 410 | 0.005 | -0.054 |  | 430 | 0.034 | 0.146 |  | 450 | -0.027 | -0.048 |  | 470 | 0.053 | 0.115 |
| 411 | 0.016 | 0.031 |  | 431 | -0.122 | -0.016 |  | 451 | 0.112 | -0.093 |  | 471 | 0.139 | -0.058 |
| 412 | -0.103 | 0.187 |  | 432 | -0.237 | -0.081 |  | 452 | 0.117 | -0.080 |  | 472 | 0.038 | -0.058 |
| 413 | -0.127 | 0.084 |  | 433 | -0.045 | 0.075 |  | 453 | 0.077 | 0.061 |  | 473 | -0.009 | 0.030 |
| 414 | 0.141 | -0.018 |  | 434 | 0.064 | -0.084 |  | 454 | -0.007 | 0.057 |  | 474 | 0.005 | -0.054 |
| 415 | 0.027 | 0.066 |  | 435 | -0.162 | -0.013 |  | 455 | -0.128 | 0.097 |  | 475 | 0.016 | 0.031 |
| 416 | -0.048 | 0.049 |  | 436 | -0.110 | 0.028 |  | 456 | 0.055 | 0.066 |  | 476 | -0.103 | 0.187 |
| 417 | 0.126 | -0.148 |  | 437 | 0.030 | -0.040 |  | 457 | 0.015 | -0.110 |  | 477 | -0.127 | 0.084 |
| 418 | -0.002 | -0.102 |  | 438 | 0.006 | -0.067 |  | 458 | -0.042 | -0.093 |  | 478 | 0.141 | -0.018 |
| 419 | 0.014 | 0.094 |  | 439 | 0.071 | -0.123 |  | 459 | 0.075 | 0.028 |  | 479 | 0.027 | 0.066 |
| 480 | -0.004 | 0.090 |  | 500 | -0.044 | -0.090 |  | 520 | -0.097 | 0.027 |  | 540 | -0.102 | -0.042 |
| 481 | 0.001 | 0.099 |  | 501 | 0.014 | -0.038 |  | 521 | 0.121 | -0.156 |  | 541 | 0.040 | 0.117 |
| 482 | 0.013 | -0.050 |  | 502 | 0.001 | -0.044 |  | 522 | 0.067 | -0.055 |  | 542 | 0.062 | 0.075 |
| 483 | -0.013 | -0.021 |  | 503 | -0.074 | 0.134 |  | 523 | -0.105 | 0.004 |  | 543 | -0.017 | 0.091 |
| 484 | -0.093 | 0.074 |  | 504 | -0.042 | 0.236 |  | 524 | 0.063 | -0.142 |  | 544 | 0.040 | 0.130 |
| 485 | -0.072 | 0.049 |  | 505 | 0.137 | 0.027 |  | 525 | 0.045 | -0.056 |  | 545 | 0.001 | 0.099 |
| 486 | -0.065 | 0.048 |  | 506 | 0.135 | -0.041 |  | 526 | 0.056 | -0.086 |  | 546 | 0.013 | -0.050 |
| 487 | -0.012 | 0.044 |  | 507 | -0.020 | 0.007 |  | 527 | 0.131 | -0.016 |  | 547 | -0.013 | -0.021 |
| 488 | 0.018 | 0.052 |  | 508 | -0.030 | 0.019 |  | 528 | 0.018 | 0.049 |  | 548 | -0.093 | 0.074 |
| 489 | 0.025 | -0.039 |  | 509 | -0.083 | 0.048 |  | 529 | -0.062 | -0.066 |  | 549 | -0.072 | 0.049 |
| 490 | 0.029 | 0.010 |  | 510 | -0.036 | 0.075 |  | 530 | -0.017 | 0.034 |  | 550 | -0.065 | 0.048 |
| 491 | -0.074 | 0.154 |  | 511 | 0.014 | 0.002 |  | 531 | -0.013 | 0.001 |  | 551 | -0.012 | 0.044 |
| 492 | 0.029 | 0.007 |  | 512 | -0.040 | -0.091 |  | 532 | -0.059 | -0.084 |  | 552 | 0.018 | 0.052 |
| 493 | 0.167 | -0.067 |  | 513 | 0.079 | 0.014 |  | 533 | 0.077 | -0.104 |  | 553 | 0.025 | -0.039 |
| 494 | 0.047 | -0.025 |  | 514 | -0.079 | 0.082 |  | 534 | 0.061 | -0.034 |  | 554 | 0.029 | 0.010 |
| 495 | 0.098 | 0.020 |  | 515 | -0.094 | -0.120 |  | 535 | -0.153 | 0.106 |  | 555 | -0.074 | 0.154 |
| 496 | 0.179 | -0.010 |  | 516 | 0.157 | -0.137 |  | 536 | -0.156 | -0.157 |  | 556 | 0.029 | 0.007 |
| 497 | 0.057 | -0.113 |  | 517 | -0.142 | 0.117 |  | 537 | -0.088 | -0.148 |  | 557 | 0.167 | -0.067 |
| 498 | 0.033 | 0.028 |  | 518 | -0.205 | 0.069 |  | 538 | 0.056 | 0.070 |  | 558 | 0.047 | -0.025 |
| 499 | 0.003 | 0.025 |  | 519 | -0.033 | 0.002 |  | 539 | 0.046 | -0.115 |  | 559 | 0.098 | 0.020 |
| 560 | 0.045 | 0.070 |  | 580 | -0.137 | -0.055 |  | 600 | -0.086 | 0.173 |  | 620 | 0.075 | -0.094 |
| 561 | -0.041 | -0.019 |  | 581 | -0.146 | -0.068 |  | 601 | -0.065 | 0.029 |  | 621 | -0.010 | -0.003 |
| 562 | 0.067 | -0.025 |  | 582 | 0.014 | 0.072 |  | 602 | 0.087 | -0.035 |  | 622 | -0.043 | 0.108 |
| 563 | -0.037 | 0.031 |  | 583 | 0.000 | -0.120 |  | 603 | 0.075 | -0.032 |  | 623 | 0.024 | 0.158 |
| 564 | -0.124 | 0.016 |  | 584 | 0.028 | 0.014 |  | 604 | -0.022 | 0.000 |  | 624 | -0.089 | 0.150 |
| 565 | -0.024 | -0.075 |  | 585 | 0.064 | 0.089 |  | 605 | 0.075 | 0.048 |  | 625 | -0.041 | -0.019 |
| 566 | 0.061 | -0.112 |  | 586 | -0.006 | -0.020 |  | 606 | 0.015 | -0.038 |  | 626 | 0.067 | -0.025 |
| 567 | 0.025 | 0.007 |  | 587 | -0.016 | 0.154 |  | 607 | -0.088 | -0.110 |  | 627 | -0.037 | 0.031 |
| 568 | 0.126 | -0.094 |  | 588 | 0.060 | -0.066 |  | 608 | -0.021 | 0.000 |  | 628 | -0.124 | 0.016 |
| 569 | 0.156 | -0.133 |  | 589 | -0.013 | -0.093 |  | 609 | -0.019 | 0.061 |  | 629 | -0.024 | -0.075 |
| 570 | -0.035 | -0.063 |  | 590 | -0.095 | 0.086 |  | 610 | -0.033 | -0.003 |  | 630 | 0.061 | -0.112 |
| 571 | -0.086 | -0.018 |  | 591 | 0.039 | -0.056 |  | 611 | -0.073 | 0.082 |  | 631 | 0.025 | 0.007 |
| 572 | 0.045 | 0.120 |  | 592 | 0.128 | 0.087 |  | 612 | -0.021 | 0.052 |  | 632 | 0.126 | -0.094 |
| 573 | 0.166 | -0.045 |  | 593 | 0.051 | 0.137 |  | 613 | 0.061 | -0.019 |  | 633 | 0.156 | -0.133 |
| 574 | 0.084 | -0.117 |  | 594 | 0.002 | -0.032 |  | 614 | -0.113 | 0.111 |  | 634 | -0.035 | -0.063 |
| 575 | 0.012 | 0.019 |  | 595 | -0.011 | -0.130 |  | 615 | -0.183 | 0.082 |  | 635 | -0.086 | -0.018 |
| 576 | 0.061 | -0.040 |  | 596 | -0.034 | -0.132 |  | 616 | 0.012 | 0.026 |  | 636 | 0.045 | 0.120 |
| 577 | 0.003 | -0.062 |  | 597 | -0.007 | 0.008 |  | 617 | 0.065 | -0.015 |  | 637 | 0.166 | -0.045 |
| 578 | 0.115 | -0.002 |  | 598 | -0.001 | -0.031 |  | 618 | -0.118 | -0.057 |  | 638 | 0.084 | -0.117 |
| 579 | 0.152 | 0.085 |  | 599 | -0.043 | 0.043 |  | 619 | -0.107 | -0.037 |  | 639 | 0.012 | 0.019 |
| 640 | 0.040 | -0.085 |  | 660 | 0.006 | -0.014 |  | 680 | 0.072 | 0.033 |  | 700 | 0.068 | 0.090 |
| 641 | 0.134 | 0.034 |  | 661 | -0.012 | -0.122 |  | 681 | -0.043 | 0.013 |  | 701 | 0.017 | -0.092 |
| 642 | 0.068 | -0.047 |  | 662 | -0.029 | 0.006 |  | 682 | -0.022 | 0.116 |  | 702 | -0.005 | 0.098 |
| 643 | 0.062 | -0.033 |  | 663 | -0.095 | -0.039 |  | 683 | -0.027 | -0.017 |  | 703 | -0.033 | 0.055 |
| 644 | 0.130 | 0.109 |  | 664 | -0.002 | -0.030 |  | 684 | 0.101 | -0.020 |  | 704 | 0.020 | -0.130 |
| 645 | 0.007 | -0.112 |  | 665 | 0.035 | 0.098 |  | 685 | 0.069 | 0.051 |  | 705 | 0.134 | 0.034 |
| 646 | -0.016 | -0.171 |  | 666 | -0.108 | -0.070 |  | 686 | -0.032 | 0.041 |  | 706 | 0.068 | -0.047 |
| 647 | -0.001 | 0.065 |  | 667 | 0.033 | -0.055 |  | 687 | 0.101 | 0.112 |  | 707 | 0.062 | -0.033 |
| 648 | -0.112 | 0.105 |  | 668 | 0.141 | 0.043 |  | 688 | 0.071 | -0.079 |  | 708 | 0.130 | 0.109 |
| 649 | 0.005 | -0.033 |  | 669 | 0.002 | 0.028 |  | 689 | -0.027 | -0.089 |  | 709 | 0.007 | -0.112 |
| 650 | 0.091 | -0.019 |  | 670 | 0.006 | 0.000 |  | 690 | 0.017 | 0.011 |  | 710 | -0.016 | -0.171 |
| 651 | -0.033 | 0.137 |  | 671 | 0.060 | -0.047 |  | 691 | 0.005 | -0.078 |  | 711 | -0.001 | 0.065 |
| 652 | -0.033 | 0.125 |  | 672 | 0.000 | 0.091 |  | 692 | -0.073 | -0.016 |  | 712 | -0.112 | 0.105 |
| 653 | 0.077 | 0.019 |  | 673 | -0.065 | 0.068 |  | 693 | -0.030 | -0.087 |  | 713 | 0.005 | -0.033 |
| 654 | 0.155 | 0.034 |  | 674 | -0.138 | 0.000 |  | 694 | 0.118 | -0.090 |  | 714 | 0.091 | -0.019 |
| 655 | -0.001 | -0.065 |  | 675 | -0.118 | 0.089 |  | 695 | 0.102 | -0.046 |  | 715 | -0.033 | 0.137 |
| 656 | -0.170 | -0.079 |  | 676 | 0.055 | 0.001 |  | 696 | 0.042 | -0.069 |  | 716 | -0.033 | 0.125 |
| 657 | -0.144 | 0.112 |  | 677 | -0.019 | -0.099 |  | 697 | -0.008 | 0.052 |  | 717 | 0.077 | 0.019 |
| 658 | -0.172 | 0.064 |  | 678 | -0.117 | -0.074 |  | 698 | -0.131 | -0.054 |  | 718 | 0.155 | 0.034 |
| 659 | -0.098 | 0.025 |  | 679 | 0.091 | 0.044 |  | 699 | -0.048 | 0.011 |  | 719 | -0.001 | -0.065 |
| 720 | -0.085 | -0.055 |  | 740 | 0.207 | 0.041 |  | 760 | 0.129 | 0.009 |  | 780 | -0.039 | 0.089 |
| 721 | -0.010 | 0.102 |  | 741 | -0.042 | -0.063 |  | 761 | -0.085 | -0.035 |  | 781 | 0.024 | -0.011 |
| 722 | 0.138 | 0.110 |  | 742 | -0.043 | -0.056 |  | 762 | 0.021 | -0.055 |  | 782 | 0.123 | 0.020 |
| 723 | 0.103 | -0.026 |  | 743 | 0.029 | 0.040 |  | 763 | 0.173 | -0.043 |  | 783 | 0.005 | 0.043 |
| 724 | -0.076 | 0.053 |  | 744 | -0.081 | 0.018 |  | 764 | -0.134 | -0.026 |  | 784 | 0.000 | -0.031 |
| 725 | -0.005 | 0.058 |  | 745 | -0.068 | -0.016 |  | 765 | -0.137 | -0.006 |  | 785 | -0.010 | 0.102 |
| 726 | -0.001 | 0.061 |  | 746 | 0.005 | -0.003 |  | 766 | 0.014 | -0.103 |  | 786 | 0.138 | 0.110 |
| 727 | -0.017 | 0.078 |  | 747 | 0.081 | -0.203 |  | 767 | -0.085 | -0.051 |  | 787 | 0.103 | -0.026 |
| 728 | -0.069 | 0.011 |  | 748 | 0.006 | -0.202 |  | 768 | 0.100 | -0.010 |  | 788 | -0.076 | 0.053 |
| 729 | -0.077 | 0.006 |  | 749 | -0.097 | 0.100 |  | 769 | 0.158 | -0.135 |  | 789 | -0.005 | 0.058 |
| 730 | 0.047 | -0.030 |  | 750 | -0.103 | 0.151 |  | 770 | -0.022 | 0.038 |  | 790 | -0.001 | 0.061 |
| 731 | 0.106 | -0.030 |  | 751 | -0.042 | 0.016 |  | 771 | -0.008 | 0.074 |  | 791 | -0.017 | 0.078 |
| 732 | 0.129 | -0.059 |  | 752 | 0.040 | -0.087 |  | 772 | -0.117 | -0.024 |  | 792 | -0.069 | 0.011 |
| 733 | -0.030 | -0.130 |  | 753 | -0.094 | -0.069 |  | 773 | -0.047 | 0.108 |  | 793 | -0.077 | 0.006 |
| 734 | -0.113 | -0.107 |  | 754 | -0.106 | 0.032 |  | 774 | 0.043 | -0.021 |  | 794 | 0.047 | -0.030 |
| 735 | -0.047 | -0.068 |  | 755 | 0.041 | 0.039 |  | 775 | -0.085 | -0.022 |  | 795 | 0.106 | -0.030 |
| 736 | -0.021 | 0.010 |  | 756 | 0.026 | 0.048 |  | 776 | 0.061 | 0.160 |  | 796 | 0.129 | -0.059 |
| 737 | 0.038 | 0.003 |  | 757 | -0.016 | 0.056 |  | 777 | 0.012 | 0.031 |  | 797 | -0.030 | -0.130 |
| 738 | -0.065 | -0.028 |  | 758 | -0.079 | -0.023 |  | 778 | -0.017 | 0.015 |  | 798 | -0.113 | -0.107 |
| 739 | 0.051 | 0.059 |  | 759 | 0.032 | -0.036 |  | 779 | 0.138 | 0.130 |  | 799 | -0.047 | -0.068 |
| 800 | 0.004 | -0.016 |  | 820 | -0.031 | 0.013 |  | 840 | 0.064 | 0.016 |  | 860 | -0.069 | -0.025 |
| 801 | 0.023 | -0.112 |  | 821 | -0.028 | -0.046 |  | 841 | -0.053 | 0.100 |  | 861 | -0.098 | -0.038 |
| 802 | 0.014 | 0.078 |  | 822 | 0.177 | -0.002 |  | 842 | -0.064 | -0.034 |  | 862 | -0.126 | -0.060 |
| 803 | -0.050 | -0.198 |  | 823 | 0.035 | -0.035 |  | 843 | -0.004 | -0.016 |  | 863 | -0.070 | 0.116 |
| 804 | -0.059 | -0.037 |  | 824 | -0.130 | -0.114 |  | 844 | -0.024 | 0.080 |  | 864 | 0.030 | -0.041 |
| 805 | 0.112 | 0.236 |  | 825 | 0.041 | 0.097 |  | 845 | -0.039 | -0.015 |  | 865 | 0.023 | -0.112 |
| 806 | -0.051 | -0.088 |  | 826 | 0.053 | 0.022 |  | 846 | -0.129 | -0.001 |  | 866 | 0.014 | 0.078 |
| 807 | -0.107 | -0.048 |  | 827 | 0.035 | -0.073 |  | 847 | -0.104 | 0.044 |  | 867 | -0.050 | -0.198 |
| 808 | 0.173 | 0.102 |  | 828 | -0.057 | 0.088 |  | 848 | 0.011 | 0.079 |  | 868 | -0.059 | -0.037 |
| 809 | 0.045 | 0.047 |  | 829 | -0.094 | -0.007 |  | 849 | 0.095 | 0.127 |  | 869 | 0.112 | 0.236 |
| 810 | -0.092 | -0.009 |  | 830 | 0.140 | -0.023 |  | 850 | 0.109 | 0.052 |  | 870 | -0.051 | -0.088 |
| 811 | -0.021 | -0.017 |  | 831 | 0.093 | 0.128 |  | 851 | 0.014 | -0.027 |  | 871 | -0.107 | -0.048 |
| 812 | -0.047 | 0.094 |  | 832 | -0.030 | 0.002 |  | 852 | 0.028 | 0.027 |  | 872 | 0.173 | 0.102 |
| 813 | -0.052 | -0.012 |  | 833 | -0.071 | -0.123 |  | 853 | 0.024 | 0.094 |  | 873 | 0.045 | 0.047 |
| 814 | -0.085 | -0.052 |  | 834 | -0.100 | -0.014 |  | 854 | 0.019 | -0.046 |  | 874 | -0.092 | -0.009 |
| 815 | -0.045 | -0.049 |  | 835 | -0.017 | 0.101 |  | 855 | 0.071 | -0.117 |  | 875 | -0.021 | -0.017 |
| 816 | 0.028 | -0.119 |  | 836 | -0.056 | 0.076 |  | 856 | 0.012 | -0.084 |  | 876 | -0.047 | 0.094 |
| 817 | -0.025 | 0.047 |  | 837 | 0.065 | 0.057 |  | 857 | 0.057 | -0.136 |  | 877 | -0.052 | -0.012 |
| 818 | -0.023 | -0.060 |  | 838 | 0.214 | 0.113 |  | 858 | 0.104 | -0.035 |  | 878 | -0.085 | -0.052 |
| 819 | 0.044 | -0.153 |  | 839 | 0.126 | 0.026 |  | 859 | -0.001 | 0.000 |  | 879 | -0.045 | -0.049 |
|  |  |  |  |  |  |  |  |  |  |  |  | 880 | 0.014 | -0.593 |

Table 24: The entire packet