IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15.3 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3 System Subcommittee SAP and PIB Living List |  |
| Date Originated | 17 May 2001 |  |
| Source | Rick Roberts XtremeSpectum, Inc. 8133 Leesburg Pike, Ste. 700 Vienna, Va. 22182 | Voice: 703.269.3043 Fax: 703.749.0249 E-mail: rroberts@xtremespectrum.com |
| Re: | Revision 3 |  |
| Abstract | This document represents the working document for the System Subcommittee. |  |
| Purpose | Working Document |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Clause Status for Draft 0.4

| Clause 6.1 Overview | Ready |
| --- | --- |
| Clause 6.2 Generic | Minor changes done … ready |
| Clause 6.3 MLME SAP | Work for Orlando meeting |
| Clause 6.4 PLME SAP | Minor changes done … ready |
| Clause 6.5 MAC PIB | Work for Orlando meeting |
| Clause 6.6 PHY PIB | Minor changes done … ready |
| Clause 6.7 MAC SAP | Work for Orlando meeting |
| Clause 6.8 PHY SAP | Minor changes done … ready |

Closing Report for Orlando – 17 May 2001

During the Orlando meeting the System Subcommittee worked on the MLME section of the document … clause 6.3 in draft 0.4.  It is proposed that these changes be reflected in draft 0.5.  The following amendments are proposed:

6.3.2.1 MLME-SCAN.request

- Changed PiconetID  PNID

- Added ChannelScanDuration and deleted Min and Max ChannelTime

- Proposed additional text to indicate PNID being scanned can either be a particular ID or any available ID.

6.3.2.2 MLME-SCAN.confirm

- Clarified that a PiconetDescriptionSet is an array of PiconetDescriptions

6.3.3 MLME-JOIN  MLME-Synch

6.3.3.1 MLME-Synch.request

- Some minor editorial corrections

- Added back in the OperationalRateSet as an open item needing resolution

6.3.4 Authenticate

- Add a note above authenticate and de-authenticate to indicate this section is under review.  Did not make any changes in this section.

6.3.6.1 MLME-ASSOCIATE.request

- Change PeerStaAddress to DeviceID

- Move CapabilityInformation field after DeviceID

- Add AllocatedDeviceAddress field after CapInfo field, see clause 7.3.5

- AssociateFailureTimeout becomes AssociateTimeoutPeriod, clause 7.3.5

- ListenInterval goes away

6.3.6.2 MLME-ASSOCIATE.confirm

- Change ResultCode as per clause 7.3.6, codes 0, 1 and 2.

6.3.6.3 MLME-ASSOCIATE.indication

- Replace PeerSTAAddress  DeviceID

6.3.7 Re-associate

- Delete section 6.3.7 and all sub-clauses

6.3.8.1 MLME-DISASSOCIATE.request

- Replace PeerSTAAddress  DeviceID

6.3.8.3 MLME-DISASSOCIATE.indication

- Replace PeerSTAAddress  DeviceID

Other Editorial Issues

- MLME-Join-network  use ASSOCIATE primitive

- MLME-Establish-network  use START primitive

- MLME-Join-or-establish  delete these combo commands, let SME do this task serially.

Subsystem Committee SAP and PIB Living List

This document tracks primitives and parameters that are being passed over the SAPs.  It also tracks items that are being stored in the PIBs.

Notation Key

Blue Reflects the Text of Clause 6.0 going into the Hilton Head Meeting

Red Reflects additional text added due to the Hilton Head Meeting

Green Reflects text added between the Hilton Head and Orlando Meeting

Text that is double strikethrough has been deleted

Pink is Resolution of an Issue as of the Opening at the Orlando Meeting

Turquoise Text added during Orlando Meeting

PHY Committee Issues List

| Item | Issue |
| --- | --- |
| PLME-RESET.request() | Roberts … what is the default Receipt of this primitive by the PHY sublayer shall cause the PHY entity to reset both the transmit and the receive state machines and place the PHY into the off state. |
| PLME-TESTMODE.request | Roberts … Missing DATA_TYPE parameter, add it Added |
| PLME-TESTMODE.request | Roberts … should we have loop back testing similar to 802.15.1? Not Specified … Not Added |
| Add PLME Power Management | Roberts … need to add PLME power management? Not Added as of Date |
| PhyPib_RegDomainsSupported | Roberts … how do we numerate these? TBD |
| PhyPib_MaxDataRate | Roberts … delete, unnecessary (see note below) Removed |
| PhyPib_MinDataRate | Roberts … delete, unnecessary (see note below) Removed |
| PhyPib_DataRateStepSize | Roberts … replace with PhyPib_DataRateVector. Index into vector returns a data rate. Added |
| PhyPib_CurrentDataRate | Roberts … Should this be reported instead in MAC PIB? Can this report be done on a per packet basis. Is this the only access point for the SME to determine/set data rate for scalable data rate (i.e. to combat poor channel conditions)? Left in PhyPib … SME entry point |
| PhyPib_CurrentPowerLevel | Roberts … Can this be done on a per packet basis is SME wants to do power control? Can the SME set the power here also? Yes |

| PhyPib RX RSSI Group | Heberling … need to revisit policy issue during the CAP in conjunction with RSSI group. Still need policy review The RX RSSI group provides information on the current RSSI value. The interpretation of the RSSI value is PHY dependent. |
| --- | --- |
| PhyPib_CCAModesSupported | Roberts … how many modes will 15.3 have? More than one? Subject to removal |
| Need Ranging Support Group | Roberts … ranging is in the evaluation requirements document … needs to be defined in PHY PIB? Added |
| In the PHY SAP clause, PHY-TXSTART.request, TXVECTOR | Shvodian … should the TXVECTOR pass the DA (destination address) and the SA (source address)? TXVECTOR definition work is in progress, will probably contain complete MAC header |
| In the PHY SAP clause, PHY-RXSTART.request, TXVECTOR | Shvodian … should the RXVECTOR pass the DA (destination address) and the SA (source address)? RXVECTOR definition work is in progress, will probably contain complete MAC header |
| PHY-CCARESET.request() | Roberts … is this needed? Left In |
| PHY-CCARESET.confirm() | Roberts … is this needed? Left In |

PHY Committee Issues List (cont.)

| PHY-CCA.indication | Heberling … need to work out policy Under Review |
| --- | --- |
| In PHY SAP, add PHY-RXNAP.request | Shovdian … now does RX go to sleep if the incoming packet is not for this station? Added |
| In PHY SAP, add PHY-RXNAP.confirm | Shovdian … see above comment Added |

MAC Committee Issues List

MAC SAP

| Item | Issue |
| --- | --- |
| MA-UNITDATA.request | Bill Shvodian … there is a stream index in the MAC header, where does this stream index come from? Is this accomplished via priority or service classes. Added to MAC Issues List |
| MA-UNITDATA.indication | Heberling … is the reception status adequate to support 15.3? What should be the reported statuses? e.g. reception status overflow Added to MAC Issues List |
| MA-UNITDATA-STATUS.indicateindication | Heberling … is the transmission status adequate to support 15.3? What should be the reported statuses? e.g. transmission status underflow Added to MAC Issues List |

MLME SAP

| Item | Issue |
| --- | --- |
| MLME-POWERMGT.request | Roberts … this needs to be defined in the 15.3 standard Added to MAC Issues List |
| MLME-SCAN.request | Heberling … Many of the parameters need to be examined for relevance to 802.15.3 Added to MAC Issues List Reviewed and resolved |
| MLME-SCAN.confirm | Heberling … Many of the parameters need to be examined for relevance to 802.15.3 Added to MAC Issues List Reviewed and resolved |

| MLME-JOIN.request | Heberling … (overloaded … not the same in 15.3, this is in response to the scan confirm, need a different name … A.H. What is difference between Join and Join-Network) Added to MAC Issues List Replace JOIN with Synch |
| --- | --- |
| MLME-AUTHENTICATE.request | Roberts … undefined in 802.15.3 Added to MAC Issues List |
| MLME-AUTHENTICATE.confirm | Roberts … undefined in 802.15.3 Added to MAC Issues List |
| MLME-AUTHENTICATE.indication | Roberts … undefined in 802.15.3 Added to MAC Issues List |
| MLME-DEAUTHENTICATE.request | Roberts … undefined in 802.15.3 Added to MAC Issues List |
| MLME-DEAUTHENTICATE.confirm | Roberts … undefined in 802.15.3 Added to MAC Issues List |
| MLME-DEAUTHENTICATE.indication | Roberts … undefined in 802.15.3 Added to MAC Issues List |

MLME SAP Continued

| MLME-ASSOCIATE.request | Montano … seems that device registration is missing? Define usage scenarios and review parameter list. Up date against 7.2.4 Added to MAC Issues List |
| --- | --- |
| MLME-REASSOCIATE.request | Heberling … roaming not supported by 15.3 … not needed Needs to be deleted … not deleted yet |
| MLME-REASSOCIATE.confirm | Heberling … roaming not supported by 15.3 … not needed Needs to be deleted … not deleted yet |
| MLME-REASSOCIATE.indication | Heberling … roaming not supported by 15.3 … not needed Needs to be deleted … not deleted yet |
| MLME-START.request | Heberling … Needs to be called Establish-Piconet … are the parameters adequate Done |

| MLME-START.confirm | Heberling … Needs to be called Establish-Piconet … are the parameters adequate Done |
| --- | --- |

MAC PIB

| MacPibStationDeviceID | Shvodian … IEEE 48 bit address In Progress |
| --- | --- |
| MacPibMediumOccupancyLimit | Roberts … Is this needed for 802,15,3? Added to Issues List |
| MacPibNetworkPiconetID | Roberts … Add this Added to Issues List |
| MacPibNodeID | Roberts … Add this Added to Issues List |
| MacPibCFPollable | Heberling … delete Need to Delete this |
| MacPibCFPPeriod | Roberts … clarify intent of this parameter Added to Issues List |

| MacPibCFPMaxDuration | Roberts … clarify intent of this parameter … should this be a CAP max duration Added to Issues List |
| --- | --- |
| MacPibAuthenticationResponseTimeOut | Heberling … not defined Added to Issues List |
| MacPibPrivacyOptionImplemented | Heberling … not defined Added to Issues List |
| MacPibPowerManagementMode | Roberts … What is it for 802.15.3 Added to Issues List |
| MacPibDesiredSSID | Roberts … Delete Deleted |
| MacPibDesiredPiconetType | Roberts … Delete Deleted |
| MacPibOperationalRateSet | Lynch … if bit rate, what are the rates we are supporting … need to coordinate with the PHY Added to Issues List |
| MacPibBeaconPeriod | Montano … need a reasonable limit and range Added to Issues List |

MAC PIB Continue

| MacPibDTIMPeriod | Heberling … does and how does 802.15.3 handle delivery traffic indication messages Added to Issues List |
| --- | --- |
| MacPibAssociationResponseTimeOutPeriod | Montano … K uS … see page 56 & 57 Added to Issues List |
| MacPibDisassociateStation | Lynch … (use 48 bit or Octet stationID/device ID?) Make this consistent with notation in section 7.2.6 Added to Issues List |
| MacPibDeauthenticateReason | Heberling … (not defined) Added to Issues List |
| MacPibDeauthenticateStation | Heberling … (not defined) Added to Issues List |
| MacPibAuthenticateFailStatus | Heberling … (not defined) Added to Issues List |
| MacPibAuthenticateFailStation | Heberling … (not defined) Added to Issues List |
| Need to define a MacPib data base to support multiple stream types …… | Shvodian … Added to Issues List |
| Do we need to specify the number of retries for CAP and CFP retransmissions. | Shvodian … The number of retries is probably data type dependent Added to Issues List |
| ###### MLME-Join-network.request | Need parameter definition Added to Issues List |
| MLME-Join-network.confirm | Need parameter definition Added to Issues List |
| MLME-Establish-network.request | Need parameter definition Added to Issues List |
| MLME-Establish-network.confirm | Need parameter definition Added to Issues List |
| MLME-Join-or-establish-network.request | Need parameter definition Added to Issues List |
| MLME-Join-or-establish-network.confirm | Need parameter definition Added to Issues List |

6.3 MLME SAP

(global change on MIB’s  PIB’s)

MLME-GET.request (PIBMIBattribute) Requests the value of the given PIBMIBattribute

MLME-GET.confirm (status, PIBMIBattribute, PIBMIBattributevalue) Returns the appropriate PIBMIB attribute value if status = “success,” otherwise returns an error indication in the Status field. Possible error status values include “invalid PIBMIB attribute” and “attempt to get write only PIBMIB attribute.

MLME-SET.request (PIBMIBattribute, PIBMIBattributevalue) Requests that the indicated PIBMIBattribute be set to the given value. If this PIBMIB attribute implies a specific action, then this requests that the action be performed.

MLME-SET.confirm (status, PIBMIBattribute) If status = “success,” this confirms that the indicated PIBMIB attribute was set to the requested value, otherwise it returns an error condition in status field. If this PIBMIBattribute implies a specific action, then this confirms that the action was performed. Possible error status values include “invalid PIBMIB attribute” and attempt to set read-only PIBMIB attribute.

MLME-POWERMGT.request (PowerManagementMode, WakeUp, ReceiveDTIMs) (this needs to be defined in the 15.3 standard)

PowerManagementMode = {ACTIVE, POWER_SAVE} Wakeup = {True, False} ReceiveDTIMs = {True, False}

MLME-POWERMGT.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, NOT_SUPPORTED}

MLME-SCAN.request (PiconetType, PiconetIDPNID, SSID, ScanType, ProbeDelay, ChannelList, ChannelScanDuration MinChannelTime, MaxChannelTime)

PiconetType = {INFRASTRUCTURE, INDEPENDENT, ANY_BSS} (is there more than one piconet type) ### PiconetID = “MACAddress” (appears this is not the MAC address …. Why is it that on a SCAN you enter the PiconetID … is this the ID of the piconet you are looking for –or- is this the Piconet of your current exising piconet during active scan?) Either looking for a particular PNID or a null value if you are just scanning. Need additional intro text to indicate this fact … could look for specific PN to re-join or join. Under when generated … add the re-join issue. ### SSID = “Octet String 0 – 32 octets” ScanType = {ACTIVE, PASSIVE} (seems 15.3 needs to support both … needs to be addressed in the specification … Do we need an active scan? Do we need the ScanType parameter) ProbeDelay = “Integer, in us, used with Probe frame during active scanning” (only needed for active scanning … do we need this for passive scanning only?) ChannelList = “Specifies a list of channels examined when scanning” (Is the channel list needed? Does this relate to regulatory domains? Can this be done at the SME level? Should we be using just a channel and not a channel list, the channel being supplied by the SME.) needed MinChannelTime = “greater/equal to Probe Delay” (Is MinChannelTime needed? Does this relate to regulatory domains? Can this be done at the SME level? Should we be using just a channel time and not a MinChannelTIme, which is supplied by the SME.) ChannelScanDuration Table 61 has superframe duration … dependent on superframe duration. Leave this TBD for now in duration for now. MaxChannelTime = “greater/equal to MinChannelTime” (Is MaxChannelTime needed? Does this relate to regulatory domains? Can this be done at the SME level? Should we be using just a channel time and not a MaxChannelTime, which is supplied by the SME.)

MLME-SCAN.confirm (PiconetDescriptionSet, ResultCode)

PiconetDescriptionSet = “indicates the results of the scan request” (see below) A piconet description set is an array of piconetDescriptions … it can contain zero or more PiconetDescriptions which is defined in table 5 ResultCode = {SUCCESS, INVALID_PARAMETERS} (do you get a success code for everything or do we get partial success?) OK … ignore note in 01/114r2.

Rename Table 5 to be “Elements of PiconetDescription” and remove current title PiconetDescription.PNPiconetID = “MACAddress” (Should this be the Piconet ID and not MAC address?) ### PiconetDescription.SSID = “1 – 32 octets, the SSID of the found BSS” ### PiconetDescription.PiconetType = {INFRASTRUCTURE, INDEPENDENT} PiconetDescription.BeaconPeriod = “The Beacon period of the found piconet (in TU)” Needs to be consistent with superframe duration (see table 61) PiconetDescription.DTIM_Period = “The DTIM period of the BSS (in beacon periods)” (the whole concept of the DTIM needs to be reviewed by the committee) This is still under study and needs to be put back into the draft standard … table 5 ### PiconetDescription.Timestamp = “timestamp of the received frame from found piconet” PiconetDescription.LocalTime = “value of the STA’s TSF timer at the start of reception” PiconetDescription.PHY_parameter_set = “parameter set relevant to PHY” (is this needed in 15.3? … reference sections 7.3.2 and 7.2.1) PiconetDescription.CF = “parameter set for the CF periods, if found BSS supports CF mode” PiconetDescription.IBSS = “parameter set for the IBSS, if found BSS is an IBSS” PiconetDescription.CapabilityInformation = “The advertised capabilities of the piconet” (it appears you can NOT get this from the beacon?) PiconetDescription.PiconetBasicRateSet = “The set of data rates that must be supported by all STA’s that desire to join this BSS” (what is the difference between the basic rate set and the operational rate set and all the other places where the rate set is defined … aren’t these specified in the association process and negotiated when requesting a time slot allocation?)

MLME-JOINSynch.request (PiconetBSSDescription, SynchJoinFailureTimeout, ProbeDelay, OperationalRateSet) (overloaded … not the same in 15.3, this is in response to the scan confirm, need a different name … A.H. What is difference between Join and Join-Network)

PiconetDescription = “see above” (table 5) SynchJoinFailureTimeout = “The time limit, in units of beacon intervals, after which the join procedure will be terminated” ProbeDelay = “Delay (in us) to be used prior to transmitting a Probe frame during active scanning” OperationalRateSet = “see above” Add back in OperationalRateSet for now and remove ProbeDelay … keep as an open item and resolve off line and put on MAC list.

MLME-SynchJOIN.confirm (ResultCode)

### ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT}

Put note that says authenticate and de-authenticate are under review

MLME-AUTHENTICATE.request (PeerSTAAddress, AuthenticationType, AuthenticateFailureTimeout) (undefined in 802.15.3)

PeerSTAAddress = “MACAddress” AuthenticationType = {OPEN_SYSTEM, SHARED_KEY} AuthenticationFailureTimeout = “Specifies a time limit (in TU) after which the authentication procedure will be terminated”

MLME-AUTHENTICATE.confirm (PeerSTAAddress, AuthenticationType, ResultCode) (undefined in 802.15.3)

MLME-AUTHENTICATE.indication (PeerSTAAddress, AuthenticationType) (undefined in 802.15.3)

MLME-DEAUTHENTICATE.request (PeerSTAAddress, ReasonCode) (undefined in 802.15.3)

MLME-DEAUTHENTICATE.confirm (PeerSTAAddress, ResultCode) (undefined in 802.15.3)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TOO_MANY_SIMULTANEOUS_REQUESTS}

MLME-DEAUTHENTICATE.indication (PeerSTAAddress, ReasonCode) (undefined in 802.15.3)

MLME-ASSOCIATE.request (PeerSTAAddress, AssociateFailureTimeout, CapabilityInformation, ListenInterval) (seems that device registration is missing? Define usage scenarios and review parameter list. Up date against 7.2.4.) Change PeerStaAddress to DeviceID Move CapabilityInformation field after DeviceID Add AllocatedDeviceAddress field after CapInfo field, see clause 7.3.5 AssociateFailureTimeout becomes AssociateTimeoutPeriod, clause 7.3.5 ListenInterval goes away

### PeerSTAAddress = “MACAddress” AssociateFailureTimeout = “Specifies a time limit (in TU) after which the associate procedure will be terminated” CapabilityInformation = “As defined in frame format Specifies the operational capability definitions to be used by the MAC entity” ListenInterval = “Specifies the number of beacon intervals that may pass before the STA awakens and listens for the next beacon”

MLME-ASSOCIATE.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED} Change result codes … valid results codes are zero, 1 and 2 in 7.3.6 … max devices, lack bandwidth, poor channel

MLME-ASSOCIATE.indication (PeerSTAAddressDeviceID)

MLME-REASSOCIATE.request (NewAPAddress, ReassociateFailureTimeout, CapabilityInformation, ListenInterval) (roaming not supported by 15.3 … not needed)

NewAPAddress = “MACAddress” ReassociateFailureTimeout = “Specifies a time limit (in TU) after which the reassociate procedure will be terminated”

MLME-REASSOCIATE.confirm (ResultCode) (roaming not supported by 15.3 … not needed)

### ResultCode = “SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED”

MLME-REASSOCIATE.indication (PeerSTAAddress) (roaming not supported by 15.3 … not needed)

MLME-DISASSOCIATE.request (DevIDPeerSTAAddress, ReasonCode)

MLME-DISASSOCIATE.confirm (ResultCode)

ResultCode = {SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED}

MLME-DISASSOCIATE.indication (DevIDPeerSTAAddress, ReasonCode)

MLME-RESET.request (DEVAddress, SetDefaultMPIB)

MLME-RESET.confirm (ResultCode)

ResultCode = {SUCCESS}

MLME-START.request (SSID, PiconetType, BeaconPeriod, DTIMPeriod, CF parameter set, PHY parameter set, IBSS parameter set, ProbeDelay, CapabilityInformation, PiconetBasicRateSet, OperationalRateSet) (Needs to be called Establish-Piconet … are the parameters adequate)

### SSID = “Octet string, the SSID of the BSS” PiconetID ### PiconetType = {INFRASTRUCTURE, INDEPENDENT} DTIM_Period = “The DTIM Period of the BSS (inbeacon periods)” (needs review to determine how and if DTIM’s are used) ### CF_parameter_set = “The parameter set for CF periods, if the BSS supports CF mode” PHY_parameter_set = “The parameter set relevant to the PHY” (Is this needed for 802.15.3) ### IBSS_parameter_set = “The parameter set for the IBSS, if BSS is an IBSS” ### ProbeDelay = “Delay (in us) to be used prior to transmitting a Probe frame during active scanning” CapabilityInformation = “The capabilities to be advertised for the BSS” (how is this used?) BSSBasicRateSet = “The set of data rate (in units of 500 kbit/s) that must be supported by all STAs to join this BSS” (why is this needed?) OperationalRateSet = “The set of data rate (in units of 500 kbit/s) that the STA may use for communication within the BSS” (why is this needed?)

MLME-START.confirm (ResultCode) (Needs to be called Establish-Piconet … are the parameters adequate)

ResultCode = {SUCCESS, INVALID_PARAMETERS, BSSPICONET_ALREADY_STARTED_OR_JOINED}

###### MLME-Join-network.request Use associate primitive

###### MLME-Join-network.confirm Use associate primitive

###### MLME-Establish-network.request Use start primitive … change 6.10

###### MLME-Establish-network.confirm Use start primitive … change 6.10

###### MLME-Join-or-establish-network.request Delete this command

###### MLME-Join-or-establish-network.confirm Delete this command

6.4 PLME SAP

PLME-GET.request (PIBattribute) ## Requests the value of the given PIBattribute

PLME-GET.confirm (status, PIBattribute, PIBattributevalue) Returns the appropriate PIB attribute value if status = “success,” otherwise returns an error indication in the Status field. Possible error status values include “invalid PIB attribute” and “attempt to get write only PIB attribute.

PLME-SET.request (PIBattribute, PIBattributevalue) Requests that the indicated PIBattribute be set to the given value. If this PIB attribute implies a specific action, then this requests that the action be performed.

PLME-SET.confirm (status, PMIBattribute) If status = “success,” this confirms that the indicated PIB attribute was set to the requested value, otherwise it returns an error condition in status field. If this PIBattribute implies a specific action, then this confirms that the action was performed. Possible error status values include “invalid PIB attribute” and attempt to set read-only PIB attribute.

PLME-RESET.request () “what is default … vendor definition (turns TX off … RX should be off probably also … turns everything off)”

PLME-TESTMODE.request (TEST_ENABLE, TEST_MODE, DATA_TYPE, DATA_RATE)

TEST_ENABLE = {True, false} TEST_MODE = “01 = transparent receive, 02 = continuous transmit, 03 = 50% duty cycle” DATA_TYPE = “1, 2, 3 … Selects one of three data patterns to be used for the transmit portions of the tests” ### DATA_RATE = “Data Rate in Mbps” (do we want Mbps or data rate vector index) (question: should we have loop back testing similar to IEEE802.15.1 … vendor specific)

PLME-TESTOUTPUT.request (TEST_OUTPUT)

TEST_OUTPUT = {True, false}

##### PLME Power Management ??? … vendor specific

Remove generic reset in 6.2 and use just the specifiecs.

6.5 MAC PIB

| Managed Object | Data Type |
| --- | --- |
| MacPibStationDeviceID (see page 53) | MacAddress Note: IEEE 48 bit address, vendors need to apply for a range of addresses |
| MacPibMediumOccupancyLimit (Is this needed for 802,15,3?) | INTEGER |
| MacPibNetworkPiconetID | Two Octets |
| MacPibNodeID (need to be consistent about node or station notation) | Octet |
| MacPibCFPollable | TruthValue |
| MacPibCFPPeriod (see 7.3.2 page 61) clarify intent of this parameter | INTEGER (what is the time units? … does integer represent a multiple of a base time unit?) |
| MacPibCFPMaxDuration (clarify intent of this parameter … should this be a CAP max duration) | INTEGER |
| MacPibAuthenticationResponseTimeOut (not defined) | INTEGER (not defined) |
| MacPibPrivacyOptionImplemented (not defined) | Boolean (not defined) |
| MacPibPowerManagementMode | INTEGER (wake, dose, active, power save? What is it for 802.15.3) |
| MacPibDesiredSSID | OCTET STRING |
| MacPibDesiredPiconetType | INTEGER |
| MacPibOperationalRateSet | OCTET STRING (if bit rate, what are the rates we are supporting … need to coordinate with the PHY) |
| MacPibBeaconPeriod (fixed for a piconet, can vary between piconets) | INTEGER (need a reasonable limit and range) |
| MacPibDTIMPeriod (does and how does 802.15.3 handle delivery traffic indication messages) | INTEGER |
| MacPibAssociationResponseTimeOutPeriod (does this represent what we want?) | INTEGER (k uS … see page 56 & 57) |
| MacPibDisassociateReason | INTEGER Octet |
| MacPibDisassociateStation | MacAddress (use 48 bit or Octet stationID/device ID?) Make this consistent with notation in section 7.2.6 |
| MacPibDeauthenticateReason (not defined) | INTEGER |
| MacPibDeauthenticateStation (not defined) | MacAddress (use 48 bit or Octet stationID/device ID?) Make this consistent with notation in section 7.2.6 |
| MacPibAuthenticateFailStatus (not defined) | INTEGER |
| MacPibAuthenticateFailStation (not defined) | MacAddress (use 48 bit or Octet stationID/device ID?) Make this consistent with notation in section 7.2.6 |
| Need to define a MacPib data base to support multiple stream types …… |  |
| Do we need to specify the number of retries for CAP and CFP retransmissions. | The number of retries is probably data type dependent. |

6.6 PHY PIB

| Managed Object | Default Value/Range | Static or Dynamic |
| --- | --- | --- |
| PhyPib Regulatory Group |  |  |
| PhyPib_Type | Phy Dependent | Static |
| PhyPib_RegDomainsSupported (note: how are these numerated?) for current PHY defined as the world | Phy Dependent | Static |
| PhyPib_CurrentRegDomain | Phy Dependent | Dynamic |
| PhyPib Rate Group |  |  |
| PhyPib_MaxDataRate | Phy Dependent | Static |
| PhyPib_MinDateRate | Phy Dependent | Static |
| ### PhyPib_DataRateStepSize (PhyPib_DataRateVector) add … ¼ and ½ rates | Phy Dependent | Static |
| PhyPib_CurrentDataRate (should this be in MAC PIB?) (can this be done on a per packet basis) | Phy Dependent | Dynamic |
| PhyPib Antenna Group |  |  |
| PhyPib_DiversitySupported | Phy Dependent | Static |
| PhyPib_CurrentTXAntenna | Phy Dependent | Dynamic |
| PhyPib_CurrentRXAntenna | Phy Dependent | Dynamic |
| PhyPib TX Power Group |  |  |
| PhyPib_MaxPowerLevel | Phy Dependent | Static |
| PhyPib_MinPowerLevel | Phy Dependent | Static |
| PhyPib_PowerLevelStepSize | Phy Dependent | Static |
| PhyPib_PowerLevelVector |  |  |
| PhyPib_CurrentPowerLevel (can this be done on a per packet basis) | Phy Dependent | Dynamic |

| Managed Object | Default Value/Range | Static or Dynamic |
| --- | --- | --- |
| PhyPib Channelization Group |  |  |
| PhyPib_NumChannelsSupported | Phy Dependent | Static |
| PhyPib_CurrentChannel (note: per piconet) | Phy Dependent | Dynamic |
| PhyPib RX RSSI Group (need to revisit policy issue on CAP) |  |  |
| PhyPib_RSSI_max | Phy Dependent | Static |
| PhyPib_RSSI_min | Phy Dependent | Static |
| PhyPib_RSSI_StepSize | Phy Dependent | Static |
| PhyPib_RSSI_Vector | Phy Dependent | Dynamic |
| PhyPib MPDU Length Group |  |  |
| PhyPib_MPDU_LengthMax | Phy Dependent | Static |
| PHYMib CCA Compliance Group |  |  |
| PhyPib_CCAModesSupported (how many modes will 15.3 have) comment … to be removed next revision unless someone can justify this | Phy Dependent | Static |
| PhyPib_CurrentCCAMode | Phy Dependent | StaticDynamic |
| PhyPib_CCA_Threshold | Phy Dependent | StaticDynamic |
| PhyPib Short Turnaround Group |  |  |
| PhyPib_ShortTurnaroundSupported | Phy Dependent | Static |
| PhyPib IEEE802.15.1 Support Group |  |  |
| PhyPib_IEEE802.15.1_PhySupported | Phy Dependent | Static |
| #### Ranging Support Group |  |  |
| PhyPib_Range | Phy Dependent | Dynamic 2 bytes, 1st byte is meters, 2nd byte is centimeters |

Add text to explain what the entries mean … both MAC & PHY PIB

6.7 MAC SAP

MA-UNITDATA.request(source address, destination address, routing information, data, priority, service class) (there is a stream index in the MAC header, where does this stream index come from? Is this accomplished via priority or service classes.)

MA-UNITDATA.indication(source address, destination address, routing information, data, reception status, priority, service class) (is the reception status adequate to support 15.3? What should be the reported statuses?) e.g. reception status overflow

MA-UNITDATA-STATUS.indicateindication(source address, destination address, transmission status, provided priority, provided service class) (is the transmission status adequate to support 15.3? What should be the reported statuses?) e.g. transmission status underflow

6.8 PHY SAP

PHY-DATA.request (DATA)

PHY-DATA.indication (DATA)

DATA = {octet value}

PHY-DATA.confirm ()

PHY-TXSTART.request (TXVECTOR)

TXVECTOR = {DataRate, Length, TxPowerLevel, DA, SA, ???} Note: “Length” is number of octets … add MAC header to the TXVECTOR, working towards to fixed MAC header, 12 bytes … J. Gilb (include diversity)

PHY-TXSTART.confirm ()

PHY-TXEND.request ()

PHY-TXEND.confirm ()

PHY-CCARESET.request () needed? … used to reset timers. TBD … unresolved until we define what the CCA is and does.

PHY-CCARESET.requestconfirm() needed? ? … used to reset timers. TBD … unresolved until we define what the CCA is and does.

PHY-CCA.indication (STATE) need to work out policy for alternate PHY’s? … TBD … unresolved until we define what the CCA is and does.

# STATE = “BUSY” or ”IDLE”

PHY-RXSTART.indication (RXVECTOR)

RXVECTOR = {DataRate, Length, RSSI, DA, SA, ???} Note: “Length” is number of octets octets … add MAC header to the TXVECTOR, working towards to fixed MAC header, 12 bytes … J. Gilb SNR for RSSI?

PHY-RXEND.indication (RXERROR) … reception of the in the current state

RXERROR = {NoError or FormatViolation or CarrierLost or UnsupportedRate}

PHY-RXNAP.request (NAP, WAKE … different degrees of sleeping vector)

PHY-RXNAP.confirm ()

RXSTART.request(diversity? … J.G.) … starting listening for valid packets, checks for valid CRC

RXSTART().confirm