| Radiocommunication Study Groups |  |
| --- | --- |
|  | Document 5D/TEMP/226-E |
|  | 16 June 2009 |
|  | English only |
| Sub-Working Group Evaluation |  |
| Update of the ITU-R IMT-Advanced submission and evaluation process web page with informative materials provided by Evaluation Groups and ITU-R Member |  |

# 1	Introduction

At the 5th ITU-R Working Party 5D meeting,

•	the Evaluation Group TTA PG707 provided C-code for Report ITU-R M.2135 channel model implementation in the contribution 5D/476 (submitted by Republic of Korea on behalf);

•	the Chinese Evaluation Group provided evaluation calibration results in the contribution 5D/486 (submitted by China on behalf);

•	Finland provided Matlab software implementation of the channel model in the contribution 5D/478;

•	Finland provided guidelines for using IMT-Advanced channel models in the contribution 5D/469.

It is agreed by the WP 5D meeting that these materials should be posted on the ITU-R IMT-Advanced submission and evaluation process web page after the meeting in order to share the information to facilitate the work of other organizations, including other Evaluation Groups and candidate technology proponents. As a consequence, the web page should be updated accordingly.

It is noted by the meeting that:

•	These materials are provided by Evaluation Groups and ITU-R Member as informative materials, which are considered as valuable information to other Evaluation Groups, potential candidate technology proponents and other relevant groups.

•	These materials may be updated by their authors respectively in future.

•	ITU-R WP 5D appreciates the supports and contributions from the contributors. ITU-R takes no responsibility on the materials and does not mandate any Evaluation Groups or candidate technology proponents to use them.

# 2	Update of the ITU-R IMT-Advanced submission and evaluation process web page

In the ITU-R IMT-Advanced submission and evaluation process web page,

•	a new sub-section “Informative Materials” should be created under “Submission and evaluation process for candidate RIT/SRITs”; some notes should be given at beginning of this sub-section;

•	four items should be added in that sub-section, and web links to documents of the informative materials of TTA PG707, Chinese Evaluation Group and Finland should be created respectively.

The updated web page should look like this:

IMT-Advanced submission and evaluation process

…

Submission and evaluation process for candidate RIT/SRITs

…

Evaluation Reports

- The evaluation reports will be made available here. They will also be prepared as inputs to Working Party 5D.

Informative Materials

The following materials are provided by Members of ITU-R, registered Evaluation Groups and other external organizations for purpose of information sharing. They should not be regarded as mandated by ITU-R in any way. Please contact the authors for latest version of the materials.

- TTA PG707: C-Code for Report ITU-R M.2135 Channel Model Implementation

- Chinese Evaluation Group: Calibration Activities in Chinese Evaluation Group

- Finland: Software Implementation of IMT.EVAL Channel Model

- Finland: Guidelines for Using IMT-Advanced Channel Models

…

The documents of the informative materials to be linked on the web page are in Annex 1, 2, 3 and 4.

Annex 1:	TTA PG707: C-Code for Report ITU-R M.2135 Channel Model Implementation

Annex 2:	Chinese Evaluation Group: Calibration Activities in Chinese Evaluation Group

Annex 3:	Finland: Software Implementation of IMT.EVAL Channel Model

Annex 4:	Finland: Guidelines for Using IMT-Advanced Channel Models

Annex 1

TTA PG707

C-Code for Report ITU-R M.2135 Channel Model Implementation

# 1	Introduction

Report ITU-R M.2135 has been agreed as the guideline for the evaluation of IMT-Advanced candidate RITs/SRITs. One of the key ideas in Report ITU-R M.2135 is how to generate the channel model used in the system-level simulation, and the Matlab® functions implementing the channel model in Report ITU-R M.2135 are available in [1]. TTA PG 707 provides the equivalent implementation in C in this contribution.

This contribution covers the current version of Report ITU-R M.2135 radio channel model. The implementation has been developed for convenience and it runs on ANSI C environment, and is available as attachment. More importantly, the channel implementation is analysed by the deterministic blocks and numerous independent random numbers. The functional operations of the C-code in this contribution are verified against the ones in Matlab® functions [1], so therefore the statistical characteristics of the C-code outputs are the same as the one from the Matlab® functions [1].

# 2	Software package

The attached file contains C functions and documentation for the Report ITU-R M.2135 IMT-Advanced channel model. The functions and documents have been packed by the WinZip® program.

# References

[1]	Finland and China (People’s Republic of), SOFTWARE IMPLEMENTATION OF IMT.EVAL CHANNEL MODEL, Contribution 5D/313, October 2008, presented in ITU-R WP 5D #3 in Seoul, October 2008.

Contact information:	please refer to the ITU-R IMT-Advanced submission and evaluation process web page for the contact information of the TTA PG707 Evaluation Group

Annex 2

Chinese Evaluation Group

Calibration Activities in Chinese Evaluation Group

# 1	Introduction

According to the ITU-R Working Party 5D work plan, the IMT-Advanced candidate RIT/SRITs may be received from different SDO or countries before the deadline at the October 2009. The evaluation reports for these received candidate RIT/SRITs will be submitted before the cut off time in June of 2010.

In preparation for the upcoming evaluation activities of the candidate technologies, Chinese Evaluation Group (ChEG) started the activities of simulation calibration among its members. The purpose of this document is to provide information about ChEG and its relevant activities. The contents of this document are informative.

# 2	Introduction to Chinese Evaluation Group

So far, plenty of Evaluation Groups have registered to ITU-R Working Party 5D. In order to facilitate the evaluation work of ITU-R Working Party 5D efficiently, ChEG, one of the Evaluation Groups, has already started the calibration activities to prepare for the coming IMT-Advanced evaluation activities that require simulation effort.

There are 13 members participating in the ChEG activities:

–	China Academy of Telecommunication Research of MIIT,

–	Southeast University,

–	China Mobile Communications Corporation,

–	China United Network Communications Corporation,

–	China Telecom Corporation Limited,

–	Tsinghua University,

–	Beijing University of Posts and Telecommunications,

–	The State Radio Monitoring Center,

–	54th Research Institute, Electronic Technology Corporation, China,

–	Datang Mobile Communication Equipment Co., Ltd.,

–	Huawei Technologies Co., Ltd.,

–	ZTE Corporation,

–	Alcatel Shanghai Bell Co., Ltd.

# 3	Calibration activities in ChEG

For calibration among its members, a step-by-step approach has been taken by ChEG. Following steps have been defined and preliminary results are included in next section for reference.

## 3.1	Channel model calibration

The intention of this step is to calibrate the implementation of newly defined IMT-Advanced MIMO channel. The output is in form of statistical characteristics of MIMO fast fading, which are expected to match with the theoretical values that can be derived from the definition of the MIMO channel in Report ITU-R M.2135.

## 3.2	Link level calibration

1)	Link performance in AWGN channel

This step is to calibrate the implementation of basic transceiver algorithms among ChEG members. The output is in form of link level curve.

2)	Link performance in MIMO fading channel (with ideal channel estimation)

This step is to calibrate the implementation of MIMO transceiver algorithms among ChEG members. The output is in form of link level curve.

3) 	Link performance in MIMO fading channel (with real channel estimation)

This step is to calibrate the implementation of channel estimation algorithms among ChEG members. The output is in form of link level curve.

4)	Link performance in MIMO fading channel with adaptive scheduling and HARQ

This step is to calibrate the implementation of full link level processes, including channel measurement / feedback, Adaptive Scheduling, AMC, HARQ, etc. The output is in form of link level spectrum efficiency.

## 3.3	System level calibration

Here ChEG referred the methodologies taken by 3GPP in LTE-advanced self evaluation. Those include:

1)	Large scale fading characteristics of system level platform.

This step is to calibrate the basic characteristics of system level platform among ChEG members. Those characteristics include distribution of downlink wideband SINR (also known as geometry) as well as coupling loss.

2)	System performance in simple scenario

This step is to calibrate the system level performance with simple system level simulation assumption, e.g. SIMO, Round Robin scheduling, etc. Output is in form of the distribution of user SINR and throughput.

# 4	Some result of calibration in ChEG

In this section, we provide some calibration results which are the latest outcomes of calibration activities of ChEG.

## 4.1	Channel model calibration

Compared with the theoretical values, calibration results about statistical characteristic of IMT-Advanced MIMO fast fading channel of different scenarios (indoor, UMa, UMi and RMa) are shown in the following embedded documents.

## 4.2	Link level calibration

1)	Link performance in AWGN channel

•	Simulation assumptions

–	Antenna configuration: 1x1;

–	Turbo decoding algorithm: Max-Log-MAP with 8 iterations;

–	Others:

|  | Short code length | Middle code length | Long code length |
| --- | --- | --- | --- |
| CQI | 5 | 9 | 15 |
| Modulation | 2 | 4 | 6 |
| TBS | 392 | 1128 | 3112 |
| rate coding | 0.4385 | 0.601 | 0.9258 |
| SNR | [−1 : 0.5 : 2] | [7.8 : 0.2 : 9] | [19.4 : 0.2 : 20.6] |

•	Result calibration

2)	Link performance in MIMO fading channel (with ideal channel estimation)

•	Simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | UMA NLoS, 30 km/h (CDL) |
| Transmission scheme | 4*2 Precoding vector: [1,0;1,0;0,1;0,1] |
| Channel estimation | Ideal |
| RB/block size | 4RB Pair (1 ms)/(1 128 + 24)*2 codeword)/16QAM |
| simulation time | 10000 packages |
| Antenna configuration in base station | 10 lambda spacing |
| Antenna configuration in UE | 0.5 lambda spacing |
| MIMO receiver | MMSE |

•	Result calibration

3)	Link performance in MIMO fading channel (with real channel estimation)

•	Simulation assumption

| Channel estimation algorithm | 2D Viena filter (CATR, Huawei) LMMSE with linear interpolation (Datang Mobile) |
| --- | --- |

•	Result calibration

4)	Link performance in MIMO fading channel with adaptive scheduling and HARQ

•	Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenarios | UMA NLoS, 30 km/h |
| Channel model | CDL |
| Antenna configuration | 1*4, 10 lambda spacing |
| Channel estimation | Real |
| RB size | 4RB Pair (1 ms) |
| HARQ Timing | Refer to LTE R8 uplink |
| SNR | 4.8 dB |
| Output | Spectrum Efficiency |
| AMC | On, refer to LTE R8 uplink |

•	Result calibration

Output Spectrum Efficiency from members: 1.50~1.56 bps/Hz

## 4.3	System level calibration

1)	Large-scale fading catachrestic of system level platform

ChEG plans to perform system level calibration from June to August 2009, and the results will be provided later to the 6th ITU-R WP 5D meeting in October 2009.

2)	System performance in simple scenario

ChEG plans to perform system level calibration from June to August 2009, and the results will be provided later to the 6th ITU-R WP 5D meeting in October 2009.

Contact information:	please refer to the ITU-R IMT-Advanced submission and evaluation process web page for the contact information of the Chinese Evaluation Group

Annex 3

Finland

Software Implementation of IMT.EVAL Channel Model

# 1	Introduction

IMT-Advanced channel models were agreed in the 3rd ITU-R Working Party 5D meeting in Seoul, Korea in October 2008, and confirmed in ITU-R Study Group 5 meeting in Geneva in November 2008. The channel models were published in Report ITU-R M.2135 [1]. Reference implementation of channel models was contributed to the 3rd ITU-R WP 5D meeting as well [2].

However, the implementation is updated according to the recent findings and interpretation of the Report ITU-R M.2135.

This contribution covers an updated reference implementation of the current version of Report ITU-R M.2135 radio channel model. The implementation has been developed for convenience and it runs on Matlab® environment, and is available as attachment. The implementation is modified from [2].

To make it easier to use the IMT-Advanced channel models by the external parties, public implementation of the channel models is helpful. Therefore, the attached Matlab software implementation of the channel models is provided to ITU-R for publishing at the ITU-R IMT-Advanced submission and evaluation process web page for easy reference.

# 2	Software package

Attached file contains the current version of the Matlab® functions and documentation for the IMT-Advanced channel model in Report ITU-R M.2135. The functions and documents have been packed by WinZip® program. The material may be updated by the author in future. Please contact the author to confirm whether there is any update of the material.

# 3	References

[1]	Guidelines for evaluation of radio interface technologies for IMT-Advanced, Report ITU-R M.2135, 2008.

[2]	Finland and China (People’s Republic of), SOFTWARE IMPLEMENTATION OF IMT.EVAL CHANNEL MODEL, Document 5D/313-E, ITU-R WP 5D meeting #3 in Seoul, South Korea, October 2008.

Contact person:	Johan Nyström (Email address: johan.nystrom@ericsson.com)

Annex 4

Finland

Guidelines for Using IMT-Advanced Channel Models

# 1	Introduction

IMT-Advanced channel models were agreed in the 3rd ITU-R Working Party 5D meeting in Seoul, Korea in October 2008, and approved in the ITU-R Study Group 5 meeting in Geneva in November 2008. The channel models were published in Report ITU-R M.2135 – Guidelines for evaluation of radio interface technologies for IMT-Advanced.

In Report ITU-R M.2135 the channel models for four deployment scenarios and another optional deployment scenario are specified. The simulation cases specify the usage of the corresponding channel models.

The purpose of this document is to provide guidelines to how the channel models should be interpreted in the different cases. It should be noted that these complementary guidelines are not in any way contradictory to the description in Report ITU-R M.2135.

Some deployment scenarios specified in [1] match exactly with the corresponding channel model. In other scenarios one or two additional conditions are specified: propagation from outdoor to indoor environment (Outdoor-to-Indoor case) and/or propagation from outdoor to vehicle (Outdoor-to-Vehicle case). For these cases the instructions are needed. In the following, we will specify the following characteristics:

•	Path-losses in the Outdoor-to-Indoor and Outdoor-to-Vehicle paths.

•	Shadow fading for the different connections.

There is also a proposal for the used channel model parameters for each of these deployment scenarios. Drop concept has been used below. It has been defined in [1]. For the purpose of this document it is sufficient to say that the drop is a simulation interval, where all channel model parameters are kept constant, except the fast fading.

# 2	Path-loss matters

## 2.1	Deployment scenarios

The following deployment scenarios are mandatory [1]:

Indoor Hotspot (InH), Urban Microcell (UMi), Urban Macrocell (UMa), Rural Macrocell (RMa).

Indoor Hotspot and Urban Microcell outdoor (LoS and NLoS) deployment scenarios match exactly with the corresponding channel models. The InH channel model is not discussed further in this document. The UMi outdoor channel model is unambiguous in [1]. Same is true also for the UMi Outdoor-to-Indoor model, but it will be discussed below for completeness and to avoid misunderstandings. In the UMa and RMa deployment scenarios there are only Outdoor-to-Vehicle connections in the evaluation cases and they need to be considered.

Suburban Macrocell (SMa) is an optional deployment scenario. There are two kinds of connections in SMa: Outdoor-to-Vehicle connections for 50 % of the cases and Outdoor-to-Indoor connections for other 50 % of the cases.

## 2.2	Path-loss

Path-loss stands for the mean path-loss. The variable component, shadowing, is modelled as a log-normal random number and is discussed in section 0. Mean and variable components together form the instantaneous path-loss. Excluding the Indoor Hotspot, the following cases for the path-loss can be found:

•	Outdoor path-loss

•	Outdoor-to-Indoor path-loss and

•	Outdoor-to-Vehicle path-loss.

It should be noted that there is always the outdoor part in the Outdoor-to-Indoor and Outdoor-to-Vehicle path-losses. This outdoor part can be line-of-sight (LoS) or non-LoS (NLoS) connection.

### 2.2.1	Outdoor path-loss

The path-loss for each outdoor deployment scenario can be written as:

PLout =  PLout1,  for LoS below breakpoint (LoS1)

PLout =  PLout2,  for LoS above breakpoint (LoS2)	(1)

PLout =  PLout3,  for NLoS connections.

These three path-loss formulas for the three different conditions LoS1, LoS2 and NLoS are given in [1] for all the deployment scenarios containing outdoor parts.

### 2.2.2	Outdoor-to-Indoor path-loss

Outdoor-to-Indoor path-loss for the UMi case is:

PL = PLout(dout + din) + PLtw + PLin(din)	 (2)

where:

PLout 	is the outdoor path-loss from the BS to the UT. One of the three cases has to be selected depending on, if the connection from the BS to the wall next to the UT is LoS1, LoS2 or NLoS. (See the section 0.)

PLtw 	is through-wall penetration loss (20 dB)

PLin = αdin 	is the path-loss inside the building, α is the attenuation coefficient (0.5)

din 	is the indoor distance from wall to the UT and dout is the distance from the base station to the wall next to the UT.

The penetration loss PLtw describes the attenuation through the wall and the indoor path-loss PLin describes the extra path-loss caused by the indoor conditions.

In the scenario SMa PLin has been set to zero for simplicity. Then the equation reads:

PL = PLout(d)+ PLtw 	 (3)

where the explanations are given in (2). Now PLout must be calculated by using one of the three outdoor formulas for SMa depending on, if the connection between the BS and the wall next to the UT is LoS1, LoS2 or NLoS. d is the Euclidean distance between the BS and UT.

In both the equations the formula for the PLout must be selected according to the conditions: First LoS/NLoS condition has to be randomly drawn according to LoS probability (see 0). Then, if the LoS condition is encountered, the below or above break-point formula has to be used depending on the distance from the BS to the wall of the building.

For UT located in indoor, the indoor segment length from UT to the wall is determined by drawing a random number between 0 and 25 (m). ITU UMi scenario required min distance (dout + din) is 10 m. It is possible to generate the indoor segment length din to exceed the distance from the BS to UT (dout + din). Explicitly, this phenomenon should be avoided. Therefore the distance from UT to wall should be specified as below:

din = 	distance from wall to UT (in the LoS direction towards the base station site), assumed uniformly distributed between 0 and 25 m for distances from UT to BS above 25 m.

din = 	distance from wall to UT (in the LoS direction towards the Base Station site), assumed uniformly distributed between 0 and (dout + din) for distances from UT to BS below 25 m.

### 2.2.3	Outdoor-to-Vehicle path-loss

Exactly the same equation as in (3) is valid for the Outdoor-to-Vehicle connections, provided that the through-wall penetration loss is replaced with the Vehicle Penetration Loss PLv:

PL = PLout + PLv 	 (4)

where PLout must be calculated by using one of the three outdoor formulas for the deployment scenario depending on the propagation conditions. (See the section 0.) The Vehicle Penetration Loss is modelled with a lognormal random number. PLv in the equation is the mean value of it, 9 dB [1].

### 2.2.4	Line of Sight probability

Line of Sight (LoS) probability is given in the table A1-3 in [1]. It specifies the probability of LoS condition for the outdoor connections as function of distance. This LoS probability applies also for the Outdoor-to-Indoor and Outdoor-to-Vehicle connections as regards the outdoor path. For Outdoor-to-Indoor connections the LoS probability should be calculated based on the distance from the base station site to the penetration point of the building wall, i.e. PLoS = PLoS(dout). Note that the statement (for outdoor users only) for the UMi case in Table A1-3 in [1] may be misleading. The outdoor LoS/NLoS condition for each connection shall be drawn according to this probability.

## 2.3	Shadow fading

Shadowing in the outdoor and Outdoor-to-Indoor connections is modelled as a log-normal shadow fading and it has been specified in [1]. Shadowing can be described by one zero-mean log-normal random variable, the standard deviation of which has been given in Table A1-7 of [1].

Shadowing in the Outdoor-to-Vehicle connections has two random components: i) The basic log-normal shadow fading described above and related to the outdoor path and ii) the additional shadow fading connected to the vehicle penetration loss. Also the latter is log-normal and the standard deviation is 5 dB as given in [1].

### 2.3.1	Shadowing for the Outdoor connections

The shadow fading for the Outdoor connections is modelled to be independent between different base station sites.

PLinst =  PL + X1	 (5)

where PLinst is the instantaneous path-loss, PL is the mean path-loss and X1 is the ordinary shadow fading random variable.

In cases, where the instantaneous path-losses to one UT from several, say N, Base Stations sites have to be calculated, the different random variables X1n for all N connections are modelled independent. All have the same standard deviation that is given in [1] for the different environments. On the contrary the random variables from different UTs to a single Base Station site are correlated. The shadow fading has exponentially distance dependent correlation between two UTs [1].

### 2.3.2	Shadowing for the Outdoor-to-Indoor connections

The shadow fading for the Outdoor-to-Indoor (O2I) connections can be calculated similarly as for the outdoor connections. The correct shadow fading standard deviation from Table A1-7 of [1] has to be applied.

### 2.3.3	Shadowing for the Outdoor-to-Vehicle connections

In the case of Outdoor-to-Vehicle (O2V) connections, there are two shadowing components. One for the outdoor connection, the other for the shadow fading connected to the vehicle penetration. The additional shadow fading connected to the vehicle penetration is modelled to be fully correlated between the connections of one UT and different Base Stations. The reason is that the variation is due to the differences between different vehicles. Then the instantaneous path-loss for the BS n is:

PLinst,n =  PLn + X1n + X2n 	 (6)

where PLinst,n is the instantaneous path-loss (constant over one drop) for the link n, PLn is the mean path-loss, X1n is the outdoor shadow fading related random variable and X2n is the vehicle penetration related shadow fading random variable for link n. X2n is equal for all links terminating to the same UT (or vehicle).

Now X1n is independent for links from one UT to different Base Stations, as described in section 0. On the other hand X2n has the same value for all Base Stations connected to the same UT. This means that for the Outdoor-to-Vehicle connections, the two shadow fading components have to be calculated separately. X1 is an independent random variable specified in the [1]. X2n is fully correlated random variable between any UT and all the Base Stations associated to it.

Note that the above interpretation is based on the understanding that the variation in the O2V loss comes from the differences between the different vehicles that has been noted in the references [2], [6], [5].

## 2.4	Path-loss summary

The summary table for the path-loss and shadow fading formulas for the Outdoor-to-Indoor and Outdoor-to-Vehicle connections is given in Table 1.

Table 1

Path-loss and shadow fading parameters for the Outdoor-to-Indoor 
and Outdoor-to-Vehicle connections

| Scenario |  | Path loss (dB) | Shadow fading std of X1 (dB) | Shadow fading std of X2 (dB) | Outdoor channel model and propagation condition |
| --- | --- | --- | --- | --- | --- |
| UMi | O2I | PL = PLout1(dout + din) + 20 + 0.5 din + X1 PLout2(dout + din) + 20 + 0.5 din + X1 PLout3(dout + din) + 20 + 0.5 din + X1 | 7 7 7 | N.A. N.A. N.A. | LoS1 LoS2 NLoS |
| UMa | O2V | PL = PLout1 + 9 + X1 + X2 PLout2 + 9 + X1 + X2 PLout3 + 9 + X1 + X2 | 4 4 6 | 5 5 5 | LoS1 LoS2 NLoS |
| RMa | O2V | PL = PLout1 + 9 + X1 + X2 PLout2 + 9 + X1 + X2 PLout3 + 9 + X1 + X2 | 4 4 8 | 5 5 5 | LoS1 LoS2 NLoS |
| SMa | O2I | PL = PLout1 + 20 + X1 PLout2 + 20 + X1 PLout2 + 20 + X1 | 4 4 8 | N.A. N.A. N.A. | LoS1 LoS2 NLoS |
|  | O2V | PL = PLout1 + 9 + X1 + X2 PLout2 + 9 + X1 + X2 PLout3 + 9 + X1 + X2 | 4 4 8 | 5 5 5 | LoS1 LoS2 NLoS |

## 2.5	Path-loss calculation for Outdoor-to-Vehicle and Outdoor-to-Indoor paths

Assume that the locations of the BSs and the UTs are given as well as the deployment scenarios. It is also assumed that the fact whether UT is located indoors or in vehicle has been determined. Further, it is assumed that the BS to UT distances are known. Then the instantaneous path-losses for O2I and O2V paths can be calculated as follows:

Instantaneous path-loss for O2I and O2V paths can be calculated as follows:

Mean path-loss:

1)	For the UMi O2I case: determine the indoor segment length by drawing a random number between 0 and 25 (m). Calculate the outdoor segment length.

2)	Determine the LoS condition from BS to the building wall or vehicle, where the UT is located, using LoS probability given in Table A1-3 in [1].

3)	If the condition is LoS, calculate the outdoor part using LoS1 or LoS2 formula depending on whether the distance is below or above the break-point distance. Otherwise use the NLoS formula. (Note that in the UMi case the outdoor part contains also the length of the indoor segment.)

4)	For the UMi case: calculate the indoor path-loss and sum to the outdoor path-loss.

5)	For the O2I cases: sum the wall penetration loss, PLtw (20 dB) to the outdoor path-loss .

6)	For the O2V case: sum the vehicle penetration loss, PLv (9 dB) to the outdoor path-loss.

Random variations:

7)	Draw the random variable X1n representing the ordinary shadowing and sum to the path-loss. The standard deviation for each deployment scenario is given in Table A1-7 of [1] and section 2.4 of this document. The random variables X1n are independent.

8)	For the O2V connections: draw the random variable X2n and sum to the path-loss including X1n. The standard deviation for each deployment scenario is given in Table 8-2 of [1] and section 2.4 of this document. The random variables X2n are equal for all Base Stations connected to same UT but independent for connections to different UTs and represent the shadowing of vehicle penetration.

9)	Repeat 1 – 8 for all O2I/O2V connections in the simulation.

## 2.6	Channel model parameters

For the mandatory InH and UMi channel models all the parameters have been specified in [1].

For the mandatory UMa and RMa channel models, use the path-loss and shadow fading parameters given above. Other channel model parameters (than path-loss and shadowing) in the UMa and RMa environments shall be used according to the conditions in the outdoor path: use LoS parameters for LoS outdoor conditions and NLoS parameters for NLoS outdoor conditions also in the O2V cases.

For the optional SMa channel model the same rule is applied both in the O2I and O2V cases.

## 2.7	Antenna effects

Antenna patterns according to [1] have been used. The field pattern is three dimensional, although the channel model itself is two dimensional. Down-tilt can be applied by calculating the antenna elevation angle at the base station from the geometry and calculating the antenna gain using this figure together with the down-tilt and azimuth angles. Note that the Indoor BS antennas are assumed omnidirectional as well as all the UT antennas, which means that the elevation effects of them can be neglected.

# 3	Implementation of IMT-Advanced channel model

## 3.1	Implementation guidelines

The reference implementations [3][4] have to be updated to include the modifications presented in this document. Following updates have to be applied:

•	For the O2I case SMa (optional):

–	Add the building penetration loss to the path-loss formula.

–	Check that the LoS1 / LoS2 / NLoS condition is interpreted correctly in the path-loss calculations.

–	Use the channel parameters given in [1] for the specified propagation condition (LoS/NLoS) and generate the channel coefficients.

•	For the O2V cases UMa, RMa (mandatory) and SMa (optional):

–	Add the vehicle penetration loss to the path-loss formula.

–	Add the lognormal random variable (X2 above) with a standard deviation of 5 dB to the instantaneous path-losses. Ensure that the random variable is fully correlated (constant) within the connections to the same UT.

–	Check that the LoS1 / LoS2 / NLoS condition is interpreted correctly in the path-loss calculations.

–	Use the channel parameters given in [1] for the specified propagation condition (LoS/NLoS) and generate the channel coefficients.

## 3.2	Implementation issues

Issue No. 1:

Shadowing standard deviation for LoS connections in RMa and SMa channel model for distances lower and higher than the breakpoint distance are equal in Table A1-7, but partly different in Table A1-2 of [1]. Finland proposes all Evaluation Groups and candidate technology proponents to consider use of the equal standard deviations of Table A1-7 for simplicity.

Issue No. 2:

In [1], section 1.3.2.1, page 38, step 10b) there is description of an alternative method to generate fading channel coefficients with so called correlation matrix based method. There is specified that “Unique covariance matrices are determined for each drop and radio link based on the parameters defined in the previous steps”. Finland believes that unique Doppler spectra should be determined for each drop and radio link based on the parameters defined in the previous steps; thus Finland encourages all Evaluation Groups and candidate technology proponents to consider this approach.

# 4	References

[1]	Guidelines for evaluation of radio interface technologies for IMT-Advanced, ITU-R Report M.2135, 2008.

[2]	Friedbert Berens et al. “UWB car attenuation measurements”. Mobile and Wireless Communications Summit, 2007.

[3]	Finland and China (People’s Republic of), SOFTWARE IMPLEMENTATION OF IMT.EVAL CHANNEL MODEL, Document 5D/313-E, ITU-R WP 5D meeting #3 in Seoul, South Korea, October 2008. , updated in a new contribution in ITU-R WP 5D meeting #5 in Geneva, Switzerland, June 2009.

[4]	Korea(Republic of), C-CODE FOR REPORT ITU-R M.2135 CHANNEL MODEL IMPLEMENTATION, Document 5D/476-E, ITU-R WP 5D meeting #5 in Geneva, Switzerland, June 2009.

[5]	ITU-R recommendation P.679-1. ITU-R WG 3M. 28 May 1998.

[6]	Joaquim Fortuny-Guasch et al. “UWB screening attenuation measurements of cars study by IPSC of the JRC and ETSI TG31C on the measurement of the screening attenuation of cars in the frequency range between 0.85 GHz AND 11 GHz”. IPSC, Sensors Radar Technologies and Cyber Security Unit, European Commission Joint Research Centre. October 2006. EUR 22532 EN.

Contact person:	Johan Nyström (Email address: johan.nystrom@ericsson.com)

_______________

This work has been performed in the framework of the CELTIC project CP5-026 WINNER+.