| Radiocommunication Study Groups |  |
| --- | --- |
|  | **DRAFT** |
| Received: Subject: Question ITU-R 229-2/5 | Document 5D/xxx |
|  | Date: |
|  | English only TECHNOLOGY ASPECTS |
| Institute of Electrical and Electronics Engineers, Inc. (IEEE) |  |
| [draft] submission of A candidate imt-advanced rit based on ieee 802.16 (Part 4 of 4) |  |

This contribution was developed by IEEE Project 802®, the Local and Metropolitan Area Network Standards Committee (“IEEE 802”), an international standards development committee organized under the IEEE and the IEEE Standards Association (“IEEE-SA”).

The content herein was prepared by a group of technical experts in IEEE 802 and industry and was approved for submission by the IEEE 802.16™ Working Group on Wireless Metropolitan Area Networks, the IEEE 802.18 Radio Regulatory Technical Advisory Group, and the IEEE 802 Executive Committee, in accordance with the IEEE 802 policies and procedures, and represents the view of IEEE 802.

This proposal consists of four parts, each submitted as a contribution, including a Part 1 overview document that lists the contents of all four parts. The four documents together constitute a complete submission of a candidate IMT-Advanced RIT.

This document is Part 4.

Table of Contents – Part 4

7	Self-evaluation report

7.1	Overview

7.2	Evaluation results

8	Compliance templates

8.1	Compliance template for services

8.2 	Compliance template for spectrum

8.3	Compliance template for technical performance

Annex 3 – Simulation Assumptions and Configuration Parameters for IMT-Advanced Test Environments

Annex 4 – Details of Simulation-related Results in the Compliance Template for Technical Performance

# 7	Self-evaluation report

IEEE confirms that the proposed RIT meets all the requirements for IMT-Advanced as specified by the service related minimum capabilities (Section 4.2.4.1), the spectrum capability requirements (Section 4.2.4.2), and all minimum technical requirements (Section 4.2.4.3) of Report ITU-R M.2133 {5}.

# 7.1	Overview

This section provides the details of the proposed RIT’s evaluation results for the completion of the compliance template for technical performance in Report ITU-R M.2133 {5}. The performance evaluation results are presented for the entries of cell spectral efficiency (Section 7.2.1), cell edge user spectral efficiency (Section 7.2.1), number of supported VoIP users (Section 7.2.2), mobility traffic channel link data rates (Section 7.2.3), peak spectral efficiency (Section 7.2.4), control plane latency (Section 7.2.5), user plane latency (Section 7.2.5), intra-frequency hand-over interruption time (Section 7.2.5), inter-frequency handover interruption time within a spectrum band (Section 7.2.5), and inter-frequency handover interruption time between spectrum bands (Section 7.2.5). The evaluation results for other entries in the compliance template for technical performance, i.e. bandwidth, mobility classes, and inter-system handover, are provided in entries of comments in its compliance template in Section 8.3. The evaluation results for the completions of the compliance template for services and compliance template for spectrum in Report ITU-R M.2133 {5} are presented in entries of evaluator’s comments and spectrum capability requirements in each compliance template in Sections 8.1 and 8.2, respectively.

All results for the completion of the compliance template for technical performance were generated based on both Report ITU-R M.2135 {7} and IMT-ADV/3 following ITU-R guidelines. Evaluation assumptions used for evaluating the performance of the RIT through simulations that are in addition to those in Error: Reference source not found{7} are based on {4} and {6} and are outlined in Annex 3. Any additional simulation assumptions not explicitly specified in Annex 3 are presented in each relevant section below. System-level simulation calibration information is contained in {14}.

All final results obtained through evaluations in this section are reflected in the compliance template for technical performance in Section 8.3.

# 7.2	Evaluation results

# 7.2.1	Cell spectral efficiency and cell-edge user spectral efficiency

# 7.2.1.1	Simulation assumptions and parameters for full-buffer data traffic

The simulation assumptions and configuration parameters used in simulations for full-buffer data traffic are described in the tables in Annex 3. Compared to the tables in Annex 3, the following modifications/additions are made for full-buffer data traffic in FDD/TDD operations:

DL configuration

- Subchannelization for data allocations: In the case of the InH and UMi, Scheme-1 (subband-based CRU subchannelization) was selected in the simulations. In the case of UMa and RMa, Scheme-2 (miniband-based CRU subchannelization) was simulated.

- Multi-antenna Transmission Format for data: In the case of InH and UMi, the 6-bit transformed codebook based MU-MIMO scheme was simulated using the 4 x 2 configuration, i.e., adaptive switching among rank-1, rank-2, rank-3, and rank-4 transmission. In the case of UMa and RMa, MU-MIMO with long term beamforming was simulated using 4 x 2 configuration, i.e., adaptive switching among rank-1, rank-2, rank-3, and rank-4 transmission.

- Receiver Structure: The MMSE receiver was simulated for the AMS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with maximum retransmission delay of four frames was used for the simulations.

- Control signalling: Signalling errors were modelled for A-A-MAP and HF-A-MAP, and sounding estimation errors were modelled for A-MIDAMBLE.

- Control Channel Overhead: Dynamic overhead modelling for A-A-MAP and HF-A-MAP and fixed overhead modelling for non-user specific A-MAP (NUS-A-MAP), A-PREAMBLE, A-MIDAMBLE, and SFH are used in this contribution. Detailed calculation of the DL control channel overhead for each test environment is shown in Section 7.2. 1.2

UL configuration

- Subchannelization for data allocations: In the case of the InH and UMi, Scheme-1 (subband-based CRU subchannelization) was selected in the simulations. In the case of UMa and RMa, Scheme-2 (miniband-based CRU subchannelization) was simulated.

- Multi-antenna transmission format for data: In the case of the InH and UMi, the 3-bit codebook-based MU-MIMO scheme was selected with adaptive switching between a single user and collaborative spatial multiplexing using 2x4 antenna configuration. In the case of UMa and RMa, the MU-MIMO scheme with long-term beamforming and adaptive switching between a single user and collaborative spatial multiplexing was used in the simulations using 2x4 antenna configuration.

- Receiver structure: The MMSE receiver was simulated for the ABS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with retransmission delay of four frames was used for the simulations.

- Control signalling: Signalling errors were modeled for Primary Feedback Control Channel (P-FBCH) (UMa, RMa), Secondary Feedback Control Channel (S-FBCH) (InH, UMi), and HARQ Feedback Control Channel (H-FBCH), and sounding estimation errors were modelled for the Sounding channel.

- Control Channel Overhead: Dynamic overhead modeling for P-FBCH, S-FBCH and H-FBCH and fixed overhead modeling for Long-Term Covariance Matrix (LT-CM), Initial Ranging (IR), and Bandwidth Request (BW-REQ) are adopted. Detailed calculation of the UL control channel overhead for each test environment is shown in Section 7.2.1.2.

# 7.2.1.2	Control channel overhead calculations

In this section, details of the control overhead calculations for DL and UL are presented.

# DL calculations

The following assumptions were made for the DL control channels:

- A-A-MAP: The A-A-MAP control overhead is dynamically calculated based on the scheduler allocations in each simulated frame of both DL and UL in each test environment. The average DL A-A-MAP and UL A-A-MAP overhead is accounted for in the estimation of the cell spectral efficiency and cell-edge user spectral efficiency.

- HF-A-MAP: The overhead calculation for the HF-A-MAP channel is based on the dynamic calculation of the required ACKs/NACKs from the UL system level simulations for each test environment.

- A-PREAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed for both TDD and FDD.

- A-MIDAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed for both TDD and FDD.

- SFH: Fixed overhead of 20 Logical Resource Unit (LRU)s per superframe is assumed for both FDD and TDD.

Table 7-1 contains the summary of the DL control channel overhead calculations for TDD and FDD.

Table 7-1: Summary of DL total control channel overhead percentage per mode (TDD/FDD) and test environment (InH, UMi, UMa, RMa)

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| TDD | 9.19% | 12.33% | 11.17% | 11.15% |
| FDD | 9.74% | 16.28% | 13.77% | 13.63% |

# UL calculations

The following assumptions were made for the UL control channels:

- P-FBCH: The UL control overhead of the P-FBCH is calculated according to the P-FBCH reporting period of 5 ms for UMa and RMa, and by assuming that all 10 users/sector transmit the PFBCH in each 5 ms frame.

- S-FBCH/UL MAC management: The feedback of CQI/PMI is done either through S-FBCH or UL MAC management messages. In both cases, the reporting period is 5 ms for both TDD and FDD.

- H-FBCH: The overhead of the H-FBCH was based on the dynamic calculation of the required ACKs/NACKs from the DL system level simulations for each test environment.

- LT-CM: Although the feedback of the LT-CM is not assumed to be received through a dedicated UL control channel but through a regular UL MAC management burst, its overhead is included in the calculation of the UL control overhead. Fixed overhead of the LT-CM feedback is assumed assuming a reporting period of 20 ms for both UMa and RMa.

- Sounding: To enable the required sounding capabilities for both FDD and TDD, fixed overhead of 1 symbol per long TTI for both TDD and FDD is assumed.

- IR: Fixed overhead of 4 LRUs per superframe is assumed for both TDD and FDD.

- BW-REQ: Fixed overhead of 4 LRUs per superframe is assumed for both TDD and FDD.

Table 7-2 contains the summary of the UL control channel overhead calculations for TDD and FDD.

Table 7-2: Summary of UL total control channel overhead percentage per mode (TDD/FDD) and test environment (InH, UMi, UMa, RMa)

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| TDD | 7.85% | 12.60% | 9.23% | 8.34% |
| FDD | 6.02% | 10.58% | 8.01% | 6.51% |

# 7.2.1.3	Cell spectral efficiency and cell edge user spectral efficiency results

In Section 7.2.1.3.1, DL cell spectral efficiency and cell edge user spectral efficiency results obtained from performance evaluation of the RIT are presented for both TDD and FDD. The corresponding numbers for the UL are presented in Section 7.2.1.3.2.

# 7.2.1.3.1	DL results

Table 7-5 and Table 7-6 present the TDD DL cell spectral efficiency and cell-edge user spectral efficiency, respectively, for all test environments. Table 7-7 and Table 7-8 present the corresponding numbers for FDD. In each table, the ITU-R requirement for each test environment is also shown.

Table 7-5: DL cell spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.93 | 3.22 | 2.41 | 3.23 |
| ITU-R requirement | 3.0 | 2.6 | 2.2 | 1.1 |

Table 7-6: DL cell edge user spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.260 | 0.092 | 0.069 | 0.093 |
| ITU-R requirement | 0.1 | 0.075 | 0.06 | 0.04 |

Table 7-7: DL cell spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.87 | 3.27 | 2.41 | 3.15 |
| ITU-R requirement | 3.0 | 2.6 | 2.2 | 1.1 |

Table 7-8: DL cell edge user spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.253 | 0.097 | 0.069 | 0.091 |
| ITU-R requirement | 0.1 | 0.075 | 0.06 | 0.04 |

# 7.2.1.3.2	UL results

Table 7-9 and Table 7-10 present the TDD UL cell spectral efficiency and cell edge user spectral efficiency, respectively, for all test environments. Table 7-11 and table 7-12 present the corresponding numbers for FDD. In each table, the ITU-R requirements for each test environment are also shown.

Table 7-9: UL cell spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 5.99 | 2.58 | 2.57 | 2.66 |
| ITU-R requirement | 2.25 | 1.8 | 1.4 | 0.7 |

Table 7-10: UL cell edge user spectral efficiency in bit/s/Hz/cell for TDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.426 | 0.111 | 0.109 | 0.119 |
| ITU-R requirement | 0.07 | 0.05 | 0.03 | 0.015 |

Table 7-11: UL cell spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 6.23 | 2.72 | 2.69 | 2.77 |
| ITU-R requirement | 2.25 | 1.8 | 1.4 | 0.7 |

Table 7-12: UL cell edge user spectral efficiency in bit/s/Hz/cell for FDD

|  | InH | UMi | UMa | RMa |
| --- | --- | --- | --- | --- |
| Cell spectral efficiency | 0.444 | 0.119 | 0.114 | 0.124 |
| ITU-R requirement | 0.07 | 0.05 | 0.03 | 0.015 |

# Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced cell spectral efficiency and cell edge spectral efficiency. See Annex 4 for supporting details on the results presented above.

# 7.2.2	VoIP capacity

# 7.2.2.1	Simulation assumptions and parameters for VoIP traffic

The modeling assumptions and configurations used in the system level simulations for VoIP are as described in tables in Annex 3. Additional details relevant to system level simulations of the RIT for VoIP are listed in this section. In relations to information in Annex 3, the following items are specific to VoIP traffic:

DL configuration

- Subchannelization for VoIP allocations: A mix of DRU and miniband CRU subchannelization schemes is used in the simulations. Since the control channels are always allocated in distributed resources, a fixed partition is used in each subframe between the DRU and miniband CRU resources.

- Multi-antenna transmission format for VoIP: In the case of DRU, SFBC with non-adaptive precoding is used. In the case of miniband CRU, rank-1 tranmission with wideband beamforming is used.

- Receiver structure: The MMSE receiver was simulated for the AMS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with maximum retransmission delay of 4 frames was used.

- Control channel overhead:

  - A-MAP Region: A fixed overhead is assumed for NUS-A-AMAP and HF-A-MAP. The A-A-MAP control overhead is explicitly simulated based on the scheduler allocations in each simulated subframe on both DL and UL in each test environment.

  - A-PREAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed.

  - A-MIDAMBLE: Fixed overhead of 1 OFDMA symbol per frame is assumed. Midamble estimation errors are included in the presented results.

  - SFH: 20 LRUs are reserved for the SFH in the first subframe of every superframe.

- Control channel signalling errors: Explicitly modelled in the VoIP simulations for both FDD and TDD.

- Persistent scheduling: Persistent scheduling for individual VoIP connections as described in {6} is modelled. The MAC procedure and dynamic control overhead based on the persistent allocation A-MAP IE for initial transmissions is modelled. HARQ retransmissions are not scheduled persistently.

- Control channel overhead: A dynamic overhead model for the A-A-MAP and SFH and a fixed overhead model for the other DL control channels are used.

UL configuration

- Subchannelization for VoIP allocations: DRU-based subchannelization is used.

- Multi-antenna transmission format for VoIP: Open loop SFBC with a 2x4 antenna configuration is used.

- Receiver structure: The MMSE receiver was simulated for the ABS for both channel estimation and data detection.

- HARQ: Chase combining HARQ with retransmission delay of four frames was used for the simulations.

- Control signalling:

  - P-FBCH: The P-FBCH is utilized for feeding back CQI information. Additionally, when wideband beamforming is used, wideband PMI is also fed back. UL control overhead induced by the P-FBCH is calculated in a fixed manner according to the reporting period derived from the number of active users, voice activity factor, and duration of persistence for persistent scheduling.

  - H-FBCH: The overhead from the H-FBCH was based on the maximum number (30) of required ACKs/NACKs in each subframe from the DL system level VoIP simulations for each test environment.

  - Sounding channel: To facilitate sounding for 16 users per frame when the system is operating at the maximum VoIP capacity, a fixed overhead of 2 OFDMA symbols per frame is assumed.

  - IR: Fixed overhead of 1 LRU per frame is used.

  - BW-REQ channels: Fixed overhead of 1 LRU per frame is used.

- Control channel signalling errors: Explicitly modelled in the VoIP simulations for both FDD and TDD.

  - Persistent scheduling: Persistent scheduling for individual VoIP connections as described in {6} is modelled. The MAC procedure and dynamic control overhead based on the persistent allocation A-MAP IE for initial transmissions is modelled. HARQ retransmissions are not scheduled persistently.

  - Control channel overhead: A fixed overhead model for the UL control channels is used.

# 7.2.2.2	VoIP capacity results

Table 7-13 and Table 7-14 summarize the VoIP capacity results for the RIT based on the assumptions of Section 7.2.2.1.

Table 7-13: VoIP capacity (users/sector/MHz) for TDD

| Test environment | DL | UL | Minimum {DL, UL} | ITU-R required |
| --- | --- | --- | --- | --- |
| Indoor (InH) | 140 | 165 | 140 | 50 |
| Microcellular (UMi) | 82 | 104 | 82 | 40 |
| Base coverage urban (UMa) | 74 | 95 | 74 | 40 |
| High speed (RMa) | 89 | 103 | 89 | 30 |

Table 7-14: VoIP capacity (users/sector/MHz) for FDD

| Test environment | DL | UL | Minimum {DL, UL} | ITU-R required |
| --- | --- | --- | --- | --- |
| Indoor (InH) | 139 | 166 | 139 | 50 |
| Microcellular (UMi) | 77 | 102 | 77 | 40 |
| Base coverage urban (UMa) | 72 | 95 | 72 | 40 |
| High speed (RMa) | 90 | 101 | 90 | 30 |

# Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced VoIP capacity. See Annex 4 for supporting details on the results presented above.

# 7.2.3	Mobility requirements

Following the evaluation methodology for the mobility requirement {7}, in a first step UL system-level simulations were run for each test environment to obtain the UL SINR distribution. From the UL SINR distribution the median (or 50th percentile) value is obtained for each test environment. In a second step, link-level simulations were run to generate the spectral efficiency versus SNR curves for each test environment. The generated spectral efficiency numbers include the impact of control channel overhead by accounting for the average control channel overhead from the UL system-level simulations for each test environment. To assess whether the RIT meets the mobility requirement, the spectral efficiency value from the link-level curves is obtained for the median SINR value from the system-level simulations.

Table 7-15 and Table 7-16 summarize the results for TDD and FDD, respectively. It is noted that the link-level curves were generated for the NLOS channel models of each test environment.

Table 7-15: Mobility requirement data for TDD

| Test environment | Median SINR (in dB) | Achieved spectral efficiency (in bit/s/Hz) LoS | Achieved spectral efficiency (in bit/s/Hz) NLoS | ITU-R Required spectral efficiency (in bit/s/Hz) |
| --- | --- | --- | --- | --- |
| InH (10 km/h) | 16.6 | 3.76 | 3.41 | 1.0 |
| UMi (30 km/h) | 5.0 | 1.81 | 1.50 | 0.75 |
| UMa (120 km/h) | 4.3 | 1.72 | 1.30 | 0.55 |
| RMa (350 km/h) | 5.6 | 1.70 | 1.23 | 0.25 |

Table 7-16: Mobility requirement data for FDD

| Test environment | Median SINR (in dB) | Achieved spectral efficiency (in bit/s/Hz) LoS | Achieved spectral efficiency (in bit/s/Hz) NLoS | ITU-R Required spectral efficiency (in bit/s/Hz) |
| --- | --- | --- | --- | --- |
| InH (10 km/h) | 16.6 | 3.86 | 3.56 | 1.0 |
| UMi (30 km/h) | 5.0 | 1.72 | 1.51 | 0.75 |
| UMa (120 km/h) | 4.3 | 1.63 | 1.34 | 0.55 |
| RMa (350 km/h) | 5.6 | 1.61 | 1.27 | 0.25 |

# Therefore, IEEE confirms that the proposed RIT meets the IMT-Advanced mobility requirements. See Annex 4 for supporting details on the results presented above.

# 7.2.4	Peak spectral efficiency

This section shows the methodology to calculate the peak spectral efficiency and its results for the proposed RIT. The analysis follows the guideline and definition provided in Reports ITU-R M.2134 {8} and Report ITU-R M.2135 {7}. The FDD and TDD modes of the RIT are considered and the case for the channel bandwidth of 20 MHz TDD and 2x20 MHz FDD with the FFT size of 2048 and the size of cyclic prefix (CP) of 1/16 both in DL and UL are addressed to calculate the peak data rate. In addition, 4 and 2 streams are considered in DL and UL, respectively, based on the constraints of antenna configurations specified in {8} and {7}.

One OFDMA symbol with the length of 97.143 µs for preamble exists in each frame with the length of 5 ms and the idle time of 45.71 µs is allocated at the end of each frame. The FDD frame has 5 type-1 subframes, consisting of 6 OFDMA symbols, and 3 type-2 subframes, consisting of 7 OFDMA symbols. The TDD frame has 3 type-1 subframes in DL, consisting of 6 OFDMA symbols, and 1 type-2 subframe in DL, consisting of 7 OFDMA symbols. The same configuration is used in the UL assuming a DL to UL ratio of 1:1. The switching gap has been accounted for in the calculations.

For both TDD and FDD, the number of pilots per resource unit (RU) for both type-1 and type-2 subframes in case of 4 streams for DL is 16, while the numbers of subcarriers per RU for type-1 and type-2 subframes are 108 (= 18x6) and 126 (= 18x7), respectively. Therefore the numbers of available data subcarriers per RU for type-1 and type-2 subframes in case of 4 for DL streams are 92 and 110, respectively. On the other hand, in case of 2 streams for UL, the number of pilots per RU for type-1 and type-2 subframes are 12 and 14, respectively. Therefore the numbers of available data subcarriers per RU for type-1 and type-2 subframes in case of 2 streams for UL are 96 and 112, respectively.

For FDD, the number of RUs for 20 MHz is 96 both in DL and UL. The number of available data subcarriers for 5 type-1 and 3 type-2 subframes with 4 streams for DL are 44160 (= 5x92x96), denoted by NType1Subcarrier4 and 31680 (= 3x110x96), denoted by NType2Subcarrier4, respectively. On the other hand, the number of available data subcarriers for 5 type-1 and 3 type-2 subframes with 2 streams for UL are 46080 (= 5x96x96), denoted by NType1Subcarrier2, and 32256 (= 3x112x96), denoted by NType2Subcarrier2, respectively. Note that in 20 MHz, the 1728 subcarriers per frame is utilized in DL as preamble for physical layer synchronization.

Assuming repetition rate of 1, coding rate of 1, and the maximum MCS level, i.e. the modulation scheme of 64QAM (6 bit/symbol), the peak data rates in DL and UL are defined as follows:

Peak data rate in DL = (NType1Subcarrier4 + NType2Subcarrier4 - 1728) x 6 x 4 / 5 ms = 356 Mbit/s

Peak data rate in UL = (NType1Subcarrier2 + NType2Subcarrier2) x 6 x 2 / 5 ms = 188 Mbit/s

The peak spectral efficiencies in DL and UL can be calculated by dividing the peak data rates by 20 MHz, resulting 17.79 bit/s/Hz and 9.40 bit/s/Hz, for DL and UL respectively.

For TDD, the number of RUs for 20 MHz is 96 both in DL and UL. Assuming and UL:DL ratio of 1:1, the number of available data subcarriers for 3 type-1 and 1 type-2 subframes with 4 streams for DL are 26496 (= 3x92x96), denoted by N’Type1Subcarrier4 and 10560 (= 1x110x96), denoted by N’Type2Subcarrier4, respectively. On the other hand, the number of available data subcarriers for 3 type-1 and 1 type-2 subframes with 2 streams for UL are 27648 (= 3x96x96), denoted by N’Type1Subcarrier2, and 10752 (= 1x112x96), denoted by N’Type2Subcarrier2, respectively. Note that in 20 MHz, the 1728 subcarriers per frame is utilized in DL as preamble for physical layer synchronization.

Assuming repetition rate of 1, coding rate of 1, and the maximum MCS level, i.e. the modulation scheme of 64QAM (6 bit/symbol), the peak data rates in DL and UL are defined as follows:

Peak data rate in DL = (N’Type1Subcarrier4 + N’Type2Subcarrier4 - 1728) x 6 x 4 / 5 ms = 169.57 Mbit/s

Peak data rate in UL = (N’Type1Subcarrier2 + N’Type2Subcarrier2) x 6 x 2 / 5 ms = 92.16 Mbit/s

The peak spectral efficiencies in DL and UL can be calculated by dividing the peak data rates by 20 MHz and adjusting by the DL to UL ratio, resulting 16.96 bit/s/Hz and 9.22 bit/s/Hz, for DL and UL respectively.

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced peak spectral efficiency.

# 7.2.5	User-plane/control-plane latency and handover interruption time

The analyses in this section use the network reference model {4}. In these analyses, it is assumed that each 5 ms radio frame of the proposed RIT consists of eight subframes of 0.617 ms length and the transmission time interval (TTI) is equal to one subframe. The mobile station and base station processing times are assumed to be three subframes (3*TTI).

Figure 7-1: Network reference model {4}

This analysis further assumes an intra-ASN handover mechanism where the serving and target base stations belong to the same ASN entity.

7.2.5.1	User-Plane Latency

HARQ retransmission probability is estimated based on both 10% and 30% HARQ retransmission probabilities. The HARQ RTT is estimated as 8*TTI as shown below for FDD duplex scheme and a synchronous HARQ scheme. The HARQ RTT for the TDD mode assuming DL:UL ratio of 5:3 and one DL/UL switching point per radio frame is 8*TTI.

Figure 7-2: Example synchronous HARQ in FDD duplex mode

The following reference model is used to estimate the user-plane latency.

MS

BS

1.85 ms

1.85 ms

HARQ RTT

5 ms

1.85 ms

1.85 ms

TTI + Frame

Alignment

0.92 ms

0.92 ms

Figure 7-3: User-plane latency calculation model

The following table summarizes the result of the analysis:

Table 7- 17: User-plane latencies for 10% and 30% Probability of HARQ Retransmissions

| Step | Description | User-Plane Latency (10% HARQ Retransmission Probability) | User-Plane Latency (30% HARQ Retransmission Probability) |
| --- | --- | --- | --- |
| 0 | AMS wakeup time | Implementation Dependent | Implementation Dependent |
| 1 | AMS Processing Delay | 3 * 0.617 = 1.85 ms | 3 * 0.617 = 1.85 ms |
| 2 | Queuing/Frame Alignment | FDD: 0.31 TDD: 2.5 ms | FDD: 0.31 TDD: 2.5 ms |
| 3 | TTI for UL data packet (Piggy back scheduling information) | 0.617 ms | 0.617 ms |
| 4 | HARQ Retransmission | 0.1 * 5 ms | 0.3 * 5 ms |
| 5 | ABS Processing Delay | 3 * 0.617 = 1.85 ms | 3 * 0.617 = 1.85 ms |
|  | Total one way access delay | FDD: 5.13 ms TDD: 7.32 ms | FDD: 6.13 ms TDD: 8.32 ms |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced user-plane latency.

7.2.5.2	Control-Plane Latency

Table 7-18 contains assumptions and results for control-plane latency analysis for the proposed RIT.

Table 7-18: Control-plane Latency for 10% and 30% Probability of HARQ Retransmissions

| Step | Description | Control-Plane Latency (10% HARQ Retransmission Probability) | Control-Plane Latency (30% HARQ Retransmission Probability) |
| --- | --- | --- | --- |
| 0 | AMS wakeup time | Implementation dependent | Implementation dependent |
| 1 | DL scanning and synchronization + Acquisition of the broadcast channel (system configuration information) for initial system entry | < 50 ms Assuming that S-SFH SP2 that contains network re-entry information is transmitted every 50 ms. This could further reduce if SP2 is transmitted more frequently | < 50 ms Assuming that S-SFH SP2 that contains network re-entry information is transmitted every 50 ms. This could further reduce if SP2 is transmitted more frequently |
| 2 | Random access procedure (UL CDMA Code + ABS Processing + DL CDMA_ALLOC_IE) | 5 ms | 5 ms |
| 3 | Initial ranging (RNG-REQ + ABS processing + RNG-RSP) + HARQ retransmission of one message at 10% or 30%, only first-order estimation | HARQ case: 1 frame * 0.9*0.9 + 2 frame * 2*0.1*0.9 + 3 frame * 0.1*0.1 = 1.2 frame = 6 ms The assumption is the message will either succeed in #1 transmission with probability=0.9 or succeed in #2 transmission with probability=0.1 | HARQ case: 1 frame * 0.7*0.7 + 2 frame * 2*0.3*0.7 + 3 frame * 0.3*0.3 = 1.6 frame = 8 ms The assumption is the message will either succeed in #1 transmission with probability=0.7 or succeed in #2 transmission with probability=0.3 |
| 4 | Capability negotiation (SBC-REQ + ABS processing + SBC-RSP) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 5 | Authorization and authentication/key exchange (PKM-REQ + ABS processing + PKM-RSP + …) +HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 6 | Registration (REG-REQ + ABS/ASN-GW processing + REG-RSP) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
| 7 | RRC connection establishment (DSA-REQ + ABS processing + DSA-RSP + DSA-ACK) + HARQ retransmission | < 5 ms (0.1 * 5 ms for HARQ ReTX) | < 5 ms (0.3 * 5 ms for HARQ ReTX) |
|  | Total C-plane connection establishment delay | < 31 ms | < 33 ms |
|  | Total IDLE_STATE –> ACTIVE_ACTIVE delay | < 81 ms | < 83 ms |

Therefore, IEEE confirms that the proposed RIT meets the requirements for IMT-Advanced control-plane latency.

7.2.5.3	Intra-RIT Handover Interruption Time

In the proposed RIT, the handover (HO) can be initiated by either the AMS or the S-ABS. The HO process is initiated when the AMS issues a HO request to the S-ABS or when the S-ABS issues a HO command to the AMS. The HO request/command is issued following Mobile-Assisted HO (MAHO) procedures. The AMS acquires the network topology through either S-ABS broadcasts or unicast signaling messages. The S-ABS also provides the AMS with the relevant trigger conditions to initiate or cancel neighbor-cell measurements, measurement reporting and HO requests. If required, scanning intervals for neighbor-cell measurements are provided by the S-ABS unilaterally or at the request of the AMS. The HO request/command is an outcome of these measurements and reporting.

The HO procedure is divided into four phases; namely, HO initiation, HO preparation, HO execution, and network reentry. In addition, HO cancellation procedure is defined to allow AMS cancel a HO procedure.

Figure 7-4: Generic HO Procedure in the Proposed RIT

7.2.5.4	Intra-RIT, Intra-frequency handover

The following describes the HO procedure with reference to the message sequence flow shown in Figure 7-4.

HO Initiation

During handover initiation phase, the AMS requests HO based on trigger conditions defined in ABS AAI_NBR-ADV broadcast message {4}. ABS may also initiate HO without AMS request by sending AAI_HO-CMD message to the AMS {4}.

HO Preparation

During HO preparation phase, the T-ABS and AMS obtain relevant information for HO optimization (e.g., AMS identity, AMS security context, T-ABS system information, service flows, etc.) via the S-ABS and the backbone network. For optimized network re-entry, the T-ABS(s) may allocate a dedicated ranging code/opportunity and resource allocations to the AMS via the S-ABS through the AAI-HO-CMD message. The message includes one or more T-ABSs, network re-entry action time and S-ABS link disconnect time. The S-ABS also indicates in the HO command (AAI-HO-CMD) whether the AMS performs CDMA ranging. The HO preparation phase completes with the exchange of AAI-HO-CMD/AAI-HO-IND messages resulting in the selection of a T-ABS and the S-ABS disconnect time.

HO execution

During HO execution, AMS responds with its T-ABS selection in the AAI-HO-IND message if multiple T-ABSs are included in AAI-HO-CMD. The AMS continue data communications with S-ABS, until the S-BS stops sending DL data and providing UL allocations to the AMS after expiration of the action/disconnect time included in the AAI-HO-CMD message. Any AMS bandwidth requests before the disconnect time that cannot be provided by the S-ABS are forwarded to the T-ABS. The S-ABS retains the connections, MAC state machine, and un-transmitted/unacknowledged data associated with the AMS for service continuation in case of HO cancellation until the expiration of resource-related timers.

Network Re-entry

If AMS performs network re-entry without CDMA ranging, the T-ABS and AMS can start data communications after the defined action time. If AMS performs CDMA ranging based network re-entry, the AMS performs network re-entry at the T-ABS using either a dedicated ranging code if allocated through AAI-HO-CMD or a random handover ranging code at the designated ranging opportunity if specified or otherwise the earliest ranging opportunity. Upon reception of the ranging code, the T-ABS allocates UL resources for AMS to send RNG-REQ message and UL data if needed. The T-ABS and AMS finishes AAI-RNG-REQ/RSP transactions by the predefined timer, which serves the purpose of key validation for each other. This validation process does not impact data communication and can happen in parallel as defined in seamless handover procedure in {1}.

HO Cancellation

The handover can be canceled by the AMS at any time during the HO process. The HO cancellation is initiated before the expiration of resource-related timers. The network can advertise HO cancellation trigger conditions.

7.2.5.5	Intra-RIT, Inter-frequency handover

The first three phases are identical with the case of Intra-RIT, Intra-frequency handover. The last phase, network re-entry, differs in that there is no option to skip CDMA ranging in the T-ABS.

For inter-frequency HO, the S-ABS always commands the AMS to perform network re-entry with CDMA ranging process. Depending on the method indicated by the HO command from the S-ABS, the AMS performs network re-entry at the T-ABS using either a dedicated ranging code if allocated through AAI-HO-CMD or a random handover ranging code at the designated ranging opportunity if specified or otherwise the earliest ranging opportunity. Upon reception of the ranging code, the T-ABS responds with AAI-RNG-RSP to instruct the AMS to adjust its UL transmission timing and power, allocate UL resources for AMS to send AAI-RNG-REQ message and UL data if needed. The rest of the procedure is the same as in intra-frequency HO case.

Table 7-19: handover interruption time

| Scenario | Interruption Time |
| --- | --- |
| Intra-RIT, Intra-frequency | 5 ms |
| Intra-RIT, Inter-frequency | 15 to 25ms (additional time for AMS to adjust RF and obtain CDMA ranging opportunity) |

Additionally, Inter-RIT, Intra-RIT, Intra-frequency, and Inter-system handover to other IMT systems is supported by the RIT.

The following diagram illustrates the seamless HO service flow in the proposed RIT.

Figure 7-5: Example of a seamless HO service flow in the proposed RIT

7.2.5.6	Summary of the Handover procedure

- Initiation

- HO triggered at AMS, based on AMS measurements and S-ABS-defined trigger levels

- HO request from AMS to S-ABS, containing T-ABS list with preferences and measurement reports

- Preparation

- S-ABS and T-ABS backbone pre-notification procedures

- HO command from S-ABS to AMS with T-ABS details, Disconnect time (may allow communication during network re-entry)

- Execution

- AMS acknowledges S-ABS HO command with selected T-ABS and confirmation/rejection of Disconnect time

- S-ABS disconnects at Disconnect time, transfers AMS context to T-ABS over backbone network

IV.   Network re-entry

- HO Ranging in T-ABS, optional for intra-FA

- T-ABS notifies S-ABS of HO completion

- Network re-entry complete with established data path in T-ABS

Assumptions

- Intra-ASN Handovers (the serving and target base stations belong to the same ASN)

- Mobile-assisted Handover (predominant case)

- Mobile-initiated Handover (worst-case compared to ABS-initiated)

- Optimized Hard Handover (for intra-frequency AMS is frame-synchronized with S-ABS and T-ABS, AMS contexts including security context, transferred to T-ABS by S-ABS over the backbone network)

All measurements are based on the synchronization channels.

The following steps capture the HO procedure delay budget (Intra-RIT, Intra-Frequency):

Table 7-20: Intra-RAT Intra-FA Handover interruption time

| Step | Procedure | Estimated Latency (ms) |
| --- | --- | --- |
| 1 | The AMS initiates HO by sending an AAI-HO-REQ to the S-ABS. | 4 to 7 frames, 20 to 35ms |
| 2 | The S-ABS processes AAI-HO-REQ and sends HO REQUEST to one or more T-ABS. | 1 frames, 5 ms (HO-REQ from S-ABS to T-ABSs) |
| 3 | T-ABSs reply S-ABS with HO RESPONSE, which may include HO optimization related MAC pre-update information. | 2 frames, 10ms (T-ABSs process and reply + HO-RSP from T-ABSs to S-ABS) |
| 4 | S-ABS responds to AMS with AAI-HO-CMD containing T-ABS list, Disconnect time | 1 frame, 5 ms |
| 5 | AMS acknowledges S-ABS with AAI-HO-IND containing selected T-ABS and confirmation/rejection of Disconnect time (unsolicited UL grant) | 1 frame, 5 ms |
| 6 | At/After Action Time (=Disconnect Time), S-ABS transfer un-acknowledged data and new data, if any, to T-ABS for AMS data continuity at T-ABS | 0 to 2 frames, 0 to 10ms (R8 interface latency, see Figure 7-1) |
| 7 | AMS switches to T-ABS, acquires DL signal | 1 frame, 5 ms |
| 8 | AMS reads UL-MAP for unsolicited UL grant for AMS to send RNG-REQ message and data | 2 frames, 10 ms |
| 9 | AMS sends RNG-REQ to T-ABS | 1 frame, 5 ms |
| 10 | T-ABS responds with RNG-RSP with necessary information for AMS to perform UL synchronization. | 2 frames, 10 ms |
| 11 | AMS processes RNG-RSP | 1 frame, 5 ms |
| 12 | If necessary, repeat steps 8 – 11 K-times Note: The maximum value of K is calculated based upon the number of times that steps 8 to 11 could be repeated before expiration of a timer specified by S-ABS. | K * 5 frames, 0 to 25K ms |
| 13 | T-ABS and AMS continue data communication | 0 |
| HO Interruption time (Using Seamless HO) |  | summation of time required for steps 6 and 7= 0 to 15 ms |

Using seamless HO procedure, HO interruption time includes steps 6 and 7 in Table 6-6. Adding the times required for completion of steps 6 and 7results in a value of 0 to 15 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 10 ms.

For intra-RIT/Inter-Frequency HO, there will be following two changes:

Step-7 will be longer (implementation dependent) so that the AMS can calibrate its RF for inter-FA operation.

An additional step 7.1 is to be inserted between Step-7 and Step-8, which will be counted into HO interruption time.

Table 7-21: Additional step in calculation of inter-FA, intra-RIT handover interruption time

| 7.1 | The AMS waits for HO ranging opportunity to perform UL synchronization with dedicated ranging code (assigned by TABS during HO preparation.) (after CDMA ranging, UL synchronization procedures are not counted into HO interruption time according to the definition) | 1 to 4 frames, 5 to 20 ms (20ms is the worst case when no dedicated ranging opportunity is allocated for this HO instance. Most cases, T-ABS has knowledge of AMS capability and how fast it can switch RF, and therefore T-ABS can prepare the ranging opportunity right at the next frame, in which case it will be 5 ms) |
| --- | --- | --- |

In this case (inter-FA, intra-RIT) , the HO interruption time is the summation of time required for steps  6,  7, and 7.1, which results in HO interruption time of 5 to 35 ms. Considering the fact that steps 6 and 7 can be performed in parallel, maximum value for HO interruption could be reduced to 30 ms.

Therefore, IEEE confirms that the proposed RIT meets the IMT-Advanced HO interruption time requirements.

8	Compliance templates

# 8.1	Compliance template for services

|  | Service related minimum capabilities within the RIT/SRIT | Evaluator’s comments |
| --- | --- | --- |
| 4.2.4.1.1 | Support of a wide range of services Does the proposal support a wide range of services?: If bullets 4.2.4.1.1.1 - 4.2.4.1.1.3 are marked as "yes" then 4.2.4.1.1 is a "yes". YES / NO | The proposed RIT supports a number of QoS classes (see Section 10.10 of {4} for more details) that are designed to enable a wide range of services and applications. These services include but are not limited to the following: - Multicast and broadcast services (see Section 16 of {4} for more details) would allow support of IP-based multimedia applications such as real-time and non-real-time audio and video streaming, IP-TV, web-casts, etc. - Location based services (see Section 12 of {4} for more details) would allow support of location based applications such as interactive maps and navigation applications, etc. - Low one-way air-link transmission latency of less than 5 ms and short handoff interruption time of less than 20 ms would allow real-time applications such as interactive gaming, on-line collaborations, etc. - High capacity VoIP service is enabled (see Section 10 and 11 of {4} for more details) through efficient DL/UL control channel design, advanced DL/UL MIMO techniques, persistent and group scheduling schemes, low user and control plane latencies. - IP-based data services such as HTTP, e-mail, web-browsing, file transfer are enabled through high spectral efficiency, low user and control plane latencies and flexible QoS classes (see Section 7.2.5 for more details) |
| 4.2.4.1.1.1 | Ability to support basic conversational service class Is the proposal able to support basic conversational service class?: YES / NO | - Given that basic conversational service* is typically characterized by per user throughputs of 20 kbps and latencies of less than 50 ms, using baseline antenna configuration and 10 MHz bandwidth, the proposed RIT with average user throughput of greater than 2.6 Mbps in the DL and 1.3 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The handover interruption time for intra-Frequency Assignment (FA) is 5-10 ms and for inter-FA is 5 to 35 ms which both are significantly less the IMT-Advanced corresponding requirements which would enable a large number of service classes using the proposed RIT. - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. *see Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |
| 4.2.4.1.1.2 | Support of rich conversational service class Is the proposal able to support rich conversational service class?: YES / NO | - Given that rich conversational service* is typically characterized by per user throughputs of 5 Mbps and latencies of less than 20 ms, using baseline antenna configuration and 20 MHz bandwidth, the proposed RIT with average user throughput of greater than 5.2 Mbps in the DL and 2.6 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. *see Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |
| 4.2.4.1.1.3 | Support of conversational low delay service class Is the proposal able to support conversational low-delay service class? YES / NO | - Given that conversational low delay service* is typically characterized by per user throughputs of 150 kbps and latencies of less than 10 ms, using baseline antenna configuration and 10 MHz bandwidth, the proposed RIT with average user throughput of greater than 2.6 Mbps in the DL and 1.3 Mbps in the UL as well as one-way access latency of less than 5 ms does support this kind of service (see Sections 10 and 11 of {4} for more details) - The RIT minimum data rates for other supported bandwidths can be derived by scaling the above data rates by the bandwidth ratio. *see Section 3.2 of IST-2003-507581 WINNER D1.3 version 1.0, Final usage scenarios, at http://www.ist-winner.org/deliverables_older.html |

# 8.2	Compliance template for spectrum

|  | Spectrum capability requirements |
| --- | --- |
| 4.2.4.2.1 | Spectrum bands Is the proposal able to utilize at least one band identified for IMT?: YES / NO Specify in which band(s) the candidate RIT or candidate SRIT can be deployed. The proposed RIT supports deployment in all bands identified for IMT in ITU-R Radio Regulations. In addition, proposed RIT supports non-IMT bands below 6 GHz allocated to the Fixed Service and/or Mobile Service. (See below for more details on some of the band classes in which the proposed RIT can be deployed). Table 8-1: Some of supported frequency bands Band Class; UL AMS Transmit Frequency (MHz); DL AMS Receive Frequency (MHz); Duplex Mode. 1; 2300-2400; 2300-2400; TDD. 2; 2305-2320, 2345-2360; 2305-2320, 2345-2360; TDD. 2345-2360; 2305-2320; FDD. 3; 2496-2690; 2496-2690; TDD. 2496-2572; 2614-2690; FDD. 4; 3300-3400; 3300-3400; TDD. 5L; 3400-3600; 3400-3600; TDD. 3400-3500; 3500-3600; FDD. 5H; 3600-3800; 3600-3800; TDD. 6; 1710-1770; 2110-2170; FDD. 1920-1980; 2110-2170; FDD. 1710-1755; 2110-2155; FDD. 1710-1785; 1805-1880; FDD. 1850-1910; 1930-1990; FDD. 1710-1785, 1920-1980; 1805-1880, 2110-2170; FDD. 1850-1910, 1710-1770; 1930-1990, 2110-2170; FDD. 7; 698-862; 698-862; TDD. 776-787; 746-757; FDD. 788-793, 793-798; 758-763, 763-768; FDD. 788-798; 758-768; FDD. 698-862; 698-862; TDD/FDD. 824-849; 869-894; FDD. 880-915; 925-960; FDD. 698-716, 776-793; 728-746, 746-763; FDD. 8; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920; TDD. 9; 450-470; 450-470; TDD. 450.0-457.5; 462.5-470.0; FDD. |

# 8.3	Compliance template for technical performance

| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.1 Cell spectral efficiency (bit/s/Hz/cell) (4.1) | Indoor | Downlink | 3 | TDD: 6.93 FDD: 6.87 |  Yes  No | The proposed RIT exceeds required values in all test environments. See Section 7.2.1 for details. |
|  |  | Uplink | 2.25 | TDD: 5.99 FDD: 6.23 |  Yes  No |  |
|  | Microcellular | Downlink | 2.6 | TDD: 3.22 FDD: 3.27 |  Yes  No |  |
|  |  | Uplink | 1.8 | TDD: 2.58 FDD: 2.72 |  Yes  No |  |
|  | Base coverage urban | Downlink | 2.2 | TDD: 2.41 FDD: 2.41 |  Yes  No |  |
|  |  | Uplink | 1.4 | TDD: 2.57 FDD: 2.69 |  Yes  No |  |
|  | High speed | Downlink | 1.1 | TDD: 3.23 FDD: 3.15 |  Yes  No |  |
|  |  | Uplink | 0.7 | TDD: 2.66 FDD: 2.77 |  Yes  No |  |
| 4.2.4.3.2 Peak spectral efficiency (bit/s/Hz) (4.2) | Not applicable | Downlink | 15 | TDD: 16.96 FDD: 17.79 |  Yes  No | The proposed RIT exceeds required values. See Section 7.2.4 for details. |
|  |  | Uplink | 6.75 | TDD: 9.22 FDD: 9.40 |  Yes  No |  |
| 4.2.4.3.3 Bandwidth (4.3) | Not applicable | Up to and including (MHz) | 40 | 20 MHz with single carrier; Up to and including 100 MHz with multi-carrier |  Yes  No | The proposed RIT supports multi-carrier operation that allows operation in any bandwidth as wide as 100 MHz by aggregating contiguous and/or non-contiguous RF carriers. |
|  |  | Scalability | Support of at least three band-width values(4) | 5, 7, 8.75, 10, and 20 MHz with single carrier operation, Maximum 100 MHz with multi-carrier operation |  Yes  No | The proposed RIT supports 5, 7, 8.75, 10 and 20 MHz with single carrier and also supports up to 100 MHz with multi-carrier operation |
| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Require-ment met? | Comments |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.4 Cell edge user spectral efficiency (bit/s/Hz) (4.4) | Indoor | Downlink | 0.1 | TDD: 0.260 FDD: 0.253 |  Yes  No | The proposed RIT exceeds required values in all test environments. See Section 7.2.1 for details. |
|  |  | Uplink | 0.07 | TDD: 0.426 FDD: 0.444 |  Yes  No |  |
|  | Microcellular | Downlink | 0.075 | TDD: 0.092 FDD: 0.097 |  Yes  No |  |
|  |  | Uplink | 0.05 | TDD: 0.111 FDD: 0.119 |  Yes  No |  |
|  | Base coverage urban | Downlink | 0.06 | TDD: 0.069 FDD: 0.069 |  Yes  No |  |
|  |  | Uplink | 0.03 | TDD: 0.109 FDD: 0.114 |  Yes  No |  |
|  | High speed | Downlink | 0.04 | TDD: 0.093 FDD: 0.091 |  Yes  No |  |
|  |  | Uplink | 0.015 | TDD: 0.119 FDD: 0.124 |  Yes  No |  |
| 4.2.4.3.5 Control plane latency (ms) (4.5.1) | Not applicable | Not applicable | Less than 100 ms | < 81 ms |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.6 User plane latency (ms) (4.5.2) | Not applicable | Not applicable | Less than 10 ms | TDD: 7.32 ms FDD: 5.13 ms |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.7 Mobility classes (4.6) | Indoor | Uplink | Stationary, pedestrian | supported |  Yes  No | Proposed RIT supports required Mobility classes in all test environments. |
|  | Microcellular | Uplink | Stationary, pedestrian, vehicular up to 30 km/h | supported |  Yes  No |  |
|  | Base coverage urban | Uplink | Stationary, pedestrian, vehicular | supported |  Yes  No |  |
|  | High speed | Uplink | High speed vehicular, vehicular | supported |  Yes  No |  |
| 4.2.4.3.8 Mobility Traffic channel link data rates (bit/s/Hz) (4.6) | Indoor | Uplink | 1.0 | LoS TDD: 3.76 NLoS TDD: 3.41 LoS FDD: 3.86 NLoS FDD: 3.56 |  Yes  No | Proposed RIT exceeds required values in all test environments. See Section 7.2.3 for detailed information. |
|  | Microcellular | Uplink | 0.75 | LoS TDD: 1.81 NLoS TDD: 1.50 LoS FDD: 1.72 NLoS FDD: 1.51 |  Yes  No |  |
|  | Base coverage urban | Uplink | 0.55 | LoS TDD: 1.72 NLoS TDD: 1.30 LoS FDD: 1.63 NLoS FDD: 1.34 |  Yes  No |  |
|  | High speed | Uplink | 0.25 | LoS TDD: 1.70 NLoS TDD: 1.23 LoS FDD: 1.61 NLoS FDD: 1.27 |  Yes  No |  |
| Minimum technical requirements item (4.2.4.3.x), units, and Report ITU-R M.2134 section reference(1) | Category |  | Required value | Value(2), (3) | Requirement met? | Comments |
|  | Test environment | Downlink or uplink |  |  |  |  |
| 4.2.4.3.9 Intra-frequency hand-over interruption time (ms) (4.7) | Not applicable | Not applicable | 27.5 | 5-10 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.10 Inter-frequency handover interruption time within a spectrum band (ms) (4.7) | Not applicable | Not applicable | 40 | 5-35 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.11 Inter-frequency handover interruption time between spectrum bands (ms) (4.7) | Not applicable | Not applicable | 60 | 5-35 |  Yes  No | Proposed RIT meets required value. See Section 7.2.5 for detailed information. |
| 4.2.4.3.12 Inter-system handover (4.7) | Not applicable | Not applicable | Not applicable | Not applicable |  Yes  No | The proposed RIT supports inter-RAT handover to and from other IMT-2000 and IMT-Advanced technologies. See Section 10.3 of {4} for detailed information on inter-RAT handover in the proposed RIT. |
| 4.2.4.3.13 Number of supported VoIP users (active users/ sector/MHz) (4.8) | Indoor | As defined in Report ITU-R M.2134 | 50 | TDD: 140 FDD: 139 |  Yes  No | Reported numbers are minimum of UL and DL. Proposed RIT exceeds required values in all test environments. See Section 7.2.2 for detailed information. |
|  | Microcellular | As defined in Report ITU-R M.2134 | 40 | TDD: 82 FDD: 77 |  Yes  No |  |
|  | Base coverage urban | As defined in Report ITU-R M.2134 | 40 | TDD: 74 FDD: 72 |  Yes  No |  |
|  | High speed | As defined in Report ITU-R M.2134 | 30 | TDD: 89 FDD: 90 |  Yes  No |  |
| (1) As defined in Report ITU-R M.2134. (2) According to the evaluation methodology specified in Report ITU-R M.2135. (3) Mandatory when “no” is checked, optional when “yes” is checked. (4) Refer to Report ITU-R M.2135, § 7.4.1. |  |  |  |  |  |  |

Annex 3

Simulation Assumptions and Configuration Parameters for IMT-Advanced Test Environments

## Table A3-1: OFDMA and frame parameters

| Parameter | Description | RIT Parameters for Indoor Environment |  | RIT Parameters for Urban Micro-cellular, Urban Macro-cellular, High Speed Environments |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TDD | FDD | TDD | FDD |  |
| fc | Carrier frequency | 3.4 GHz |  | Urban Micro-cellular: 2.5 GHz |  |  |
|  |  |  |  | Urban Macro-cellular: 2.0 GHz |  |  |
|  |  |  |  | High Speed: 0.8 GHz |  |  |
| BW | Total bandwidth | 40 MHz for data-only (2x20 MHz) | 2x20 MHz for data-only | 20 MHz for data-only |  | 2x10 MHz for data-only |
|  |  | 10 MHz for VoIP | 5 + 5 MHz for VoIP | 10 MHz for VoIP |  | 5 + 5 MHz for VoIP |
| NFFT | Number of points in full FFT | 2x2048 for data-only | 2048 for data-only | 2048 for data-only |  | 1024 for data-only |
|  |  | 1024 for VoIP | 512 for VoIP | 1024 for VoIP |  | 512 for VoIP |
| FS | Sampling frequency | 44.8 MHz for data-only | 22.4 MHz for data-only | 22.4 MHz for data-only |  | 11.2 MHz for data-only |
|  |  | 11.2 MHz for VoIP | 5.6 MHz for VoIP | 11.2 MHz for VoIP |  | 5.6 MHz for VoIP |
| Δf | Subcarrier spacing | 10.9375 kHz |  |  |  |  |
| To=1Δf | OFDMA symbol duration without cyclic prefix | 91.43 us |  |  |  |  |
| CP | Cyclic prefix length (fraction of To) | 1/16 |  |  |  |  |
| Ts | OFDMA symbol duration with cyclic prefix | 97.143 us |  |  |  |  |
| TF | Frame length | 5 ms |  |  |  |  |
| NF | Number of OFDMA symbols in frame (excluding switching gaps) | 50 | 51 | 50 | 51 |  |
| RDL−UL | Ratio of DL to UL (TDD mode) | 5 DL subframes, 3 UL subframes for data-only | 8 DL subframes for DL and UL | 5 DL subframes, 3 UL subframes for data-only | 8 DL subframes for DL and UL |  |
|  |  | 4 DL subframes, 4 UL subframes for VoIP | 8 DL subframes for DL and UL | 4 DL subframes, 4 UL subframes for VoIP | 8 DL subframes for DL and UL |  |
| Tduplex | Duplex time | TTG+RTG = 142.85 µs | N/A | TTG+RTG = 142.85 µs | N/A |  |

## Table A3-2: DL configuration

| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| --- | --- | --- | --- |
| Basic modulation for data | Modulation schemes for data | QPSK, 16QAM, 64QAM |  |
| Basic modulation for control | Modulation schemes for control | QPSK |  |
| Duplexing scheme | TDD or FDD | TDD/FDD |  |
| Subchannelization for data allocations | Contiguous/Distributed Resource Units and associated permutations | - Scheme for InH and UMi: Sub-band CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6};12 equal-size allocations spanning the complete duration of the time resources (DL portion of the TDD frame, DL FDD frame) Scheme for UMa and RMa: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6}; 6 equal-size allocations spanning the complete duration of the time resources (DL portion of the TDD frame, DL FDD frame) |  |
| Subchannelization for VoIP allocations | Contiguous/Distributed Resource Units and associated permutations | Combination of the following two schemes (selection should be based on the maximization of the VoIP capacity): - Scheme 1: DRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6}; Scheme 2: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6} |  |
| Subchannelization for A-A-MAP | Contiguous/Distributed Resource Units and associated permutations | Distributed LRUs with frequency reuse 1 without FFR as defined in Sections 3.3.1.1-3.3.1.3 of {6} |  |
| DL pilot structure | Pilot structure, density etc. | Depends on the number of streams per allocation: 1 or 2 pilot streams from 2 stream interlaced pilot structure as defined in Section 3.3.1.4.1 of {6}with 2 dB pilot power boosting over data tone 3 or 4 pilot streams from 4 stream pilot structure as defined in in Section 3.3.1.4.1 of {6}with 0 dB pilot power boosting over data tone |  |
| Multi-antenna transmission format for data | Multi-antenna configuration and transmission scheme | - Scheme for InH and UMi: 6-bit Transformed Codebook or analog feedback based MU-MIMO using 4x2 configuration; adaptive switching among rank-1 CL-SU-MIMO, two stream CL-MU-MIMO, three stream CL-MU-MIMO and four stream CL-MU-MIMO - Scheme for UMa and RMa: MU-MIMO with long term beamforming using 4x2 configuration (20 ms reporting period for the long-term covariance matrix); adaptive switching among rank-1 CL-SU-MIMO, two stream CL-MU-MIMO, three stream CL-MU-MIMO and four stream CL-MU-MIMO |  |
| Multi-antenna transmission format for voip | Multi-antenna configuration and transmission scheme | Combination of the following two schemes (selection should be based on the maximization of the VoIP capacity): - Scheme 1: SU-MIMO using 4x2 configuration with SFBC + non-adaptive precoding Scheme 2: SU-MIMO with wideband beamforming using 4x2 configuration |  |
| Multi-antenna transmission format for A-A-MAP | Multi-antenna configuration and transmission scheme | Open-loop SFBC + non-adaptive precoding (TX diversity) |  |
| Receiver structure | MMSE/ML/MRC, Receiver interference awareness | MMSE for channel estimation and MMSE/MLD data detection |  |
| Data channel coding | Channel coding schemes | Convolutional Turbo Coding (CTC) 1/3 |  |
| Control channel coding for A-A-MAP | Channel coding schemes and block sizes | As described in Section 3.3.2.3.2.2 of {6} with MLRU size equal to 56 tones |  |
| Scheduling | Demonstrate performance / fairness criteria in accordance to traffic mix | Proportional Fair for full buffer data and delay-weighted Proportional Fair with persistent scheduling for VoIP |  |
| Link adaptation | Modulation and Coding Schemes (MCS), CQI feedback delay / error | Choice of 16 MCS schemes inclusive of coding rate and rate matching, see Section 11.13 of {4} |  |
| Link to system mapping | MI based PHY abstraction | RBIR or MMIB by providing the necessary supporting PHY abstraction link curves |  |
| HARQ | Chase combining/ incremental redundancy, synchronous/asynchronous, adaptive/non-adaptive ACK/NACK delay, Maximum number of retransmissions, retransmission delay | Incremental Redundancy Asynchronous, adaptive, 3 subframe ACK/NACK delay, maximum 4 HARQ retransmissions, minimum retransmission delay 3 subframes |  |
| Power allocation | Subcarrier power allocation | Equal power per subcarrier; power boosting should be used only for A-A-MAP allocations (separate power boosting for each A-A-MAP allocation) |  |
| Interference model | Co-channel interference model, fading model for interferers, number of major interferers, threshold | Explicitly modeled Average interference on used subcarriers per CRU (subband or miniband) in PHY abstraction {3} |  |
| Frequency reuse | Frequency reuse pattern | 3 sectors with frequency reuse 1 |  |
| Control signalling | Message/signaling format, overheads | SFH, A-MAP, ACK/NACK, and Power Control overhead/channels as described in Section 3.3.2.2 of {6} Dynamic transmission modeled for A-A-MAP; percentage of signalling error modeled for fixed control overhead, should be derived from corresponding link curve for each test environment; |  |
| Persistent scheduling | Persistent scheduling for VoIP only | Persistent scheduling for individual VoIP connections as described in Section 3.2.1 of {6} |  |
| Control channel overhead | L1/L2 Overhead | Dynamic overhead model whenever possible |  |
| Antenna tilt angle | Antenna tilt angle | InH: 0 deg. UMi: 12 deg. UMa: 12 deg. RMa: 6 deg. |  |

## Table A3-3: UL configuration

| Topic | Description | RIT Parameters for Indoor and Urban Micro-Cellular Environments | RIT Parameters for Urban Macro-Cellular and High Speed Environments |
| --- | --- | --- | --- |
| Basic modulation for data | Modulation schemes for data | QPSK, 16QAM, 64QAM |  |
| Basic modulation for control | Modulation schemes for control | Dependent on feedback channel (FBCH, HARQ,BW-REQ, Ranging, Sounding) as defined in Section 3.3.5 of {6} |  |
| Duplexing scheme | TDD or FDD | TDD/FDD |  |
| Subchannelization for data allocations | Contiguous/Distributed Resource Units and associated permutations | - Scheme for InH and UMi: Sub-band CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.4.1-3.3.4.3 of {6}; 12 equal-size allocations spanning the complete duration of the time resources (UL portion of the TDD frame, UL FDD frame) - Scheme for UMa and RMa: Miniband CRUs with frequency reuse 1 without FFR as defined in Sections 3.3.4.1-3.3.4.3 of {6}; 6 equal size allocations spanning the complete duration of the time resources (UL portion of the TDD frame, UL FDD frame) |  |
| Subchannelization for VoIP allocations | Contiguous/Distributed Resource Units and associated permutations | DRUs with frequency reuse 1 without FFR as defined in 3.3.4.1-3.3.4.3 of {6} |  |
| UL pilot structure | Pilot structure, density etc. | 1/2 stream pilot structure as defined in 3.3.4.4 of {6} without pilot power boosting |  |
| Multi-antenna transmission format for data | Multi-antenna configuration and transmission scheme | - Scheme for InH and UMi: 3-bit Codebook based MU- MIMO using 2x4 configuration; adaptive switching between single user and collaborative spatial multiplexing - Scheme for UMa and RMa: MU-MIMO with long term beamforming using 2x4 configuration; adaptive switching between single-user and collaborative spatial multiplexing |  |
| Multi-antenna transmission format for VoIP | Multi-antenna configuration and transmission scheme | SU-MIMO using 2x4 configuration with SFBC + non-adaptive precoding |  |
| Multi-antenna transmission format for control | Multi-antenna configuration and transmission scheme | 2x4 antenna configuration |  |
| Receiver structure | MMSE/ML/MRC, Receiver interference awareness | MMSE for channel estimation and MMSE/MLD for data detection |  |
| Data channel coding | Channel coding schemes | Convolutional Turbo Coding (CTC) 1/3 |  |
| Control channel coding | Channel coding schemes and block sizes | As described in Section 3.3.5 of {6} for FBCH, HARQ, BW-REQ, Ranging, Sounding |  |
| Scheduling | Demonstrate performance / fairness criteria in accordance to traffic mix | Proportional Fair for full buffer data and delay-weighted Proportional Fair with persistent scheduling for VoIP |  |
| Link adaptation | Modulation and Coding Schemes (MCS), CQI feedback delay / error | Choice of 16 MCS schemes inclusive of coding rate and rate matching, see Section 11.13 of {4} |  |
| Link to system mapping | MI based PHY abstraction | RBIR or MMIB by providing the necessary supporting PHY abstraction link curves |  |
| HARQ | Chase combining/ incremental redundancy, synchronous/asynchronous, adaptive/non-adaptive ACK/NACK delay, Maximum number of retransmissions, retransmission delay | Incremental Redundancy Synchronous, non-adaptive, 3 subframe ACK/NACK delay, maximum 4 HARQ retransmissions, minimum retransmission delay 3 subframes |  |
| Power control | Open loop/closed loop | Open loop power control as described in 3.3.5.4 of {6}; values for γ and SINRMIN should be chosen such that the average IoT meets the IMT-Advanced requirement |  |
| Interference model | Co-channel interference model, fading model for interferers, number of major interferers, threshold | Explicitly modeled Average interference on used subcarriers per CRU (subband or miniband) in PHY abstraction {3} |  |
| Frequency reuse | Frequency reuse pattern | 3 sectors with frequency reuse 1 |  |
| Control signalling | Message/signaling format, overheads | Initial ranging, periodic ranging, handover ranging, bandwidth request fast feedback/CQI channel, sounding overheads modeled as described in 3.3.5 of {6} Percentage of signalling error modeled for fixed control overhead should be derived from corresponding link curve for each test environment |  |
| Persistent scheduling | Persistent scheduling for VoIP only | Persistent scheduling for individual VoIP connections as described in Section 3.2.1 of {6} |  |
| Control channel overhead | L1/L2 Overhead | Dynamic overhead model whenever possible |  |
| Antenna tilt angle | Antenna tilt angle | InH: 0 deg. UMi: 12 deg. UMa: 12 deg. RMa: 6 deg. |  |

## Table A3-4: VoIP configuration

| Parameter | Characterization |
| --- | --- |
| Codec | Source rate 12.2 kbit/s |
| Voice Activity Factor (VAF) | 50% (c=0.01, d=0.99) |
| Speech payload | 33 bytes, encoder frame every 20 ms |
| Silence Insertion Descriptor (SID) payload | 7 bytes, SID packet every 160ms during silence |
| Protocol overhead with compressed header | 4 byte MAC Header + Extended Header 4 byte (RTP/UDP/IP) 4 byte (RLC/security) 16 bits (CRC) |
| Packet size | 47 bytes (talk),/ 21 bytes (silence) |

Annex 4

Details of Simulation-related Results in the Compliance Template for Technical Performance

This Annex provides the results from all input contributors submitted for filling in the self-evaluation of the proposed RIT. The information below consists of 11 tables which include results for the following cases:

- Full buffer

  - DL and UL control overhead (Tables A4-1 and A4-2)

  - DL and UL cell spectral efficiency and cell edge SE for TDD (Tables A4-3 and A4-4) and FDD (Tables A4-5 and A4-6) systems

- VoIP capacity

  - DL and UL VoIP capacity numbers for TDD (Table A4-7) and FDD (Table A4-8)

- Mobility requirement

  - UL SINR CDF for link simulation (Table A4-9)

  - TDD and FDD systems (Tables A4-10 and A4-11)

In the tables below, reported data is based on {12}, {13}, and {14} with data attributed to

Alcatel-Lucent Shanghai Bell, Clearwire, ETRI, Fujitsu, Hitachi, Intel Corporation, ITRI, KDDI R&D Laboratories, LG Electronics, MediaTek Inc., Mitsubishi Electric, Motorola Inc., NEC, Samsung Electronics, Toshiba, and UQ Communications.

In some cases, data from individual sources is reported along with an average value.

Table A4-1: Control overhead (in %) for TDD per test environment and link (downlink/uplink)

| Test Environment | Downlink | Uplink |
| --- | --- | --- |
| Indoor (InH) | 9.19 | 7.85 |
| Mirocellular (UMi) | 12.33 | 12.60 |
| Base coverage urban (UMa) | 11.17 | 9.23 |
| High speed (RMa) | 11.15 | 8.34 |

Table A4-2: Control overhead (in %) for FDD per test environment and link (downlink/uplink)

| Test Environment | Downlink | Uplink |
| --- | --- | --- |
| Indoor (InH) | 9.74 | 6.02 |
| Mirocellular (UMi) | 16.28 | 10.58 |
| Base coverage urban (UMa) | 13.77 | 8.01 |
| High speed (RMa) | 13.63 | 6.51 |

Table A4-3: DL spectral efficiency for TDD

| Test environment | ITU Requirement |  | Source 10 | Source 3 | Source 1 | Source 6 | Source 8 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 3.0 |  |  | 6.75 |  |  | 7.10 | 6.93 |
|  | Cell-edge user | 0.10 |  |  | 0.235 |  |  | 0.286 | 0.260 |
| Microcellular (UMi) | Cell | 2.6 | 3.04 | 3.18 | 3.45 | 3.13 |  | 3.28 | 3.22 |
|  | Cell-edge user | 0.075 | 0.076 | 0.089 | 0.087 | 0.097 |  | 0.112 | 0.092 |
| Base coverage urban (UMa) | Cell | 2.2 | 2.36 | 2.32 | 2.62 | 2.40 | 2.35 | 2.41 | 2.41 |
|  | Cell-edge user | 0.06 | 0.060 | 0.071 | 0.071 | 0.077 | 0.064 | 0.069 | 0.069 |
| High speed (RMa) | Cell | 1.1 |  |  | 3.58 |  |  | 2.88 | 3.23 |
|  | Cell-edge user | 0.04 |  |  | 0.095 |  |  | 0.092 | 0.093 |

Table A4-4: UL spectral efficiency for TDD

| Test environment | ITU Requirement |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 2.25 | 5.20 |  | 6.79 | 5.99 |
|  | Cell-edge user | 0.07 | 0.361 |  | 0.491 | 0.426 |
|  | Average IoT (dB) < 10 dB |  | 9.37 |  | 8.70 |  |
| Microcellular (UMi) | Cell | 1.8 | 2.60 | 2.53 | 2.62 | 2.58 |
|  | Cell-edge user | 0.05 | 0.137 | 0.091 | 0.106 | 0.111 |
|  | Average IoT (dB) < 10 dB |  | 9.00 | 9.30 | 9.80 |  |
| Base coverage urban (UMa) | Cell | 1.4 | 2.38 | 2.49 | 2.86 | 2.57 |
|  | Cell-edge user | 0.03 | 0.113 | 0.091 | 0.123 | 0.109 |
|  | Average IoT (dB) < 10 dB |  | 9.59 | 9.90 | 9.90 |  |
| High speed (RMa) | Cell | 0.7 | 2.45 |  | 2.87 | 2.66 |
|  | Cell-edge user | 0.015 | 0.125 |  | 0.114 | 0.119 |
|  | Average IoT (dB) < 10 dB |  | 8.60 |  | 9.80 |  |

Table A4-5: DL spectral efficiency for FDD

| Test environment | ITU Requirement |  | Source 3 | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 3.0 |  | 6.85 |  | 6.89 | 6.87 |
|  | Cell-edge user | 0.10 |  | 0.239 |  | 0.266 | 0.253 |
| Microcellular (UMi) | Cell | 2.6 | 3.05 | 3.57 | 3.30 | 3.14 | 3.27 |
|  | Cell-edge user | 0.075 | 0.089 | 0.090 | 0.101 | 0.108 | 0.097 |
| Base coverage urban (UMa) | Cell | 2.2 | 2.23 | 2.63 | 2.48 | 2.30 | 2.41 |
|  | Cell-edge user | 0.06 | 0.060 | 0.069 | 0.082 | 0.063 | 0.069 |
| High speed (RMa) | Cell | 1.1 |  | 3.58 |  | 2.72 | 3.15 |
|  | Cell-edge user | 0.04 |  | 0.095 |  | 0.087 | 0.091 |

Table A4-6: UL spectral efficiency for FDD

| Test environment | ITU Requirement |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor (InH) | Cell | 2.25 | 5.40 |  | 7.06 | 6.23 |
|  | Cell-edge user | 0.07 | 0.377 |  | 0.511 | 0.444 |
|  | Average IoT (dB) < 10 dB |  | 9.45 |  | 8.70 |  |
| Microcellular (UMi) | Cell | 1.8 | 2.66 | 2.75 | 2.73 | 2.72 |
|  | Cell-edge user | 0.05 | 0.141 | 0.107 | 0.110 | 0.119 |
|  | Average IoT (dB) < 10 dB |  | 9.10 | 9.40 | 9.80 |  |
| Base coverage urban (UMa) | Cell | 1.4 | 2.46 | 2.66 | 2.96 | 2.69 |
|  | Cell-edge user | 0.03 | 0.117 | 0.097 | 0.128 | 0.114 |
|  | Average IoT (dB) < 10 dB |  | 9.39 | 9.80 | 9.90 |  |
| High speed (RMa) | Cell | 0.7 | 2.54 |  | 2.99 | 2.77 |
|  | Cell-edge user | 0.015 | 0.130 |  | 0.118 | 0.124 |
|  | Average IoT (dB) < 10 dB |  | 9.05 |  | 9.80 |  |

Table A4-7: VoIP capacity for TDD

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | UL | 154 |  | 176 |  |
|  | DL | 146 |  | 135 |  |
|  | Min of UL and DL | 146 |  | 135 | 140 |
|  | ITU requirement | 50 | 50 | 50 | 50 |
| Microcellular (UMi) | UL | 99 |  | 110 |  |
|  | DL | 84 | 86 | 77 |  |
|  | Min of UL and DL | 84 | 86 | 77 | 82 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| Base coverage urban (UMa) | UL | 93 |  | 98 |  |
|  | DL | 78 | 78 | 68 |  |
|  | Min of UL and DL | 78 | 78 | 68 | 74 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| High speed (RMa) | UL | 101 |  | 106 |  |
|  | DL | 99 |  | 80 |  |
|  | Min of UL and DL | 99 |  | 80 | 89 |
|  | ITU requirement | 30 | 30 | 30 | 30 |

Table A4-8: VoIP capacity for FDD

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | UL | 156 |  | 176 |  |
|  | DL | 144 |  | 134 |  |
|  | Min of UL and DL | 144 |  | 134 | 139 |
|  | ITU requirement | 50 | 50 | 50 | 50 |
| Microcellular (UMi) | UL | 100 |  | 104 |  |
|  | DL | 80 | 84 | 68 |  |
|  | Min of UL and DL | 80 | 84 | 68 | 77 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| Base coverage urban (UMa) | UL | 94 |  | 96 |  |
|  | DL | 74 | 80 | 64 |  |
|  | Min of UL and DL | 74 | 80 | 64 | 72 |
|  | ITU requirement | 40 | 40 | 40 | 40 |
| High speed (RMa) | UL | 102 |  | 100 |  |
|  | DL | 96 |  | 84 |  |
|  | Min of UL and DL | 96 |  | 84 | 90 |
|  | ITU requirement | 30 | 30 | 30 | 30 |

Table A4-9: UL SINR distribution for mobility requirement

Table A4-10: Mobility spectral efficiency (TDD)

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | LOS | 3.51 |  | 4.00 | 3.76 |
|  | NLOS | 3.41 |  |  | 3.41 |
|  | ITU requirement | 1.0 | 1.0 | 1.0 | 1.0 |
| Microcellular (UMi) | LOS | 1.64 |  | 1.98 | 1.81 |
|  | NLOS | 1.33 | 1.66 |  | 1.50 |
|  | ITU requirement | 0.75 | 0.75 | 0.75 | 0.75 |
| Base coverage urban (UMa) | LOS | 1.58 |  | 1.85 | 1.72 |
|  | NLOS | 1.26 | 1.33 |  | 1.30 |
|  | ITU requirement | 0.55 | 0.55 | 0.55 | 0.55 |
| High speed (RMa) | LOS | 1.54 |  | 1.85 | 1.70 |
|  | NLOS | 1.23 |  |  | 1.23 |
|  | ITU requirement | 0.25 | 0.25 | 0.25 | 0.25 |

Table A4-11: Mobility spectral efficiency (FDD)

| Test environment |  | Source 1 | Source 6 | Source 9 | Average |
| --- | --- | --- | --- | --- | --- |
| Indoor (InH) | LOS | 3.64 |  | 4.08 | 3.86 |
|  | NLOS | 3.56 |  |  | 3.56 |
|  | ITU requirement | 1.0 | 1.0 | 1.0 | 1.0 |
| Microcellular (UMi) | LOS | 1.70 |  | 1.74 | 1.72 |
|  | NLOS | 1.41 | 1.60 |  | 1.51 |
|  | ITU requirement | 0.75 | 0.75 | 0.75 | 0.75 |
| Base coverage urban (UMa) | LOS | 1.66 |  | 1.60 | 1.63 |
|  | NLOS | 1.30 | 1.38 |  | 1.34 |
|  | ITU requirement | 0.55 | 0.55 | 0.55 | 0.55 |
| High speed (RMa) | LOS | 1.61 |  | 1.60 | 1.61 |
|  | NLOS | 1.27 |  |  | 1.27 |
|  | ITU requirement | 0.25 | 0.25 | 0.25 | 0.25 |

Contact:	Michael Lynch
		E-mail: freqmgr@ieee.org

_______________