| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Features Restriction |  |
| Date Submitted | 2011-05-07 |  |
| Source(s) | Joey Chou Shantidev Mohant Kerstin Johnsson, Intel | Joey.Chou@intel.com |
| Re: | P802.16p SRD 80216p-10-0004r2 |  |
| Abstract | Propose the M2M Features Restriction |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Features Restriction

Joeyn Chou,

Shantidev Mohant,

Kerstin Johnsson,

Intel

- Introduction

The IEEE 802.16e and WiMAX Forum specification are mainly designed to support mobile applications. Therefore, some features are not required for certain M2M applicarions.

For example, the text below that is extracted from WiMAX Forum requirements specification [1] indicates that idle mode paging is not required for nomadic and portable usage models.

10.8.1 Sleep-Mode Operation (Normative)

Low power operation is essential for mobile, small, battery-operated devices. Sleep mode is critical for Simple Mobility and Full Mobility usage for that reason. It is also desired for Nomadic and Portable usage.

R-[328] The WiMAX network MAY provide support for sleep-mode operation in the Nomadic and Portable usage model scenarios.

R-[329] The WiMAX network SHALL provide support for sleep-mode operation in the full Simple Mobility and Full Mobility usage model.

10.8.2 Idle-Mode Operation and Paging Support (Normative)

Low power operation is essential for mobile, small, battery-operated devices. Idle mode is critical for Simple Mobility and Full Mobility usage for that reason. IP services using client pull does not require paging. Paging for other type of services is an open issue to be worked out, and should not incur high device and network complexity, and scalability issues. Paging support is not a requirement for nomadic and portable usage models.

R-[474] The WiMAX network SHALL provide support for idle-mode operation in the Simple Mobility and Full Mobility usage scenarios. Standby time SHOULD match or exceed averages for 3G (3GPP/3GPP2) devices providing similar services

This contribution proposes adding feature restriction to requirement 6.1.1 to disable certain features that are not required for certain M2M applications. This Change has the following benefit:

- Simplify the M2M devices that can reduce the power consumption.

- Reduce the CAPEX and OPEX for M2M operator and Service peovider.

2. References

- DRAFT-T31-001-R015v02-A, “WiMAX Forum® Network Requirements Recommendations and Requirements for Networks based on WiMAX Forum CertifiedTM Products” Sep. 2010, WiMAX Forum.

3. Proposed Text

3.1 Proposed Text #1

[Insert the following text at page 6:]

----------------------------------------------  Text Start -----------------------------------------------

6.1		Low Power Consumption

The 802.16p system shall support mechanisms for low power consumption in M2M devices.

6.1.1	The 802.16p system shall provide optimized operation for M2M devices with no/low mobility to conserve power by disabling certain unnecessary features (e.g. sleep mode, idle mode paging).

----------------------------------------------  Text End -----------------------------------------------