| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of a new MGID field for DSA and DSC messages in IEEE 802.16p system |  |
| Date Submitted | 2011-07-04 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on 802.16p Amendment Working Document (AWD) Content for Session #74 |  |
| Abstract | This contribution provides a text for MGID reassignment procedure in IEEE 802.16p system. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of a new MGID field for DSA and DSC messages in IEEE 802.16p system

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon, Kwangjae Lim

ETRI

# Proposal

According to the section 6.3.1 in the AWD [1], the MGID may be added and changed by DSA and DSC procedure based on the IEEE 802.16e system, respectively. However, there is no MGID field in DSA and DSC messages in the current AWD. Therefore, we propose a new MGID field for DSA-REQ and DSC-REQ messages.

# References

[1] IEEE 802.16p-10/0018r2, “IEEE 802.16p Machine to Machine (M2M) Amendment Working Document (AWD),” May 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts.

- Texts deleted by this contribution: red strikeout texts.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Add Section 6.3.2.3.10 as follows;]

### 6.3.2.3.10 DSA-REQ message

Add a new subclause at the end of 6.3.2.3.10

When the DSA-REQ message is sent to an M2M device, the following TLV may be included:

MGID Tuple (see 11.13.43)

MGID which is added

[Remedy 2: Add Section 6.3.2.3.13 as follows;]

### 6.3.2.3.13 DSC-REQ (DSC request) message

Add a new subclause at the end of 6.3.2.3.13

When the DSC-REQ message is sent to an M2M device, the following TLV may be included:

MGID Tuple (see 11.13.43)

MGID which is changed

[Remedy 3: Add section 11.13.43 as follows;]

### 11.13 Service flow management

Add a new subclause at the end of 11.13

### 11.13.43 MGID field

The value of this field specifies MGID which is used for the associated flow. During connected mode, the MGID may be added by DSA-REQ message and may be changed by DSC-REQ message.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| MGID | [145/146].54 | 2 | Bits 0-14: Indicates MGID; Bit 15: Padding, Will be set to 0 | DSA-REQ DSC-REQ |

----------------- End of the text proposal ---------------------------------------------------------------------------------------