| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distinguish normal user and M2M device by MAC version |  |
| Date Submitted | 2011-7-10 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes distinguishing scheme between MS/AMS and M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distinguish normal user and M2M device by MAC version

Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee

Institute for Information Industry (III)

# Introduction

Since the M2M devices may perform initial network entry in the normal ranging resource, the 16p BS needs additional information to distinguish between normal users and M2M devices. Such that the 16p BS can perform admission control for the M2M devices if congestion happened in the backhaul network, or reply to the M2M devices with specific backoff parameters when ranging procedure is congested.

In this contribution, new MAC version encoding is proposed for IEEE 802.16p. When an M2M device performs initial network entry, the M2M device shall carry the new MAC version in the AAI-RNG-REQ/MOB_RNG-REQ message. The 16p BS can recognize the type of the entering subscriber and then reply to the subscriber according to the MAC version.

# Reference

[1] IEEE 802.16m-2011, “Part 16: Air Interface for Broadband Wireless Access Systems: Amendment 3: Advanced Air Interface,” May 2011.

[2] IEEE 802.16p-10/0018r2, “P802.16p Amendment Working Document (AWD),” May 2011.

- Text proposal for inclusion in the 802.16p AWD

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

11.1.3 MAC version encoding

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 148 | 1 | Maximum version number supported by the device sending the message. 0: Reserved 1–7: Indicates conformance with an earlier and/or obsolete version of IEEE Std 802.16 8: Indicates conformance with IEEE Std 802.16-2009 9: Indicates conformance with IEEE Std 802.16-2009 as amended by IEEE Std 802.16j-2009 10: Indicates conformance with IEEE Std 802.16-2009, as amended by IEEE Std 802.16j-2009, IEEE Std 802.16h-2010 and IEEE Std 802.16m-2011 11: Indicates conformance with IEEE Std 802.16-2009, as amended by IEEE Std 802.16m-2011 and IEEE Std 802.16p 1112–255: Reserved | PMP: DCD, RNG-REQ |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------