| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Handover in 16p System |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Guanghui Fan, Erik Colban Huawei Technologies | E-mail: fanguanghui@huawei.com ecolban@huawei.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document <IEEE 802.16p-10/0018r2> |  |
| Abstract | Proposal for handover scheme in 16p |  |
| Purpose | To be discussed and adopted in IEEE 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Handover in 16p System

Guanghui Fan, Erik Colban

Huawei Technologies

# Introduction

In 16m and 16e, the NBR-ADV message is broadcast periodically to expedite the scanning and handover procedures. The NBR-ADV message broadcasting interval is defined for each as a global value. However, most M2M devices are expected to be fixed or seldom move. In some deployment scenarios, the M2M devices may coexist with a number of legacy MSs or AMSs that may vary over time and sometimes be zero. For this reason, this proposal suggests to allow the BS to dynamically change the broadcasting interval of the NBR-ADV message.

This proposal also suggests that the devices report their mobility capabilities during the network entry or reentry procedure and also be able to request the ABS to transmit neighbor base station information and scanning opportunities by sending an AAI-SCN-REP message.

# Text Proposal

On page 11, add the following modification to Table 678 (AAI-RNG-REQ)

------------------------------------------------Start of the Proposed Text Part 1---------------------------------------------

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Device type | 1 | Type indication of M2M device 0b0: fixed M2M device 0b1: mobile M2M device | Present for M2M devices during network entry |

------------------------------------------------End of the Proposed Text Part 1----------------------------------------------