| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Authenticated Validity for M2M devices (reply contribution on comment #53) |  |
| Date Submitted | 2011-09-21 |  |
| Source(s) | Hyunjeong Kang Samsung Electronics Eldad Zeira InterDigital Erik Colban Huawei | E-mail: hyunjeong.kang@samsung.com Eldad.Zeira@InterDigital.com ecolban@huawei.com |
| Re: | Call for Reply Comments on the M2M 802.16Rev3 |  |
| Abstract | The contribution proposes texts for device validity check as a reply on comment #53 at 802.16p-11/0023.cmt. |  |
| Purpose | To be discussed and adopted by TGp M2M |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Authenticated Validity for M2M devices (reply contribution on comment #53)

# Introduction

This contribution proposes texts to handle M2M device’s validity check as a reply on comment #53 at 802.16p-11/0023.cmt

# Proposed text change

Adopt the following remedy in IEEE 802.16p-11/0021.

[Remedy 1: Add the following texts on line 2 of page 13:]

Modify 7.2.2.2 as indicated

7.2.2.2 Key derivation

The PKMv2 key hierarchy defines what keys are present in the system and how the keys are generated.

Since there are two authentication schemes, one based on RSA and one based on EAP, there are two primary sources of keying material.

The keys used to protect management message integrity and transport the TEKs are derived from source key material generated by the authentication and authorization processes. The RSA-based authorization process yields the pre-Primary AK (pre-PAK) and the EAP based authentication process yields the MSK. Keys used to protect MBS traffic are derived from the MBSAK, which is supplied by means outside the scope of this specification. These keys form the roots of the key hierarchy.

Only the EAP based authentication process is applied to M2M devices unless the M2M devices and BS decide “No authorization”.

[Remedy 2: Add new subclause 7.10 on line 37 of page 15:]

Add new subclause 7.10 as indicated

7.10 M2M device validity check

An M2M device that has device validity checking implemented and has failed its validity test shall refrain from accessing the network. During the PKM exchange an EAP based authentication procedure (IETF RFC 3748) may be used to carry a device validity certificate to the network.