| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Operation of paging message transmission |  |
| Date Submitted | Nov 06, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This is to clarify the operation of paging message transmission. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Operation of paging message transmission

Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] does not describe how the M2M device determines its paging frame to monitor the paging message. Especially fixed M2M device may not apply the ‘m’ of the PGID-Info message to its paging frame calculation because the ‘m’ of the PGID-Info message depends on the paging group and this value is only applicable to the AMS. Thus, we additionally define ‘m’ for fixed M2M device and clarify the paging frame determination of idle mode M2M device.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add the new text in 6.2.18.7.1 (page 35, line 42) as follows:]

6.2.18.7.1 Paging operation

The M2M device shall monitor a pre-determined frame for paging message. The pre-determined frame  for an M2M device is implicitly determined as follows:

= M2M device’s deregistration identifier mod m

Where m = 1 or 2 or 3 or 4

m is indicated by an ABS using the PGID-Info message.

[Remedy 2. Add the new text in 6.2.18.7.3.1 (page 38, line 22) as follows:]

6.2.18.7.3.1 Idle mode operation for fixed M2M devices

PGID information of the PGID-Info message is not applicable to the fixed M2M device because the Paging Group ID is not assigned to the fixed M2M device.

The Fixed M2M device shall monitor a pre-determined frame for paging message. The pre-determined frame  for an M2M device is implicitly determined as follows:

= Fixed M2M device’s Deregistration Identifier (i.e., FMDID) mod fixed M2M device specific ‘m’

Where m = 1 or 2 or 3 or 4

m is indicated by an ABS using the PGID-Info message.

[Remedy 3. Add the table in page 22, line 55 as follows:]

6.2.3.24 PGID-Info (paging group information) message

Table – PGID-Info message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| For(i=0; i<Num_PGIDs; i++) { |  | Indicate the number of PGID included in PGID-Info message 1..4 |  |
| PGID | 16 | Indicate Paging group identifier where the ABS belongs 0..-1 | Shall be present |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation Npaging frame = AMS’s deregistration identifier mod m to determine a predefined paging frame for the AMS 1..4 | Shall be present |
| } |  |  |  |
| Fixed M2M device specific ‘m’ | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for paging message transmission of an idle mode fixed M2M device. Indicate m in the equation = FMDID mod m to determine a predefined paging frame for the fixed M2M device 1..4 |  |
| … |  |  |  |

# References

[1] P802.16p/11-0033