| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | [Title of Contribution; e.g., IEEE 802.16 Document Submission Template, Rev. 9] |  |
| Date Submitted | [The date this revision of the document is uploaded, in the format 2007-06-19. Tell the truth. Automatic numbering is prohibited.] |  |
| Source(s) | Muthaiah Venkatachalam Intel Corporation Suresh Nair Alcatel Lucent | Voice: 503 629 0238 Fax: Muthaiah.Venkatachalam@intel.com |
| Re: | 802.16 Revision |  |
| Abstract | ROHC operation clarifications |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# ROHC operation clarifications

Muthaiah Venkatachalam, Suresh Nair

5.2.7.2 Compressed IP-header classification rules

Current text:

Proposed text:

The term ‘ROHC channel’ is defined in RFC3095 and further clarified in RFC3759. A single ROHC channel, which may have multiple ROHC contexts, shall have a one-to-one mapping to a single service Flow (SFID). ROHC classification SHALL neither use ROHC context ID nor ROHC channel ID.

The ROHC parameters associated with the ROHC Channel shall be negotiated by including the ROHC Parameter Payload TLV (11.13.38) in the DSA-REQ/RSP messages (for a new Service Flow creation) or the DSC-REQ/RSP messages (for an existing Service Flow).

Downlink:

For every incoming packet into the BS MAC, ROHC compressor operation will be performed in some higher layer entity, since ROHC is not part of the MAC layer. The higher layer entity SHALL add a “ROHC index” to each of the ROHC compressed SDUs. The exact definition of the “ROHC index” is dependent on the end to end implementation and is out of scope of 802.16. For instance, “ROHC index” MAY be realized using some backhaul specific parameters.

For the downlink ROHC compressed packets, the BS shall classify the packets onto the appropriate MAC CID using the “ROHC index” classification parameter. Please see figure xxx.

Uplink:

ROHC compression will be performed in the MS above the MAC layer. The classification of the ROHC compressed packets to the appropriate CIDs in the MS MAC is an implementation specific issue and is out of the scope of the 802.16 standard.

For the incoming ROHC enabled packets on the uplink, the BS shall lookup the “ROHC index” using the MAC CID as a classification parameter. This ROHC index is then used to communicate with the higher layer entity. Please see figure xxx.

Figure xxx: ROHC classification operation.