| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Quick Paging |  |
| Date Submitted | 2008-03-19 |  |
| Source(s) | Havish Koorapaty, Per Ernström Ericsson AB SE-164 80 Stockholm, Sweden | Voice: +1 – 919-472-7524 mailto:havish.koorapaty@ericsson.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot #26 |  |
| Abstract | This contribution proposes a design for a quick paging signal for IEEE 802.16e |  |
| Purpose | Discuss and Adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Quick Paging Signal for IEEE 802.16e

Havish Koorapaty, Per Ernström

Ericsson AB

# Summary

In this document, we propose a design for a quick paging signal in IEEE 802.16e. The quick paging signal notifies a subset of MS whether to read a full paging message in the near future, and thus helps improve standby time. We propose a design based on a bi-orthogonal Walsh-Hadamard code to be used in the preamble part of the frame. The performance of this design is evaluated and the design is found to work effectively without causing any noticeable effects on legacy users.

We recommend that quick paging functionality be included in the revision to the baseline specification. In addition, we recommend that the proposal in this contribution be included as the method to achieve quick paging functionality.

# Introduction

A quick paging signal is used in systems such as CDMA2000 and WCDMA in order to extend battery life. The quick paging signal provides an indication to selected mobiles to read the full paging message that occurs in a subsequent frame. Due to the significantly shorter duration of the quick paging message, the wake-up time of the MS is lowered, thereby saving battery life.

The quick paging mechanism is not currently available in the IEEE 802.16e-2005 standard [1]. A similar quick paging concept for IEEE 802.16m has been proposed in [2]. In this contribution, we propose a design for the quick paging signal using the unused part of the preamble portion of the downlink frame. We use a signal that is significantly lower in power than the preamble signal in order to minimize effects on legacy equipment, and in order not to affect common functions for which the preamble is needed. The signal is designed to be received reliably in spite of the low signal level. We perform a detailed performance analysis of the signal using link and system simulations, and demonstrate its efficacy.

# Background

The frame structure as specified in IEEE 802.16e, is shown in Figure 1.

Figure 1	Frame Structure Used in IEEE 802.16e

Each frame starts with a preamble signal that consists of a known binary signal sent every third OFDM tone. In the specification, the preamble signal is defined by the segment (i.e., one of the three sets of tones to be used), and the parameter IDCell.  Mobile stations use the preamble for initial synchronization to the system, and to determine the location of the FCH message, which gives further information on the signal parameters. Mobile stations also use the preambles in neighbor cell signals in order to synchronize to them for purposes of measurement for handover.  The use of every third tone for the preamble signal allows for a 1/3 reuse of the preamble signal, thereby improving its reception for the mobile stations. The use of every third tone also implies a repetitive structure in the time domain, which can be used by mobiles for initial acquisition. The DL-MAP signal contains downlink assignments in time and frequency for identified mobile stations, which can then receive the data in the particular location. The structure of the preamble in the frequency domain for a 2048-FFT system is shown in Figure 2.

Figure 2	Structure of Preamble in Frequency Domain. Every third tone (or subcarrier) is occupied.

Sleep modes are used in IEEE 802.16e to extend battery life.  A mobile station requests sleep mode from the system, which assigns it a sleep time and a particular cycle of frame numbers on which to wake up and listen for a paging message. In order to maximize the use of paging resources, it makes sense for the system to form groups with multiple MS in each group, and have the MS in a group wake up during a particular frame in order to receive a paging message. On waking up, a MS has to receive the preamble, then decode the FCH and the DL-MAP message to determine whether it has an assignment in the particular frame, and decode the message if it finds an assignment in the DL-MAP. The DL-MAP may be quite long, especially if a significant number of assignments are carried in it.

The quick paging message is used to minimize the wake-up time of the MS in order to extend battery life. In general, the probability of having a paging message addressed to the MS is quite low, but it needs to read the paging message in order not to miss pages which can occur at any time. Instead, if a message of a short duration is provided that indicates whether the MS can expect a page, and thus has to read the full paging message at a subsequent block, then the MS only reads the ‘quick paging’ message, and can avoid reading the full paging message most of the time. Due to its shorter duration, receiving the quick paging message lowers the power consumption at the MS.

# Signal Design

In this contribution, we propose a quick paging message that can be implemented during the duration of the preamble message. As stated before, the preamble utilizes only every third OFDM tone, thus there are empty spaces available to utilize for the quick paging message. For example, in Figure 2, the quick paging message would be used on subcarriers 1, 2, 4, 5 and so on. However, care has to be taken not to affect any of the essential functions of the preamble, which are mainly for initial acquisition, and for neighbor cell measurement purposes. Thus, any quick paging signal implemented in the duration of the preamble needs to have very low power compared to the preamble signal. This will be the focus of the design of a quick paging signal.

The quick paging signal needs to operate at a very low signal to noise ratio. However, it can rely on good channel estimates that can be obtained using the preamble signal, and the significant amount of frequency coherence that can be expected even in a highly dispersive channel. In addition, the quick paging signal should be able to address a large number of users. For example, it should be possible to divide the pool of MS into many groups, and signal the identity of the group of the MS that needs to read the full paging message. By using many such groups, the number of users that need to read a full paging message can be minimized.

It is well known that orthogonal codes perform well at low signal to noise ratios [3]. In addition, an orthogonal code of a long length can provide many codewords that can be used to address multiple groups. For example, an orthogonal code of length 512 can address 512 groups of users. In addition, due to the presence of reliable channel information obtainable from the preamble, we can choose a bi-orthogonal code that uses both a code and its inverse. The distance between a codeword and its inverse is large, and the use of reliable channel information helps make discrimination between these codewords highly reliable. Thus, 1024 groups of users can be addressed using codewords of length 512.

Of the orthogonal codes, the best known are the Hadamard codes, which exist for all lengths that are multiples of 4, though lengths that are a power of 2 (i.e., the Walsh-Hadamard codes) are most popular. The use of a Walsh-Hadamard code permits the use of a Fast Walsh Transform in the decoding, so that the relative merit of many codewords can be assayed with low complexity.

We thus propose the use of a Walsh-Hadamard code as the signal for quick paging. With a 10 MHz WiMAX channel (using a FFT of length 1024), the preamble used is of length 284 bits. Thus, we have 568 unused positions that can be used for the quick paging signal, and a Walsh-Hadamard code of length 512 fits well. For a 5 MHz 802.16e channel, the FFT size is 512, the preamble used is of length 143, and 286 unused positions are available for the quick paging signal, thereby suggesting a Walsh-Hadamard code of length 256.

However, we also note that the Walsh-Hadamard codewords do not have particularly desirable spectral properties. For example, the all zeros codeword has a spectral line at zero frequencies. To whiten the spectrum, we suggest applying a pseudo-random masking sequence to the codeword.  This masking sequence could be chosen as a subsequence of a length 1023 PN-sequence that can be generated using a shift register. Different cells in the system can use different shifts of the PN-sequence. Alternatively, each cell can use a periodically varying subsequence with different cells starting at different indices.

The quick paging signal indicates the possible presence of a page in the next frame to an MS that belongs to the group for which a quick paging signal is sent. The one frame delay allows a further reduction in the wake-up time thereby enhancing the power savings that may be obtained using the quick paging signal.

# Performance Evaluation

We characterize the performance of the proposed quick paging signal using a combination of link and system simulations. Using a system simulation, including path loss, antenna patterns and shadow fading, we obtain the path losses from all base stations (3 sectors per site) in the system to a set of mobile stations uniformly distributed through the system. A wrap-around procedure is used to eliminate edge-effects. The losses for segments of the base stations are also logged. For each mobile station, we generate the preamble with a randomly chosen IDCell, and a quick paging codeword at a specified power level below the preamble, and apply the composite path loss to the whole signal. For the interfering sectors, we generate different preambles at a signal level as obtained from the system simulation, in the corresponding segments, and quick paging signals in the rest of the tones at a similar offset to that between the serving cell preamble and the serving cell codeword.  Additive white Gaussian noise is also added at a level corresponding to a given noise figure.  The signals from the different BS to the MS pass through different radio channels. This is assumed true for different sectors from the same site also, since the transmit antennas used are different. The channel is applied to the signal from each BS, and the combined signal is then received by the MS.  Since the relative signal levels from the system simulation, corresponding to a BS transmit power, are used, the average CINR achieved is fixed. For each set of channel realizations from the different base stations, many noise realizations are used for averaging, and multiple sets of channel realizations are also used.

Three receiver algorithms are used in the evaluation: (1) A conventional receiver based on correlations, similar to an MRC receiver (2) An interference-cancelling MMSE-based receiver that exploits the structure of the signal and (3) An enhanced interference-cancelling receiver that uses knowledge of the two strongest neighbor cell preambles to subtract them before applying the MMSE-based receiver algorithm. A threshold can be used to determine if an assigned codeword has been detected. The relative power of the quick paging codeword to the preamble signal in the same cell is assumed to be known. This information can be sent in the DCD message.

The performance of the scheme is characterized in terms of the Missed Detection probability and the False Alarm probability. A missed detection occurs when the BS sends the assigned codeword to a MS, but the MS does not detect the codeword. Thus, the MS will not read the full page intended to it. A missed detection is detrimental to the paging performance of the system. A false alarm occurs when the MS detects that its assigned codeword has been sent when the BS sent a different codeword. A false alarm will cause the MS to read the full paging message. A false alarm is not detrimental to the paging performance of the system, though it does reduce the battery life savings obtained from use of the quick paging signal. The probability of missed detection and the probability of false alarm can be traded off by choosing appropriate thresholds for detection of the assigned codeword.

The parameters used in the simulation are shown in Table 1.

| Aspect | Value |
| --- | --- |
| Number of Cells/Sites | 57/19 |
| Reuse | 1/1 (1/3 for preamble) |
| Path Loss Model | 20 +35 log(d) (Similar to ITU Vehicular) |
| Site-to-Site Distance | 2.8km |
| Penetration Loss | None |
| Shadowing Standard Deviation | 8dB |
| Shadowing Correlation Distance | 100m |
| Bandwidth | 10 MHz |
| FFT Size | 1024 |
| Transmit Power (for Preamble) | 20 W |
| Noise Figure | 9 dB |
| Channel Model | Pedestrian B |
| Relative Power of Paging Signal | -20dB |
| Codeword Size | 512 |

Table 1	Simulation Parameters

The geometry characteristics of the simulation are shown in Figure 3, assuming full occupancy of the signal and the noise. The interference-limited nature of the environment is clearly seen, the signal to noise ratio is quite high in most cases.

Figure 3	Geometry Characteristics of Simulation Used for Quick Paging Evaluation

The results of this evaluation are shown in Figure 4.  The CDF of the probability of missed detection and the probability of false alarm are shown with the traditional receiver (denoted AWGN in the figure, since it makes an AWGN assumption for the impairment), the interference cancelling receiver (denoted Blind IC in the figure, since it assumes no knowledge of the interferer preambles), and the enhanced interference cancelling receiver. The results are shown for three different threshold values. With more relaxed threshold values, the probability of missed detection is reduced at the price of an increased probability of false alarm.

With the traditional receiver, a significant number of users seem to experience missed detection probabilities of 10% or above. This would be unacceptable in a practical system. For example, we see that 20-25% of the users employing a traditional receiver experience missed detection probabilities of 10% or above. The probability of missed detection improves with the degree of interference cancellation used.  With the blind interference cancelling receiver, we see that only 6-10% of the users have missed detection probabilities of 10% or above. With the enhanced interference cancelling receiver, we see that 4-6% of the users have missed detection probabilities of 10% or above. It is seen from the figure that the probability of false alarm is maintained at around 1% or below for all users. Using more relaxed thresholds, the probability of missed detection can be improved further with an increase in the probability of false alarm.

Finally, we note that the improved performance of the enhanced interference cancelling receiver also indicates the benign effect of the quick paging signal on the performance of neighbor cell preamble measurements. The improvement indicates that the neighbor cell preamble can be detected reliably. The reliable detection leads to a reliable subtraction, which improves the performance.

Figure 4	Performance of Quick Paging Mechanism

# Conclusions

We have presented a design for a quick paging signal in IEEE 802.16e that utilizes the unoccupied space available in the preamble signal to send an indication of a full paging signal in a succeeding frame. The performance of this scheme was evaluated using a combination of link and system simulations for multiple types of receivers. By utilizing the structure of the interferer, it is possible to achieve significant gains in performance and arrive at low probabilities of missed detection along with acceptable probabilities of false alarm.  In addition, due to its low power, the proposed scheme has negligible impact on the other functionalities that require the preamble, a fact that was confirmed by the results of the simulation. Thus, the proposed scheme is a viable candidate for implementing quick paging functionality in IEEE 802.16e.

# Text Proposal

- Change to SS Capabilities: Please change table in section 11.7.14.1 as below

11.7.14. 1 Mobility Features Supported

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 31 | 1 | Bit 0: Mobility (HO) support Bit 1: Sleep mode support Bit 2: Idle mode support Bit 3: Quick Paging Support | REG-REQ REG-REQ, REG-RSP |

- Add the following TLV and its description to the DREG-CMD message and the MOB_SLP-RSP (sleep response) message :

6.3.2.3.26 DREG-CMD (de/reregister command) message

The DREG-CMD message may include the following parameters encoded as TLV tuples:

REQ-duration

Waiting value for the DREG-REQ message re-transmission (measured in frames) if this is included with action code 0x06 in DREG-CMD. If serving BS includes REQ-duration in a DREG-CMD message including an Action Code = 0x05, the MS may initiate an Idle Mode request through a DREG-REQ with Action Code = 0x01, request for MS De-Registration from serving BS and initiation of MS Idle Mode, at REQ-duration expiration.

When the DREG-CMD message is sent with action code 0x05, the following TLVs may be included:

Quick Paging

This is a TLV that indicates the number of quick paging codewords assigned to an MS and the indices of these codewords (11.1.8.4).

6.3.2.3.40 MOB_SLP-RSP (sleep response) message

The following TLV parameter may be included in MOB_SLP-RSP message transmitted by the BS.

Enabled-Action-Triggered (11.1.8.1)

This TLV indicates the enabled action that the MS performs upon reaching trigger condition in sleep mode. Next Periodic Ranging (11.1.8.3)

This value indicates the offset of frame in which MS shall be ready to perform a periodic rang-ing with respect to the frame where MOB_SLP-RSP is transmitted.

Quick Paging

This is a TLV that indicates the number of quick paging codewords assigned to an MS and the indices of these codewords (11.1.8.4)

…

…

- Add the following section and TLV description to the sleep mode TLVs in section 11.1.8.

11.1.8.4 Quick Paging

| Type (1 byte) | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | The length of the word is equal to the ceil((Number of codewords assigned to MS)*log2(NFFT)/8), where NFFT is defined in section 8.4.2.4 for OFDMA. Padding may be used to byte-align the field. | Indices of the assigned codewords concatenated into a single word with each index spanning log2(NFFT) bits. | MOB_SLP-RSP, DREG-CMD |

Number of Codewords Assigned to MS

This indicates the number of quick paging codewords assigned by the BS to the MS. If the MS detects any of these codewords have been sent, it shall activate the MS listening interval in sleep mode, or listen to a paging message in idle mode.

- DCD  Message: Please add to Table 613 as below

11.4.1 DCD channel encodings

The DCD Channel Encoding are provided in Table 543.

Table 543—DCD channel encoding

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| Quick_Paging_Signal_Power_Offset | TBD | 1 | 0-255; steps of -0.25 dB, Gives offset of power per occupied quick paging subcarrier wrt occupied preamble subcarrier | OFDMA |
| Quick_Paging_Signal_Frame_Offset | TBD | 1 | 1-31 frames, other values invalid Gives the number of frames before the start of the MS listening interval where the quick paging message will be sent | OFDMA |
| Nr_Used_Quick_Paging_Codewords | TBD | 2 | Index of highest quick paging codeword that is in use | OFDMA |

- MAC Section:

  - Please add to section 6.3.21.1, at the end

The BS may implement a wake-up indication or quick paging signal to inform the MS in sleep mode whether it needs to be active during its listening window. The quick paging signal is sent over the duration of the frame preamble a specified number of frames before the start of the MS’s listening window. If the BS implementing the quick paging functionality requires the MS to be active during the listening window (e.g., it intends to send data or a keep-alive check to the MS, or the DCD/UCD configuration has changed), it shall send a quick paging signal to the MS in the specified time period. If the MS receives a quick paging signal intended for it, it shall be active during the listening window and perform the required tasks.

The quick paging signal is implemented using a set of codewords in OFDMA mode, as defined in section 8.4.9.2.6. The BS assigns a subset of codewords to each MS in the MOB-SLP-RSP message sent to the MS via its basic CID.  The MS determines if a codeword assigned to it was sent in the quick paging interval, and acts according to such determination.

The MS indicates its support of the quick paging functionality using the SS Capabilities message. The BS adds the TLV encodings Quick_Paging_Signal_Power_Offset  and Quick_Paging_Signal_Frame_Offset in the DCD message to indicate its support of the quick paging functionality, and also to provide important information the MS can use to receive the quick paging signal.  In addition, the BS sends the TLV encoded parameter Nr_Used_Quick_Paging_Codewords to inform all MS of the codewords currently in use. The BS shall assign codewords to various MS in sequence starting from codeword index zero.

  - Please add to section 6.3.24 around indicated text

Idle mode is intended as a mechanism to allow the MS to become periodically available for DL broadcast traffic messaging without registration at a specific BS as the MS traverses an air link environment populated by multiple BSs, typically over a large  geographic area. Idle mode benefits MS by removing the active requirement for HO, and all normal operation requirements. By restricting MS activity to scanning at discrete intervals, idle mode allows the MS to conserve power and operational resources. Additionally, the MS may be assigned a subset of quick paging codewords allowing further power savings.

…

…

  - Add to section 6.3.24.1 at the end

The serving BS may also include a Quick Paging TLV with an Action Code = 0x05 in the DREG-CMD, signaling for an MS capable of quick paging functionality, the assignment of quick paging codewords that the MS needs to detect. The BS may choose to omit such information if such has already been communicated to the MS in a different message. The quick paging signal is sent over the duration of the frame preamble a specified number of frames before the start of the MS’s paging interval. If the BS implementing the quick paging functionality requires the MS to be active during the paging interval, it shall send a quick paging signal to the MS in the specified time period. If the MS receives a quick paging signal intended for it, it shall be active during the paging interval and perform the required tasks. The MS shall ensure that it looks for quick paging signals defined for its preferred base station, as per Section 8.4.9.2.6.

The BS shall communicate information pertaining to the codewords it has assigned to the MS to other base stations within the paging group. The means for such communication are beyond the scope of this specification.

- PHY section :

  - Signal format (occupied subcarriers): Add new subsection 8.4.6.1.1.3

          - Quick Paging

The preamble carrier-sets are defined using Equation (74).  For a given preamble carrier-set, say PreambleCarrierSetn,  the quick paging carrier-set nominally consists of all subcarriers not used in the preamble carrier-set. In other words, the quick paging carrier-set consists of all subcarriers in

PreambleCarrierSet(n+1 mod 3)  and  PreambleCarrierSet(n+2 mod 3)

except that values of k equal to (6 mod 10) are omitted (In other words, values of k equal to 6,16,26… are omitted)

  - Codeword Definition and Indices: Add new subsection 8.4.9.2.6

8.4.9.2.6 Bi-Orthogonal Hadamard Encoding

Bi-orthogonal Hadamard encoding is used for the Quick Paging Signal.

Hadamard matrices of sizes that are a power of two are defined as a recursion on a basic 2x2 Hadamard matrix, as given by the following equation (TBD).

H2=111−1,H2m=HmHmHm−Hm     (TBD)

For an FFT size of  NFFT , as defined in section 8.4.2.4, the bi-orthogonal Hadamard codewords are given by the matrix C(NFFT) derived from the matrix H(NFFT/2). A(NFFT)  is defined as follows, for k=0,….( NFFT/2) -1:

Row 2k of A(NFFT) = Row k of H(NFFT/2)

Row (2k+1) of A(NFFT) = (-1)*(Row k of H(NFFT/2))

Each element e of A(NFFT) is replaced by (1-e)/2 to get matrix B(NFFT)

Each element (bit) in a row of B(NFFT) is XORed with the bits in the sequence wk as defined for the downlink in section 8.4.9.4.1, wherein the first  (NFFT/2) bits of wk are used.  This gives matrix C(NFFT).

The rows of C(NFFT) , read from left to right, define the codewords, and the index of a codeword is defined as the corresponding row number of C(NFFT).

  - Quick Paging Signal Modulation: Add new subsection 8.4.9.4.3.3

8.4.9.4.3.3 Quick Paging Signal Modulation

The BPSK modulation on the quick paging signal, real and imaginary parts, is defined by Equation (TBD):

Re{ck} = (10(GD/20))*4*2(1/2)*(1/2 – Ck)

Im{ck} =0

(TBD)

where ck is the k-th subcarrier in order of increasing frequency of the quick paging carrier-set defined in section 8.4.6.1.1.3, and Ck is the k-th bit of the codeword of the Bi-Orthogonal Hadamard code  generated according to section 8.4.9.2.6. GD is the gain differential of the Quick Paging Signal with respect to the Preamble, as given by the parameter Quick_Paging_Signal_Power_Offset in the DCD message.

# References

[1] IEEE P802.16Rev2/D3, “Draft Standard for local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,’’ Feb. 2008

[2] S. Shawn Tsai, Stefan Lindgren, and Sten Sjöberg, `Wake-up Signal for 802.16m OFDMA Idle Mode,’ IEEE C802.16m07/217r1.

[3] John G. Proakis, `Digital Communications.’

# Appendix 1

In this appendix, we briefly explore the extent of standby energy savings possible by using a quick paging signal as proposed in this contribution. With normal paging, the MS needs to wake up, read the preamble, the FCH, DL-MAP and a certain number of subsequent symbols by which time it could verify whether it has an assignment. Let us assume that this can be done in ‘n’ symbol times. The energy needed to have the RF portion of the receiver on for each symbol, is denoted E.  Thus, the energy expended in regularly reading the full paging signal is nE.

With the quick paging signal, the RF portion of the receiver needs to be awake only for one symbol if there is no indication of a full paging message, or for 1+n symbols if there is an indication of a full paging signal.  If the probability that a MS will be paged in a paging slot is p, then the probability that at least 1 out of M mobiles will be paged is s=1 - (1-p)M, where M is the number of MS in the paging group.  For very small p, we have s~Mp. A reasonable p may be of the order of one thousandth or lower. Thus, the energy consumption with the quick paging signal is given by

Etot= E + (1+n)E.s ~E+(1+n)EMp

With n of the order of 10 and M of the order of 10, we see that the energy consumption is about 10E without quick paging, and 1.11E with quick paging, a savings by a factor of about 9. Note that this calculation has ignored other aspects of energy consumption at the MS.

If the total number of MS (in the paging area) was N, and the number of paging groups is P, then M=N/P.  If an average of q mobiles is paged in every paging slot, then we have p= q/N. Thus, Mp = (N/P).(q/N) = q/P. In terms of the average number of MS paged per paging slot, the expression can be rewritten as

Etot= E + (1+n)E.s ~E+(1+n)Eq/P.

Figure 5 shows the fraction of power that may be saved when quick paging is used as a function of n, the number of symbols the MS needs to be awake with the current scheme. The total number of mobiles in the paging area is assumed to be N=250*57=14250. The probability that an MS will be paged is set to 0.001 and the number of paging groups P is set to 512. The figure shows that more than 60% of the power currently used may be saved when the MS needs to stay awake for 3 symbols to read a page which leads to significant savings in energy consumption. Higher savings are likely when there are a high number of users that need to be scheduled in a frame such as in VoIP scenarios.

Figure 5: Energy savings with quick paging

# Appendix 2

In this appendix, we show further system simulation results evaluating the CINR on the various preamble signals expected to be received by a MS, and how it may degrade due to the use of the quick paging signal. As seen in Figure 5, the degradation is minimal. Note that the CINRs seen here take the 1/3 reuse of the preamble into account.

Figure 6: CINR Distribution for various Preambles