| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improvement in MBS in 802.16REV2 |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Vladimir Yanover, Nadav Lavi, Leonid Shousterman Alvarion Jonathan Segev Comsys Jerry Chow, Tricci So ZTE Kamran Etemad Intel Steven Xu, Shahab Sayeedi Motorola Mo-Han Fong, Michael Wang Nortel | E-mail: Vladimir.yanover@alvarion.com Nadav.lavi@alvarion.com Leonid.shousterman@alvarion.com |
| Re: | P802.16Rev2/D4, LB26c |  |
| Abstract | MBS definitions and procedures in the REV2 D4 draft need adjustments and some errors need to be fixed. |  |
| Purpose | Adoption toward REV2/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Improvement to MBS in 802.16REV2

Nadav Lavi, Leonid Shousterman, Vladimir Yanover

Alvarion

Jonathan Segev
Comsys

Jerry Chow, Tricci So
ZTE

Kamran Etemad
Intel

Steven Zhu, Shahab Sayeedi
Motorola

Mo-Han Fong, Michael Wang
Nortel

# Introduction

The proposal details the following:

- A method for MCID pre-allocation scheme that enables subscribers’ movement from one MBS zone to another without service interference.

- A method to ensure the MBS maps’ daisy-chain continuity when MS moves from one MBS zone to another.

- A method to provide time and frequency diversity even when Multi-BS MBS without macro-diversity is not in used, by utilizing the information transmitted from two or more MBS zones.

The proposal is based on [1].

# Proposed Solution

The proposal’s concept is a flexible transmission scheme that enables informing MSs within a certain MBS zone regarding the MCID mapping and the map location in neighboring zones. Thus when an MS moves to a new zones, it can immediately (and internally) update its MCIDs and resume content reception without the need to interact with the new BS (i.e., no need to perform CID update through location-update or handover procedures).

In addition, the proposal enhances the reception possibilities of the MS by enabling time and frequency diversity in between zones.

The proposal main merits:

- Simple solution - no need to add new features/concepts to the standard, just use current tools (TLVs)

- Support all types of deployments - with and without macro-diversity, high and low mobility, etc.

- Minor overhead - few bytes-per-second to support an entire MBS deployment

- Flexible transmission scheme - the scheme enables the vendor to adjust the TLV distribution scheme according to the deployment and mobility:

  - In a deployment without macro-diversity only border BSs are required to transmit the update information

  - Update information can be fragmented into several transmissions (i.e., no need to send all MCIDs in a single large message)

  - Frequency of transmission can be adjusted according to the mobility characteristics

- Backward compatible - new TLV will be ignored by MSs that do not support this TLV type

# Proposed Text Modification

[In 6.3.2.3.52, modify the text after the table as:]

MCID_Preallocation and Transmission Info [see section 11.1.13.1] : is used by the BS’s in one MBS-Zone to provide information about changes in mapping of current MCID’s in the selected other MBS Zones.

MCID-Continuity and Transmission Info [see section 11.1.13.2] : is used by the BS’s in one MBS-Zone to show consistency of MCID’s mapping used in selected other MBS Zones.

Both procedures of MCID update do not make any assumptions on the way the MCIDs were allocated.

[Insert new Section 11.1.13]

11.1.13 MCID_ Update Management Encoding

The TLV encodings defined in this sub clause are specific to the MBS_MAP (6.3.2.3.52) and SII-ADV

(6.3.2.3.58) MAC management message.

The TLV enables to provide information regarding several MBS zones and the MCID used in them.

11.1.13.1 MCID Pre-allocation and Transmission Info

This TLV indicates the mapping of MCIDs used in the current MBS Zone ID to new MCID within a neighboring MBS zone and information regarding the MBS map transmission in the neighboring MBS zone.

| Type | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 131 | Variable (3+N×4) | See Table xxx | MBS_MAP, SII-ADV |

Table xxx – MCID Pre-allocation and Transmission Info definition

| Field | Length (bits) | Note |
| --- | --- | --- |
| MBS_ZONE_ID | 8 | MBS zone identifier for current MBS Zone (bit#7 is not part of identifier and set to 0) |
| Neighboring_MBS_ZONE_ID | 8 | MBS zone identifier for neighboring MBS Zone (bit#7 is not part of identifier and set to 0) |
| Next_MBS_Frame_Offset_Delta | 4 | Signed integer: -8 to +7. This value is added to the Next_MBS_Frame_Offset in MBS_DATA_IE, Extended_MBS_DATA_IE, or MBS_DATA_Time_Diversity_IE to obtain the frame offset to the next MBS MAP message in the neighboring MBS Zone. The range of this value is -7 to 7 frames. The value of -8 indicates that there is no indication regarding the transmission in the neighboring zone. |
| Reserved | 4 | Set to 0. |
| List of MCID Mappings | Variable (Nx4) | Current_MCID(1), New_MCID (1), … Current_MCID(N), New_MCID (N), |

Value of 0xFFFF in the New_MCID field indicates that the service flow corresponding to Current_MCID is not available in the MBS Zone indicated by the TLV.

To receive the MBS MAP in the neighboring MBS zone, the MS is required to receive the MBS_MAP_IE in the DL MAP in the neighboring MBS zone. This is performed using the information in the ‘Next MBS frame offset’ field in the MBS data allocation IE (i.e. MBS_DATA_IE, Extended_MBS_DATA_IE, and MBS_DATA_Time_Diversity_IE) and the Next_MBS_Frame_Offset_Delta field in the TLV. The MBS_MAP_IE is located in the frame offset indicated by the sum of the two fields (Next MBS frame offset’ field in the MBS data allocation IE and Next_MBS_Frame_Offset_Delta field in the TLV).

11.1.13.2 MCID Continuity and Transmission Info

This field indicates a certain MCID stays the same in one or more MBS Zones and information regarding the MBS map transmission in the neighboring MBS zone.

| Type | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 130 | Variable (3+N×2) | See Table yyy. | MBS_MAP, SII-ADV |

Table yyy – MCID Continuation List definition

| Field | Length (bits) | Note |
| --- | --- | --- |
| MBS_ZONE_ID | 8 | MBS zone identifier for current MBS Zone (bit#7 is not part of identifier and set to 0) |
| Neighboring_MBS_ZONE_ID | 8 | MBS zone identifier for neighboring MBS Zone (bit#7 is not part of identifier and set to 0) |
| Next_MBS_Frame_Offset_Delta | 4 | Signed integer: -8 to +7. This delta value is added to the Next_MBS_Frame_Offset in MBS_DATA_IE, Extended_MBS_DATA_IE, and MBS_DATA_Time_Diversity_IE to obtain the frame offset to the next MBS MAP message in the neighboring MBS Zone. The range of this value is -7 to 7 frames. Value of -8 indicates that there is no indication regarding the transmission in the neighboring zone. |
| Time + Frequency Diversity Supported | 1 | 0 – MBS burst transmissions in neighboring MBS Zones not guaranteed to be useful for time and frequency diversity reception. 1 – MBS burst transmissions in neighboring MBS Zones carry the same data bursts in allocations for corresponding MBS MAP messages and therefore may be used for time and frequency diversity reception. |
| MCID_Update_Info_Present | 1 | If N is not equal to 0 this flag is not valid and should be set to 0 In case N = 0: 0 – This TLV does not carry any information regarding MCID update 1 – All current MCIDs are valid also in new MBS zone |
| Reserved | 2 | Set to 0. |
| List of MCIDs Continued | Variable (Nx2) | Current_MCID(1), …, Current_MCID(N) |

To receive the MBS MAP in the neighboring MBS zone, the MS is required to receive the MBS_MAP_IE in the DL MAP in the neighboring MBS zone. This is performed using the information in the ‘Next MBS frame offset’ field in the MBS data allocation IE (i.e. MBS_DATA_IE, Extended_MBS_DATA_IE, and MBS_DATA_Time_Diversity_IE) and the Next_MBS_Frame_Offset_Delta field in the TLV. The MBS_MAP_IE is located in the frame offset indicated by the sum of the two fields (Next MBS frame offset’ field in the MBS data allocation IE and Next_MBS_Frame_Offset_Delta field in the TLV).

If there are no MCIDs listed in this TLV then all MCIDs within the current MBS Zone stay the same

in the neighboring MBS Zone indicated by the TLV and the ‘Next_MBS_Frame_Offset_Delta’ applies to all MCIDs.

6.3.2.3.58 Service Identity Information (SII-ADV) message

[Add to the end of section 6.3.2.3.58]

MCID_Preallocation and Transmission Info (see 11.1.13.1) : is used by the BSs in one MBS-Zone to provide information about changes in mapping of current MCIDs in the select other MBS Zones as determined by the serving MBSZone.

MCID-Continuity and Transmission Info (see 11.1.13.2) : is used by the BS’s in one MBS-Zone to show consistency of MCID’s mapping used in select other MBS Zones as determined by the serving MBS Zone.

[Insert row values for ‘MCID Pre-allocation and Transmission Info’, Type 131, and ‘MCID-Continuity and Transmission Info’, Type 130 into Table 529]

6.3.23.1.1 Inter-MBS Zone transition

[In 6.3.23.1.1, modify the text:]

To allow seamless transition from one MBS Zone to another without any interruption of MBS data service and

operation, the MS shall update MBS service flow management encodings including multicast CID, Target SAID parameter, Packet Classification Rule parameter(s), MBS Zone Identifier Assignment parameter, and MBS contents IDs.

Upon detection of MBS Zone change SS shall not request the new MCIDs from target MBS Zone and shall use

stored MBS configuration information when available. The method of acquiring the stored update MBS Zone

information is outside the scope of this standard.

If the SS has no MCID information regarding the new MBS Zone, then the SS is required to acquire MCID context through the other procedures, e.g., location-update, handover, or network-entry.

If the SS has an indication that the MCID has no continuity in the target MBS zone then the SS shall delete the

MCID and MBS Zone Identifier Assignment related to the MCID, while the corresponding MBS service flows

become provisioned but not active.

If the SS holds provisioned MBS service-flows and it moves to another MBS Zone then the SS shall perform MCID update procedure only for the provisioned service-flows.

To allow a seamless movement from one MBS Zone to another, and enable MBS service operation without

interruption, BSs within an MBS Zone may distribute MBS configuration information about the

neighboring MBS Zones using MCIDs Update Information TLV (section 11.22) in the MBS_MAP

message. The MCID Update Information indicates the MS whether the MCID it uses in the current MBS

Zone shall be updated upon movement to another zone or not (same MCID is used in the target MBS Zone

for the same content). BS is not required to indicate all MCIDs within a single TLV, it can divide the MCIDs into several TLVs and into several transmissions.

When the MS receives the MCID Update Information TLV it stores the zone information and the MCID

values (current MCID and new MCID). When the MS receives the MCID Continuity Information TLV, it

stores the zone information and the MCID value.

As the information may be fragmented over several TLVs, the MS accumulates the information provided within the TLVs and does not discard the gathered information upon reception of a new TLV.

Upon detection of MBS Zone change to MBS-Zone ID and in case the MS arrived to MBS Zone for which it has received sufficient information of MCIDs over MBS Update TLVs in the previous MBS zones, the MS may continue to receive MBS data without requesting new MCIDs via CID update procedure.

If the MS has no MCID information regarding the new MBS Zone, then the MS should acquire

MCID context through other procedures, e.g., location update, handover, or network entry.

If the MS has an indication that the MCID has no continuity in the target MBS zone then the MS shall

delete the MCID and MBS Zone Identifier Assignment related to the MCID., while the corresponding MBS

service flows become provisioned.

If the MS holds provisioned MBS service-flows and it moves to another MBS Zone then the MS shall

use MBS Update TLVs only for the provisioned service-flows.

# Introduction

- Clarifications and Improvement in the MBS definitions and procedures in 802.16REV2, (IEEE C802.16maint-08/064r7, 2008-03-20).