| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SII-ADV Fragmentation |  |
| Date Submitted | 2008-04-19 |  |
| Source(s) | Chris Stanaway Ajay Idnani Chris Cushing Mark Marsan Joe Schumacher Motorola* | Voice: +1(847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Revision 2 / Letter Ballot 26c |  |
| Abstract | The SII-ADV message can easily be required to contain more NSP information than could fit in a single frame. It should therefore be classified as a fragmentable message. |  |
| Purpose | Adopt for 802.16 Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# SII-ADV Fragmentation

Chris Stanaway, Ajay Idnani, Chris Cushing, Mark Marsan, Joe Schumacher

Motorola

# Problem Statement

The SII-ADV MAC management message can easily be configured to contain more NSP information than will fit in a single frame.

For example, for a 5 MHz channel with a 50/50 frame ratio at QPSK ½ and a repetition rate of 6, the SII-ADV can be no larger than 150 bytes.  This allows a very limited number of NSPs and requires relatively short verbose NSP names.  The problem is even more severe if any of the verbose NSP names use UTF-16 encoding.

# Proposed Text Changes

[Change the SII-ADV entry in Table 37 (page 81, line 35) as indicated]

| … | … | … | … |
| --- | --- | --- | --- |
| 67 | MOB_MIH-MSG | MIH Payload Transfer message | primary management |
| 68 | SII-ADV | Service Identity Broadcast Information Advertisement broadcast message | Fragmented broadcast |
| 69 | LBS-ADV | Location information broadcast for LBS | broadcast |
| … | … | … | … |

[Change subclause 6.3.2.3.58 (page 255, line 1) as indicated:)

A BS may use the SII-ADV message to broadcast a list of Network Service Provider (NSP) Identifiers. The message may be broadcast periodically without solicitation or may be solicited by an SS during network entry by including the SIQ TLV in the SBC-REQ message (see section 6.3.2.3.23). This message is sent from the BS to all SSs on the fragmentable broadcast CID. Assignment method, administration, and usage of NSP Ids are outside the scope of this standard. The list of NSP Ids to be included in this message and the message transmission frequency are programmable. The BS may use the SII-ADV message to deliver IEEE 802.21 MIHF frame which carries query response. SII-ADV is broadcasted at pre-determined cycle which was notified to the MS during initial query.