| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized OFDMA FDD/H-FDD Frame Structure Proposed Text Changes for IEEE 802.16Rev2/D4 |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Dan Gal (Alcatel-Lucent ) | dgal@alcatel-lucent.com |
|  | Sylvain Labonte |  |
|  | Vladimir Yanover |  |
|  | Sean |  |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26c on IEEE Rev2/D4 |  |
| Abstract | Proposed text for FDD/H-FDD Related Sections of IEEE Rev2/D4 |  |
| Purpose | Adopt the proposed text and incorporate in 802.16 Rev2/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized OFDMA FDD/H-FDD Frame Structure Proposed Text Changes

for

IEEE 802.16/Rev2/D4

Dan Gal, Sylvain Labonte

On Behalf of the TGmaint FDD Rapporteur Group

Status date/time:  May 13, 2008 11:00 PM (Macao)

| # | 26c Ballot Comment |  | Section | Page/ line | Issue | Contribution | Need Harmonization? |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | # | Made by |  |  |  |  |  |
|  | 3058 3191 | Sylvain | 6.3.2.3.2 | 83/ 39 | Definition of “No. OFDMA Symbols” { Frame structure formulae} | 217 | - 3058 propose to reject - 3191 propose to accept #174r1 |
|  | 3119 | Gerrit | 6.3.7.5 | 300/ | MAP Relevancy figures for H-FDD fig 63, 64 | 185r1 | No |
|  | 3161 (to be withdrawn) | Jianmin et al | 6.3.27 | 475/ 31 | Persistent Allocation Updated Procedures {numerous changes in #182r1} | 182r1 | No. |
|  | 3175 | Dan | 8.4.4.1 | 692 | H-FDD/FD Frame structure | 217 | ### Summary contribution |
|  | 3174 | Aik | 8.4.4.1 | 692/26 | H-FDD/FD Frame structure | 217 |  |
|  | 3176 | Gerrit | 8.4.4.1 | 692 | H-FDD/FD Frame structure | 184r3 |  |
|  | 3177 | Vladimir | 8.4.4.1 | 692 | H-FDD/FD Frame structure | 217 | 203r1 |
|  | 3178 | Scott | 8.4.4.1 | 692/ 52 | H-FDD/FD Frame structure | [130] | ->180? |
|  | 3179 | Joey | 8.4.4.1 | 692/ 60 | H-FDD/FD Frame structure | 217 |  |
|  | 3180 | Joey | 8.4.4.1 | 693/ 4 | H-FDD/FD Frame structure | 217 |  |
|  | 3182 | Yunsong | 8.4.4.1 | 693/27 1134/47 1139/3,30 | H-FDD/FD Frame structure | - |  |
|  | 3183 | Yair | 8.4.4.1.1 | 694/ 20 | H-FDD/FD Frame structure | 217 |  |
|  | 3185 | Sylvain | 8.4.4.1.1 | 694/ 31 | H-FDD/FD Frame structure | 217 |  |
|  | 3186 | Yunsong | 8.4.4.1.1 | 695/ 40 | H-FDD/FD Frame structure | 217 |  |
|  | 3192 | Yunsong | 8.4.4.1.3 | 695/40 | H-FDD/FD Frame structure | 217 |  |
|  | 3193 | Yunsong | 8.4.4.1.3 | 695/40 | H-FDD/FD Frame structure | 217 (partial) |  |
|  | 3194 | Vladimir | 8.4.4.2 | 695/50 |  | - |  |
|  | 3197 | Jin Young |  |  | Midamble (CL-MIMO) | 200 | withdrawn |
|  | 3187 | Vladimir | 8.4.5.3.28 |  |  |  |  |
|  | 3211 | Changyoon | 8.4.5.4.7 |  |  | 180 |  |
|  | 3216 | Yunsong | 8.4.5.4.10.16 |  |  |  |  |
|  | 3221 | Gerrit | 8.4.5.4.25 |  |  |  |  |
|  | 3223 | Yunsong | 8.4.5.4.30 |  |  | 178 |  |
|  | 3225 | Yunsong | 8.4.5.4.30 |  |  | 179 |  |
|  | 3227 | Yunsong | 8.4.5.6.2 |  |  |  |  |
|  | 3233 | Wonil | 8.4.9.4.1 |  |  |  |  |
|  | 3242 | Yunsong | 11.3.1 |  |  |  |  |
|  | 3244 | Yunsong | 11.4.1 |  |  | 226 |  |
|  | 3258 | Roger | 12.6 |  |  |  |  |
|  | 3262 | Joey | 13.2.3 |  |  |  |  |
|  | 3271 | Joey | 13.2.3 |  |  |  |  |
|  | 3278 | Joey | 13.2.3 |  |  |  |  |
|  | 3279 | Joey | 13.2.3 |  |  |  |  |
|  | 3288 | Joey | 13.2.3 |  |  |  |  |
|  | 3297 | Joey | 13.2.7 |  |  |  |  |
|  | 3300 | Aik |  |  | FD MS operation in HFDD frame | [130] |  |

FDD Contributions to IEEE 802.16Rev2 Ballot 26c:

| # | Contribution Number C80216maint-08_xxx | Title | Harmonization Status |
| --- | --- | --- | --- |
|  | 174r1 -> 174r2 | Formulas for Determining H-FDD Frame Partitions | ###### OK |
|  | 179r2 | #### Full Duplex Resource Allocation Using an Extended Allocations | OK |
|  | 180r2 | UL burst allocation for F-FDD MS in generic H-FDD frame structure | Review pending |
|  | 184r3 -> 184r5 | H-FDD Group 2 and Power Save Classes | OK? |
|  | 185 ->185r1 | MAP relevance figures for H-FDD Group 2 | ###### OK |
|  | 203r1 | Clarifications for the FDD part of 802.16 REV2 | ##### OK |
|  | 226 (WR to a reply comment) | FDD Size Signaling during FDD Group Switching | Review pending |
|  | 238 | Issues Related to Supporting Full Duplex Operation in the H-FDD Frame Structure | OK |

Section 8.4.4.1:  Proposed Text

{page 692}

8.4.4 Frame structure

In licensed bands, the duplexing method shall be either FDD or TDD. FDD SSs may be full-duplex (FDD) or half-duplex (H-FDD). The FDD BS shall support both SS types concurrently. In license-exempt bands, the duplexing method shall be TDD.

8.4.4.1 FDD frame structure

Base stations of OFDMA FDD systems shall operate in full duplex mode. SSs shall be either full duplex (FDD) or half duplex (H-FDD). The FDD frame structure supports both FDD and H-FDD SS types. The frame structure supports a coordinated transmission arrangement of two groups of H-FDD MSs (Group-1 and Group-2) .{DG SL}

Figure 223 shows the frame structure .{DG SL} The DL {SL, DG} frame  contains two subframes. {3179, DG} DL Subframe 1 comprises a preamble symbol, a MAP region (MAP1) and data symbols (DL1). DL Subframe 2 comprises a MAP region (MAP2) and data symbols (DL2). The size of the DL gap shown in figure 223 (DLGap) must be an integer number of symbols (0, 1, 2, 3).

The residual frame time, DLresidue, (the frame duration minus the total time occupied by the frame symbols) may be placed either immediately before or after DL Subframe 2.

{page 693}

{DG, SL}

The number of symbols in the DLGap {DG}and the location of DLresidue shall be signaled in the DCD {3180, DG}of both DL subframes using the "FDD DL {SL, DG} gap" TLV (see section 11.4.1 Table 563).

The UL frame {SL, DG} contains two subframes, UL2 and UL1 (in this order). Figure 223 shows the timing relationship of the UL subframes relative to the DL subframes. The four parameters TTG1, TTG2, RTG1 and RTG2 are announced in the DCD messages (see table 563) and they must be sufficiently large to accommodate the H-FDD MSs transmit receive switching time plus the round trip propagation delay.

{SL, DG}

Group-1 H-FDD MSs listen to DL Subframe 1 and transmit in uplink subframe UL1. Group-2 H-FDD MSs listen to DL Subframe 2 and transmit in uplink subframe UL2. No uplink transmission by any H-FDD MS is allowed during the preamble transmission. {3174} {???} All FDD MSs may transmit during the preamble transmission.

{3177} The MAP regions - MAP1 and MAP2 - are independent and include FCH, DL-MAP and UL-MAP, the definition of which is provided in 8.4.4.3, 8.4.5.3 and 8.4.5.4 respectively.

{3194} {???} SS allowances must be made by a SSRTG and by a SSTTG. The BS shall not transmit DL information to a station later than (SSRTG + RTD) before the beginning of its first scheduled

UL allocation in any UL subframe and shall not transmit DL information to it earlier than (SSTTG – RTD) after the end of the last scheduled UL allocation, where RTD denotes round-trip delay. In addition, the SS should be allowed to receive the DL preamble for each frame that contains DL data for it by assuring the period specified above does not overlap with the preamble

Figure 223—Generic OFDMA FDD frame structure supporting H-FDD MS in two groups

{3191}

The frame parameters broadcast by the BS shall allow the MS to locate DL Subframe 2 using any of the following formulas.

Note: PS index value 1 is the first PS of the Preamble symbol.

(1)	Index of first PS of Subframe 2 = (Symbols_Frame - Symbols_DL2) * PS_1Symbol  + Z  + 1

= (Symbols_DL1 + DL_gap) * PS_1Symbol  +  Z + 1

= (1 + Symbols_UL2) * PS_1Symbol + TTG2 + RTG2 + 1

(2)	Number of symbols in subframe 2 = Symbols_DL2

= Symbols_Frame – Symbols_DL1 – DL_gap

Where:

- DL_gap = Number of symbols announced by TLV 24 of the DCD message corresponding to the DCD count announced in MAP1 of current frame

- PS_1Symbol  = Number of PS per symbol

- PS_Residual = Number of PS per frame  – Symbols_Frame * PS_1Symbol

- RTG2 = in DCD message corresponding to the DCD count announced in the frame preceding the previous frame (assumption: RTG2 does not change from frame to frame)

- Symbols_DL1 = Number of symbols broadcast in DL MAP1 in current frame (includes preamble)

- Symbols_DL2 = Number of symbols broadcast in DL MAP2 in the previous frame

- Symbols_Frame  =  Floor(Tf / Ts)

- Symbols_UL2 = Number of symbols broadcast in UL MAP2 in the frame preceding the previous frame

- TTG2 = in DCD message corresponding to the DCD count announced in the frame preceding the previous frame (assumption: TTG2 does not change from frame to frame)

- Z = 0;     	       if Bit 0 of TLV 24 == 1

= PS_Residual;        if Bit 0 of TLV 24 == 0.

8.4.4.1.1 Group Switching 								 {page 694}

In FDD, for H-FDD MSs, the BS shall be able to switch a user (MS) from group-1 to group-2, or vice versa at its discretion. To effectuate a group switch, the BS shall use either:

a) the H-FDD Group Switch IE (8.4.5.3.28), or

b) the DL HARQ Chase Subburst IE, or

c) the DL HARQ IR CTC Subburst IE, or

d) the DL HARQ IR CC Subburst IE.

When using the H-FDD Group Switch IE method, the BS shall use the Group Indicator field to signal the H-FDD group index that the MS should be associated with. If the Group Indicator field is not equal to the current H-FDD MS's group index, the mobile station shall switch to the group whose index is indicated by the Group Indicator field.

When an MS is instructed to switch to the opposite group, it shall deem any existing periodic CQICH allocations {3183} and any persistent allocation as being de-allocated by the BS.

The BS may request the MS to explicitly acknowledge a group switch instruction, in which case the BS shall assign a one-time CQICH channel. When either the DL HARQ Chase Subburst IE or the DL HARQ IR CTC Subburst IE, or the DL HARQ IR CC Subburst IE is used for signaling a group switch, the one-time CQICH channel shall be allocated via setting the LSB #0 of the Dedicated DL Control Indicator to 1 and using the Allocation Index field to indicate the location.

When the {3185, 3186}H-FDD Group Switch IE is used for signaling a group switch, a one-time CQICH channel {3186}may be assigned in that IE (see 8.4.5.3.28). The MS shall acknowledge the group switch when the BS includes a one-time CQICH allocation in the IE that contains the group switch instruction. The MS shall acknowledge - with a MAP ACK command, as described in section 8.4.5.4.10.16 - in the assigned CQICH channel.

After the MS receives a group switch instruction in frame n, the MS shall switch to the new group and decode the downlink subframe in frame n+H-FDD_Group_Switch_Delay+m, where m denotes the current group number (1 or 2) and H-FDD_Group_Switch_Delay is specified in the UCD. The MS may ignore the downlink subframe of the current group in frame n+H-FDD_Group_Switch_Delay+1 (the subframe immediately preceding the transition). See Figure 224.

{page 695}

Figure 224—H-FDD Group transition scenarios

8.4.4.1.2 Frame partition signaling

The BS shall indicate the size (in symbols) of DL1 of the current frame using the “No. of OFDMA symbols” field (see 6.3.2.3.2) in DL-MAP1 whereas the same field in DL-MAP2 shall indicate the size of DL2 of the next frame.

For the UL, the BS shall indicate, in the “No. OFDMA symbols” in UL-MAP1 and UL-MAP2, the size of UL1 of the next frame and the size of UL2 of the next-next (n+2) frame respectively

{add new section below}

8.4.4.1.3  DCD

{3192} If the DCD Alignment for H-FDD is set to 1 (see table 563, TLV 155), for a particular configuration change count, the DCD message transmitted in H-FDD group 1 shall be equal to the DCD message transmitted in H-FDD group 2.  In a given frame, the DCD count in the DL-MAPs of both groups shall be the same.

{add new section below}

8.4.4.1.4  UCD

{3192} If the UCD Alignment for H-FDD is set to 1 (see table 559, TLV 218), for a particular configuration change count, the UCD message transmitted in H-FDD group 1 shall be equal to the UCD message transmitted in H-FDD group 2.  The UCD count in the UL-MAP of group 2 of frame N shall be equal to the UCD count in the UL-MAP of group 1 of frame N+1.

Section 11.3.1:  Proposed Text

11.3.1 UCD channel encodings

UCD channel encodings shared across PHY specifications are provided in Table 556.  For FDD/H-FDD operation, if two TLVs for a particular type occur in the UCD, the first TLV corresponds to H-FDD group 1, while the second TLV corresponds to H-FDD group 2.

Section 11.4.1:  Proposed Text

{page 1139 line 4}

11.4.1 DCD channel encodings

The DCD Channel Encoding are provided in Table 563.  For FDD/H-FDD operation, if two TLVs for a particular type occur in the DCD, the first TLV corresponds to H-FDD group 1, while the second TLV corresponds to H-FDD group 2.

{Table 563}

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| FDD DL inter-group gap | 24 | 1 | Bit 0(LSB): Indicates the location of the fractional symbols period (unusable DL time in each frame) the residual frame time (DL_residue) “0”- in between DL1 and DL2 before DL Subframe 2 “1” – end of DL2 after DL Subframe 2 Bits 1-2: integer portion of the inter-group gap (# of symbols, i.e., 0,1,2,3 symbols). Note: Fractional portion is indicated by bit-0 above # of symbols, 0,1,2,3, in DL_gap. (See 8.4.4.1) Bits 3-7: reserved, set to 0 |  |
| TTG | 7 | 2 for TDD; 4 for H-FDD | TTG (in PSs) Note: for H-FDD, the first set of 2 bytes corresponds to H-FDD group 1, while the second set of 2 bytes corresponds to H-FDD group 2. |  |
| RTG | 8 | 1 for TDD; 2 for H-FDD | RTG (in PSs) Note: for H-FDD, the first byte corresponds to H-FDD group 1, while the second byte corresponds to H-FDD group 2. |  |
| DCD Alignment for H-FDD | 155 | 1 | Bit #0 Alignment for H-FDD “0”- DCD messages in group1 and group2 are different. “1”- DCD messages in group1 and group2 are the same. Default value shall be 1. | OFDMA |

{Table 559}