| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ ACK Disabled Burst |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Ji-Yun Seol, Samsung Sean McBeath, Huawei Technologies | Voice: E-mail: jiyun.seol@samsung.com smcbeath@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot 802.16 Revision 2 |  |
| Abstract | This contribution proposes clarifications on how bursts with the ACK disable flag set to 1 are counted. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D6a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ ACK Disabled Burst

Ji-Yun Seol, Samsung

Sean McBeath, Huawei Technologies

Introduction

The limitation in the maximum allocation of downlink(uplink) bursts comes from the limitation in the decoding(encoding) performance of the MS in downlink(uplink). That is, the restriction on the burst allocation per frame per MS is imposed to accommodate the performance of MS in PHY level. However, it is not clearly specified how to treat the HARQ ACK disabled burst when counting the number of bursts per frame per MS. The suggestion is to count HARQ ACK disabled bursts (allocated with ACK disable = 1) as normal (non-HARQ) bursts.

Text Changes

[note to the editor: Make the following changes to IEEE P802.16Rev2/D6a (Section 11.8.3.6.13) as shown below;

Blue = insertions relative to R2D6a

Red = deletions relative to R2D6a

]

11.8.3.6.13 Maximum number of bursts per frame capability in HARQ

The Maximum Number of Bursts Per Frame Capability field indicates the maximum number of UL/DL data burst allocations for the SS in a single UL/DL subframe (note that the number of UL non-HARQ burst is always limited to 1).  Bursts allocated using the HARQ UL MAP IE or Persistent HARQ UL MAP IE with the ACK disable field set to 1 and bursts allocated using the HARQ DL MAP IE or Persistent HARQ DL MAP IE with the ACK disable field set to 1 shall be treated as non-HARQ bursts when counting the number of UL/DL bursts per frame per MS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 175 | 1 | Bits 0–2: One less than the maximum number of UL HARQ bursts per HARQ-enabled MS per frame. (0b000 = 1, default) Bit 3: Indicates whether the maximum number of UL HARQ bursts per frame (i.e., bits 2–00-2) includes the one non-HARQ burst. (0 = not included, default) Bits 4–7: One less than the maximum number of DL HARQ bursts per HARQ-enabled MS per frame. (0b0000 = 1, default) | SBC-REQ (see 6.2.2.3.23) SBC-RSP (see 6.2.2.3.24) |