| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uncontrolled Handover |  |
| Date Submitted | 2008-11-08 |  |
| Source(s) | Yerang Hur, Hyung-Joon Jeon POSDATA Steven Xu, Joseph Schmacher Motorola Brian Shim Samsung Electronics Vladimir Yanover Alvarion | E-mail: yehur@posdata-usa.com |
| Re: | IEEE 802.16 Sponsor Ballot on P802.16Rev2/D7 |  |
| Abstract | This proposes unexpected handover of a MS to its preferred target BS be included in the standard so the target BS can handle it properly. |  |
| Purpose | Review and adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uncontrolled Handover

Yerang Hur, Hyung-Joon Jeon, Steven Xu

Joseph Schmacher, Brian Shim, Vladimir Yanover

1. Introduction

Due to suboptimal radio planning conditions, link loss, or MS implementation, when a MS detects a drop, the MS might begin ranging at a preferred BS which was not contacted by the serving BS for handover preparation.

In such a case, if the MS provides Serving BSID in RNG-REQ, the target BS might be able to handle the unexpected handover within a reduced amount of time, comparing with the required time for full initial network entry.

This concept is missing in IEEE P802.16Rev2 although it has been considered for some time in network. This contribution proposes the notion of Uncontrolled HO should be included in IEEE P802.16Rev2 with the appropriate usage of TLVs in the air link.

2. Proposed Text

[Add a new sub-clause as indicated after the end of 6.3.26:]

6.3.27 Uncontrolled HO

Uncontrolled HO occurs when an MS starts ranging at a target BS that was not previously notified of an impending handover and did not participate in the handover preparation phase. This may occur due to suboptimal radio planning conditions or MS implementation (handover notification to the serving BS by MS is optional).

To perform Uncontrolled HO,  the MS may attempt network reentry with its preferred target BS using the HO ranging procedure as defined in Section 6.3.10.3.3 with Ranging Purpose Indication TLV Bit #0 set to 1, and Serving BSID TLV set to the previous serving BS.

3. References

[1] IEEE P802.16Rev2/D7. Draft Standard for Local and Metropolitan Area Networks Part 16: Air Interface for

Broadband Wireless Access Systems, October 2008.