| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO plus Beamforming Support |  |
| Date Submitted | 2011-3-16 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0005, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | Changes are provided to accommodate MIMO Plus Beamforming Support |  |
| Purpose | As basis of maintenance data base for IEEE 802.16-2009 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO plus Beamforming Support

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

This contribution is intended to incorporate changes agreed based on the liaison IEEE L802.16-11/0005.

# Proposed Text

[Modify section 11.8.3.5.2 on page 1227 in IEEE 802.16-2009 as follows:]

11.8.3.5.2 OFDMA SS demodulator

This field indicates the different demodulator options supported by a WirelessMAN-OFDMA PHY SS for DL reception. This field is not used for other PHY specifications. A bit value of 0 indicates “not supported” while 1 indicates “supported.” The SS or BS may include bits 0–7 only and set the TLV length field to 1, in which case the receiving entity shall assume that the options represented by bits 8–15 are not supported.

When Bit 10 is set to 1, it indicates that a DL allocation may use dedicated pilots with or without MIMO where MIMO functionality is defined by TLV 176.

When Bit 11 is set to 1, it indicates that the combination of all DL allocations to the MS in a zone with dedicated pilots and MIMO must be rectangular, where the combined allocation shall span all slots between lowest and highest numbered symbols for each allocated subchannel. If set to 0, then the combination of all DL allocations to the MS does not have to be rectangular.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 151 | 1 or 2 | Bit 0: 64-QAM Bit 1: BTC Bit 2: CTC Bit 3: STC for PUSC only Bit 4: CC with optional interleaver Bit 5: HARQ Chase Bit 6: HARQ CTC_IR Bit 7: Reserved; shall be set to zero Bit 8: HARQ CC IR Bit 9: LDPC Bit 10: Dedicated pilots for PUSC with or without MIMO Bit 11: Rectangular combination of allocations for DL MIMO zone with dedicated pilots 0b0: No restriction to rectangular combination of allocations for DL MIMO zone with dedicated pilots 0b1: Restricted to rectangular combination of allocations for DL MIMO zone with dedicated pilots Default is 0b0 Bit 12: Dedicated pilots with AMC without MIMO Bits 11 13–15: Reserved; shall be set to zero. | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

[Modify section 11.8.3.5.4 on page 1229 in IEEE 802.16-2009 as follows:]

11.8.3.5.4 OFDMA SS permutation support

This field indicates the different optional OFDMA permutation modes (optional PUSC, optional FUSC, and AMC) supported by a WirelessMAN-OFDMA SS. A bit value of 0 indicates “not supported” while 1 indicates “supported.”

This TLV only indicates support of permutation and does not indicate support for other capabilities such as MIMO or dedicated pilots.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 154 | 1 | Bit 0: Optional PUSC support Bit 1: Optional FUSC support Bit 2: AMC 1x6 support Bit 3: AMC 2x3 support Bit 4: AMC 3x2 support Bit 5: AMC support with HARQ map Bit 6: TUSC1 support Bit 7: TUSC2 support | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

[Modify section 11.8.3.5.18 on page 1236 in IEEE 802.16-2009 as follows:]

11.8.3.5.18 OFDMA parameters sets

| Sets | Items | Sub-Items | Reference |
| --- | --- | --- | --- |
| OFDMA PHY parameter set B | OFDMA SS demodulator | 64 QAM | 11.8.3.5.2 |
|  |  | CTC |  |
|  |  | STC in PUSC zone with or without dedicated pilots |  |
|  |  | HARQ chase |  |
|  |  | Dedicated pilot in PUSC zone and AMC2x3 zone |  |