| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2mBsMib |  |
| Date Submitted | 2007-11-11 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2mBsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2mBsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2mBsMib.

# Proposed ASN.1 Notations

WMAN-IF2M-BS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2mBsMib MODULE-IDENTITY

LAST-UPDATED    "200711090000Z" -- November 09, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@futurewei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Base Station

based on IEEE Std 802.16-2004 and its amendment IEEE Std

802.16e-2005. The MIB contains managed objects that are

specific to mobile Broadband Wireless Networks."

REVISION        "200711090000Z"

DESCRIPTION

"Change to wmanIf2mBsMib"

REVISION        "200710020000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Malaga meeting"

REVISION        "200705220000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Portland meeting"

REVISION        "200703280000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Orlando meeting"

REVISION        "200702100000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

London meeting"

REVISION        "200610160000Z"

DESCRIPTION

"The 1st revision of WMAN-IF2M-MIB module."

::= { iso std(0) iso8802(8802) wman(16) 3 }

wmanIf2mBsCm OBJECT IDENTIFIER ::= { wmanIf2mBsMib 1 }

wmanIf2mBsPm OBJECT IDENTIFIER ::= { wmanIf2mBsMib 2 }

wmanIf2mBsFm OBJECT IDENTIFIER ::= { wmanIf2mBsMib 3 }

wmanIf2mBsSm OBJECT IDENTIFIER ::= { wmanIf2mBsMib 4 }

wmanIf2mBsAm OBJECT IDENTIFIER ::= { wmanIf2mBsMib 5 }

--

-- Textual Conventions

--

WmanIf2mOfdmaMobility ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates whether or not the MS supports

mobility hand-over, Slee-pmode, and Idle-mode. A bit value

of 0 indicates 'not supported' while 1 indicates it is

supported."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

SYNTAX      BITS {handoverSupport(0),

sleepModeSupport(1),

idleModeSupport(2)}

WmanIf2mHandoverType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS and the MS

supports.

bit#0: if set to 1, MDHO/FBSS HO not supported, the BS

shall ignore all other bits.

bit#1: if set to 1, FBSS/MDHO DL RF Combining is supported

with monitoring MAPs from active BSs

bit#2: if set to 1, MDHO DL soft Combining is supported

with monitoring single MAP from anchor BS

bit#3: if set to 1, MDHO DL soft combining is supported

with monitoring MAPs from active BSs

bit#3: if set to 1, MDHO UL Multiple transmission is

supported"

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

SYNTAX      BITS {mdhcFbssHoNotSpported(0),

mdhcFbssDlMapsFromActiveBss(1),

mdhcDlMapFromAnchorBs(2),

mdhcDlMapsFromActiveBss(3),

mdhcUlMultipleTx(4)}

WmanIf2mCidType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of CID."

SYNTAX      INTEGER (0 .. 65535)

WmanIf2mPsClassId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates the index to Power Saving Classes. The ID shall

be unique within the group of Power Saving Classes

associated with the MS. This ID may be used in further

MOB_SLP-REQ/RSP messages for activation / deactivation of

Power Saving Class."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER (0..63)

WmanIf2mPsClassType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The types of power saving classes."

REFERENCE

"Table 374a in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {powerSavingClassTypeI(1),

powerSavingClassTypeII(2),

powerSavingClassTypeIII(3)}

WmanIf2mPsClassCidDir ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The direction of power saving class's CIDs.

0b00 = Unspecified. Each CID has its own direction assign

in its connection creation. Can be DL, UL, or both

(in the case of management connections).

0b01 = Downlink direction only.

0b10 = Uplink direction only."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {unspecified(0),

downlink(1),

uplink(2)}

WmanIf2mPowerSavingMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Power saving class mode active or not active."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {psNotActive(0),

psActive(1)}

WmanIf2mSkipOptBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"If set to 1, its corresponding field will be omitted."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      BITS {omitOperatorId(0),

omitNeighborBsId(1),

omitHoProcOptimization(2),

omitQosRelatedField(3)}

WmanIf2mNbrBsId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The least significant 24 bits of the Base Station ID

parameter in the DL-MAP message broadcast by the Neighbor

BS. The BSID is a 6 byte number and follows the encoding

rules of MacAddress textual convention, i.e. as if it were

transmitted least-significant bit first. The value should

be displayed with 2 parts clearly separated by a colon.

Example 001DFF:00003A - 00003A is the Base Station ID. "

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mNbrOperatorId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The most significant 24 bits of the Base Station ID

parameter in the DL-MAP message broadcast by the Neighbor

BS. The BSID is a 6 byte number and follows the encoding

rules of MacAddress textual convention, i.e. as if it were

transmitted least-significant bit first. The value should

be displayed with 2 parts clearly separated by a colon.

Example 001DFF:00003A - 001DFF is the Operator ID. "

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mPhyProfileId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For systems using OFDM or OFDMA, the definition of the PHY

Profile ID is shown as follows:

bit#0: If set to 1, BS (or FA) is co-located with the

serving BS

bit#1: If set to 1, the BS has the same number of FAs and

frequencies as the BS broadcasting the NBR-ADV

bit#2: 0b00 = Unsynchronized

bit#3: 0b01 = Time synchronization

0b10 = Time and Frequency synchronization

If time synchronization is indicated for the OFDMA

PHY, then the downlink frames transmitted by the

serving BS and the Neighbor BS shall be synchronized

to a level of at least 1/8 cyclic prefix length. If

frequency synchronization is indicated for the OFDMA

PHY, then the BS reference clocks shall be

synchronized to a level that yields RF center

frequency offset of no more than 1% of the OFDMA

carrier spacing of the Neighbor BS.

bit#4: If set to 1, the BS EIRP follows the PHY Profile ID

bit#5: 0b0- The DCD/UCD settings of this neighbor BS are

the same as those of the serving BS unless the

TLV information specifies.

0b1- The DCD/UCD settings of this neighbor BS are

the same as those of the preceding neighbor BS

unless the TLV information specifies.

bit#6: If set to 1, the FA Index follows the PHY Profile ID.

In addition, if the FA Indicator is followed, the DL

center frequency shall be omitted in the DCD/UCD

difference TLV information.

bit#7: The Trigger Reference Indicator is related to the

Neighbor BS trigger metric TLV information of this

neighbor BS.

0b0- The trigger settings of this neighbor BS are

the same as those provided by the serving BS

(via DCD).If the TLV information is present, it

overrides values inherited from preceding

neighbor BS.

0b1- The trigger settings of this neighbor BS are

the same as those of the preceding neighbor BS."

REFERENCE

"Table 109g in IEEE Std 802.16e-2005"

SYNTAX      BITS {colocatedFaInd(0),

faConfigInd(1),

timeFreqSyncInd1(2),

timeFreqSyncInd2(3),

bsEirpInd(4),

dcdUcdRefInd(5),

faIndexInd(6),

triggRefInd(7)}

WmanIf2mHoProcOptm ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For each Bit location, a value of '0' indicates the

associated reentry management messages shall be required,

a value of '1' indicates the reentry management message may

be omitted. Regardless of the HO Process Optimization TLV

settings, the target BS may send unsolicited SBC-RSP and/ or

REG-RSP management messages

bit#0: Omit SBC-REQ/RSP management messages during re-entry

processing

bit#1: Omit PKM Authentication phase except TEK phase during

current re-entry processing

bit#2: Omit PKM TEK creation phase during reentry processing

bit#3: Omit REG-REQ/RSP management during current re-entry

processing

bit#4: Omit Network Address Acquisition management messages

during current reentry processing

bit#5: Omit Time of Day Acquisition management messages

during current reentry processing

bit#6: Omit TFTP management messages during current re-entry

processing

bit#7: Full service and operational state transfer or

sharing between serving BS and target BS (ARQ, timers

, counters, MAC state machines, etc...)"

REFERENCE

"Table 109f in IEEE Std 802.16e-2005"

SYNTAX      BITS {omitSbcReq(0),

omitPkmAuth(1),

omitPkmTek(2),

omitRegReq(3),

omitNtwkAddrAcq(4),

omitTimeOfDay(5),

omitTftp(6),

fullService(7)}

WmanIf2mSchedulingSupp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Bitmap to indicate if BS supports a particular scheduling

service. 1 indicates support, 0 indicates not support:

bit#0: Unsolicited Grant Service (UGS)

bit#1: Real-time Polling Service (rtPS)

bit#2: Non-real-time Polling Service (nrtPS)

bit#3: Best Effort

bit#4: Extended real-time Polling Service (ertPS)

If the value of bit 0 through bit 4 is 0b00000, it indicates

no information on service available."

REFERENCE

"Table 109f in IEEE Std 802.16e-2005"

SYNTAX      BITS {ugs(0),

rtPs(1),

nrtPs(2),

be(3),

ertPs(4)}

WmanIf2mMacVersion ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Version number of IEEE 802.16."

SYNTAX      INTEGER {ieee802Dot16Of2001(1),

ieee802Dot16cOf2002(2),

ieee802Dot16aOf2003(3),

ieee802Dot16Of2004(4),

ieee802Dot16e(5),

tbd(6)}

WmanIf2mHarqAckDelay ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"HARQ ACK delay for UL and DL bursts

1 = one frame offset

2 = two frames offset

3 = three frames offset"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {oneframeoffset(1),

twoframesoffset(2),

threeframesoffset(3)}

WmanIf2mPowerSaveType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

SYNTAX      BITS {psClassTypeI(0),

psClassTypeII(1),

psClassTypeIII(2)}

WmanIf2mHoTrigMatrix ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates trigger metrics that MS or BS supports.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

SYNTAX      BITS {bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRoundTripDelay(3)}

WmanIf2mAssociationTyp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the association level supported by the

MS or the BS. If a bit is set to '1', then MS or BS

indicates support at the respective association type and

level. The MS may associate according to arrangements by

the BS at levels up to and including the one for which the

MS has indicated support."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

SYNTAX      BITS {scanWoAssociation(0),

scanOrAssocWoCoordination(1),

assocWithCoordination(2),

ntwkAssistAssociation(3),

directAssociation(4)}

WmanIf2mOfdmaFftSize ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FFT size for OFDMA PHY

0b000 = 2048

0b001 = 1024

0b010 = 512

0b100 = 128"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {fft2048(0),

fft1024(1),

fft512(2),

reserved(3),

fft128(4)}

WmanIf2mOfdmaCp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Cycle prefix for OFDMA PHY

0b00 = 1/4

0b01 = 1/8

0b10 = 1/16

0b11 = 1/32"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {oneForth(0),

oneEighth(1),

oneSixteenth(2),

oneThirtySecond(3)}

WmanIf2mOfdmaFrame ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Frame duration for OFDMA PHY

0b0000 = 2.0 ms

0b0001 = 2.5 ms

0b0010 = 4 ms

0b0011 = 5 ms

0b0100 = 8 ms

0b0101 = 10 ms

0b0110 = 12.5 ms

0b0111 = 20 ms"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {twoMs(0),

twoPointFiveMs(1),

fourMs(2),

fiveMs(3),

eightMs(4),

tenMs(5),

twelvePointFiveMs(6),

twentyMs(7)}

WmanIf2mPagingAction ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Paging action instruction to MS

0b00 = No Action Required

0b01 = Perform Ranging to establish location and

acknowledge message

0b10 = Enter Network"

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noAction(0),

performRanging(1),

enterNetwork(2)}

WmanIf2mSsMacAddrHash ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"24 bit SS MAC address hash that is obtained by computing a

CRC24 on the MS 48-bit MAC address."

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mSfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow.

'inactive' - A service flow is inactive, when the MS owns

this service flow has handoff to another BS.

'provisioned' - A service flow is provisioned, but not

yet activated.

'admitted' - This maps to the 1st phase of the two-phase

activation model that the bandwidth a service

flow is reserved. But, no traffic can be

sent on this service flow yet.

'active' - This maps to the 2nd phase of the two-phase

activation model that bandwidth is granted,

(e.g., is actively sending UL maps containing

unsolicited grants for a UGS service flow)."

REFERENCE

"Subclause 6.3.14.6 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {inactive(0),

provisioned(1),

admitted(2),

active(3)}

WmanIf2mGlobalSrvClass ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Global Service Class Name contains 8 information fields

that map to predefined QoS attributes as shown in

subclause 6.3.14.4.1.

bit#0:     Uplink/Downlink indicator

0 - uplink

1 - downlink

bit#1-6:   Maximum sustained traffic rate in bps that is

defined in Table 124b

bit#7:     0 - no traffic indication

1 - traffic indication

bit#8-13:  Maximum traffic burst defines the maximum burst

size that must be accommodated for the service.

bit#14-19: Minimum reserved traffic rate parameter

specifies the minimum rate, in bits per second,

reserved for this service flow.

bit#20-25: Maximum latency specifies the maximum interval

between the reception of a packet at CS of BS

or SS and the arrival of the packet to the peer

device.

bit#26:    SDU indicator specifies whether the SDUs on the

service flow are fixed-length or variable-length.

0 - variable length

1 - fixed length

bit#27:    Paging indicator of an MS preference for the

reception of paging advisory messages during

idle mode. When set, it indicates that the BS

may present paging advisory messages or other

indicative messages to the MS when data SDUs

bound for the MS are present while the MS is in

Idle Mode.

0 - no paging generation

1 - paging generation"

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

SYNTAX      BITS {ulDlIndicator(0),

maxSustainedRate0(1),

maxSustainedRate1(2),

maxSustainedRate2(3),

maxSustainedRate3(4),

maxSustainedRate4(5),

maxSustainedRate5(6),

trafficIndication(7),

maxTrafficBurst0(8),

maxTrafficBurst1(9),

maxTrafficBurst2(10),

maxTrafficBurst3(11),

maxTrafficBurst4(12),

maxTrafficBurst5(13),

minReservedRate0(14),

minReservedRate1(15),

minReservedRate2(16),

minReservedRate3(17),

minReservedRate4(18),

minReservedRate5(19),

maxLatency0(20),

maxLatency1(21),

maxLatency2(22),

maxLatency3(23),

maxLatency4(24),

maxLatency5(25),

sduIndicator(26),

pagingGeneration(27),

reserved0(28),

reserved1(29),

reserved2(30),

reserved3(31)}

WmanIf2mSfDirection ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The direction of a service flow"

SYNTAX      INTEGER {downstream(1),

upstream(2)}

WmanIf2mReqTxPolicy ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Specify certain attributes for the associated service

flow. An attribute is enabled by setting the

corresponding bit position to 1.

bit#0: Service flow shall not use broadcast bandwidth

request opportunities. (Uplink only)

bit#1: reserved

bit#2: The service flow shall not piggyback requests with

data. (Uplink only)

bit#3: The service flow shall not fragment data.

bit#4: The service flow shall not suppress payload headers

(CS parameter)

bit#5: The service flow shall not pack multiple SDUs (or

fragments) into single MAC PDUs.

bit#6: The service flow shall not include CRC in the MAC

PDU."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

WmanIf2mPhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {phsVerifyEnable(0),

phsVerifyDisable(1)}

WmanIf2mSchedulingType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The scheduling service provided by a SC for an

upstream service flow. If the parameter is omitted

from an upstream QOS Parameter Set, this object takes

the value of bestEffort (2). This parameter must be

reported as undefined (1) for downstream QOS Parameter

Sets."

SYNTAX      INTEGER {undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)}

WmanIf2mCsSpecification ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)}

WmanIf2mIpv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most

significant byte shall be set to 0x0 and disregarded."

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mClassifierBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16-2004"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12)}

-- XXX

WmanIf2mReportMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Action code for an MS's report of CINR measurement:

0b00: The MS measures channel quality of the Available BSs

without reporting.

0b01: The MS reports the result of the measurement to

Serving BS periodically. The period of reporting is

different from that of scanning.

0b10: The MS reports the result of the measurement to

Serving BS after each measurement."

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noReport(0),

periodicReport(1),

eventTriggeredReport(2)}

-- XXX

WmanIf2mReportMatric ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Bitmap indicator of trigger metrics that the serving BS

requests the MS to report. Serving BS shall indicate only

the trigger metrics agreed during SBC-REQ/RSP negotiation.

Each bit indicates whether reports will be initiated by

trigger based on the corresponding metric:

Bit 0:    BS CINR mean

Bit 1:    BS RSSI mean

Bit 2:    Relative delay

Bit 3:    BS RTD; this metric shall be only measured on

serving BS/anchor BS"

REFERENCE

"Subclause 6.3.2.3.48 in IEEE Std 802.16-2004"

SYNTAX      BITS {bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)}

-- XXX

WmanIf2mFullBsId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The 48 bit field sent in the DL-MAP message to identifies

the BS."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(6))

-- XXX

WmanIf2mScanType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of scanning or association used by the MS and

coordinated by the Serving BS:

0b000: Scanning without Association Scanning

0b001: Scanning with Association level 0: association

without coordination

0b010: Scanning with Association level 1: association

with coordination

0b011: Scanning with Association level 2: network assisted

association"

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {scanWoAssociation(0),

scanWithAssociation0(1),

scanWithAssociation1(2),

scanWithAssociation2(3)}

-- XXX

WmanIf2mBsSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER {xxxReq (1),

xxxRsp (2)}

WmanIf2mBsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX      INTEGER {inheritedBsCap (1),

configedBsCap (2)}

--

-- wmanIf2mBsCm contain the Base Station Configuration Management

-- objects

--

--

-- Base Station configuration

--

wmanIf2mBsConfiguration OBJECT IDENTIFIER ::= { wmanIf2mBsCm 1 }

--

-- wmanIf2mBsConfigurationTable contains global parameters for BS

--

wmanIf2mBsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the BS system parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsConfiguration 1 }

wmanIf2mBsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2mBsConfigurationTable 1 }

WmanIf2mBsConfigurationEntry ::= SEQUENCE {

wmanIf2mBsMobNbrAdvInterval             INTEGER,

wmanIf2mBsAscAgingTimer                 INTEGER,

wmanIf2mBsPagingRetryCount              INTEGER,

wmanIf2mBsModeSelectFeedbackProcTime    INTEGER,

wmanIf2mBsIdleModeSystemTimer           Unsigned32,

wmanIf2mBsMgmtResourceHoldingTimer      INTEGER,

wmanIf2mBsDregCommandRetryCount         INTEGER,

wmanIf2mBsT46Timer                      INTEGER,

wmanIf2mBsT47Timer                      INTEGER,

wmanIf2mBsPagingInterval                INTEGER,

wmanIf2mBs2ndMgmtDlQoSProfileIndex      INTEGER,

wmanIf2mBs2ndMgmtUlQoSProfileIndex      INTEGER,

wmanIf2mBsBasicCidDlQosProfileIndex     INTEGER,

wmanIf2mBsBasicCidUlQosProfileIndex     INTEGER,

wmanIf2mBsPrimaryCidDlQosProfileIndex   INTEGER,

wmanIf2mBsPrimaryCidUlQosProfileIndex   INTEGER}

wmanIf2mBsMobNbrAdvInterval OBJECT-TYPE

SYNTAX      INTEGER (1 .. 30)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time between transmission of MOB_NBR-ADV messages"

::= { wmanIf2mBsConfigurationEntry 1 }

wmanIf2mBsAscAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of MS associations"

::= { wmanIf2mBsConfigurationEntry 2 }

wmanIf2mBsPagingRetryCount OBJECT-TYPE

SYNTAX      INTEGER (1 .. 16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on paging transmission. If the BS does

not receive RNG-REQ from the MS until this value decreases

to zero, it determines that the MS is unavailable."

DEFVAL      { 3 }

::= { wmanIf2mBsConfigurationEntry 3 }

wmanIf2mBsModeSelectFeedbackProcTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "microseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time allowed between the end of the burst carrying the

Mode Selection Feedback subheader and the start of the UL

subframe carrying the Mode Selection Feedback response.

Minimum value = 1 frame duration for TDD

1/2 Frame duration for FDD"

::= { wmanIf2mBsConfigurationEntry 4 }

wmanIf2mBsIdleModeSystemTimer OBJECT-TYPE

SYNTAX      Unsigned32 (128 .. 65536)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"For BS acting as Paging Controller, timed interval to

receive notification of MS Idle Mode Location Update. Set

timer to MS Idle Mode Timeout. Timer recycles on

successful Idle Mode Location Update."

DEFVAL      { 4096 }

::= { wmanIf2mBsConfigurationEntry 5 }

wmanIf2mBsMgmtResourceHoldingTimer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the BS maintain connection information with the MS

after the BS send DREG-CMD to the MS."

DEFVAL      { 500 }

::= { wmanIf2mBsConfigurationEntry 6 }

wmanIf2mBsDregCommandRetryCount OBJECT-TYPE

SYNTAX      INTEGER (3 .. 16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on DREG Command Message."

DEFVAL      { 3 }

::= { wmanIf2mBsConfigurationEntry 7 }

wmanIf2mBsT46Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the BS waits for DREG-REQ in case of unsolicited Idle

Mode initiation from BS."

::= { wmanIf2mBsConfigurationEntry 8 }

wmanIf2mBsT47Timer OBJECT-TYPE

SYNTAX      INTEGER (8 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"PMC_RSP Timer: BS shall send the PMC_RSP before T47 + 1

frames after BS receives PMC_REQ (confirmation = 0)

correctly."

DEFVAL      { 64 }

::= { wmanIf2mBsConfigurationEntry 9 }

wmanIf2mBsPagingInterval OBJECT-TYPE

SYNTAX      INTEGER (8 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration of Paging Interval of the BS."

DEFVAL      { 64 }

::= { wmanIf2mBsConfigurationEntry 10 }

wmanIf2mBs2ndMgmtDlQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the index of a row in

wmanIf2mBsQoSProfileTable which is used to obtain all QoS

parameters required for the BS downlink scheduler to

properly allocate and manage the bandwidth and schedule

the 2nd Management Connection traffic. The 2nd Management

Connection traffic doesn't differ from Traffic Connection

traffic in the area of QoS management."

::= { wmanIf2mBsConfigurationEntry 11 }

wmanIf2mBs2ndMgmtUlQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the index of a row in

wmanIf2mBsQoSProfileTable which is used to obtain all QoS

parameters required for the BS uplink scheduler to

properly allocate and manage the bandwidth and schedule

the 2nd Management Connection traffic. The 2nd Management

Connection traffic doesn't differ from Traffic Connection

traffic in the area of QoS management."

::= { wmanIf2mBsConfigurationEntry 12 }

wmanIf2mBsBasicCidDlQosProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the QoS parameter set for downlink basic CID."

::= { wmanIf2mBsConfigurationEntry 13 }

wmanIf2mBsBasicCidUlQosProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the QoS parameter set for uplink basic CID."

::= { wmanIf2mBsConfigurationEntry 14 }

wmanIf2mBsPrimaryCidDlQosProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the QoS parameter set for downlink primary CID

."

::= { wmanIf2mBsConfigurationEntry 15 }

wmanIf2mBsPrimaryCidUlQosProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the QoS parameter set for uplink primary CID."

::= { wmanIf2mBsConfigurationEntry 16 }

wmanIf2mBsSsReqCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the SS's capabilities that are

necessary for supporting mobility. SS reports these

capabilities in the REG-REQ messages."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsConfiguration 2 }

wmanIf2mBsSsReqCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has entered

and registered into the BS. The primary index is the

ifIndex with an ifType of ieee80216WMAN, indicating the BS

sector with which the SS is associated.

wmanIf2mBsSsMacAddress identifies the SS being registered."

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsSsReqCapabilitiesTable 1 }

WmanIf2mBsSsReqCapabilitiesEntry ::= SEQUENCE {

wmanIf2mBsSsReqCapHandoverSupported     WmanIf2mHandoverType,

wmanIf2mBsSsReqCapHoProcessTimer        Unsigned32,

wmanIf2mBsSsReqCapMobilityFeature       WmanIf2mOfdmaMobility,

wmanIf2mBsSsReqCapSleepRecoveryTime     Unsigned32,

wmanIf2mBsSsReqCapPreviousIpAddr        OCTET STRING,

wmanIf2mBsSsReqCapIdleModeTimeout       Unsigned32,

wmanIf2mBsSsReqCapHoConnProcessTime     Unsigned32,

wmanIf2mBsSsReqCapHoTekProcessTime      Unsigned32,

wmanIf2mBsSsReqCapPowerSavingType       WmanIf2mPowerSaveType,

wmanIf2mBsSsReqCapNumOfPsClassIandII    INTEGER,

wmanIf2mBsSsReqCapNumOfPsClassIII       INTEGER,

wmanIf2mBsSsReqCapHoTrigMatrix          WmanIf2mHoTrigMatrix,

wmanIf2mBsSsReqCapAssociationType       WmanIf2mAssociationTyp}

wmanIf2mBsSsReqCapHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 1 }

wmanIf2mBsSsReqCapHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization

element of the RNG-RSP message."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 2 }

wmanIf2mBsSsReqCapMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates whether or not the MS supports

mobility hand-over, Slee-pmode, and Idle-mode."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 3 }

wmanIf2mBsSsReqCapSleepRecoveryTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object indicates the time required for an MS that is

in a sleep mode to return to awake-mode. This may be used

by the BS to determine sleep interval window sizes when

initiating sleep mode with an MS."

REFERENCE

"Subclause 11.7.15 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 4 }

wmanIf2mBsSsReqCapPreviousIpAddr OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object indicates the IP address that the MS was

assigned on the secondary management connection based on an

association with its last serving BS. An IPv4 address shall

be specified in conventional dotted format; e.g.,

'134.234.2.3'. An IPv6 address may be expressed in abridged

or unabridged form; however, the form chosen shall be

consistent with RFC4291."

REFERENCE

"Subclause 11.7.16 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 5 }

wmanIf2mBsSsReqCapIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 6 }

wmanIf2mBsSsReqCapHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 7 }

wmanIf2mBsSsReqCapHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 8 }

wmanIf2mBsSsReqCapPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 9 }

wmanIf2mBsSsReqCapNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 10 }

wmanIf2mBsSsReqCapNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 11 }

wmanIf2mBsSsReqCapHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 12 }

wmanIf2mBsSsReqCapAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsReqCapabilitiesEntry 13 }

wmanIf2mBsSsRspCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the SS's capabilities that are necessary

for supporting mobility. BS acknowledges the capabilities in

the REG-RSP message in response to REG-REQ messages."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsConfiguration 3 }

wmanIf2mBsSsRspCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has entered and

registered into the BS. The primary index is the ifIndex

with an ifType of ieee80216WMAN, indicating the BS sector

with which the SS is associated. wmanIf2mBsSsMacAddress

identifies the SS being registered."

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsSsRspCapabilitiesTable 1 }

WmanIf2mBsSsRspCapabilitiesEntry ::= SEQUENCE {

wmanIf2mBsSsRspCapHandoverSupported     WmanIf2mHandoverType,

wmanIf2mBsSsRspCapRetrainTime           Unsigned32,

wmanIf2mBsSsRspCapHoProcessTimer        Unsigned32,

wmanIf2mBsSsRspCapRetransmissionTimer   Unsigned32,

wmanIf2mBsSsRspCapMobilityFeature       WmanIf2mOfdmaMobility,

wmanIf2mBsSsRspCapIdleModeTimeout       Unsigned32,

wmanIf2mBsSsRspCapHoConnProcessTime     Unsigned32,

wmanIf2mBsSsRspCapHoTekProcessTime      Unsigned32,

wmanIf2mBsSsRspCapPowerSavingType       WmanIf2mPowerSaveType,

wmanIf2mBsSsRspCapNumOfPsClassIandII    INTEGER,

wmanIf2mBsSsRspCapNumOfPsClassIII       INTEGER,

wmanIf2mBsSsRspCapHoTrigMatrix          WmanIf2mHoTrigMatrix,

wmanIf2mBsSsRspCapAssociationType       WmanIf2mAssociationTyp}

wmanIf2mBsSsRspCapHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 1 }

wmanIf2mBsSsRspCapRetrainTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "100 milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the duration for MS's connection information that

will be retained in serving BS. BS shall start

Resource_Retain_Time timer at MS notification of pending HO

attempt through MOB_HO-IND or by detecting an MS drop."

REFERENCE

"Subclause 11.7.13.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 2 }

wmanIf2mBsSsRspCapHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization element

of the RNG-RSP message. On HO Process Optimization MS Timer

timeout and while HO Process Optimization MS Timer Retries

is valid, MS shall send the network re-entry MAC management

request message corresponding to the expected and pending

network re-entry MAC management response message as

indicated in HO Process Optimization and recycle HO Process

Optimization MS Timer."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 3 }

wmanIf2mBsSsRspCapRetransmissionTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When an MS transmits MOB_MSHO-REQ to initiate a handover

process, it shall start MS Handover Retransmission Timer

and shall not transmit another MOB_MSHO-REQ until the

expiration of the MS Handover Retransmission Timer."

REFERENCE

"Subclause 11.7.13.3 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 4 }

wmanIf2mBsSsRspCapMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates the mobility hand-over, Sleep-mode, and

Idle-mode negotiated for MS."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 5 }

wmanIf2mBsSsRspCapIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 6 }

wmanIf2mBsSsRspCapHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 7 }

wmanIf2mBsSsRspCapHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 8 }

wmanIf2mBsSsRspCapPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 9 }

wmanIf2mBsSsRspCapNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 10 }

wmanIf2mBsSsRspCapNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 11 }

wmanIf2mBsSsRspCapHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 12 }

wmanIf2mBsSsRspCapAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsRspCapabilitiesEntry 13 }

wmanIf2mBsBasicCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities of the BS as

implemented in BS hardware and software. These capabilities

along with the configuration for them

(wmanIf2mBsCapabilitiesConfigTable) are used for negotiation

of basic capabilities with SS using RNG-RSP, SBC-RSP and

REG-RSP messages. The negotiated capabilities are obtained

by interSubclause of SS raw reported capabilities, BS raw

capabilities and BS configured capabilities. The objects in

the table have read-only access. The table is maintained

by BS."

::= { wmanIf2mBsConfiguration 4 }

wmanIf2mBsBasicCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2mBsBasicCapabilitiesTable 1 }

WmanIf2mBsBasicCapabilitiesEntry ::= SEQUENCE {

wmanIf2mBsCapHandoverSupported          WmanIf2mHandoverType,

wmanIf2mBsCapRetrainTime                Unsigned32,

wmanIf2mBsCapHoProcessTimer             Unsigned32,

wmanIf2mBsCapRetransmissionTimer        Unsigned32,

wmanIf2mBsCapMobilityFeature            WmanIf2mOfdmaMobility,

wmanIf2mBsCapIdleModeTimeout            Unsigned32,

wmanIf2mBsCapHoConnProcessTime          Unsigned32,

wmanIf2mBsCapHoTekProcessTime           Unsigned32,

wmanIf2mBsCapPowerSavingType            WmanIf2mPowerSaveType,

wmanIf2mBsCapNumOfPsClassIandII         INTEGER,

wmanIf2mBsCapNumOfPsClassIII            INTEGER,

wmanIf2mBsCapHoTrigMatrix               WmanIf2mHoTrigMatrix,

wmanIf2mBsCapAssociationType            WmanIf2mAssociationTyp}

wmanIf2mBsCapHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 1 }

wmanIf2mBsCapRetrainTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "100 milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the duration for MS's connection information that

will be retained in serving BS. BS shall start

Resource_Retain_Time timer at MS notification of pending HO

attempt through MOB_HO-IND or by detecting an MS drop."

REFERENCE

"Subclause 11.7.13.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 2 }

wmanIf2mBsCapHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization element

of the RNG-RSP message."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 3 }

wmanIf2mBsCapRetransmissionTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When an MS transmits MOB_MSHO-REQ to initiate a handover

process, it shall start MS Handover Retransmission Timer

and shall not transmit another MOB_MSHO-REQ until the

expiration of the MS Handover Retransmission Timer."

REFERENCE

"Subclause 11.7.13.3 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 4 }

wmanIf2mBsCapMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates the mobility hand-over, Sleep-mode, and

Idle-mode supported by BS."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 5 }

wmanIf2mBsCapIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 6 }

wmanIf2mBsCapHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 7 }

wmanIf2mBsCapHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 8 }

wmanIf2mBsCapPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 9 }

wmanIf2mBsCapNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 10 }

wmanIf2mBsCapNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 11 }

wmanIf2mBsCapHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 12 }

wmanIf2mBsCapAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 13 }

wmanIf2mBsCapabilitiesConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration for basic capabilities

of BS. The table is intended to be used to restrict the

Capabilities implemented by BS, for example in order to

comply with local regulatory requirements. The BS should use

the configuration along with the implemented Capabilities

(wmanIf2mBsBasicCapabilitiesTable) for negotiation of basic

capabilities with SS using RNG-RSP, SBC-RSP and REG-RSP

messages. The negotiated capabilities are obtained by

interSubclause of SS reported capabilities, BS raw

capabilities and BS configured capabilities. The objects in

the table have read-write access. The rows are created by

BS as a copy of wmanIf2mBsBasicCapabilitiesTable and can be

modified by NMS."

::= { wmanIf2mBsConfiguration 5 }

wmanIf2mBsCapabilitiesConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2mBsCapabilitiesConfigTable 1 }

WmanIf2mBsCapabilitiesConfigEntry ::= SEQUENCE {

wmanIf2mBsCapCfgHandoverSupported       WmanIf2mHandoverType,

wmanIf2mBsCapCfgRetrainTime             Unsigned32,

wmanIf2mBsCapCfgHoProcessTimer          Unsigned32,

wmanIf2mBsCapCfgRetransmissionTimer     Unsigned32,

wmanIf2mBsCapCfgMobilityFeature         WmanIf2mOfdmaMobility,

wmanIf2mBsCapCfgIdleModeTimeout         Unsigned32,

wmanIf2mBsCapCfgHoConnProcessTime       Unsigned32,

wmanIf2mBsCapCfgHoTekProcessTime        Unsigned32,

wmanIf2mBsCapCfgPowerSavingType         WmanIf2mPowerSaveType,

wmanIf2mBsCapCfgNumOfPsClassIandII      INTEGER,

wmanIf2mBsCapCfgNumOfPsClassIII         INTEGER,

wmanIf2mBsCapCfgHoTrigMatrix            WmanIf2mHoTrigMatrix,

wmanIf2mBsCapCfgAssociationType         WmanIf2mAssociationTyp}

wmanIf2mBsCapCfgHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 1 }

wmanIf2mBsCapCfgRetrainTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "100 milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the duration for MS's connection information that

will be retained in serving BS. BS shall start

Resource_Retain_Time timer at MS notification of pending HO

attempt through MOB_HO-IND or by detecting an MS drop."

REFERENCE

"Subclause 11.7.13.1 in IEEE Std 802.16e-2005"

DEFVAL		{ 1 }

::= { wmanIf2mBsCapabilitiesConfigEntry 2 }

wmanIf2mBsCapCfgHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization element

of the RNG-RSP message."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 3 }

wmanIf2mBsCapCfgRetransmissionTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When an MS transmits MOB_MSHO-REQ to initiate a handover

process, it shall start MS Handover Retransmission Timer

and shall not transmit another MOB_MSHO-REQ until the

expiration of the MS Handover Retransmission Timer."

REFERENCE

"Subclause 11.7.13.3 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 4 }

wmanIf2mBsCapCfgMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The field indicates the mobility hand-over, Sleep-mode, and

Idle-mode configured for the BS."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 5 }

wmanIf2mBsCapCfgIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

DEFVAL         { 4096 }

::= { wmanIf2mBsCapabilitiesConfigEntry 6 }

wmanIf2mBsCapCfgHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 7 }

wmanIf2mBsCapCfgHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 8 }

wmanIf2mBsCapCfgPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 9 }

wmanIf2mBsCapCfgNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 10 }

wmanIf2mBsCapCfgNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 11 }

wmanIf2mBsCapCfgHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 12 }

wmanIf2mBsCapCfgAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilitiesConfigEntry 13 }

wmanIf2mBsSsCidUpdateTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsCidUpdateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the 'CID update' TLV that is send in

the REG-RSP message to allow an MS to update its service

flows and connection information so that it may continue

service after a handover to a new serving BS.

The wmanIf2BsCid and wmanIf2BsSfTargetSaid objects in

wmanIf2BsServiceFlowTable in wmanIf2Mib shall be updated

with the CIDs and SAIDs included in the 'CID update' TLV.

If the service flow parameters changes are included in the

'Connection Info' TLV, the service flow information can be

found in wmanIf2BsServiceFlowTable."

REFERENCE

"Subclause 6.3.2.3.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsConfiguration 6 }

wmanIf2mBsSsCidUpdateEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsCidUpdateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each service flow."

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsSfId }

::= { wmanIf2mBsSsCidUpdateTable 1 }

WmanIf2mBsSsCidUpdateEntry ::= SEQUENCE {

wmanIf2mBsSsSfId                        Unsigned32,

wmanIf2mBsSsNewCid                      WmanIf2mCidType,

wmanIf2mBsSsNewSaid                     Integer32,

wmanIf2mBsSsOldSaid                     Integer32}

wmanIf2mBsSsSfId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow."

::= { wmanIf2mBsSsCidUpdateEntry 1 }

wmanIf2mBsSsNewCid OBJECT-TYPE

SYNTAX      WmanIf2mCidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The new CID at the target BS for a service flow that was

used by MS in the previous serving BS."

REFERENCE

"Subclause 11.7.10 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCidUpdateEntry 2 }

wmanIf2mBsSsNewSaid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates New SAID after handover to new BS. It

provides a translation table that allows an MS to update

its security associations so that it may continue security

service after a handover to a new serving BS."

REFERENCE

"Subclause 11.7.18 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCidUpdateEntry 3 }

wmanIf2mBsSsOldSaid OBJECT-TYPE

SYNTAX      Integer32 (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates Old SAID after handover to new BS. It

provides a translation table that allows an MS to update

its security associations so that it may continue security

service after a handover to a new serving BS."

REFERENCE

"Subclause 11.7.18 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCidUpdateEntry 4 }

--

-- Base Station Power Saving Mode

--

wmanIf2mBsPowerSavingMode OBJECT IDENTIFIER ::= { wmanIf2mBsCm 2 }

--

-- wmanIf2mBsSsPowerSavingStatusTable contains the power saving status

--

wmanIf2mBsSsPwrSaving2CidMapTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsPwrSaving2CidMapEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the list of CIDs for each power saving

class of every MS supporting sleep mode. When the MS roams

to a different BS, all entries associated with such MS will

be deleted."

::= { wmanIf2mBsPowerSavingMode 1 }

wmanIf2mBsSsPwrSaving2CidMapEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsPwrSaving2CidMapEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each CID in an MS."

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsCid  }

::= { wmanIf2mBsSsPwrSaving2CidMapTable 1 }

WmanIf2mBsSsPwrSaving2CidMapEntry::= SEQUENCE {

wmanIf2mBsSsCid                         WmanIf2mCidType,

wmanIf2mBsSsPowerSavingClassId          WmanIf2mPsClassId}

wmanIf2mBsSsCid OBJECT-TYPE

SYNTAX      WmanIf2mCidType

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 16 bit channel identifier to identify a connection."

::= { wmanIf2mBsSsPwrSaving2CidMapEntry 1 }

wmanIf2mBsSsPowerSavingClassId OBJECT-TYPE

SYNTAX      WmanIf2mPsClassId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2mBsSsPowerSavingClassId identifies the power saving

class associated with this CID. It maps to an entry in

wmanIf2mBsSsPowerSavingClassesTable."

::= { wmanIf2mBsSsPwrSaving2CidMapEntry 2 }

--

-- wmanIf2mBsSsPowerSavingClassesTable contains the power saving classes

-- information

--

wmanIf2mBsSsPowerSavingClassesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsPowerSavingClassesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table reports the parameters of all the power saving

classes negotiated between BS and MS using MOB_SLP-REQ and

MOB_SLP-RSP messages. When the MS roams to a different BS,

all entries associated with such MS will be deleted."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPowerSavingMode 2 }

wmanIf2mBsSsPowerSavingClassesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsPowerSavingClassesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"It is intended to support both unicast and multicast

service flows. wmanIf2mBsSsMacAddress contains the MAC

address of the MS to which the power saving classes are

associated."

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsPsClassId }

::= { wmanIf2mBsSsPowerSavingClassesTable 1 }

WmanIf2mBsSsPowerSavingClassesEntry ::= SEQUENCE {

wmanIf2mBsSsPsClassId                   WmanIf2mPsClassId,

wmanIf2mBsSsStartFrameNumber            INTEGER,

wmanIf2mBsSsPowerSavingClassType        WmanIf2mPsClassType,

wmanIf2mBsSsPsClassCidDirection         WmanIf2mPsClassCidDir,

wmanIf2mBsSsTrafficTriggeredWakening    INTEGER,

wmanIf2mBsSsInitialSleepWindow          INTEGER,

wmanIf2mBsSsFinalSleepWindowBase        INTEGER,

wmanIf2mBsSsFinalSleepWindowExponent    INTEGER,

wmanIf2mBsSsListeningWindow             INTEGER,

wmanIf2mBsSsPowerSavingMode             WmanIf2mPowerSavingMode,

wmanIf2mBsSsSlpId                       INTEGER}

wmanIf2mBsSsPsClassId OBJECT-TYPE

SYNTAX      WmanIf2mPsClassId

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This object uniquely identifies the power saving classes

in a MS."

::= { wmanIf2mBsSsPowerSavingClassesEntry 1 }

wmanIf2mBsSsStartFrameNumber OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Start frame number for first sleep window."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 2 }

wmanIf2mBsSsPowerSavingClassType OBJECT-TYPE

SYNTAX      WmanIf2mPsClassType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Power saving classes type I - BE & NRT-VR,

Power saving classes type II - UGS & RT-VR,

Power saving classes type III - multicast, management CID"

REFERENCE

"Subclause 6.3.21.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 3 }

wmanIf2mBsSsPsClassCidDirection OBJECT-TYPE

SYNTAX      WmanIf2mPsClassCidDir

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The direction of power saving class's CIDs."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 4 }

wmanIf2mBsSsTrafficTriggeredWakening OBJECT-TYPE

SYNTAX      INTEGER {notBeDeactivated (0),

beDeactivated (1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"0 = Power Saving Class shall not be deactivated if traffic

appears at the connection as per 6.3.19.2.

1 = Power Saving Class shall be deactivated if traffic

appears at the connection as 6.3.19.2."

REFERENCE

"Subclause 6.3.19.2, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 5 }

wmanIf2mBsSsInitialSleepWindow OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "frame"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The initial duration for the sleep window. It is not

relevant for Power Saving Class type III, and shall

return '0'."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 6 }

wmanIf2mBsSsFinalSleepWindowBase OBJECT-TYPE

SYNTAX      INTEGER (0..1023)

UNITS       "frame"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The final value for the sleep interval. It is not

relevant for Power Saving Class type II, and shall

return '0'. For Power Saving Class type III, it is the

base for duration of single sleep window request."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 7 }

wmanIf2mBsSsFinalSleepWindowExponent OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The factor by which the final-sleep window base is

multiplied in order to calculate the final-sleep window.

The following formula is used:

final-sleep window = final-sleep window base x

2^(final-sleep window exponent)

For Power Saving Class type III, it is the exponent for

the duration of single sleep window request."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 8 }

wmanIf2mBsSsListeningWindow OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "frame"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Duration of MS listening window. It is not relevant for

Power Saving Class type III, and shall return '0'."

REFERENCE

"Subclause 6.3.2.3.44, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 9 }

wmanIf2mBsSsPowerSavingMode OBJECT-TYPE

SYNTAX      WmanIf2mPowerSavingMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicate whether the power saving class is active or not."

REFERENCE

"Subclause 6.3.2.3.45, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 10 }

wmanIf2mBsSsSlpId OBJECT-TYPE

SYNTAX      INTEGER (0..1023)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2mBsSsSlpId is assigned by the BS whenever an MS is

instructed to enter sleep mode. This number shall be unique

among all MSs that are in sleep mode."

REFERENCE

"Subclause 6.3.2.3.45, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsPowerSavingClassesEntry 11 }

--

--

-- Neighbor Base Stations Advertizement

--

wmanIf2mBsNeighborAdv OBJECT IDENTIFIER ::= { wmanIf2mBsCm 3 }

wmanIf2mBsNeighborAdvCommonTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborAdvCommonEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the common attributes for the

MOB_NBR-ADV message."

REFERENCE

"Subclause 6.3.2.3.47, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdv 1 }

wmanIf2mBsNeighborAdvCommonEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborAdvCommonEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2mBsNeighborAdvCommonTable 1 }

WmanIf2mBsNeighborAdvCommonEntry::= SEQUENCE {

wmanIf2mBsSkipOptions                   WmanIf2mSkipOptBitMap,

wmanIf2mBsOperatorId                    WmanIf2mNbrOperatorId,

wmanIf2mBsNumOfNeighbors                Unsigned32,

wmanIf2mBsConfigChangeCount             INTEGER}

wmanIf2mBsSkipOptions OBJECT-TYPE

SYNTAX      WmanIf2mSkipOptBitMap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When a bit is set to 1, its corresponding field will be

omitted."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvCommonEntry 1 }

wmanIf2mBsOperatorId OBJECT-TYPE

SYNTAX      WmanIf2mNbrOperatorId

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The unique network ID shared by an association of BS. The

'Operator IE' field is present only if Bit #0 of

wmanIf2mBsSkipOptions is 0."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvCommonEntry 2 }

wmanIf2mBsNumOfNeighbors OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The count of the unique combination of Neighbor BSID,

Preamble Index, and DCD."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvCommonEntry 3 }

-- XXX

wmanIf2mBsConfigChangeCount OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Incremented by one (modulo 256) whenever any of the values

relating to any included data element changes, including

DCD and UCD parameters. If the value of this count in a

subsequent MOB_NBR-ADV message remains the same, the MS

can quickly disregard the entire message."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvCommonEntry 4 }

wmanIf2mBsNeighborAdvertizementTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborAdvertizementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes specific to each neighbor

BS for the MOB_NBR-ADV message."

REFERENCE

"Subclause 6.3.2.3.47, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdv 2 }

wmanIf2mBsNeighborAdvertizementEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborAdvertizementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each neighboring BSs."

INDEX     { ifIndex, wmanIf2mBsNeighborBsIndex }

::= { wmanIf2mBsNeighborAdvertizementTable 1 }

WmanIf2mBsNeighborAdvertizementEntry::= SEQUENCE {

wmanIf2mBsNeighborBsIndex               INTEGER,

wmanIf2mBsNeighborBsId                  WmanIf2mNbrBsId,

wmanIf2mBsPhyProfileId                  WmanIf2mPhyProfileId,

wmanIf2mBsFaIndex                       Unsigned32,

wmanIf2mBsEirp                          INTEGER,

wmanIf2mBsPreampleSubchIndex            Unsigned32,

wmanIf2mBsHandoverProcOptimization      WmanIf2mHoProcOptm,

wmanIf2mBsSchedulingService             WmanIf2mSchedulingSupp,

wmanIf2mBsChannelBandwidth              INTEGER,

wmanIf2mBsFftSize                       WmanIf2mOfdmaFftSize,

wmanIf2mBsCyclicPrefix                  WmanIf2mOfdmaCp,

wmanIf2mBsFrameDurationCode             WmanIf2mOfdmaFrame,

wmanIf2mBsMobilityFeatureSupported      WmanIf2mOfdmaMobility,

wmanIf2mBsNrbBsPagingGroupListIndex     INTEGER,

wmanIf2BsNeighborAdvertizementRowStatus RowStatus}

wmanIf2mBsNeighborBsIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Index to entries in wmanIf2mBsNeighborAdvertizementTable."

::= { wmanIf2mBsNeighborAdvertizementEntry 1 }

wmanIf2mBsNeighborBsId OBJECT-TYPE

SYNTAX      WmanIf2mNbrBsId

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The least significant 24 bits of the Base Station ID

parameter in the DL-MAP message of the Neighbor BS. The

'Neighbor BSID' field is present only if Bit #1 of

wmanIf2mBsSkipOptions bitmap is 0."

::= { wmanIf2mBsNeighborAdvertizementEntry 2 }

wmanIf2mBsPhyProfileId OBJECT-TYPE

SYNTAX      WmanIf2mPhyProfileId

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Aggregated IDs of Co-located FA Indicator, FA Configuration

Indicator, FFT size, Bandwidth, Operation Mode of the

starting subchannelization of a frame, and Channel Number."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 3 }

wmanIf2mBsFaIndex OBJECT-TYPE

SYNTAX      Unsigned32 (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field is present only if the faIndexInd bit in

WmanIf2mPhyProfileId is set to 1. Its definition shall be

determined by a service provider or a governmental body

like FCC after the licensed band is determined."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 4 }

wmanIf2mBsEirp OBJECT-TYPE

SYNTAX      INTEGER (-128 .. 127)

UNITS       "dBm"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field is present only if the bsEirpInd bit in

WmanIf2mPhyProfileId is not set. Otherwise, the BS has the

same EIRP as the serving BS."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 5 }

wmanIf2mBsPreampleSubchIndex OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"SCa and OFDMA PHY - this field defines the PHY specific

preamble.

OFDM PHY - the 5 LSB contain the active DL subchannel

index. The 3 MSB shall be Reserved and set to

'0b000'"

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 6 }

wmanIf2mBsHandoverProcOptimization OBJECT-TYPE

SYNTAX      WmanIf2mHoProcOptm

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field is present only if omitHoProcOptimization bit in

WmanIf2mPhyProfileId is not set. Each bit in this field

indicates certain reentry message may be omitted."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 7 }

wmanIf2mBsSchedulingService OBJECT-TYPE

SYNTAX      WmanIf2mSchedulingSupp

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field is present only if omitQosRelatedField bit in

WmanIf2mPhyProfileId is not set."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 8 }

wmanIf2mBsChannelBandwidth OBJECT-TYPE

SYNTAX      INTEGER (0 .. 127)

UNITS       "125KHz"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field indicates the channel BW in units of 125 kHz."

REFERENCE

"Subclause 11.18.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 9 }

wmanIf2mBsFftSize OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaFftSize

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field indicates the channel BW in units of 125 kHz

for OFDMA PHY."

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 10 }

wmanIf2mBsCyclicPrefix OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaCp

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field indicates the CP for OFDMA PHY."

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 11 }

wmanIf2mBsFrameDurationCode OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaFrame

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field indicates the frame duration for ODMA PHY."

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 12 }

wmanIf2mBsMobilityFeatureSupported OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This field indicates whether the neighbor BS supports

mobility features."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborAdvertizementEntry 13 }

wmanIf2mBsNrbBsPagingGroupListIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"If idle mode is supported, this index maps to

wmanIf2mBsPagingGroupListId in wmanIf2mBsPagingGroupsTable

, and is used to identify the list of paging group IDs,

assigned to a neighbor BS."

::= { wmanIf2mBsNeighborAdvertizementEntry 14 }

wmanIf2BsNeighborAdvertizementRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or delete

an existing row in this table. If the implementator of this

MIB has choosen not to implement 'dynamic assignment' of

profiles, this object is not useful and should return

noSuchName upon SNMP request."

::= { wmanIf2mBsNeighborAdvertizementEntry 15 }

wmanIf2mBsNeighborBsOfdmaUcdTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborBsOfdmaUcdEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes of the UCD message for

the neighboring BSs."

REFERENCE

"Table 349 and Table 353, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborAdv 3 }

wmanIf2mBsNeighborBsOfdmaUcdEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborBsOfdmaUcdEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each neighboring BS."

INDEX     { wmanIf2mBsNeighborBsId }

::= { wmanIf2mBsNeighborBsOfdmaUcdTable 1 }

WmanIf2mBsNeighborBsOfdmaUcdEntry ::= SEQUENCE {

wmanIf2mBsOfdmaCtBasedResvTimeout       INTEGER,

wmanIf2mBsOfdmaUplinkCenterFreq         Unsigned32,

wmanIf2mBsOfdmaStartOfRngCodes          INTEGER,

wmanIf2mBsOfdmaPermutationBase          INTEGER,

wmanIf2mBsOfdmaULAllocSubchBitmap       OCTET STRING,

wmanIf2mBsOfdmaOptPermULAlloSubchBitmap OCTET STRING,

wmanIf2mBsOfdmaBandAMCAllocThreshold    INTEGER,

wmanIf2mBsOfdmaBandAMCReleaseThreshold  INTEGER,

wmanIf2mBsOfdmaBandAMCAllocTimer        INTEGER,

wmanIf2mBsOfdmaBandAMCReleaseTimer      INTEGER,

wmanIf2mBsOfdmaBandStatRepMAXPeriod     INTEGER,

wmanIf2mBsOfdmaBandAMCRetryTimer        INTEGER,

wmanIf2mBsOfdmaHandoverRangingStart     INTEGER,

wmanIf2mBsOfdmaHandoverRangingEnd       INTEGER,

wmanIf2mBsOfdmaHARQAackDelayDLBurst     WmanIf2mHarqAckDelay,

wmanIf2mBsOfdmaUlAmcAlloPhyBandsBitmap  OCTET STRING,

wmanIf2mBsOfdmaMaxRetransmission        INTEGER,

wmanIf2mBsOfdmaNormalizedCnOverride     OCTET STRING,

wmanIf2mBsOfdmaSizeOfCqichId            INTEGER,

wmanIf2mBsOfdmaNormalizedCnValue        INTEGER,

wmanIf2mBsOfdmaNormalizedCnOverride2    OCTET STRING,

wmanIf2mBsOfdmaBandAmcEntryAvgCinr      INTEGER,

wmanIf2mBsOfdmaAasPreambleUpperBond     INTEGER,

wmanIf2mBsOfdmaAasPreambleLowerBond     INTEGER,

wmanIf2mBsOfdmaAasBeamSelectAllowed     INTEGER,

wmanIf2mBsOfdmaCqichIndicationFlag      OCTET STRING,

wmanIf2mBsOfdmaUpPowerAdjStep           Unsigned32,

wmanIf2mBsOfdmaDownPowerAdjStep         Unsigned32,

wmanIf2mBsOfdmaMinPowerOffsetAdj        INTEGER,

wmanIf2mBsOfdmaMaxPowerOffsetAdj        INTEGER,

wmanIf2mBsOfdmaHandoverRngCodes         INTEGER,

wmanIf2mBsOfdmaInitialRngInterval       INTEGER,

wmanIf2mBsOfdmaTxPwrRepThreshold        INTEGER,

wmanIf2mBsOfdmaTprPower                 INTEGER,

wmanIf2mBsOfdmaAlphaPavg                INTEGER,

wmanIf2mBsOfdmaCqichTxPwrRepThreshold   INTEGER,

wmanIf2mBsOfdmaCqichTprPower            INTEGER,

wmanIf2mBsOfdmaCqichAlphaPavg           INTEGER,

wmanIf2mBsOfdmaNormalizedCnChSounding   INTEGER,

wmanIf2mBsOfdmaInitialRngBackoffStart   INTEGER,

wmanIf2mBsOfdmaInitialRngBackoffEnd     INTEGER,

wmanIf2mBsOfdmaBwRequestBackoffStart    INTEGER,

wmanIf2mBsOfdmaBwRequestBackoffEnd      INTEGER}

wmanIf2mBsOfdmaCtBasedResvTimeout OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The number of UL-MAPs to receive before contention-based

reservation is attempted again for the same connection."

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 1 }

wmanIf2mBsOfdmaUplinkCenterFreq OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

" Uplink center frequency (kHz)"

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 2 }

wmanIf2mBsOfdmaStartOfRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the starting number, S, of the group of codes

used for this uplink.All the ranging codes used on this

uplink will be between S and ((S+N+M+L) mod 256). Where,

N: the number of initial-ranging codes

M: the number of periodic-ranging codes

L: the number of bandwidth-request codes

O: the number of handover-ranging codes"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL 	    { 0 }

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 3 }

wmanIf2mBsOfdmaPermutationBase OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Determines the UL_PermBase parameter for the subcarrier

permutation to be used on this uplink channel.

UL_PermBase = 7 LSBs of Permutation base."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 0 }

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 4 }

wmanIf2mBsOfdmaULAllocSubchBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (9))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a bitmap describing the physical sub-channels

allocated to the segment in the UL, when using the uplink

PUSC permutation. The LSB of the first byte shall corrspond

to subchannel 0. For any bit that is not set, the

corresponding subchannel shall not be used by the SS on

that segment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 5 }

wmanIf2mBsOfdmaOptPermULAlloSubchBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (13))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a bitmap describing the sub-channels allocated to

the segment in the UL, when using the uplink optional PUSC

permutation (see 8.4.6.2.5 in IEEE Std 802.16-2004). The

LSB of the first byte shall correspond to subchannel 0.

For any bit that is not set, the corresponding subchannel

shall not be used by the SS on that segment. When this TLV

is not present, BS may allocate any subchannels to an SS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 6 }

wmanIf2mBsOfdmaBandAMCAllocThreshold OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the maximum of the standard deviations of the

individual bands CINR measurements over time to trigger

mode transition from normal subchannel to Band AMC"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 7 }

wmanIf2mBsOfdmaBandAMCReleaseThreshold OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the maximum of the standard deviations of the

individual bands CINR measurements over time to trigger

mode transition from Band AMC to normal subchannel"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 8 }

wmanIf2mBsOfdmaBandAMCAllocTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum required number of frames to measure the average

and standard deviation for the event of Band AMC triggering"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 9 }

wmanIf2mBsOfdmaBandAMCReleaseTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum required number of frames to measure the average

and standard deviation for the event triggering from Band

AMC to normal subchannel"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 10 }

wmanIf2mBsOfdmaBandStatRepMAXPeriod OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum period between refreshing the Band CINR

measurement by the unsolicited REP-RSP"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 11 }

wmanIf2mBsOfdmaBandAMCRetryTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Backoff timer between consecutive mode transitions from

normal subchannel to Band AMC when the previous request

is failed"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 12 }

wmanIf2mBsOfdmaHandoverRangingStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for MS performing initial

ranging during handover process, expressed as a power of 2."

REFERENCE

"Table 349, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 13 }

wmanIf2mBsOfdmaHandoverRangingEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for MS performing initial

ranging during handover process, expressed as a power

of 2."

REFERENCE

"Table 349, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 14 }

wmanIf2mBsOfdmaHARQAackDelayDLBurst OBJECT-TYPE

SYNTAX      WmanIf2mHarqAckDelay

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the OFDMA H-ARQ ACK delay for DL

burst."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 15 }

wmanIf2mBsOfdmaUlAmcAlloPhyBandsBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (6))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"A bitmap describing the physical bands allocated to the

segment in the UL, when using the optional AMC permutation

with regular MAPs (see 8.4.6.3). The LSB of the first byte

shall correspond to the physical band 0. For any bit that

is not set, the corresponding physical bands shall not be

used by the SS on that segment. When this TLV is not

present, BS may allocate any physical bands to an SS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 16 }

wmanIf2mBsOfdmaMaxRetransmission OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of retransmission in UL HARQ."

REFERENCE

"Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { 4 }

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 17 }

wmanIf2mBsOfdmaNormalizedCnOverride OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (8))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a list of numbers, where each number is encoded by

one nibble, and interpreted as a signed integer. The

nibbles correspond in order to the list define by Table

334, starting from the second line, such that the LS

nibble of the first byte corresponds to the second line in

the table. The number encoded by each nibble represents

the difference in normalized C/N relative to the previous

line in the table."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 18 }

wmanIf2mBsOfdmaSizeOfCqichId OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Size of CQICH ID field.

0 = 0 bits

1 = 3 bits

2 = 4 bits

3 = 5 bits

4 = 6 bits

5 = 7 bits

6 = 8 bits

7 = 9 bits"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 19 }

wmanIf2mBsOfdmaNormalizedCnValue OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"It shall be interpreted as signed integer in dB. It

corresponds to the normalized C/N value in the first line

(counting except for header cell of table)"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 20 }

wmanIf2mBsOfdmaNormalizedCnOverride2 OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (7))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a list of numbers, where each number is encoded by

one nibble, and interpreted as a signed integer. The

nibbles correspond in order to the list define by Table 334

, starting from the second line (counting except for the

header cell of table), such that the LS nibble of the first

byte corresponds to the second line in the table. The

number encoded by each nibble represents the difference in

normalized C/N relative to the previous line in the table."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 21 }

wmanIf2mBsOfdmaBandAmcEntryAvgCinr OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the average CINR of the whole bandwidth to

trigger mode transition from normal subchannel to AMC"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 22 }

wmanIf2mBsOfdmaAasPreambleUpperBond OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Upper bound of AAS preamble."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 23 }

wmanIf2mBsOfdmaAasPreambleLowerBond OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Lower bound of AAS preamble."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 24 }

wmanIf2mBsOfdmaAasBeamSelectAllowed OBJECT-TYPE

SYNTAX      INTEGER {notAllowed(0),

allowed(1)}

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicate whether unsolicited AAS Beam Select messages (see

6.3.2.3.41 in IEEE 802.16e-2005) should be sent by the MS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { allowed }

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 25 }

wmanIf2mBsOfdmaCqichIndicationFlag OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (1))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The N MSB values of this field represents the N-bit payload

value on the Fast-Feedback channel reserved as indication

flag for MS to initiate feedback on the Feedback header,

where N is the number of payload bits used for S/N

measurement feedback on the Fast-Feedback channel. The

value shall not be set to all zeros."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 26 }

wmanIf2mBsOfdmaUpPowerAdjStep OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "0.01 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS-specific up power offset adjustment step"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 27 }

wmanIf2mBsOfdmaDownPowerAdjStep OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "0.01 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS-specific down power offset adjustment step"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 28 }

wmanIf2mBsOfdmaMinPowerOffsetAdj OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "0.1 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum level of power offset adjustment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 29 }

wmanIf2mBsOfdmaMaxPowerOffsetAdj OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "0.1 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum level of power offset adjustment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 30 }

wmanIf2mBsOfdmaHandoverRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of handover ranging CDMA codes"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 31 }

wmanIf2mBsOfdmaInitialRngInterval OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of frames between initial ranging interval allocation

."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 32 }

wmanIf2mBsOfdmaTxPwrRepThreshold OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report threshold.

wmanIf2mBsOfdmaTxPwrRepThreshold = 0b1111 means infinite."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 33 }

wmanIf2mBsOfdmaTprPower OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report interval = 2 ^ wmanIf2mBsOfdmaTprPower.

The unit of Tx power report interval is frame.

wmanIf2mBsOfdmaTprPower = 0b1111 means infinite."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 34 }

wmanIf2mBsOfdmaAlphaPavg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Aplha p_avg parameter as shown in equation 138d in IEEE

802.16e-2005 indicates the multiple of 1/16. For example

'0' means 1/16, 15 means 16/16. "

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 35 }

wmanIf2mBsOfdmaCqichTxPwrRepThreshold OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report threshold.

wmanIf2mBsOfdmaTxPwrRepThreshold = 0b1111 means infinite.

It shall be used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 36 }

wmanIf2mBsOfdmaCqichTprPower OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report interval = 2 ^ wmanIf2mBsOfdmaTprPower.

The unit of Tx power report interval is frame.

wmanIf2mBsOfdmaTprPower = 0b1111 means infinite.

It shall be used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 37 }

wmanIf2mBsOfdmaCqichAlphaPavg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Aplha p_avg parameter as shown in equation 138d in

IEEE 802.16e-2005 indicates the multiple of 1/16. For

example '0' means 1/16, 15 means 16/16. It shall be

used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 38 }

wmanIf2mBsOfdmaNormalizedCnChSounding OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Signed integer for the required C/N (dB) for Channel

Sounding."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 39 }

wmanIf2mBsOfdmaInitialRngBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for initial ranging

contention, expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 40 }

wmanIf2mBsOfdmaInitialRngBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for initial ranging

contention, expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 41 }

wmanIf2mBsOfdmaBwRequestBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for contention BW requests,

expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 42 }

wmanIf2mBsOfdmaBwRequestBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for contention BW requests,

expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaUcdEntry 43 }

wmanIf2mBsNeighborBsOfdmaDcdTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborBsOfdmaDcdEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes of the DCD message for

the neighboring BSs."

::= { wmanIf2mBsNeighborAdv 4 }

wmanIf2mBsNeighborBsOfdmaDcdEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborBsOfdmaDcdEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each neighboring BS."

INDEX     { wmanIf2mBsNeighborBsId }

::= { wmanIf2mBsNeighborBsOfdmaDcdTable 1 }

WmanIf2mBsNeighborBsOfdmaDcdEntry ::= SEQUENCE {

wmanIf2mBsOfdmaBsEIRP                   INTEGER,

wmanIf2mBsOfdmaChannelNumber            INTEGER,

wmanIf2mBsOfdmaTTG                      INTEGER,

wmanIf2mBsOfdmaRTG                      INTEGER,

wmanIf2mBsOfdmaInitRngMaxRSS            INTEGER,

wmanIf2mBsOfdmaDownlinkCenterFreq       Unsigned32,

wmanIf2mBsOfdmaBsId                     OCTET STRING,

wmanIf2mBsOfdmaMacVersion               WmanIf2mMacVersion,

wmanIf2mBsOfdmaFrameDurationCode        INTEGER,

wmanIf2mBsOfdmaHARQAackDelayULBurst     WmanIf2mHarqAckDelay,

wmanIf2mBsOfdmaHarqZonePermutation      INTEGER,

wmanIf2mBsOfdmaHMaxRetransmission       INTEGER,

wmanIf2mBsOfdmaCinrAlphaAvg             INTEGER,

wmanIf2mBsOfdmaRssiAlphaAvg             INTEGER,

wmanIf2mBsOfdmaDlAmcAlloPhyBandsBitmap  OCTET STRING,

wmanIf2mBsOfdmaHandoverSupported        BITS,

wmanIf2mBsOfdmaThresholdAddBsDivSet     INTEGER,

wmanIf2mBsOfdmaThresholdDelBsDivSet     INTEGER,

wmanIf2mBsOfdmaAsrSlotLength            INTEGER,

wmanIf2mBsOfdmaAsrSwitchingPeriod       INTEGER,

wmanIf2mBsOfdmaHysteresisMargin         INTEGER,

wmanIf2mBsOfdmaTimeToTrigger            INTEGER,

wmanIf2mBsOfdmaRestartCount             INTEGER}

wmanIf2mBsOfdmaBsEIRP OBJECT-TYPE

SYNTAX      INTEGER (-32768..32767)

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The EIRP is the equivalent isotropic radiated power of the

base station, which is computed for a simple

single-antenna transmitter."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 1 }

wmanIf2mBsOfdmaChannelNumber OBJECT-TYPE

SYNTAX      INTEGER (0 .. 199)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Downlink channel number as defined in 8.5. Used for

license-exempt operation only."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 2 }

wmanIf2mBsOfdmaTTG OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "PS"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Transmit / Receive Transition Gap. Used on TDD system only."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 3 }

wmanIf2mBsOfdmaRTG OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "PS"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Receive / Transmit Transition Gap. Used on TDD system only."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 4 }

wmanIf2mBsOfdmaInitRngMaxRSS OBJECT-TYPE

SYNTAX      INTEGER (-32768..32767)

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial Ranging Max. equivalent isotropic received power

at BS Signed in units of 1 dBm."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 5 }

wmanIf2mBsOfdmaDownlinkCenterFreq OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Downlink center frequency (kHz)."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 6 }

wmanIf2mBsOfdmaBsId OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(6))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Defines the encoding of BSID. The BSID is a 6 byte number

and follows the encoding rules of MacAddress textual

convention, i.e. as if it were transmitted

least-significant bit first. The value should be displayed

with 2 parts clearly separated by a colon e.g:

001DFF:00003A. The most significant part is representing

the Operator ID."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 7 }

wmanIf2mBsOfdmaMacVersion OBJECT-TYPE

SYNTAX      WmanIf2mMacVersion

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This parameter specifies the version of 802.16 to which

the message originator conforms."

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 8 }

wmanIf2mBsOfdmaFrameDurationCode OBJECT-TYPE

SYNTAX      INTEGER {aasGap(0),

duration2ms(1),

duration2dot5ms(2),

duration4ms(3),

duration5ms(4),

duration8ms(5),

duration10ms(6),

duration12dot5ms(7),

duration20ms(8)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration of the frame. The frame duration code values

are specified in Table 274."

REFERENCE

"Table 274, in IEEE Std 802.16-2004"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 9 }

wmanIf2mBsOfdmaHARQAackDelayULBurst OBJECT-TYPE

SYNTAX      WmanIf2mHarqAckDelay

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the OFDMA H-ARQ ACK delay for UL

burst."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 10 }

wmanIf2mBsOfdmaHarqZonePermutation OBJECT-TYPE

SYNTAX      INTEGER {pusc(1),

fusc(2),

optionalFusc(3),

amc(4)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Permutation type for broadcast region in HARQ zone"

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 11 }

wmanIf2mBsOfdmaHMaxRetransmission OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of retransmission in DL HARQ."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 4 }

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 12 }

wmanIf2mBsOfdmaCinrAlphaAvg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of Default RSSI and CINR averaging parameter TLV.

Default averaging parameter Alpha Avg for physical CINR

measurements, in multiples of 1/16. For example

'0' means 1/16, 15 means 16/16."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 3 }

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 13 }

wmanIf2mBsOfdmaRssiAlphaAvg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of Default RSSI and CINR averaging parameter TLV.

Default averaging parameter Alpha Avg for physical RSSI

measurements, in multiples of 1/16. For example

'0' means 1/16, 15 means 16/16."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 3 }

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 14 }

wmanIf2mBsOfdmaDlAmcAlloPhyBandsBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (6))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"A bitmap describing the physical bands allocated to the

segment in the DL, when allocating AMC subchannels

through the HARQ MAP, or through the Normal MAP, or for

Band-AMC CINR reports, or using the optional AMC

permutation (see 8.4.6.3). The LSB of the first byte

shall correspond to band 0. For any bit that is not set,

the corresponding band shall not be used by the SS on

that segment. When this TLV is not present, BS may

allocate any physical bands to an SS."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 15 }

wmanIf2mBsOfdmaHandoverSupported OBJECT-TYPE

SYNTAX      BITS {handover(0),

mdHandover(1),

fbssHandover(2)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the types of handover supported.

Bit #0 = HO

Bit #1 = MDHO

Bit #2 = FBSS HO."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 16 }

wmanIf2mBsOfdmaThresholdAddBsDivSet OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold used by the MS to add a neighbor BS to the

diversity set. When the CINR of a neighbor BS is higher

than H_Add_Threshold, the MS should send MOB_MSHO-REQ to

request adding this neighbor BS to the diversity set.

This threshold is used for the MS that is performing

MDHO/FBSS HO. If the BS does not support FBSS HO/MDHO,

this value is not set."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 17 }

wmanIf2mBsOfdmaThresholdDelBsDivSet OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold used by the MS to delete a neighbor BS to the

diversity set. When the CINR of a neighbor BS is lower

than H_Add_Threshold, the MS should send MOB_MSHO-REQ to

request dropping this neighbor BS to the diversity set.

This threshold is used for the MS that is performing

MDHO/FBSS HO. If the BS does not support FBSS HO/MDHO,

this value is not set."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 18 }

wmanIf2mBsOfdmaAsrSlotLength OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "Frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of ASR Slot Length and Switching Period. For FBSS

operation, the time axis is slotted by an ASR (Anchor

Switch Reporting) slot that is

wmanIf2mBsOfdmaAsrSlotLength frame long."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 19 }

wmanIf2mBsOfdmaAsrSwitchingPeriod OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "ASR slots"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of ASR Slot Length and Switching Period.

A switching period is introduced whose duration is equals

to wmanIf2mBsOfdmaAsrSwitchingPeriod ASR slots that should

be long enough such that certain process (e.g., HARQ

transmission, backhaul context transfer) can be completed

at the current anchor BS before the MS switches to the new

anchor BS."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 20 }

wmanIf2mBsOfdmaHysteresisMargin OBJECT-TYPE

SYNTAX      INTEGER (0..57)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When the CINR of a neighbor BS is larger than the sum of

the CINR of the current serving BS and

wmanIf2mBsOfdmaHysteresisMargin for the time-to-trigger

duration, then the neighbor BS is included in the list

of possible target BSs in MOB_MSHO-REQ."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 21 }

wmanIf2mBsOfdmaTimeToTrigger OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the time duration for MS decides to select a

neighbor BS as a possible target BS. It is  applicable only

for HHO."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry 22 }

wmanIf2mBsOfdmaRestartCount OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value is incremented by one whenever BS restarts (see

6.3.9.11). The value rolls over from 0 to 255."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2mBsNeighborBsOfdmaDcdEntry  23 }

wmanIf2mBsPaging OBJECT IDENTIFIER ::= { wmanIf2mBsCm 4 }

--

-- wmanIf2mBsPagingAdvertizementTable

--

wmanIf2mBsPagingAdvertizementTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsPagingAdvertizementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that BS broadcasts in

the MOB_PAG-ADV message."

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPaging 1 }

wmanIf2mBsPagingAdvertizementEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsPagingAdvertizementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2mBsPagingAdvertizementTable 1 }

WmanIf2mBsPagingAdvertizementEntry ::= SEQUENCE {

wmanIf2mBsPagingGroupListIndex          INTEGER,

wmanIf2mBsPagingRspWindow               INTEGER,

wmanIf2mBsPagingAdvRowStatus            RowStatus}

wmanIf2mBsPagingGroupListIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"wmanIf2mBsPagingGroupListIndex maps to

wmanIf2mBsPagingGroupListId in wmanIf2mBsPagingGroupsTable

, and is used to identify the list of paging group IDs."

::= { wmanIf2mBsPagingAdvertizementEntry 1 }

wmanIf2mBsPagingRspWindow OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frames"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"OFDMA-PHY specific parameter used to indicate the time

window during which the MS shall transmit the CDMA code at

the transmission opportunity assigned in the CDMA code and

transmission opportunity assignment TLV. The start of the

window is the next frame after receiving the MOB_PAG-ADV."

REFERENCE

"Subclause 11.17.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPagingAdvertizementEntry 2 }

wmanIf2mBsPagingAdvRowStatus OBJECT-TYPE

SYNTAX          RowStatus

MAX-ACCESS      read-create

STATUS          current

DESCRIPTION

"This object is used to ensure that the write, create,

delete operation to multiple columns is guaranteed to be

treated as atomic operation by agent."

::= { wmanIf2mBsPagingAdvertizementEntry 3 }

wmanIf2mBsMsPagedTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMsPagedEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the MSs that are paged in the

MOB_PAG-ADV message."

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPaging 2 }

wmanIf2mBsMsPagedEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMsPagedEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsMsPagedTable 1 }

WmanIf2mBsMsPagedEntry ::= SEQUENCE {

wmanIf2mBsSsMacAddrHash                 WmanIf2mSsMacAddrHash,

wmanIf2mBsPagingActionCode              WmanIf2mPagingAction,

wmanIf2mBsCdmaCode                      INTEGER,

wmanIf2mBsTransmitOpportunity           INTEGER}

wmanIf2mBsSsMacAddrHash OBJECT-TYPE

SYNTAX      WmanIf2mSsMacAddrHash

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The hash is obtained by computing a CRC24 on the MS 48-bit

MAC address. The polynomial for the calculation is

0x1864CFB"

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

::= { wmanIf2mBsMsPagedEntry 1 }

wmanIf2mBsPagingActionCode OBJECT-TYPE

SYNTAX      WmanIf2mPagingAction

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Paging action instruction to MS."

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

::= { wmanIf2mBsMsPagedEntry 2 }

wmanIf2mBsCdmaCode OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"OFDMA-PHY specific parameter used to indicate CDMA code

and assigned to one or more MSs being paged in this

message. One CDMA code assignment in the TLV corresponds

to one MS paged. If wmanIf2mBsPagingActionCode is 'No

Action Required', then it should return 0."

REFERENCE

"Subclause 11.17.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsMsPagedEntry 3 }

wmanIf2mBsTransmitOpportunity OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"OFDMA-PHY specific parameter used to indicate transmission

opportunity assigned to one or more MSs being paged in this

message. One transmission opportunity assignment in the TLV

corresponds to one MS paged. If wmanIf2mBsPagingActionCode

is 'No Action Required', then it should return 0."

REFERENCE

"Subclause 11.17.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsMsPagedEntry 4 }

wmanIf2mBsPagingGroupsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsPagingGroupsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains paging group IDs that BS can broadcast

in the MOB_PAG-ADV message."

REFERENCE

"Table 109f and Table 358 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPaging 3 }

wmanIf2mBsPagingGroupsEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsPagingGroupsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains a paging group ID. If multiple paging

group IDs are to be formed in a list that will be

broadcast by a BS, these paging group IDs should be

identified by the same wmanIf2mBsPagingGroupListId value."

INDEX     { wmanIf2mBsPagingGroupListId,

wmanIf2mBsPagingGroupId }

::= { wmanIf2mBsPagingGroupsTable 1 }

WmanIf2mBsPagingGroupsEntry ::= SEQUENCE {

wmanIf2mBsPagingGroupListId             INTEGER,

wmanIf2mBsPagingGroupId                 INTEGER,

wmanIf2BsPagingGroupsRowStatus          RowStatus}

wmanIf2mBsPagingGroupListId OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index to the wmanIf2mBsPagingGroupsTable."

REFERENCE

"Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPagingGroupsEntry 1 }

wmanIf2mBsPagingGroupId OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This field indicates the ID of the paging group."

REFERENCE

"Subclause 6.3.2.3.47, Table 109f in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPagingGroupsEntry 2 }

wmanIf2BsPagingGroupsRowStatus OBJECT-TYPE

SYNTAX          RowStatus

MAX-ACCESS      read-create

STATUS          current

DESCRIPTION

"This object is used to ensure that the write, create,

delete operation to multiple columns is guaranteed to be

treated as atomic operation by agent."

::= { wmanIf2mBsPagingGroupsEntry 3 }

wmanIf2mBsServiceFlow OBJECT IDENTIFIER ::= { wmanIf2mBsCm 5 }

wmanIf2mBsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the service flow database. When an SS

first registers at the BS, the BS should download the

SS' service flow profile (e.g. QoS parameter set and

classification rules) from the home AAA server.

For fixed or normadic SS, its service flow profile will

not be changed in the entire duration of the session.

For portable or mobile SS, when the SS handoffs to another

BS, as part of the context transfer, the serving BS should

transfer service flow profile to the target BS. After the

handoff, the old serving BS shall change the

wmanIf2mBsServiceFlowState of the service flows, previously

used by the SS to 'inactive'.

The BS may cleanup wmanIf2mBsServiceFlowTable periodically,

by removing those entries with wmanIf2mBsServiceFlowState

= 'inactive'."

REFERENCE

"Subclause 6.3.14 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsServiceFlow 1 }

wmanIf2mBsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each service flow. It

supports both unicast and multicast service flows:

Unicast - a SS (wmanIf2mBsSsMacAddress) may contain

multiple service flows (wmanIf2mBsSsSfId)

Multicast - a service flow (wmanIf2mBsSsSfId) may be

multicast to multiple SS

(wmanIf2mBsSsMacAddress)"

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsSfId }

::= { wmanIf2mBsServiceFlowTable 1 }

WmanIf2mBsServiceFlowEntry ::= SEQUENCE {

wmanIf2mBsSsMacAddress                  MacAddress,

wmanIf2mBsServiceFlowDirection          WmanIf2mSfDirection,

wmanIf2mBsProvisionedGlobalServiceClass WmanIf2mGlobalSrvClass,

wmanIf2mBsAdmittedGlobalServiceClass    WmanIf2mGlobalSrvClass,

wmanIf2mBsActiveGlobalServiceClass      WmanIf2mGlobalSrvClass,

wmanIf2mBsProvisionedQoSProfileIndex    INTEGER,

wmanIf2mBsAdmittedQoSProfileIndex       INTEGER,

wmanIf2mBsActiveQoSProfileIndex         INTEGER,

wmanIf2mBsClassifierRuleIndex           Unsigned32,

wmanIf2mBsPhsRuleIndex                  INTEGER,

wmanIf2mBsArqAttributeIndex             INTEGER,

wmanIf2mBsServiceFlowState              WmanIf2mSfState,

wmanIf2mBsCid                           INTEGER,

wmanIf2mBsSfCsSpecification             WmanIf2mCsSpecification,

wmanIf2mBsSfMinTolerableTrafficRate     Unsigned32,

wmanIf2mBsSfReqTxPolicy                 WmanIf2mReqTxPolicy,

wmanIf2mBsSfTargetSaid                  INTEGER,

wmanIf2mBsSfEstablishTime               TimeStamp,

wmanIf2mBsSfTerminateTime               TimeStamp}

wmanIf2mBsSsMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The MAC address of the SS that the service flow is

associated with."

::= { wmanIf2mBsServiceFlowEntry 1 }

wmanIf2mBsServiceFlowDirection OBJECT-TYPE

SYNTAX      WmanIf2mSfDirection

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An attribute indicating the direction of a service flow."

::= { wmanIf2mBsServiceFlowEntry 2 }

wmanIf2mBsProvisionedGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ProvisionedQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2mBsProvisionedQoSProfileIndex."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mBsServiceFlowEntry 3 }

wmanIf2mBsAdmittedGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the AdmittededQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2mBsAdmittedQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mBsServiceFlowEntry 4 }

wmanIf2mBsActiveGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ActiveQoSParamSet for this service

flow. When '0' is returned from reading this object, it

means either no global service class is defined, or its Qos

profile may be defined in wmanIf2mBsActiveQoSProfileIndex.

ActiveQoSParamSet is a subset of AdmittededQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mBsServiceFlowEntry 5 }

wmanIf2mBsProvisionedQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the ProvisionedQoSParamSet of a service flow.

If WmanIf2mSfState = 'provisioned', then

ProvisionedQoSParamSet is the QoS profile for this service

flow. When '0' is returned from reading this object, it

means the QoS profile either is not defined, or is defined

in wmanIf2mBsProvisionedQoSProfileIndex."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 6 }

wmanIf2mBsAdmittedQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the AdmittedQoSParamSet of a service flow. If

WmanIf2mSfState = 'admitted', then AdmittedQoSParamSet is

the QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2mBsAdmittedQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 7 }

wmanIf2mBsActiveQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsQoSProfileTable

that defines the ActiveQoSParamSet of a service flow. If

WmanIf2mSfState = 'active', then ActiveQoSParamSet is the

QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2mBsActiveQoSProfileIndex. ActiveQoSParamSet is a

subset of AdmittedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 8 }

wmanIf2mBsClassifierRuleIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in

wmanIf2mBsClassifierRuleTable that defines the

classification rules for a service flow. When '0' is

returned from reading this object, it means the

classification rules are not defined for this service

flow."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 9 }

wmanIf2mBsPhsRuleIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mBsPhsRuleTable

that defines the packet suppression rules for a service

flow. When '0' is returned from reading this object, it

means the PHS rules are not defined for this service

flow."

REFERENCE

"Subclause 11.13.19.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 10 }

wmanIf2mBsArqAttributeIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in

wmanIf2mBsArqAttributeTable that defines the ARQ

attributes for a service flow. When '0' is returned from

reading this object, it means the ARQ attributes are not

defined for this service flow."

REFERENCE

"Subclause 11.13.19.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 11 }

wmanIf2mBsServiceFlowState OBJECT-TYPE

SYNTAX      WmanIf2mSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2SsServiceFlowState determines the state of a service

flow."

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 12 }

wmanIf2mBsCid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier points to the connection being

created by DSA for this service flow. When '0' is returned

from reading this object, it means no CID has been assigned

to this service flow yet."

::= { wmanIf2mBsServiceFlowEntry 13 }

wmanIf2mBsSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2mCsSpecification

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 14 }

wmanIf2mBsSfMinTolerableTrafficRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with time

base T(sec) means the following. Let S denote additional

demand accumulated at the MAC SAP of the transmitter

during an arbitrary time interval of the length T. Then the

amount of data forwarded at the receiver to CS (in bits)

during this interval should be not less than min {S, R * T}

."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 15 }

wmanIf2mBsSfReqTxPolicy OBJECT-TYPE

SYNTAX      WmanIf2mReqTxPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service flow.

An attribute is enabled by setting the corresponding bit

position to 1."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlowEntry 16 }

wmanIf2mBsSfTargetSaid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto which the

service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsServiceFlowEntry 17 }

wmanIf2mBsSfEstablishTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

established that means wmanIf2mBsServiceFlowState is

either in 'provisioned', 'admitted', or 'active' state."

::= { wmanIf2mBsServiceFlowEntry 18 }

wmanIf2mBsSfTerminateTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

terminated that means wmanIf2mBsServiceFlowState is

in 'inactive' state."

::= { wmanIf2mBsServiceFlowEntry 19 }

wmanIf2mBsClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with service flows."

::= { wmanIf2mBsServiceFlow 2 }

wmanIf2mBsClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier rule

."

INDEX     { ifIndex, wmanIf2mBsClassifierRuleId }

::= { wmanIf2mBsClassifierRuleTable 1 }

WmanIf2mBsClassifierRuleEntry::= SEQUENCE {

wmanIf2mBsClassifierRuleId              Unsigned32,

wmanIf2mBsClassifierRulePriority        INTEGER,

wmanIf2mBsClassifierRuleIpTosLow        INTEGER,

wmanIf2mBsClassifierRuleIpTosHigh       INTEGER,

wmanIf2mBsClassifierRuleIpTosMask       INTEGER,

wmanIf2mBsClassifierRuleIpProtocol      Integer32,

wmanIf2mBsClassifierRuleIpSrcAddr       InetAddress,

wmanIf2mBsClassifierRuleIpSrcMask       InetAddress,

wmanIf2mBsClassifierRuleIpDestAddr      InetAddress,

wmanIf2mBsClassifierRuleIpDestMask      InetAddress,

wmanIf2mBsClassifierRuleSrcPortStart    Integer32,

wmanIf2mBsClassifierRuleSrcPortEnd      Integer32,

wmanIf2mBsClassifierRuleDestPortStart   Integer32,

wmanIf2mBsClassifierRuleDestPortEnd     Integer32,

wmanIf2mBsClassifierRuleDestMacAddr     MacAddress,

wmanIf2mBsClassifierRuleDestMacMask     MacAddress,

wmanIf2mBsClassifierRuleSrcMacAddr      MacAddress,

wmanIf2mBsClassifierRuleSrcMacMask      MacAddress,

wmanIf2mBsClassifierRuleEnetType        INTEGER,

wmanIf2mBsClassifierRuleEnetProtocol    Integer32,

wmanIf2mBsClassifierRuleUserPriLow      Integer32,

wmanIf2mBsClassifierRuleUserPriHigh     Integer32,

wmanIf2mBsClassifierRuleVlanId          Integer32,

wmanIf2mBsClassifierRuleIpv6FlowLabel   WmanIf2mIpv6FlowLabel,

wmanIf2mBsClassifierRulePkts            Counter64,

wmanIf2mBsClassifierRuleBitMap          WmanIf2mClassifierBitMap

}

wmanIf2mBsClassifierRuleId  OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 1 }

wmanIf2mBsClassifierRulePriority OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the priority.

The value of 0 is used as default in provisioned service

flows classifiers. The default value of 64 is used for

dynamic service flow classifiers. If the referenced

parameter is not present in a classifier, this object

reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2mBsClassifierRuleEntry 2 }

wmanIf2mBsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 3 }

wmanIf2mBsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 4 }

wmanIf2mBsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 5 }

wmanIf2mBsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 6 }

wmanIf2mBsClassifierRuleIpSrcAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2mBsClassifierRuleIpSrcMask value

equals the wmanIf2mBsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 7 }

wmanIf2mBsClassifierRuleIpSrcMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2mBsClassifierRuleIpSrcMask value equals the

wmanIf2mBsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 8 }

wmanIf2mBsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2mBsClassifierRuleIpDestMask value equals the

wmanIf2mBsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 9 }

wmanIf2mBsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2mBsClassifierRuleIpDestMask value equals the

wmanIf2mBsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 10 }

wmanIf2mBsClassifierRuleSrcPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 11 }

wmanIf2mBsClassifierRuleSrcPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 12 }

wmanIf2mBsClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 13 }

wmanIf2mBsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 14 }

wmanIf2mBsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mBsClassifierRuleDestMacMask equals the value of

wmanIf2mBsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 15 }

wmanIf2mBsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mBsClassifierRuleDestMacMask equals the value of

wmanIf2mBsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 16 }

wmanIf2mBsClassifierRuleSrcMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source MAC

address  bitwise ANDed with

wmanIf2mBsClassifierRuleSrcMacMask equals the value of

wmanIf2mBsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 17 }

wmanIf2mBsClassifierRuleSrcMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mBsClassifierRuleSrcMacMask equals the value of

wmanIf2mBsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 18 }

wmanIf2mBsClassifierRuleEnetType OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 19 }

wmanIf2mBsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If wmanIf2mBsClassifierRuleEnetType is none(0), this

object is ignored when considering whether a packet matches

the current rule. If wmanIf2mBsClassifierRuleEnetType is

ethertype(1), this object gives the 16-bit value of the

EtherType that the packet must match in order to match the

rule. If wmanIf2mBsClassifierRuleEnetType is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 20 }

wmanIf2mBsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2mBsClassifierRulePriLow and

wmanIf2mBsClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 21 }

wmanIf2mBsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2mBsClassifierRulePriLow

and wmanIf2mBsClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 22 }

wmanIf2mBsClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2mBsClassifierRuleEntry 23 }

wmanIf2mBsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2mIpv6FlowLabel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2mBsClassifierRuleEntry 24 }

wmanIf2mBsClassifierRulePkts OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2mBsClassifierRuleEntry 25 }

wmanIf2mBsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2mClassifierBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2mBsClassifierRuleEntry 26 }

wmanIf2mBsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlow 3 }

wmanIf2mBsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex, wmanIf2mBsPhsRuleId }

::= { wmanIf2mBsPhsRuleTable 1 }

WmanIf2mBsPhsRuleEntry::= SEQUENCE {

wmanIf2mBsPhsRuleId                     INTEGER,

wmanIf2mBsPhsRulePhsField               OCTET STRING,

wmanIf2mBsPhsRulePhsMask                OCTET STRING,

wmanIf2mBsPhsRulePhsSize                Integer32,

wmanIf2mBsPhsRulePhsVerify              WmanIf2mPhsRuleVerify}

wmanIf2mBsPhsRuleId OBJECT-TYPE

SYNTAX      INTEGER (1 .. 255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2mBsPhsRuleEntry 1 }

wmanIf2mBsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2mBsPhsRuleEntry 2 }

wmanIf2mBsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2mBsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2mBsPhsRuleEntry 3 }

wmanIf2mBsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2mBsPhsRuleEntry 4 }

wmanIf2mBsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2mPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2mBsPhsRuleEntry 5 }

wmanIf2mBsQoSProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains QoS profiles that are associated with

service flows or CIDs via the wmanIf2mBsQoSProfileIndex.

The following table shows the required parameters for

different UL grant scheduling type.

0   - not required

1   - required

0-1 - optional

QoS Parameters                BE  ertPS  UGS  rtPS  nrtPS

---------------------------------------------------------

Traffic priority              0-1  0-1    0    0-1   0-1

Max sustained traffic rate    0-1  0-1    1    0-1   0-1

Min reserved traffic rate      0    1    0-1    1     1

Minimum traffic burst          0   0-1    0    0-1   0-1

Tolerated jitter               0   0-1    1     0     0

Maximum latency                0    1     1     1     0

Unsolicited Grant Interval     0    1     1     0     0

SDU size                       0    0    0-1    0     0

Unsolicited Polling Interval   0    0     0     1     0"

REFERENCE

"Subclause 6.3.14.4 in IEEE Std 802.16-2004"

::= { wmanIf2mBsServiceFlow 4 }

wmanIf2mBsQoSProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each QoS parameter Set."

INDEX     { ifIndex, wmanIf2mBsQoSProfileIndex }

::= { wmanIf2mBsQoSProfileTable 1 }

WmanIf2mBsQoSProfileEntry ::= SEQUENCE {

wmanIf2mBsQoSProfileIndex               INTEGER,

wmanIf2mBsQosServiceClassName           OCTET STRING,

wmanIf2mBsQosUlGrantScheduleType        WmanIf2mSchedulingType,

wmanIf2mBsQosTrafficPriority            INTEGER,

wmanIf2mBsQosMaximumSustainedRate       Unsigned32,

wmanIf2mBsQosMinimumReservedRate        Unsigned32,

wmanIf2mBsQosMaximumTrafficBurst        Unsigned32,

wmanIf2mBsQosToleratedJitter            Unsigned32,

wmanIf2mBsQosMaxLatency                 Unsigned32,

wmanIf2mBsQosUnsolicitedGrantInterval   Unsigned32,

wmanIf2mBsQosSduSize                    Unsigned32,

wmanIf2mBsQosUnsolicitedPollInterval    Unsigned32}

wmanIf2mBsQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

wmanIf2mBsQoSProfileTable"

::= { wmanIf2mBsQoSProfileEntry 1 }

wmanIf2mBsQosServiceClassName  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(2..128))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the Null-terminated string of ASCII

characters. It refers to a predefined BS service

configuration to be used for a service flow."

REFERENCE

"Subclause 11.13.3 in IEEE Std 802.16-2004"

::= { wmanIf2mBsQoSProfileEntry 2 }

wmanIf2mBsQosUlGrantScheduleType OBJECT-TYPE

SYNTAX      WmanIf2mSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the Uplink grant scheduling type

that shall be enabled for the associated uplink service

flow upstream service flow. If the parameter is not

present in the corresponding 802.16 QOS Parameter Set of

an upstream service flow, the default value is assumed."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16e-2004"

DEFVAL	    { bestEffort }

::= { wmanIf2mBsQoSProfileEntry 3 }

wmanIf2mBsQosTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority assigned

to a service flow. For uplink service flows, the BS should

use this parameter when determining precedence in request

service and grant generation, Higher numbers indicate

higher priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsQoSProfileEntry 4 }

wmanIf2mBsQosMaximumSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate of the

service. The rate is expressed in bits per second and

pertains to the SDUs at the input to the Convergence

Sublayer."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsQoSProfileEntry 5 }

wmanIf2mBsQosMinimumReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved for this

service flow. It specifies the minimum amount of data to be

transported on behalf of the service flow when averaged

over time."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16e-2004"

::= { wmanIf2mBsQoSProfileEntry 6 }

wmanIf2mBsQosMaximumTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that must be

accommodated for the service. It defines the maximum

continuous burst the system should accommodate for the

service assuming the service is not currently using any of

its available resources."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2mBsQoSProfileEntry 7 }

wmanIf2mBsQosToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay variation (jitter)

for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2mBsQoSProfileEntry 8 }

wmanIf2mBsQosMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the maximum latency between the

ingress of a packet to the Convergence Sublayer and the

forwarding of the SDU to its Air Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2mBsQoSProfileEntry 9 }

wmanIf2mBsQosUnsolicitedGrantInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the nominal interval between

successive data grant opportunities for a service flow."

REFERENCE

"Subclause 11.13.20 in IEEE Std 802.16e-2004"

::= { wmanIf2mBsQoSProfileEntry 10 }

wmanIf2mBsQosSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the length of the SDU for a

fixed-length SDU service flow. It is used only if packing

is on and the service flow is indicated as carrying

fixed-length SDUs. If this object is omitted in the QoS

parameter set, it should return 0 that means the

variable-length service flow."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

::= { wmanIf2mBsQoSProfileEntry 11 }

wmanIf2mBsQosUnsolicitedPollInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximal nominal interval between

successive polling grants opportunities for this Service

Flow."

REFERENCE

"Subclause 11.13.21 in IEEE Std 802.16e-2004"

::= { wmanIf2mBsQoSProfileEntry 12 }

wmanIf2mBsArqAttributeTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains ARQ parameters that are associated

with the Service Flows."

::= { wmanIf2mBsServiceFlow 5 }

wmanIf2mBsArqAttributeEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service flow

for a given MacAddress."

INDEX     { ifIndex, wmanIf2mBsArqIndex  }

::= { wmanIf2mBsArqAttributeTable 1 }

WmanIf2mBsArqAttributeEntry::= SEQUENCE {

wmanIf2mBsArqIndex                      INTEGER,

wmanIf2mBsArqEnable                     TruthValue,

wmanIf2mBsArqWindowSize                 INTEGER,

wmanIf2mBsArqBlockLifetime              INTEGER,

wmanIf2mBsArqSyncLossTimeout            INTEGER,

wmanIf2mBsArqDeliverInOrder             TruthValue,

wmanIf2mBsArqRxPurgeTimeout             INTEGER,

wmanIf2mBsArqBlockSize                  INTEGER,

wmanIf2mBsArqAckProcessingTime          INTEGER}

wmanIf2mBsArqIndex OBJECT-TYPE

SYNTAX      INTEGER ( 1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

in the wmanIf2mBsArqAttributeTable."

::= { wmanIf2mBsArqAttributeEntry 1 }

wmanIf2mBsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

::= { wmanIf2mBsArqAttributeEntry 2 }

wmanIf2mBsArqWindowSize	 OBJECT-TYPE

SYNTAX	    INTEGER (1..1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged fragments

at any time."

::= { wmanIf2mBsArqAttributeEntry 3 }

wmanIf2mBsArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be managed

by the transmitter ARQ machine, once initial transmission

of the fragment has occured. If transmission or

retransmission of the fragment is not acknowledged by the

receiver before the time limit is reached, the fragment is

discarded. A value of 0 means Infinite."

::= { wmanIf2mBsArqAttributeEntry 4 }

wmanIf2mBsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss of

synchronization of the sender and receiver state machines.

A value of 0 means Infinite."

::= { wmanIf2mBsArqAttributeEntry 5 }

wmanIf2mBsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered by the

receiving MAC to its client application in the order in

which data was handed off to the originating MAC."

::= { wmanIf2mBsArqAttributeEntry 6 }

wmanIf2mBsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

::= { wmanIf2mBsArqAttributeEntry 7 }

wmanIf2mBsArqBlockSize OBJECT-TYPE

SYNTAX	    INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an ARQ

block. This parameter shall be established by negotiation

during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2mBsArqAttributeEntry 8 }

wmanIf2mBsArqAckProcessingTime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 255)

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the number of ms required by the

ARQ receiver to process the received ARQ blocks and provide

a valid ACK or NAK."

REFERENCE

"Subclause 11.13.18.9 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsArqAttributeEntry 9 }

--

-- Mobile Station Sleep Mode Statistics Table

--

wmanIf2mBsSsSleepModeStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF   WmanIf2mBsSsSleepModeStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the sleep mode statistic for MS. This

table shall be maintained as FIFO to store the sleep mode

statistics over a period of time that is subject to

implementation. This statistics information can be to

monitor, fine tuning, or debugging the power saving

performance of each MS. When the statistics entry for an

MS reaches the limit, it wraps around to the beginning, and

overwrites the oldest entry with the new entry. When the BS

roams to a different BS, all entries associated with such

MS will be deleted."

REFERENCE

"6.3.21 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 1 }

wmanIf2mBsSsSleepModeStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsSleepModeStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry in the table contains the event of an MS

entering the sleep mode. wmanIf2mBsSsStatisticsIndex is the

index to sleep mode event entry in the table, and should be

increased monotonically, and wraps around when it reaches

the implementation specific limit. A time stamp is provided

in each entry to indicate when the sleep mode event took

place."

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsCid,

wmanIf2mBsSsStatisticsIndex }

::= { wmanIf2mBsSsSleepModeStatisticsTable  1 }

WmanIf2mBsSsSleepModeStatisticsEntry ::= SEQUENCE {

wmanIf2mBsSsStatisticsIndex             Unsigned32,

wmanIf2mBsSsSleepWindowStarted          Unsigned32,

wmanIf2mBsSsListeningWindowStarted      Unsigned32,

wmanIf2mBsSsPendingMsdu                 INTEGER,

wmanIf2mBsSsSleepWindowTimeStamp        DateAndTime}

wmanIf2mBsSsStatisticsIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"wmanIf2mBsSsStatisticsIndex identifies the entry in the

table where the latest sleep mode event took place."

::= { wmanIf2mBsSsSleepModeStatisticsEntry 1 }

wmanIf2mBsSsSleepWindowStarted OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 166777215)

UNITS       "frame"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2mBsSsSleepWindowStarted identifies when the sleep

mode is activated.

wmanIf2mBsSsSleepWindowStarted = current frame number +

Start_frame_number.

The frame number is provided in the DL-MAP, and is

incremented by 1 MOD 2^24 each frame."

::= { wmanIf2mBsSsSleepModeStatisticsEntry 2 }

wmanIf2mBsSsListeningWindowStarted OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 166777215)

UNITS       "frame"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2mBsSsListeningWindowStarted identifies when the sleep

mode is deactivated.

wmanIf2mBsSsListeningWindowStarted =

wmanIf2mBsSsListeningWindowStarted + sleep window

The frame number is provided in the DL-MAP, and is

incremented by 1 MOD 2^24 each frame."

::= { wmanIf2mBsSsSleepModeStatisticsEntry 3 }

wmanIf2mBsSsPendingMsdu OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicate the number of MAC SDU that are received from the

network during the sleep window."

::= { wmanIf2mBsSsSleepModeStatisticsEntry 4 }

wmanIf2mBsSsSleepWindowTimeStamp OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This is the time when sleep window is started in seconds.

The definition of time is as in IETF RFC 868."

::= { wmanIf2mBsSsSleepModeStatisticsEntry 5 }

-- XXX

-- wmanIf2mBsMobileScanRequestTable

--

wmanIf2mBsMobileScanRequestTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMobileScanRequestEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that are sent in the

MOB_SCN-REQ message."

REFERENCE

"Subclause 6.3.2.3.48, Table 109h in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 2 }

wmanIf2mBsMobileScanRequestEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMobileScanRequestEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsMobileScanRequestTable 1 }

WmanIf2mBsMobileScanRequestEntry ::= SEQUENCE {

wmanIf2mBsScanReqDuration               INTEGER,

wmanIf2mBsScanReqInterleavingInterval   INTEGER,

wmanIf2mBsScanReqIteration              INTEGER,

wmanIf2mBsNumOfRecommendedBs            INTEGER,

wmanIf2mBsScanConfigChangeCount         INTEGER}

wmanIf2mBsScanReqDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Duration of the requested scanning period."

::= { wmanIf2mBsMobileScanRequestEntry 1 }

wmanIf2mBsScanReqInterleavingInterval OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period of MS's Normal Operation which is interleaved

between Scanning Durations."

::= { wmanIf2mBsMobileScanRequestEntry 2 }

wmanIf2mBsScanReqIteration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The requested number of iterating scanning interval by an

MS."

::= { wmanIf2mBsMobileScanRequestEntry 3 }

wmanIf2mBsNumOfRecommendedBs OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of neighboring BS to be scanned or associated, which

are included in MOB_NBR-ADV message."

::= { wmanIf2mBsMobileScanRequestEntry 4 }

wmanIf2mBsScanConfigChangeCount OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of Configuration Change Count in MOB_NBR-ADV

message referred in order to compress neighbor BSID."

::= { wmanIf2mBsMobileScanRequestEntry 5 }

-- XXX

-- wmanIf2mBsMobileScanResponseTable

--

wmanIf2mBsMobileScanResponseTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMobileScanResponseEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that are sent in the

MOB_SCN-RSP message."

REFERENCE

"Subclause 6.3.2.3.49, Table 109i in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 3 }

wmanIf2mBsMobileScanResponseEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMobileScanResponseEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsMobileScanResponseTable 1 }

WmanIf2mBsMobileScanResponseEntry ::= SEQUENCE {

wmanIf2mBsScanRspDuration               INTEGER,

wmanIf2mBsScanRspInterleavingInterval   INTEGER,

wmanIf2mBsScanRspIteration              INTEGER,

wmanIf2mBsReportMode                    WmanIf2mReportMode,

wmanIf2mBsReportPeriod                  INTEGER,

wmanIf2mBsReportMatric                  WmanIf2mReportMatric,

wmanIf2mBsStartFrame                    INTEGER}

wmanIf2mBsScanRspDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Duration where the MS may perform scanning or association

for Available BSs. When MOB_SCN-RSP is sent in response to

MOB_SCN-REQ, setting Scan duration to zero indicates the

request for an allocation of scanning interval is denied."

::= { wmanIf2mBsMobileScanResponseEntry 1 }

wmanIf2mBsScanRspInterleavingInterval OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period interleaved between Scanning Intervals when MS

shall perform Normal Operation."

::= { wmanIf2mBsMobileScanResponseEntry 2 }

wmanIf2mBsScanRspIteration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of iterating scanning interval."

::= { wmanIf2mBsMobileScanResponseEntry 3 }

wmanIf2mBsReportMode OBJECT-TYPE

SYNTAX      WmanIf2mReportMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Action code for an MS's report of CINR measurement."

::= { wmanIf2mBsMobileScanResponseEntry 4 }

wmanIf2mBsReportPeriod OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period of MSis report of CINR measurement when the MS

is required to report the value periodically."

::= { wmanIf2mBsMobileScanResponseEntry 5 }

wmanIf2mBsReportMatric OBJECT-TYPE

SYNTAX      WmanIf2mReportMatric

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Bitmap indicator of trigger metrics that the serving BS

requests the MS to report."

::= { wmanIf2mBsMobileScanResponseEntry 6 }

wmanIf2mBsStartFrame OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Measured from the frame in which this message was received.

A value of zero means that first Scanning Interval starts

in the next frame."

::= { wmanIf2mBsMobileScanResponseEntry 7 }

-- XXX

-- wmanIf2mBsNeighborBsInfoTable

--

wmanIf2mBsNeighborBsInfoTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the neighbor BS information that is

sent in the MOB_SCN-RSP and MOB_SCN-REP messages."

REFERENCE

"Subclause 6.3.2.3.49 and 6.3.2.3.50, Table 109i and 109j

in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 4 }

wmanIf2mBsNeighborBsInfoEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsNeighbirBsIndex }

::= { wmanIf2mBsNeighborBsInfoTable 1 }

WmanIf2mBsNeighborBsInfoEntry ::= SEQUENCE {

wmanIf2mBsNeighbirBsIndex               INTEGER,

wmanIf2mBsFullBsId                      WmanIf2mFullBsId,

wmanIf2mBsScanningType                  WmanIf2mScanType,

wmanIf2mBsRendezvousTime                INTEGER,

wmanIf2mBsScanCdmaCode                  INTEGER,

wmanIf2mBsTxOpportunityOffset           INTEGER,

wmanIf2mBsCinrMean                      INTEGER,

wmanIf2mBsRssiMean                      INTEGER,

wmanIf2mBsRelativeDelay                 INTEGER}

wmanIf2mBsNeighbirBsIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Index to the neighbor BS."

::= { wmanIf2mBsNeighborBsInfoEntry 1 }

wmanIf2mBsFullBsId OBJECT-TYPE

SYNTAX      WmanIf2mFullBsId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object contains the BS ID if full 48 bits BS ID is

used to scan the neighbor BS. This object returns NULL if

the Neighbor_BS_index as defined in MOB_SCN-RSP is used

instead."

::= { wmanIf2mBsNeighborBsInfoEntry 2 }

wmanIf2mBsScanningType OBJECT-TYPE

SYNTAX      WmanIf2mScanType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Type of scanning or association used by the MS and

coordinated by the Serving BS."

::= { wmanIf2mBsNeighborBsInfoEntry 3 }

wmanIf2mBsRendezvousTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This is offset, measured in units of frame duration (of

Serving BS), when the corresponding Recommended BS is

expected to provide non-contention-based ranging

opportunity for the MS."

::= { wmanIf2mBsNeighborBsInfoEntry 4 }

wmanIf2mBsScanCdmaCode OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A unique code assigned to the MS, to be used for

association with the neighbor BS. Code is from the initial

ranging codeset."

::= { wmanIf2mBsNeighborBsInfoEntry 5 }

wmanIf2mBsTxOpportunityOffset OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A unique transmission opportunity assigned to the MS, to be

used for association with the Target BS in units of symbol

duration."

::= { wmanIf2mBsNeighborBsInfoEntry 6 }

wmanIf2mBsCinrMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.5 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS CINR mean parameter indicates the CINR measured by

the MS from the particular BS. The value shall be

interpreted as a signed byte with units of 0.5 dB. The

measurement shall be performed on the subcarriers of the

frame preamble that are active in the particular BS's

segment and averaged over the measurement period."

::= { wmanIf2mBsNeighborBsInfoEntry 7 }

wmanIf2mBsRssiMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.25 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RSSI mean parameter indicates the Received Signal

Strength measured by the MS from the particular BS. The

value shall be interpreted as an unsigned byte with units

of 0.25 dB."

::= { wmanIf2mBsNeighborBsInfoEntry 8 }

wmanIf2mBsRelativeDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "samples"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the delay of neighbor DL signals

relative to the serving BS, as measured by the MS for the

particular BS. The value shall be interpreted as a signed

integer in units of samples."

::= { wmanIf2mBsNeighborBsInfoEntry 9 }

-- XXX

-- wmanIf2mBsDiversityBsInfoTable

--

wmanIf2mBsDiversityBsInfoTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsDiversityBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the diversity BS information that is

sent in the MOB_SCN-REP messages."

REFERENCE

"Subclause 6.3.2.3.49 and 6.3.2.3.50, Table 109i and 109j

in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 5 }

wmanIf2mBsDiversityBsInfoEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsDiversityBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsTempBsIndex }

::= { wmanIf2mBsDiversityBsInfoTable 1 }

WmanIf2mBsDiversityBsInfoEntry ::= SEQUENCE {

wmanIf2mBsTempBsIndex                   INTEGER,

wmanIf2mBsFbssMdhoCinrMean              INTEGER,

wmanIf2mBsFbssMdhoRssiMean              INTEGER,

wmanIf2mBsFbssMdhoRelativeDelay         INTEGER,

wmanIf2mBsFbssMdhoRtd                   INTEGER}

wmanIf2mBsTempBsIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Diversity set member ID assigned to this BS. When the MS

has an empty diversity set or FBSS/MDHO is not supported,

Temp BSID shall be set to 0.."

::= { wmanIf2mBsDiversityBsInfoEntry 1 }

wmanIf2mBsFbssMdhoCinrMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.5 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS CINR mean parameter indicates the CINR measured by

the MS from the particular BS. The value shall be

interpreted as a signed byte with units of 0.5 dB. The

measurement shall be performed on the subcarriers of the

frame preamble that are active in the particular BS's

segment and averaged over the measurement period."

::= { wmanIf2mBsDiversityBsInfoEntry 2 }

wmanIf2mBsFbssMdhoRssiMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.25 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RSSI mean parameter indicates the Received Signal

Strength measured by the MS from the particular BS. The

value shall be interpreted as an unsigned byte with units

of 0.25 dB."

::= { wmanIf2mBsDiversityBsInfoEntry 3 }

wmanIf2mBsFbssMdhoRelativeDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "samples"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the delay of neighbor DL signals

relative to the serving BS, as measured by the MS for the

particular BS. The value shall be interpreted as a signed

integer in units of samples."

::= { wmanIf2mBsDiversityBsInfoEntry 4 }

wmanIf2mBsFbssMdhoRtd OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "1/Fs"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RTD parameter indicates the round trip delay (RTD)

measured by the MS from the serving BS. RTD can be given by

the latest time advance taken by MS. The value shall be

interpreted as an unsigned byte with units of 1/Fs (see

10.3.4.3). This parameter shall be only measured on serving

BS/anchor BS."

::= { wmanIf2mBsDiversityBsInfoEntry 5 }

END