3GPP TSG-RAN Meeting #84bis 	R1-163945

- Busan, Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 627 | rev | 1 | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for LAA CSI processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA |  |  |  |  |  |  | Date: |  | 2016-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current procedure requires the UE to use subframes in CSI measurements when PDCCH with DCI CRC scrambled by CC-RNTI is received containing only reserved values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a condition on the subframe configuration being set to a valid value for the subframe to be used for CSI measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Future use of reserved values would result in unspecified UE behaviour regarding CSI processing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact |  |  |  |  |  |  |  |  |

# 13A	Subframe configuration for Frame Structure Type 3

If a UE detects PDCCH with DCI CRC scrambled by CC-RNTI in subframe n-1 or subframe n of a LAA Scell, the UE may assume the configuration of occupied OFDM symbols in subframe n of the LAA Scell according to the ‘Subframe configuration for LAA’ field in the detected DCI in subframe n-1 or subframe n.

The ‘Subframe configuration for LAA’ field indicates the configuration of occupied OFDM symbols (i.e., OFDM symbols used for transmission of downlink physical channels and/or physical signals) in current and/or next subframe according to Table 13A-1.

If the configuration of occupied OFDM symbols for subframe n is indicated by the Subframe configuration for LAA field in both subframe n-1 and subframe n, the UE may assume that the same configuration of occupied OFDM symbols is indicated in both subframe n-1 and subframe n.

If a UE detects PDCCH with DCI CRC scrambled by CC-RNTI in subframe n, and the UE does not detect PDCCH with DCI CRC scrambled by CC-RNTI in subframe n-1, and if the number of occupied OFDM symbols for subframe n indicated by the Subframe configuration for LAA field in subframe n is less than 14, the UE is not required to receive any other physical channels in subframe n.

If a UE does not detect PDCCH with DCI CRC scrambled by CC-RNTI containing ‘Subframe Configuration for LAA’ field set to other than ‘1110’ and ‘1111’ in subframe n and the UE does not detect PDCCH with DCI CRC scrambled by CC-RNTI containing ‘Subframe Configuration for LAA’ field set to other than ‘1110’ and ‘1111’ in subframe n-1, the UE is not required to use subframe n for updating CSI measurement.

The UE may detect PDCCH with DCI CRC scrambled by CC-RNTI by monitoring the following PDCCH candidates according to DCI Format 1C.

-	one PDCCH candidate at aggregation level L=4 with the CCEs corresponding to the PDCCH candidate given by CCEs numbered 0,1,2,3

-	one PDCCH candidate at aggregation level L=8 with the CCEs corresponding to the PDCCH candidate given by CCEs numbered 0,1,2,3,4,5,6,7

If a serving cell is a LAA Scell, and if the higher layer parameter subframeStartPosition for the Scell indicates ‘s07’, and if the UE detects PDCCH/EPDCCH intended for the UE starting in the second slot of a subframe, the UE may assume that OFDM symbols in the first slot of the subframe are not occupied, and all OFDM symbols in the second slot of the subframe are occupied,

If subframe n is a subframe in which OFDM symbols in the first slot are not occupied, the UE may assume that all the OFDM symbols are occupied in subframe n+1.

Table 13A-1: Subframe configuration for LAA in current and next subframe

| Value of ‘Subframe configuration for LAA’ field in current subframe | Configuration of occupied OFDM symbols (current subframe, next subframe) |
| --- | --- |
| 0000 | (-,14) |
| 0001 | (-,12) |
| 0010 | (-,11) |
| 0011 | (-,10) |
| 0100 | (-,9) |
| 0101 | (-,6) |
| 0110 | (-,3) |
| 0111 | (14,*) |
| 1000 | (12,-) |
| 1001 | (11,-) |
| 1010 | (10,-) |
| 1011 | (9,-) |
| 1100 | (6,-) |
| 1101 | (3,-) |
| 1110 | reserved |
| 1111 | reserved |
| NOTE: - (-, Y) means UE may assume the first Y symbols are occupied in next subframe and other symbols in the next subframe are not occupied. - (X,-) means UE may assume the first X symbols are occupied in current subframe and other symbols in the current subframe are not occupied. - (X,*) means UE may assume the first X symbols are occupied in current subframe, and at least the first OFDM symbol of the next subframe is not occupied. |  |