3GPP TSG-RAN Meeting #85 	R1-166039

- Nanjing, China, 23rd - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 687 | rev |  | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CWp adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The LS received in R1-163375 recommends calrificaton to the CWp adjustment procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when the contention window size is increased, it is done so for every access priority class |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent implementation due to no applying the window size increase to every access priority class |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact |  |  |  |  |  |  |  |  |

### 15.1.3	Contention window adjustment procedure

If the eNB transmits transmissions that are associated with channel access priority class pon a channel, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in sub clause 15.1.1 for those transmissions using the following steps:

1)	for every priority class set CWp=CWmin,p

2)	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe kare determined as NACK, increase CWp for every priority class to the next higher allowed value and remain in step 2; otherwise, go to step 1.

Reference subframe k is the starting subframe of the most recent transmission on the channel made by the eNB, for which HARQ-ACK feedback is available.

If CWp=CWmax,p, the next higher allowed value for adjusting CWpis CWmax,p.

For determining Z,

-	if the eNB transmission(s) for which HARQ-ACK feedback is available start in the second slot of subframe k, HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k+1are also used in addition to the HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k.

-	‘DTX’ state and ‘any’ state are counted as NACK, if the HARQ-ACK values corresponding to the PDSCH transmission(s) are assigned by (E)PDCCH transmitted on the channel

-	if a PDSCH transmission has two codewords, the HARQ-ACK value of each codeword is considered separately

-	bundled HARQ-ACK across M subframes is considered as M HARQ-ACK responses.

If the CWp=CWmax,p is consecutively used K times for generation of Ninit,CWp is reset to CWmin,p only for that priority class pfor which CWp=CWmax,p is consecutively used K times for generation of Ninit.   K is selected by eNB from the set of values {1, 2, …,8} for each priority class .