3GPP TSG-RAN WG1 Meeting #84bis 	R1-163531

- Busan, Korea 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 631 | rev |  | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on "special subframe" for frame structure type 3 in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, CATT, Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current specification only differentiates normal subframe and special subframe. But the ending partial subframe is the subframe following DwPTS duration. The clarification on "special subframe" for frame structure type 3 for EPDCCH assignment procedure (specified in clause 9.1.4) has been captured in the current specification with one place missing. In addition, such clarification was not modified for TBS determination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on "special subframe" for frame structure type 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE procedure for TBS determination and EPDCCH assignment procedure in the ending partial subframe is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7, 9.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact. |  |  |  |  |  |  |  |  |

### 7.1.7	Modulation order and transport block size determination

To determine the modulation order and transport block size(s) in the physical downlink shared channel, the UE shall first

-	if the UE is a BL/CE UE

-	if PDSCH is assigned by MPDCCH DCI format 6-1A or 6-2

-	read the 4-bit "modulation and coding scheme (IMCS1)" field in the DCI

-	The UE is not expected to receive a DCI format 6-1A indicating  or a DCI format 6-2 indicating IMCS1>7

-	else if PDSCH is assigned by MPDCCH DCI format 6-1B

-	read the 4-bit "modulation and coding scheme (IMCS1)" field in the DCI and set ITBS1=IMCS1.

-	else if the UE is a BL/CE UE and if PDSCH carriers SystemInformationBlockType1-BR

-	set ITBS to the value of the parameter schedulingInfoSIB1-BR-r13 configured by higher-layers

-	otherwise

-	read the 5-bit "modulation and coding scheme" field (IMCS) in the DCI

and second if the PDCCH DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI then

-	for DCI format 1A:

-	set the Table 7.1.7.2.1-1 column indicatorNPRB to from subclause 5.3.3.1.3 in [4]

-	for DCI format 1C:

-	use Table 7.1.7.2.3-1 for determining its transport block size.

else

-	if the UE is a BL/CE UE

-	if PDSCH is assigned by MPDCCH DCI format 6-2

-	use Table 7.1.7.2.3-1 for determining its transport block size.

-	else if PDSCH carriers SystemInformationBlockType1-BR

-	use subclause 7.1.7.2.7 for determining its transport block size.

-	otherwise,

-	set NPRB' to the total number of allocated PRBs based on the procedure defined in subclause 7.1.6.

-	set the Table 7.1.7.2.1-1 column indicator NPRB=NPRB'.

-	if PDSCH is assigned by MPDCCH DCI format 6-1B

-	use subclause 7.1.7.2.6 for determining its transport block size.

-	otherwise

-	set NPRB' to the total number of allocated PRBs based on the procedure defined in subclause 7.1.6.

if the transport block is transmitted in DwPTS of the special subframe in frame structure type 2, or is transmitted in the subframes with the same duration as the DwPTS duration of a special subframe configuration in frame structure type 3, then

-	for special subframe configuration 9 with normal cyclic prefix or special subframe configuration 7 with extended cyclic prefix:

-	set the Table 7.1.7.2.1-1 column indicator

-	for other special subframe configurations:

-	set the Table 7.1.7.2.1-1 column indicator ,

else, set the Table 7.1.7.2.1-1 column indicator NPRB=NPRB'.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.931, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded. For the special subframe configurations 0 and 5 with normal downlink CP or configurations 0 and 4 with extended downlink CPin frame structure type 2, or for subframes with the same duration as the DwPTS duration of the special subframe configuration 0 and 5 in frame structure type 3, with the special subframe configurations shown in Table 4.2-1 of [3], there shall be no PDSCH transmission in DwPTS of the special subframe.

#### < Unchanged parts are omitted >

### 9.1.4	EPDCCH assignment procedure

For each serving cell, higher layer signalling can configure a UE with one or two EPDCCH-PRB-sets for EPDCCH monitoring. The PRB-pairs corresponding to an EPDCCH-PRB-set are indicated by higher layers as described in subclause 9.1.4.4. Each EPDCCH-PRB-set consists of set of ECCEs numbered from 0 to where is the number of ECCEs in EPDCCH-PRB-set  of subframe . Each EPDCCH-PRB-set can be configured for either localized EPDCCH transmission or distributed EPDCCH transmission.

#### < Unchanged parts are omitted >

The UE shall not monitor EPDCCH

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0 and 5, or for frame structure type 3, in the subframe with the same duration as the DwPTS duration of the special subframe configurations 0 and 5, shown in Table 4.2-1 of [3].

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4 and 7 shown in Table 4.2-1 of [3].

-	In subframes indicated by higher layers to decode PMCH.

-	For TDD and if the UE is configured with different UL/DL configurations for the primary and a secondary cell, in a downlink subframe on the secondary cell when the same subframe on the primary cell is a special subframe and the UE is not capable of simultaneous reception and transmission on the primary and secondary cells.

An EPDCCH UE-specific search space at aggregation level  is defined by a set of EPDCCH candidates.

#### < Unchanged parts are omitted >