3GPP TSG-RAN WG4 Meeting #79	R4-164802

- Nanjing, China, 23-27 May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 0867 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LBT performance test for LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-05-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | This CR introduce LBT performance test for Licensed assis access (LAA) Rel-13 operation in unlicensed spectrum of 5 GHz. Introduced core requirements for channel access procedure in TS 36.104 require introduction of new test in TS 36.141. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of downlik channel access test procedure in TS 36.141 which verify new funcionalities: - Energy Detection (ED) threshold and detection accuracy - Testing Max Channel Occupancy Time (MCOT) - Minimum Idle time |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test for channel access procedure would be missing in TS 36.141. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, newly introduced: Clause 9, Clause I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 36.101 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 36.141 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of proposal >

#### 6.1.1	E-UTRA Test Models

The set-up of physical channels for transmitter tests shall be according to one of the E-UTRA test models (E-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all E-UTRA test models:

- The test models are defined for a single antenna port (using p = 0); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise

- Duration is 10 subframes (10 ms)

- Normal CP

- Virtual resource blocks of localized type, no intra-subframe hopping for PDSCH

- UE-specific reference signals are not used

Power settings of physical channels are defined by physical channel EPRE relative to the EPRE of the RS. The relative accuracy of the physical channel EPRE as referred to the EPRE of the RS shall have a tolerance of ±0.5 dB.

For E-UTRA TDD, test models are derived based on the uplink/downlink configuration 3 and special subframe configuration 8 defined in TS36.211, i.e. as showing in the table 6.1.1-1 (excluding Channel access procedure test for downlink operation in Band 46 where Frame structure Type 3 is defined in TS 36.211 clause 4.3 is used). Number of frames for the test models is 2.

Table 6.1.1-1: Configurations of TDD eNB test models

| Downlink-to-Uplink Switch-point periodicity | Number of UL/DL sub-frames per radio frame (10 ms) |  | DwPTS | GP | UpPTS |
| --- | --- | --- | --- | --- | --- |
|  | DL | UL |  |  |  |
| 10ms | 6 | 3 | 24144⋅Ts | 2192⋅Ts | 4384⋅Ts |

- <Next Section>

- 9	Channel access procedures

- 9.1	Downlink channel access procedure

- 9.1.1	Definition and applicability

Channel access procedure for downlink operation in Band 46 for PDSCH transmission is described in TS 36.213, Clause 15.

- 9.1.2	Minimum requirement

The minimum requirement is in TS 36.104 [2] subclause 9.1.

- 9.1.3 	Test purpose

The test purpose is to verify the accuracy of the energy detection threshold, maximum channel occupancy time (MCOT) and minimum idle time in band 46 and under normal conditions for all band 46 transmitters in the BS.

- 9.1.4	Method of test

- 9.1.4.1	Initial conditions

Test environment: 			normal; see Annex D.2.

RF channels to be tested for single carrier: 	B, M and T; see subclause 4.7.

Connect the signal analyzer to the base station antenna connector as shown in Annex I..

- 9.1.4.2	Procedure

MCOT and minimum idle time

- Set the base station to transmit a signal according to E-TM 1.1 at manufacturer’s declared rated output power with 20 MHz channel bandwidth. Channel Access Priority Class 3 parameters are selected to be tested based on Table 15.1.1-1 in TS 36.213.

- Measure the transmitter ON period during the continuous transmission (after initial CCA).

- Measure the transmitter OFF period between two consecutive transmitter ON periods.

- Verify minimum idle time as follows:

The transmitter OFF period between two consecutive transmitter ON periods shall not be less than [25 µs].

- Verify maximum channel occupancy time (MCOT) as follows:

- The duration of each transmitter ON period continuous transmission shall not exceed the maximum channel occupancy time (MCOT) requirement specified in clause 9.1.5.

Energy detection accuracy

- Generate the interfering signal of AWGN with 20 MHz channel bandwidth at the same centre frequency as the tested channel. The interfering signal shall be at a level of (-72dBm+ [4dB])/20MHz. The base station shall stop transmission on the current operating channel and will not resume normal transmissions as long as the interference signal is present.

- The step 6) is repeated multiple times considering the following sub-steps:

- Interferer ON: if the interfering signal is present, the interfering signal should be present for [10ms].

- Interferer OFF: if the interfering signal is removed, the interfering signal should be absent for [10ms].

Note: the total number of interferer ON duration is assumed to be N and the total number of interferer OFF duration is assumed to be M. The value N, M and the sequence of interferer ON/OFF pattern shall be generated randomly for the test.

- To pass the test, the BS must stop transmission in a number of times in the presence of interfering signal that shall not be less than N*0.9.

- 9.1.5	Test Requirements

In normal conditions, the measurement result shall remain channel access related test requirements for PDSCH as listed in Table 9.1.5-1.

- Table 9.1.5-1: Channel access test requirements for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| LBT measurement bandwidth | MHz | 20 |
| Maximum energy detection threshold | dBm/20MHz | -72 + [4dB] |
| Maximum channel occupancy time | ms | 8 |

The Base Station shall be able to assess whether the medium is busy or idle with at least 90% probability, using a channel access procedure with the parameters in Table 9.1.5-1.

- <Next Section>

- I.4	Channel access procedures

- Figure I.4: Measuring system Set-up for Channel access procedures

- <End of Changes>