3GPP TSG RAN WG1 Meeting #84bis 	R1-163831

- Busan, South Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 651 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for the contention window adjustment procedure in LAA downlink channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, LGE, WILUS, Nokia, Alcatel-Lucent Shanghai Bell, Broadcom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-3-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN1#83, the following agreements were reached: The eNB considers HARQ-ACK feedback (ACK, NACK, or DTX) only for the self-scheduled TB(s) with DTX and ‘any’ state (defined for TDD channel selection case) counted as NACK. HARQ-ACK DTX is considered as NACK with the exception of following cases. • HARQ-ACK DTX from a non-scheduled TB is ignored. • HARQ-ACK DTX from the TB which is scheduled by licensed carrier is ignored. • Note: HARQ-ACK DTX means explicit DTX response or implies that eNB does not detect any HARQ-ACK response corresponding to a PDSCH transmitted in an unlicensed carrier. However, there are three aspects that are not properly captured in the current 36.213. - The behaviour for the case when the eNB does not detect any HARQ-ACK response is not specified. - The agreement that DTX should be ignored for the cross-carrier scheduling case is not captured. - The ‘NACK/DTX’ state is not included. Currently the definition of “reference subframe” is given by “Reference subframe k is the starting subframe of the most recent transmission on the channel made by the eNB, for which HARQ-ACK feedback is available.” However, the phrase “for which HARQ-ACK feedback is available” is ambiguous because: - The eNB may not detect HARQ-ACK feedback even if HARQ-ACK feedback should be available according to HARQ-ACK feedback timeline. - Not all HARQ-ACK feedback for PDSCH transmission(s) in the reference subframes may be expected to be available at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the definition of “reference subframe”, the phrase “for which HARQ-ACK feedback is available” is changed to “for which at least some HARQ-ACK feedback is expected to be available.” The rules for determining Z are clarified for the case when the eNB does not detect HARQ-ACK feedback, the cross-carrier scheduling case, and the “NACK/DTX” state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The HARQ-ACK values for determining Z are not counted correctly. There is some ambiguity in the definition of “reference subframe”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 15.1.3	Contention window adjustment procedure

If the eNB transmits transmissions that are associated with channel access priority class pon a channel, the eNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in sub clause 15.1.1 for those transmissions using the following steps:

1)	for each priority class set CWp=CWmin,p

2)	if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe kare determined as NACK, increase CWp for each priority class to the next higher allowed value and remain in step 2; otherwise, go to step 1.

Reference subframe k is the starting subframe of the most recent transmission on the channel made by the eNB, for which at least some HARQ-ACK feedback is expected to be available.

If CWp=CWmax,p, the next higher allowed value for adjusting CWpis CWmax,p.

For determining Z,

-	if the eNB transmission(s) for which HARQ-ACK feedback is available start in the second slot of subframe k, HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k+1are also used in addition to the HARQ-ACK values corresponding to PDSCH transmission(s) in subframe k.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on the same LAA SCell,

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB, or if the eNB detects ‘DTX’, ‘NACK/DTX’ or ‘any’ state, it is counted as NACK.

-	if the HARQ-ACK values correspond to PDSCH transmission(s) on an LAA SCell that are assigned by (E)PDCCH transmitted on another serving cell,

-	if the HARQ-ACK feedback for a PDSCH transmission is detected by the eNB, ‘NACK/DTX’ or ‘any’ state is counted as NACK, and ‘DTX’ state is ignored.

-	if no HARQ-ACK feedback is detected for a PDSCH transmission by the eNB

- 	if PUCCH format 1b with channel selection is expected to be used by the UE, ‘NACK/DTX’ state corresponding to ‘no transmission’ as described in subclauses 10.1.2.2.1, 10.1.3.1 and 10.1.3.2.1 is counted as NACK, and ‘DTX’ state corresponding to ‘no transmission’ is ignored.

-	Otherwise, the HARQ-ACK for the PDSCH transmission is ignored.

-	if a PDSCH transmission has two codewords, the HARQ-ACK value of each codeword is considered separately

-	bundled HARQ-ACK across M subframes is considered as M HARQ-ACK responses.

If the CWp=CWmax,p is consecutively used K times for generation of Ninit,CWp is reset to CWmin,p only for that priority class pfor which CWp=CWmax,p is consecutively used K times for generation of Ninit.   K is selected by eNB from the set of values {1, 2, …,8} for each priority class .