3GPP TSG RAN WG1 Meeting #84bis 	R1-163528

- Busan, South Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 235 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the support of ending partial subframe in LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell, CATT, Ericsson, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In the current specification, the following are not correctly defined for the ending partial subframe in LAA: - The number of OFDM symbol used for PDCCH - The number of EREGs per ECCE for EPDCCH and the antenna port to use for localized EPDCCH - UE-specific RS associated with PDSCH - DMRS associated with EPDCCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the above three cases, it is clarified that the ending partial subframe in LAA follows the same behavior as DwPTS in a special subframe. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above behaviors for the ending partial subframe in LAA are not correctly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7, 6.8A, 6.10.3, 6.10.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 6.7	Physical control format indicator channel

The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1.

Table 6.7-1: Number of OFDM symbols used for PDCCH

| Subframe | Number of OFDM symbols for PDCCH when NRBDL>10 | Number of OFDM symbols for PDCCH when NRBDL≤10 |
| --- | --- | --- |
| Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration | 1, 2 | 2 |
| MBSFN subframes on a carrier supporting PDSCH, configured with 1 or 2 cell-specific antenna ports | 1, 2 | 2 |
| MBSFN subframes on a carrier supporting PDSCH, configured with 4 cell-specific antenna ports | 2 | 2 |
| Subframes on a carrier not supporting PDSCH | 0 | 0 |
| Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals | 1, 2, 3 | 2, 3 |
| All other cases | 1, 2, 3 | 2, 3, 4 |

…

## 6.8A	Enhanced physical downlink control channel

For frame structure type 3, for a subframe with the same duration as the DwPTS duration of a special subframe configuration, the enhanced physical downlink control channel is defined the same as that for the corresponding special subframe configuration.

### 6.8A.1	EPDCCH formats

The enhanced physical downlink control channel (EPDCCH) carries scheduling assignments. An enhanced physical downlink control channel is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A. The number of ECCEs used for one EPDCCH depends on the EPDCCH format as given by Table 6.8A.1-2 and the number of EREGs per ECCE is given by Table 6.8A.1-1.Both localized and distributed transmission is supported.

An EPDCCH can use either localized or distributed transmission, differing in the mapping of ECCEs to EREGs and PRB pairs.

A UE shall monitor multiple EPDCCHs as defined in 3GPP TS 36.213 [4]. One or two sets of physical resource-block pairs which a UE shall monitor for EPDCCH transmissions can be configured. All EPDCCH candidates in EPDCCH set Xm use either only localized or only distributed transmission as configured by higher layers. Within EPDCCH set Xm in subframe i, the ECCEs available for transmission of EPDCCHs are numbered from 0 to NECCE,m,i−1 and ECCE number n corresponds to

-	EREGs numbered nmodNECCERB+jNECCERB in PRB index nNECCERB for localized mapping, and

-	EREGs numbered n/NRBXm+jNECCERB in PRB indices n+jmax1,NRBXm/NEREGECCEmodNRBXm  for distributed mapping,

where j=0,1,...,NEREGECCE−1, NEREGECCE is the number of EREGs per ECCE, and NECCERB=16/NEREGECCE is the number of ECCEs per resource-block pair. The physical resource-block pairs constituting EPDCCH set Xm are in this paragraph assumed to be numbered in ascending order from 0 to NRBXm−1.

Table 6.8A.1-1: Number of EREGs per ECCE, NEREGECCE

| Normal cyclic prefix |  |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Special subframe, configuration 1, 2, 6, 7, 9 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |  |

…

### 6.8A.5	Mapping to resource elements

The block of complex-valued symbols y(0),...,y(Msymb−1) shall be mapped in sequence starting with y(0) to resource elements k,l on the associated antenna port which meet all of the following criteria:

-	they are part of the EREGs assigned for the EPDCCH transmission, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 unless other values for these parameters are provided by clause 9.1.4.3 in 3GPP TS 36.213 [4], and

-	they are assumed by the UE not to be used for transmission of:

-	zero-power CSI reference signals, where the positions of the CSI reference signals are given by clause 6.10.5.2. The configuration for zero power CSI reference signals is

-	obtained as described in clause 6.10.5.2 unless other values are provided by clause 9.1.4.3 in 3GPP TS 36.213 [4], and

-	obtained by higher-layer configuration of up to five reserved CSI-RS resources as part of the discovery signal configuration following the procedure for zero-power CSI-RS in clause 6.10.5.2.

-	non-zero-power CSI reference signals for CSI reporting with the configuration for non-zero power CSI reference signals for CSI reporting obtained as described in clause 6.10.5.2, and

-	for frame structure type 1 and 2, the index l in the first slot in a subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause 9.1.4.1 of 3GPP TS 36.213 [4], and

-	for frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates ‘s07’ and if the downlink transmission starts in the second slot of a subframe

-	the index l in the second slot in the subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause 7.1.6.4 of 3GPP TS 36.213  [4],

-	otherwise

-	the index l in the first slot in the subframe fulfils l≥lEPDCCHStart where lEPDCCHStart is given by clause 7.6.1.4 of 3GPP TS 36.213  [4].

The mapping to resource elements k,l on antenna port p meeting the criteria above shall be in increasing order of first the index k and then the indexl, starting with the first slot and ending with the second slot in a subframe.

For localized transmission, the single antenna port p to use is given by Table 6.8A.5-1 with

n'=nECCE,lowmodNECCERB+nRNTImodmin(NECCEEPDCCH,NECCERB)

where nECCE,low is the lowest ECCE index used by this EPDCCH transmission in the EPDCCH set, nRNTI equals the C-RNTI, and NECCEEPDCCH is the number of ECCEs used for this EPDCCH.

Table 6.8A.5-1: Antenna port to use for localized EPDCCH transmission

| n' | Normal cyclic prefix |  | Extended cyclic prefix |
| --- | --- | --- | --- |
|  | Normal subframes, Special subframes, configurations 3, 4, 8 | Special subframes, configurations 1, 2, 6, 7, 9 | Any subframe |
| 0 | 107 | 107 | 107 |
| 1 | 108 | 109 | 108 |
| 2 | 109 | - | - |
| 3 | 110 | - | - |

For distributed transmission, each resource element in an EREG is associated with one out of two antenna ports in an alternating manner, starting with antenna port 107, where p∈107,109 for normal cyclic prefix and p∈107,108 for extended cyclic prefix.

…

### 6.10.3	UE-specific reference signals associated with PDSCH

UE-specific reference signals associated with PDSCH

-	are transmitted on antenna port(s) p=5, p=7,  ,, or p=7,8,...,υ+6, where υ is the number of layers used for transmission of the PDSCH;

-	are present and are a valid reference for PDSCH demodulation only if the PDSCH transmission is associated with the corresponding antenna port according to clause 7.1 of 3GPP TS 36.213 [4];

-	are transmitted only on the physical resource blocks upon which the corresponding PDSCH is mapped.

A UE-specific reference signal associated with PDSCH is not transmitted in resource elements k,l in which one of the physical channels or physical signals other than the UE-specific reference signals defined in 6.1 are transmitted using resource elements with the same index pair k,l regardless of their antenna port p.

For frame structure type 3, for PDSCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the UE-specific reference signals are defined the same as that for the corresponding special subframe configuration.

#### 6.10.3.1	Sequence generation

For antenna port 5, the UE-specific reference-signal sequence rns(m) is defined by

rns(m)=121−2⋅c(2m)+j121−2⋅c(2m+1),          m=0,1,...,12NRBPDSCH−1

…

### 6.10.3A	Demodulation reference signals associated with EPDCCH or MPDCCH

….

#### 6.10.3A.2	Mapping to resource elements

For the antenna port p∈107,108,109,110 in a physical resource block nPRB assigned for the associated EPDCCH/MPDCCH, a part of the reference signal sequence r(m) shall be mapped to complex-valued modulation symbols ak,l(p) in a subframe according to

Normal cyclic prefix:

ak,l(p)=wp(l')⋅r(3⋅l'⋅NRBmax,DL+3⋅nPRB+m')

where

wp(i)=wp(i)(m'+nPRB)mod2=0wp(3−i)(m'+nPRB)mod2=1k=5m'+NscRBnPRB+k'k'=1p∈107,1080p∈109,110l=l'mod2+2if in a special subframe with configuration 3, 4,  8 or 9 (see Table 4.2-1)l'mod2+2+3l'/2if in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)l'mod2+5if not in a special subframel'=0,1,2,3if nsmod2=0 and in a special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)0,1if nsmod2=0 and not in special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)2,3if nsmod2=1 and not in special subframe with configuration 1, 2, 6, or 7 (see Table 4.2-1)m'=0,1,2

The sequence wp(i) is given by Table 6.10.3A.2-1.

Table 6.10.3A.2-1: The sequence wp(i) for normal cyclic prefix

| Antenna port p | wp(0)wp(1)wp(2)wp(3) |
| --- | --- |
| 107 | +1+1+1+1 |
| 108 | +1−1+1−1 |
| 109 | +1+1+1+1 |
| 110 | +1−1+1−1 |

Extended cyclic prefix:

ak,l(p)=wp(l'mod2)⋅r(4⋅l'⋅NRBmax,DL+4⋅nPRB+m')

where

wp(i)=wp(i)m'mod2=0wp(1−i)m'mod2=1k=3m'+NscRBnPRB+k'k'=1if nsmod2=0 and p∈107,1082if nsmod2=1 and p∈107,108l=l'mod2+4l'=0,1if nsmod2=0 and in a special subframe with configuration 1, 2, 3, 5 or 6 (see Table 4.2-1)0,12,3if nsmod2=0 and not in a special subframeif nsmod2=1 and not in a special subframem'=0,1,2,3

The sequence wp(i) is given by Table 6.10.3A.2-2.

Table 6.10.3A.2-2: The sequence wp(i) for extended cyclic prefix

| Antenna port p | wp(0)wp(1) |
| --- | --- |
| 107 | +1+1 |
| 108 | −1+1 |

For extended cyclic prefix, demodulation reference signals are not supported on antenna ports 109 to 110.

Resource elements k,l used for transmission of demodulation reference signals to one UE on any of the antenna ports in the set S, where S=107,108 or S=109,110 shall

-	not be used for transmission of EPDCCH/MPDCCH on any antenna port in the same slot, and

-	not be used for demodulation reference signals to the same UE on any antenna port other than those in S in the same slot.

Replacing antenna port numbers 7 – 10 by 107 – 110 in Figure 6.10.3.2-3 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for normal cyclic prefix. Replacing antenna port numbers 7 – 8 by 107 – 108 in Figure 6.10.3.2-4 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for extended cyclic prefix.

For frame structure type 3, for EPDCCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the mapping of the demodulation reference signals to the resource elements is the same as that for the corresponding special subframe configuration.