3GPP TSG-RAN WG4 Meeting #78bis 	R4-162683

- San Jose del Cabo, Mexico, 11th April 2016 - 16th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 0780 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Band 46 sub-bands indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-03-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Sub-bands concept was introduced for the first time for unlicensed 5GHz band for LAA in Rel’13 3GPP specification. To mark different sub-bands in Band 46, uppercase letters are currently used in specification: A, B, C and D (the following sub-bands are used: 46A, 46B, 46C and 46D). Uppercase letters are used in 3GPP specifications to mark CA Bandwidth Classes. Currently in TS 36.101 V13.2.1 there are A, B, C, D, E, F and I classes (Table 5.6A-1). Carrier aggregation bandwidth class is defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by the UE. These CA Bandwidth Classes are very widely used in different 3GPP specifications to mark carrier aggregation combinations with different number of component carriers. With introduction of Band 46, some CA combinations of licensed bands and Band 46 were also introduced, e.g. CA_2A-46A. Using uppercase letters for sub-bands can be very misleading and ambiguous as for 46A one can assume “A” is a BW class while the other one sub-band A. For example, for CA_2A-46A is it a CA combination with Band 2 and Band 46, or is this CA with Band 2 and sub-band 46A. As in 3GPP specification such ambiguity should be avoided, this CR changes uppercase letters to lowercase letters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sub-bands symbol changed from uppercase letters A, B, C, D to lowercase letters a, b, c, d. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would be ambiguous in relation to carrier agrregation combinations and sub-bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS 36.101 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.141 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Unless stated otherwise, requirements specified for the TDD duplex mode apply for downlink and uplink operations in Frame Structure Type 2 [4].

Table 5.5-1 E-UTRA frequency bands

| E-UTRA Operating Band |  |  | Uplink (UL) operating band BS receive UE transmit |  |  |  |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  |  |  |  |  |  |  |  | Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FUL_low – FUL_high |  |  |  |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1920 MHz |  |  |  |  | – |  | 1980 MHz |  |  | 2110 MHz |  |  |  |  | – |  |  | 2170 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 2 | 1850 MHz |  |  |  |  | – |  | 1910 MHz |  |  | 1930 MHz |  |  |  |  | – |  |  | 1990 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 3 | 1710 MHz |  |  |  |  | – |  | 1785 MHz |  |  | 1805 MHz |  |  |  |  | – |  |  | 1880 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 4 | 1710 MHz |  |  |  |  | – |  | 1755 MHz |  |  | 2110 MHz |  |  |  |  | – |  |  | 2155 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 5 | 824 MHz |  |  |  |  | – |  | 849 MHz |  |  | 869 MHz |  |  |  |  | – |  |  | 894MHz |  |  |  |  |  | FDD |  |  |  |  |
| 6 (NOTE 1) | 830 MHz |  |  |  |  | – |  | 840 MHz |  |  | 875 MHz |  |  |  |  | – |  |  | 885 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 7 | 2500 MHz |  |  |  |  | – |  | 2570 MHz |  |  | 2620 MHz |  |  |  |  | – |  |  | 2690 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 8 | 880 MHz |  |  |  |  | – |  | 915 MHz |  |  | 925 MHz |  |  |  |  | – |  |  | 960 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 9 | 1749.9 MHz |  |  |  |  | – |  | 1784.9 MHz |  |  | 1844.9 MHz |  |  |  |  | – |  |  | 1879.9 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 10 | 1710 MHz |  |  |  |  | – |  | 1770 MHz |  |  | 2110 MHz |  |  |  |  | – |  |  | 2170 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 11 | 1427.9 MHz |  |  |  |  | – |  | 1447.9 MHz |  |  | 1475.9 MHz |  |  |  |  | – |  |  | 1495.9 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 12 | 699 MHz |  |  |  |  | – |  | 716 MHz |  |  | 729 MHz |  |  |  |  | – |  |  | 746 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 13 | 777 MHz |  |  |  |  | – |  | 787 MHz |  |  | 746 MHz |  |  |  |  | – |  |  | 756 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 14 | 788 MHz |  |  |  |  | – |  | 798 MHz |  |  | 758 MHz |  |  |  |  | – |  |  | 768 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 15 | Reserved |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |
| 16 | Reserved |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |
| 17 | 704 MHz |  |  |  |  | – |  | 716 MHz |  |  | 734 MHz |  |  |  |  | – |  |  | 746 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 18 | 815 MHz |  |  |  |  | – |  | 830 MHz |  |  | 860 MHz |  |  |  |  | – |  |  | 875 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 19 | 830 MHz |  |  |  |  | – |  | 845 MHz |  |  | 875 MHz |  |  |  |  | – |  |  | 890 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 20 | 832 MHz |  |  |  |  | – |  | 862 MHz |  |  | 791 MHz |  |  |  |  | – |  |  | 821 MHz |  |  |  |  |  |  |  |  |  |  |
| 21 | 1447.9 MHz |  |  |  |  | – |  | 1462.9 MHz |  |  | 1495.9 MHz |  |  |  |  | – |  |  | 1510.9 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 22 | 3410 MHz |  |  |  |  | – |  | 3490 MHz |  |  | 3510 MHz |  |  |  |  | – |  |  | 3590 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 23 | 2000 MHz |  |  |  |  | – |  | 2020 MHz |  |  | 2180 MHz |  |  |  |  | – |  |  | 2200 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 24 | 1626.5 MHz |  |  |  |  | – |  | 1660.5 MHz |  |  | 1525 MHz |  |  |  |  | – |  |  | 1559 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 25 | 1850 MHz |  |  |  |  | – |  | 1915 MHz |  |  | 1930 MHz |  |  |  |  | – |  |  | 1995 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 26 | 814 MHz |  |  |  |  | – |  | 849 MHz |  |  | 859 MHz |  |  |  |  | – |  |  | 894 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 27 | 807 MHz |  |  |  |  | – |  | 824 MHz |  |  | 852 MHz |  |  |  |  | – |  |  | 869 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 28 | 703 MHz |  |  |  |  | – |  | 748 MHz |  |  | 758 MHz |  |  |  |  | – |  |  | 803 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 29 |  | N/A |  |  |  |  |  |  |  | 717 MHz |  |  |  |  | – |  |  | 728 MHz |  |  |  |  |  | FDD (NOTE 2) |  |  |  |  |  |
| 30 | 2305 MHz |  |  |  |  | – |  | 2315 MHz |  |  | 2350 MHz |  |  |  |  | – |  |  | 2360 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 31 |  |  |  |  | 452.5 MHz |  | – |  | 457.5 MHz |  |  |  | 462.5 MHz |  |  |  |  |  |  | – |  | 467.5 MHz |  |  |  |  | FDD |  |  |
| 32 |  |  |  | N/A |  |  |  |  |  |  |  |  |  | 1452 MHz |  |  | – |  |  |  | 1496 MHz |  | FDD (NOTE 2) |  |  |  |  |  |  |
| 33 | 1900 MHz |  |  |  |  | – |  | 1920 MHz |  |  | 1900 MHz |  |  |  |  | – |  |  | 1920 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 34 | 2010 MHz |  |  |  |  | – |  | 2025 MHz |  |  | 2010 MHz |  |  |  |  | – |  |  | 2025 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 35 | 1850 MHz |  |  |  |  | – |  | 1910 MHz |  |  | 1850 MHz |  |  |  |  | – |  |  | 1910 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 36 | 1930 MHz |  |  |  |  | – |  | 1990 MHz |  |  | 1930 MHz |  |  |  |  | – |  |  | 1990 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 37 | 1910 MHz |  |  |  |  | – |  | 1930 MHz |  |  | 1910 MHz |  |  |  |  | – |  |  | 1930 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 38 | 2570 MHz |  |  |  |  | – |  | 2620 MHz |  |  | 2570 MHz |  |  |  |  | – |  |  | 2620 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 39 | 1880 MHz |  |  |  |  | – |  | 1920 MHz |  |  | 1880 MHz |  |  |  |  | – |  |  | 1920 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 40 | 2300 MHz |  |  |  |  | – |  | 2400 MHz |  |  | 2300 MHz |  |  |  |  | – |  |  | 2400 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 41 | 2496 MHz |  |  |  |  | – |  | 2690 MHz |  |  | 2496 MHz |  |  |  |  | – |  |  | 2690 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 42 | 3400 MHz |  |  |  |  | – |  | 3600 MHz |  |  | 3400 MHz |  |  |  |  | – |  |  | 3600 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 43 | 3600 MHz |  |  |  |  | – |  | 3800 MHz |  |  | 3600 MHz |  |  |  |  | – |  |  | 3800 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 44 | 703 MHz |  |  |  |  | – |  | 803 MHz |  |  | 703 MHz |  |  |  |  | – |  |  | 803 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 45 | 1447 MHz |  |  |  |  | – |  | 1467 MHz |  |  | 1447 MHz |  |  |  |  | – |  |  | 1467 MHz |  |  |  |  |  | TDD |  |  |  |  |
| 46 | 5150 MHz |  |  |  |  | – |  | 5925 MHz |  |  | 5150 MHz |  |  |  |  | – |  |  | 5925 MHz |  |  |  |  |  | TDD (NOTE 3, NOTE 4) |  |  |  |  |
| 65 | 1920 MHz |  |  |  |  | – |  | 2010 MHz |  |  | 2110 MHz |  |  |  |  | – |  |  | 2200 MHz |  |  |  |  |  | FDD |  |  |  |  |
| 66 | 1710 MHz |  |  |  |  | – |  | 1780 MHz |  |  | 2110 MHz |  |  |  |  | – |  |  | 2200 MHz |  |  |  |  |  | FDD (NOTE 5) |  |  |  |  |
| 67 | N/A |  |  |  |  |  |  |  |  |  | 738 MHz |  |  |  |  | – |  |  | 758 MHz |  |  |  |  |  | FDD (NOTE 2) |  |  |  |  |
| 68 | 698 MHz |  |  |  |  | – |  | 728 MHz |  |  | 753 MHz |  |  |  |  | – |  |  | 783 MHz |  |  |  |  |  | FDD |  |  |  |  |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 4: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. Band 46 is divided into four sub-bands as in Table 5.5-1A. NOTE 5: The range 2180 – 2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 5.5-1A Sub-bands for Band 46

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| 46a | 5150 MHz | – | 5250 MHz | 5150 MHz | – | 5250 MHz |
| 46b | 5250 MHz | – | 5350 MHz | 5250 MHz | – | 5350 MHz |
| 46c | 5470 MHz | – | 5725 MHz | 5470 MHz | – | 5725 MHz |
| 46d | 5725 MHz | – | 5925 MHz | 5725 MHz | – | 5925 MHz |