### 4.2.3	Adaptive applications

[Editor’s note: This needs to be discussed][Needs resolution/Berlin]. Certain applications such as audio streaming sense the available bit rate of the channel and then adjust the amount of traffic that is transmitted. Certain multi-media sessions may employ content-adaptation of images or video based on network conditions. This directly changes the amount of data that is transmitted. The adaptive nature of applications can be incorporated into the traffic model. We do not perceive a strong need for the adaptive nature of an application to be incorporated as a dynamic feature of the traffic model. Such adaptive behavior can be addressed by using traffic models with different parameters and switching between them in an appropriate manner. Thus, adaptation of traffic characteristics based on network/device conditions is outside the scope of this modeling.

[the following is replacement- text proposed by Dan Gal-- for discussion on the 10/26/04 ECG call]

An adaptive-application is capable of reacting to changes in the transport channel parameters (such as data rate, latency, overall error rate etc.) and adjusting some of its transmission attributes, in an effort to optimize the user experience. For example, a streaming audio application may want to increase the packet rate to take advantage of favorable channel conditions (higher data rate availability). Another example would be a multimedia application that can flexibly adjust the content type and attributes (such as video resolution and frame rate or audio track fidelity). Thus, adaptive applications can vary the amount and rate of the data transported to the user.

It is recommended that the 802.20 traffic models reflect the adaptation capability of applications as a set of representative static traffic parameters, rather than employ a dynamic adaptation model. Network conditions and device capabilities shall not be considered in the application adaptation modeling.