4.3.5 Voice (VoIP)

ITU-T G.107 defines an objective model known as E-Model based on Network, Speech, Terminal/Device parameters to estimate/predict the perceived quality of VoIP session. The primary output of the E-Model is transmission rating factor R (Total Value Index) that can be mapped one-to-one to an estimated MOS.

The E-model defines 20 different parameters each with a default value and their ranges of values are defined. If all parameters are set to the default values, the calculation results in a very high quality with a rating factor of Rdefault = 93.2, which is also defined as the intrinsic quality of a voice call with a mouth-to-ear delay of 0 ms. The intrinsic quality of a packetized voice call transported without packet loss in the G.711 format corresponds to this Rdefault = 93.2.

However, for MBWA system specific impairments such as Packet Loss, Delay etc considered, the effective R factor for such system needs to be estimated by incorporating equipment impairment factor, delay factor. The effective R factor is

RMBWA = 93.2 – Id – Ie-eff

Here Id, the impairment factor representing all impairments due to delay of voice signals Talker Echo, Listener Echo and Idd, a loss of interactivity, represents the impairment caused by too-long absolute delay Ta, which occurs even with perfect echo canceling.  Here we assume perfect Jitter operation resulting no packet loss and additional delay introduced by jitter.

The packet-loss dependent Effective Equipment Impairment Factor Ie-eff is derived using the codec specific value for the Equipment Impairment Factor at zero packet-loss Ie and the codec specific Packet-loss Robustness Factor Bpl

Ie represents the effect of degradation introduced by CODECs, Packet Loss. G.113 –Appendix (2002) provided provides parameters for use in calculating Ie from  CODEC type and Packet Loss rate. For a G.729 Codec Ie = 11, and  for random packet loss Bpl = 19.

VoIP performance shall be compared on the basis of the cdf of the R values generated as a result of simulating the proposed codec traffic. As a default the G.729 decoder shall be simulated with an assumed 4 byte IP header.